IEEE P802.11
Wireless LANs

| LB78 LCI-Azimuth normative text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-1-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Peter Ecclesine | Cisco Systems | 170 W. Tasman Dr., MS SJ-10-5, San Jose, CA 95134-1706 | 408-527-0815 | petere@cisco.com |

Abstract

LB78 LCI-Azimuth normative text, resolving comments on 802.11k D3.0 clauses 7.3.2.21.11, 7.3.2.22.11, 11.11.9.8, Annex I and Annex J.

LB78 ballot comment numbers (11-05/1049r22) are shown in {xxx} form in editor’s instructions

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# 3. Definitions

{328,1000,1135} Insert the following new definition in alphabetical order, renumbering as necessary:

Azimuth: The horizontal orientation of the front face of a station or of a radio beam measured clockwise from True North.

LCI subject Local: The LCI request is for the location of the requesting STA, by asking “Where am I?”

LCI subject Remote: The LCI request is for the location of the reporting STA, by asking “Where are you?”

# 7. Frame formats

## 7.3 Management frame body components

### 7.3.2 Information elements

#### 7.3.2.21 Measurement Request element

##### 7.3.2.21.11 Location Configuration Indication (LCI) Request

{328, 472, 473, 999, 1000, 1134, 1135, 1250, 1251, 1341, 1422, 1423, 1424}  Change 7.3.2.21.11 replacing Accuracy with Requested Resolution and inserting Azimuth as shown and renumber tables accordingly:

The Measurement Request field corresponding to an LCI request is shown in Figure k12.

|  | Location Subject | Latitude Requested Resolution | Longitude Requested Resolution | Altitude Requested Resolution | Azimuth |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 |

Figure k12—LCI Request field format

The Location Subject field of a LCI request is a single octet. See Table k1.

Table k1—Location subject definition

| Value | Location Subject |
| --- | --- |
| 0 | LCI Subject Local |
| 1 | LCI Subject Remote |
| 2-255 | Reserved |

The term ‘Local’ refers to the location of the requesting STA, and ‘Remote’ refers to the location of the reporting STA.

NOTE—Local LCI Measurement Request is used by requesting STA to obtain its own location by asking "Where am I?". Remote LCI Measurement Request is used by requesting STA to obtain location of reporting STA by asking "Where are you?".

Latitude requested resolution is the number of valid bits requested for the fixed-point value of Latitude in degrees. Values above 34 (decimal), the specified maximum Latitude, are reserved.

Longitude requested resolution is the number of valid bits requested for the fixed-point value of Longitude in degrees. Values above 34 (decimal), the specified maximum Longitude, are reserved.

Altitude requested resolution is the number of valid bits requested for the Altitude, which has either of two types, as described in 7.3.2.22.11. Values above 30 (decimal), the specified maximum Altitude, are reserved.

{1000, 1135} Change 7.3.2.21.11 inserting Azimuth text and table as shown and renumber tables accordingly:

The Azimuth field of a LCI request is a single octet. See Table k9.

Table k9—Azimuth definition

| Value | Azimuth |
| --- | --- |
| 0 | No azimuth report requested |
| 1-9 | Azimuth of front face of STA resolution requested |
| 10-15 | Reserved |
| 16 | No azimuth report requested |
| 17-25 | Azimuth of radio beam resolution requested |
| 26-255 | Reserved |

Azimuth resolution requested is the number of valid bits requested in the fixed-point value of Azimuth, reported in integer degrees. Values of 0 and 16 represent no azimuth report requested. Values of 10 to 15 are reserved. Values of 17 to 25 represent the number of valid bits requested plus 16. Values above 25 (decimal) are reserved.

‘Azimuth of front face of STA’ refers to the Azimuth of the front face of the station, and ‘Azimuth of radio beam’ refers to the Azimuth of radio reception, and is signified by bit 4 of Azimuth being true.

NOTE— A geographic feature is "an abstraction of a real world phenomenon; it is a geographic feature if it is associated with a location relative to the Earth”. The designation of a horizontal plane is relative to the Earth. The designation of the ‘front face’ of a station is arbitrary, but refers to an orientable surface (possessing a centerline) of the station. Non-orientable shapes like the Mobius band and Klein bottle are non-intuitive for the purpose of making an Azimuth report. It is common to use a direction cosine matrix to convert from one coordinate system to another, i.e. body coordinates to earth-centered coordinates.

#### 7.3.2.22 Measurement Report element

##### 7.3.2.22.11 Location Configuration Information Report

{8, 83, 157, 350, 919, 1000, 1135, 1266, 1363} Change 7.3.2.22.11 adding Azimuth as shown and renumber tables accordingly:

A Location Configuration Information report includes latitude, longitude, altitude and azimuth information. The LCI report format is shown in Figure 27.

Figure 27—Location Configuration Information

This structure and information fields are little-endian, per conventions defined in 7.1.1, and are based on the LCI format described in IETF RFC 3825, “Dynamic Host Configuration Protocol Option for Coordinate-based Location Configuration Information”. The length shall be 18 octets.

The definition of elements within the LCI report shall be as defined in IETF RFC3825 section 2.1 or as defined herein.

An LCI report with Latitude resolution, Longitude resolution and Altitude resolution set to zero shall indicate that the requested location cannot be reported with the requested accuracy.

The Datum value shall be 1 (World Geodetic System 1984), unless another datum is required for operation in the regulatory domain.

If the Altitude Type is 2 (Floors of Altitude), the value reported shall be as required for operation in the regulatory domain.

Azimuth type is three bit encoding the type of azimuth. See Table k10.

Table k10—Azimuth type definition

| Value | Azimuth Type |
| --- | --- |
| 0 | Azimuth of front face of STA |
| 1 | Azimuth of radio beam |
| 2-7 | Reserved |

Azimuth resolution is 4 bits, indicating the number of valid bits in the Azimuth. An Azimuth resolution set to zero shall indicate that the Azimuth cannot be reported with the requested accuracy.

Azimuth is a 9-bit unsigned integer value in degrees from True North, of the type defined by the Azimuth type field.

# 11 MLME

## 11.11 Radio measurement procedures

### 11.11.9 Specific measurement usage

#### 11.11.9.8 Location Configuration Information Report

{472, 755, 885, 1134, 1135, 1305} Change 11.11.9.8

If a STA accepts an LCI request, it shall respond with a Radio Measurement Report frame including one Location Configuration Information element (LCI Report).

An LCI request may indicate a location request for the requesting STA or the reporting STA by setting the LCI request Location Subject octet to indicate a Local or Remote request respectively. Local LCI Measurement Request is used by requesting STA to obtain its own location by asking "Where am I?" Remote LCI Measurement Request is used by requesting STA to obtain location of reporting STA by asking "Where are you?"

If the STA receiving an LCI request lacks the means to report the requested location to the requested resolution, then the LCI Report shall have that corresponding Latitude, Longitude, Altitude or Azimuth resolution set to zero, otherwise the Latitude, Longitude, Altitude and Azimuth fields shall be reported to their full resolutions, which can exceed the requested resolutions.

If the reporting STA has no information about the physical location of the ‘Local’ requestor, then it shall set the Incapable bit in the Measurement Report Mode field. If the reporting STA has no information about the Azimuth requested, then it shall set the Incapable bit in the Measurement Report Mode field. The method by which the physical location and azimuth information in the LCI Report is generated is outside the scope of this standard, to allow the accuracy of the reported location to be ‘best effort.’

NOTE—The physical location and azimuth MIB information of the STA may be set by administrative means. A STA that requested a ‘Local’ LCI and received an LCI Report with the Incapable bit set may alternatively make a ‘Remote’ LCI request to obtain the reporting STA’s physical location. A STA that requested an Azimuth report in the LCI and received an LCI Report with the Incapable bit set may alternatively request the LCI with no Azimuth report requested.

# Annex A - PICS

## A.4 PICS proforma-IEEE Std 802.11, 1999 Edition9

{1000, 1135} Insert new Items in Section A.4.13 after the RRM9 items, adjusting the subsequent table numbers as necessary:

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RRM9.3 | Azimuth | 11.11, 11.11.9.8 | CFk:O | Yes □ No □ N/A □ |
| RRM9.3.1 | Azimuth Request | 7.3.2.21.11 | CFk:O | Yes □ No □ N/A □ |
| RRM9.3.2 | Azimuth Response | 7.3.2.22.11 | CFk:O | Yes □ No □ N/A □ |

# Annex D

{1000, 1135} In SMT MIB of Annex D, insert new Items in  dot11LCIReportEntry  after dot11LCIDatum and renumber accordingly :

dot11AzimuthType INTEGER,

dot11AzimuthResolution INTEGER,

dot11Azimuth Integer32,

{1000, 1135} In SMT MIB of Annex D, insert new Items after dot11LCIDatum OBJECT-TYPE and renumber accordingly :

dot11AzimuthType OBJECT-TYPE

SYNTAX INTEGER {

front face of STA(0),

radio beam(1) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Azimuth Type is a three bit attribute encoding the type of azimuth.

Codes defined are:

front face of STA : in 2s-complement fixed-point 9-bit integer

radio beam : in 2s-complement fixed-point 9-bit integer"

::= { dot11LCIReportEntry 16 }

dot11AzimuthResolution OBJECT-TYPE

SYNTAX INTEGER (0..15)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Azimuth resolution is 4 bits indicating the number of valid

bits in the azimuth."

::= { dot11LCIReportEntry 17 }

dot11AzimuthInteger OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Azimuth is a 9 bit value defined by the Azimuth type field.

The field is encoded as a 2s-complement fixed-point 9-bit integer

horizontal angle in degrees from True North."

::= { dot11LCIReportEntry 18 }

# Annex I

(informative)

# Regulatory classes

## I.1 External regulatory references

{1010, 1139}Change the first paragraph to:

This annex and Annex J provide information and specifications for operaion in many regulatory domains.

{1011, 1140}Insert two new rows in Table I.6 as shown:

| Frequency band (GHz) | USA (Maximum output power with up to 6 dBi antenna gain) (mW) | Europe (EIRP) |
| --- | --- | --- |
| 2.400-2.4835 | 1 000 | 100 mW |
| 5.725-5.850 | 1 000 | — |

# Annex J

# (normative)

# Country information element and regulatory classes

{619, 620, 621, 955, 1012, 1141} Change the title, change channel set in row 5,  insert a new row after 5 and change the last row of Table J.1 as shown:

### Table J.1—Regulatory classes for 5 GHz bands in the USA

| Regulatory Class | Channel Starting Frequency (GHz) | Channel Spacing (MHz) | Channel set | Transmit Power limit (mW) | Emissions Limits set | Behavior Limits set |
| --- | --- | --- | --- | --- | --- | --- |
| 5 | 5 | 20 | 149, 153, 157, 161, 165 | 1 000 | 4 | 1 |
| 6 | 2.407 | 25 | 1-11 | 1 000 | 4 | 9 |
| 57-255 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

{622, 623} Change the title, insert a new row and change the last row of Table J.2 as shown:

### Table J.2—Regulatory classes for 5 GHz bands in Europe

| Regulatory Class | Channel Starting Frequency (GHz) | Channel Spacing (MHz) | Channel set | Transmit Power limit (EIRP) | Emissions Limits set | Behavior Limits set |
| --- | --- | --- | --- | --- | --- | --- |
| 4 | 2.407 | 25 | 1-13 | 100 | 4 | 9 |
| 5-255 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

{624, 625} Change the title, insert two new rows and change the last row of Table J.3 as shown:

### Table J.3—Regulatory classesfor 4.9 GHz and 5 GHz bands in Japan

| Regulatory Class | Channel Starting Frequency (GHz) | Channel Spacing (MHz) | Channel set | Transmit Power limit (dBm) | Emissions Limits set | Behavior Limits set |
| --- | --- | --- | --- | --- | --- | --- |
| 21 | 2.407 | 25 | 1-13 | 10 mW/MHz | 4 | 6, 9 |
| 22 | 2.414 | 25 | 14 | 10 mW/MHz | 4 | 6, 9 |
| 2123-255 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |