IEEE P802.11
Wireless LANs

| Normative Text Proposal for Media-Independent-Handover support |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-02-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Marian Rudolf | InterDigital | 1000 Sherbrooke W. Montreal, QC, Canada H3A 3G4 | +1 514 904 6258 | marian.rudolf@interdigital.com |
| Ulises Olvera-Hernandez | InterDigital | 1000 Sherbrooke W. Montreal, QC, Canada H3A 3G4 | +1 514 904 6282 | ulises.olvera-hernandez@interdigital.com |
| Joe Kwak | InterDigital | 482 Degas Bolingbrook, IL 60440 | +1-630-739-4159 | joekwak@sbcglobal.net |

Abstract

This document contains a normative text proposal in support of requirement R9H1 (MIH cluster) in 05/822r9.

R9H1: “Provide a mechanism  to support Media Independent Handoff capability.”

This text proposal is based on P802.11-REVma-D5.1.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Contents

7. Frame formats	3

7.2 Format of individual frame types	3

7.2.3 Management frames	3

7.2.3.1 Beacon frame format	3

7.2.3.4 Association Request frame format	3

7.2.3.5 Association Response frame format	3

7.2.3.6 Reassociation Request frame format	4

7.2.3.7 Reassociation Response frame format	4

7.2.3.9 Probe Response frame format	4

7.3 Management frame body components	4

7.3.2 Information Elements	5

7.3.2.35 Interworking Capability Information element	5

11. MLME	7

11.11 Interworking Procedures	7

11.11.1 Media-Independent Handover	7

Annex A	7

A.4 PICS proforma–IEEE Std. 802.11, 2006 Edition	7

A.4.3 IUT configuration	7

A.4.16 Interworking extensions	8

Annex D	8

# 7. Frame formats

## 7.2 Format of individual frame types

### 7.2.3 Management frames

#### 7.2.3.1 Beacon frame format

Insert a new row into table 8 as shown below:

Table 8—Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 26 | Interworking Capability | Interworking Capability shall be present if dot11InterworkingImplemented is true. |

#### 7.2.3.4 Association Request frame format

Insert a new row into table 10 as shown below:

Table 10—Association Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 11 | Interworking Capability | Interworking Capability shall be present if dot11InterworkingImplemented is true. |

#### 7.2.3.5 Association Response frame format

Insert a new row into table 11 as shown below:

Table 11—Association Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 8 | Interworking Capability | Interworking Capability shall be present if dot11InterworkingImplemented is true. |

#### 7.2.3.6 Reassociation Request frame format

Insert a new row into table 12 as shown below:

Table 12—Reassociation Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 12 | Interworking Capability | Interworking Capability shall be present if dot11InterworkingImplemented is true. |

#### 7.2.3.7 Reassociation Response frame format

Insert new row into table 13 as follows:

Table 13—Reassociation Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 8 | Interworking Capability | Interworking Capability shall be present if dot11InterworkingImplemented is true. |

#### 7.2.3.9 Probe Response frame format

Insert a new row before the last line into table 15 and modify the last row as shown below:

Table 15—Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 24 | Interworking Capability | Interworking Capability shall be present if dot11InterworkingImplemented is true. |
| 25-255 | Requested information elements | Elements requested by the Request information element of the Probe Request frame. |

## 7.3 Management frame body components

### 7.3.2 Information Elements

Insert Element ID x into Table 26 and change the Reserved row accordingly:

Table 26—Element IDs

| Information Element | Element ID |
| --- | --- |
| Interworking Capability | 51 |
| Reserved | 52-221 |

Insert the following new clauses after 7.3.2.34

#### 7.3.2.35 Interworking Capability Information element

The Interworking Capability Information element contains information about the capabilities of a STA to operate on other non-802.11 access networks and information about its non-802.11 protocol capabilities as shown in Figure u1.

|  | Element ID | Length | Interworking Capabilities |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 |

Figure u1—Interworking Capability Information element format

The Length field is variable and depends on the length of the Interworking Capabilities field.  The minimum value of the Length field is 2.

The Interworking Capabilities field is a bit field indicating the advertised capabilities of the STA. The Interworking Capabilities field is shown in Figure u2.

|  | B0 | B1 | B2 | B3 | B15 |
| --- | --- | --- | --- | --- | --- |
|  | MIH capability | MIH mode L2 transport | MIH mode L3 transport | Reserved |  |
| Bits: | 1 | 1 | 1 | 14 |  |

Figure u2—Interworking Capabilities

- The MIH capability bit set to 1 indicates the STA supports Media-Independent Handover Service as described in 802.21. The MIH capability bit set to 0 indicates that the STA does not support this service.

- The MIH mode L2 transport bit set to 1 indicates the STA supports Media-Independent Handover Service messages through the 802.21 L2 signalling transport option as described in 802.21. The MIH mode L2 transport bit set to 0 indicates that the STA does not support this service.

- The MIH mode L3 transport bit set to 1 indicates the STA supports Media-Independent Handover Service messages through the 802.21 L3 signalling transport option as described in 802.21. The MIH mode L3 transport bit set to 0 indicates that the STA does not support this service.

- All other bits are reserved and shall be set to 0 on transmission and ignored on reception

The lack of a Interworking Capability element shall be interpreted as the STA having no advertised Interworking Capabilities.

# 11. MLME

Insert the following new clauses after 11.10

## 11.11 Interworking Procedures

This clause describes the actions and the procedures to enable seamless interworking of a multi-access capable STA with other non-802.11 access technologies or when other non-802.11 specific network protocols are involved.

### 11.11.1 Media-Independent Handover

Support for Media-Independent Handover in the BSS is provided to interworking capable STAs by advertising MIH capability, MIH mode L2 transport and MIH mode L3 transport bits in the Interworking Capability Information Element in Beacon, Probe Response and (Re-)Association Response frames.

A STA shall indicate its MIH capability and support for MIH mode L2 or L3 signalling transport through the appropriate MIH capability, MIH mode L2 transport and MIH mode L3 transport bits in the Interworking Capability Information Element in (Re-)Association Request frames.

The MIH mode L2 transport bit should be set when for example Ethertype protocols are used to carry the MIH PDUs. The MIH mode L3 transport bit should be set when for example IP-control type protocols are used to carry the MIH PDUs.

If MIH capability is supported, support for Media-Independent Handover in a BSS shall be provided to a STA only when in State 3, i.e. when the STA is authenticated and associated.

Procedures in support of Media-Independent Handover beyond the indication of STA capability are out-of-scope for this amendment and should be refered to in relevant 802.21 standards.

# Annex A

## A.4 PICS proforma–IEEE Std. 802.11, 2006 Edition

### A.4.3 IUT configuration

Append this entry to the end of the IUT configuration table:

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| *CFu | Interworking supported | 11.11 | O | Yes, No, N/A |

Insert this new clause after clause A.4.16:

### A.4.16 Interworking extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
|  | Are the following Interworking capabilities supported? |  |  |  |
| IW1 | Media-Independent Handover | 11.11.1 | CFu:M | Yes, No, N/A |
| IW1.1 | MIH mode L2 transport | 11.11.1 | CFu:O | Yes, No, N/A |
| IW1.2 | MIH mode L3 transport | 11.11.1 | CFu:O | Yes, No, N/A |

Add the following MIB attributes to Annex D:

# Annex D

In the dot11StationConfig table of Annex D, insert the following entries at the end of the dot11StationConfigEntry sequence list:

Dot11StationConfigEntry ::=

SEQUENCE {

…

dot11InterworkingImplemented TruthValue }

Insert the following elements at the end of the dot11StationConfigTable element definitions:

dot11InterworkingImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station

implementation is capable of supporting either non-802.11 Network

Access services or non-802.11 specific protocol functionalities in support of seamless interworking.

The default value of this attribute is FALSE."

::= { dot11StationConfigEntry TBD }