IEEE P802.11
Wireless LANs

| Submission to address TGn draft omissions in clause 20 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-02-27 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Assaf Kasher | Intel Corporation | P.O.B 1659 Haifa, 31015 Israel |  | assaf.kasher@intel.com |
| Vinko Erceg | Broadcom | 15435 Innovation Dr. San Diego, CA 92128 |  | verceg@broadcom.com |

Abstract

This document contains proposed changes to the TGn Draft to address omissions from the TGn Draft document.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Omissions Covered

This document addresses the following omissions identified in document 11-06-0263-00-000n-tgn-draft-omissions.xls:

OM103, 0M104, OM105, OM106, OM107, OM108, OM109, OM111, OM117, OM118

OM103

TGn Editor:  Replace section 20.3.7 with the following text:

20.3.7 Regulatory requirements

Wireless LANs implemented in accordance with this standard are subject to equipment certification andoperating requirements established by regional and national regulatory administrations. The PMD specification establishes minimum technical requirements for interoperability, based upon established regulations at the time this standard was issued. These regulations are subject to revision, or may be superseded. Requirements that are subject to local geographic regulations are annotated within the PMD specification. Regulatory requirements that do not affect interoperability are not addressed in this standard. Implementers are referred to the regulatory sources in Annex I for further information. Operation in countries within defined regulatory domains may be subject to additional or alternative national regulations.

OM104

TGn Editor:  Replace section 20.3.9 with the following text:

20.3.9 Transmit and receive in-band and out-of-band spurious transmissions

The OFDM PHY shall conform to in-band and out-of-band spurious emissions as set by regulatory bodies.

OM105

TGn Editor:  Replace section 20.3.10 with the following text:

20.3.10 Transmitter RF delay

The transmitter RF delay shall follow subclause 17.3.8.5.

OM106

TGn Editor:  Replace section 20.3.11 with the following text:

20.3.11 Slot time

The slot time shall follow subclause 17.3.8.6 for 5 GHz bands and subclause 19.4.4 for 2.4 GHz bands.

OM107

TGn Editor:  Replace section 20.3.12 with the following text:

20.3.12 Transmit and receive port impedance

The transmit and receive antenna port impedance for each transmit and receive antenna shall follow subclause 17.3.8.7.

OM108

TGn Editor:  Replace section 20.3.13 with the following text:

20.3.13 Transmit and receive operating temperature range

The transmit and receive temperature range shall follow subclause 17.3.8.8.

OM109

TGn Editor:  Replace section 20.3.14.7 with the following text:

20.3.14.7 Symbol clock frequency tolerance

The symbol clock frequency tolerance shall be ±20 ppm maximum for 5 GHz bands and ±25 ppm for 2.4 GHz bands. The transmit center frequency and the symbol clock frequency for all transmit antennas shall be derived from the same reference oscillator.

20.3.14.8 Modulation accuracy

OM111

TGn Editor:  Replace section 20.3.14.8.1 with the following text:

20.3.14.8.1 Transmit center frequency leakage

The transmitter center frequency leakage shall follow subclause 17.3.9.6.1 for all 20 MHz modes of transmission. For 40 MHz modes, the center frequency leakage shall not exceed -18 dB relative to overall transmitted power, or, equivalently, +2 dB relative to the average energy of the rest of the subcarriers. The transmit center frequency leakage is specified per antenna.

OM117

TGn Editor:  Replace section 20.3.15.4 with the following text:

20.3.15.4 Receiver maximum input level

The receiver shall provide a maximum PER of 10% at a PSDU length of 1000 bytes, for a maximum input level of –30 dBm measured at each antenna for any baseband modulation.

.

OM117

TGn Editor:  Replace section 20.3.15.5 with the following text:

20.3.15.5 Clear channel assessment (CCA) sensitivity

The start of a valid High Througput transmission at a receive level equal to or greater than the minimum modulation and coding rate sensitivity (–80 dBm for a 20 MHz channel and –77 dBm for a 40 MHz channel) shall cause CCA to indicate busy with a probability > 90% within 4µs for both greenfield, mixed mode and non-HT modes.  If the  preamble portion was missed, the receiver shall hold the CS signal busy for any signal 20 dB above the minimum modulation and coding rate sensitivity (–60 dBm for a 20 MHz channel and –57 dBm for a 40 MHz channel). In a 40 MHz channel, the receiver shall provide a CCA for each of the 20 MHz channels occupying the 40 MHz bandwidth, two in total.

TGn Editor:  Insert this section after section 20.3.15.6:

20.3.15.6 Received Channel Power Indicator (RCPI) Measurement

The RCPI indicator is a measure of the received RF power in the selected channel. This parameter shall be a measure by the PHY sublayer of the received RF power in the channel measured over the entire received frame. The received power shall be the average over all receive chains.  RCPI shall be a monotonically increasing, logarithmic function of the received power level defined in dBm. The allowed values for the Received Channel Power Indicator (RCPI) parameter shall be an 8 bit value in the range from 0 through 220, with indicated values rounded to the nearest 0.5 dB as follows:

0: Power < -110 dBm

1: Power = -109.5 dBm

2: Power = -109.0 dBm

and so on where

RCPI = int{(Power in dBm +110)*2} for 0dbm > Power > -110dBm

220: Power > -0 dBm

221-254: reserved

255: Measurement not available

Accuracy for each measurement shall be +/- 5dB (95% confidence interval) within the specified dynamic

range of the receiver. The measurement shall assume a receiver noise equivalent bandwidth equal to the

channel bandwidth multipled by 1.1.