IEEE P802.11
Wireless LANs

| Normative Text Proposal for Diagnostic Alerts |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-03-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Tim Olson | Cisco Systems | 170 W. Tasman Dr. San Jose, CA 95170 | 408-853-4527 | tolson@cisco.com |
| Bob O’Hara | Cisco Systems | 3625 Cisco Way, San Jose, CA 95134 | 408-853-5513 | bob.ohara@cisco.com |
| Emily H. Qi | Intel Corp. | 2111 N.E 25th Ave. Hillsboro, OR 97124 | 503-264-7799 | Emily.h.qi@intel.com |
| Necati Canpolat | Intel Corp. | 2111 N.E 25th Ave. Hillsboro, OR 97124 | 503-264-8014 | Necati.Canpolat@intel.com |
| Simon Black | Nokia |  |  | EXT-simon.3.black @nokia.com |
| Jari Jokela | Nokia |  |  | jari.jokela@nokia.com |
| Joe Epstein | Meru Networks | 1309 S Mary Ave, Sunnyvale CA | 408-215-5345 | jepstein@merunetworks.com |

Abstract

This submission contains the diagnostic alerts part of 05/1070r3 updated to be consistent with the TGv baseline document (IEEE802.11v D0.01). It is a proposal in support of the TGv Diagnostics and Troubleshooting requirement.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# 7. Frame formats

## 7.3 Management Frame Body Components

### 7.3.2 Information Elements

#### 7.3.2.21 Measurement Request element

Change Table 29 in 7.3.2.21 as follows:

Table 29—Measurement Type definitions for measurement requests

| Name | Measurement Type | Measurement Use |
| --- | --- | --- |
| Basic Request | 0 | Spectrum Management |
| Clear Channel Assessment (CCA) request | 1 |  |
| Receive power indication (RPI) histogram request | 2 |  |
| Channel load request | 3 | Radio Resource Measurement |
| Noise histogram request | 4 |  |
| Beacon request | 5 |  |
| Frame request | 6 |  |
| STA statistics request | 7 | Radio Resource Measurement & Wireless Network Management |
| LCI request | 8 | Radio Resource Measurement |
| QoS metrics request | 9 | Radio Resource Measurement & Wireless Network Management |
| Multicast Diagnostics | 10 | Wireless Network Management |
| Reserved | 11-254 | N/A |
| Measurement Pause request | 255 | Radio Resource Measurement |

##### 7.3.2.21.8 STA Statistics Request

Change Figure k11 in 7.3.2.21.8 as follows:

|  | Randomization Interval | Measurement Duration | Group Identity | Triggered Reporting (optional) |
| --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 1 | variable |

Figure k11—Measurement Request field format for a STA Statistics Request

Change Table k4 in 7.3.2.21.8 as follows:

Table k4—Group Identity for a STA Statistics Request

| Statistics Group Name | Group Identity |
| --- | --- |
| STA Counters from dot11CountersTable | 0 |
| BSS Load as described in 7.3.2.29 | 1 |
| RSNA Counters | 2 |
| Reserved | 3 – 255 |

Insert the following at end of subclause 7.3.2.21.8:

The Triggered Reporting field is used to specify trigger conditions and thresholds for triggered STA Statistics measurements. It is only present when setting up triggered reporting for Statistics Groups STA Counters from dot11CountersTable or RSNA Counters. See 11.11.9.5.

The format of Triggered Reporting field for STA Counters is shown in Figure v1.

|  | STA Counter Trigger Condition | dot11Failed Threshold | dot11Retry Threshold | dot11Multiple Retry Threshold | dot11Frame Duplicate Threshold |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 2 | 2 | 2 | 2 |
|  | dot11RTS Failure Threshold | dot11ACK Failure Threshold | dot11FCS Error Threshold | Measurement Count | Trigger Timeout |
|  | 2 | 2 | 2 | 2 | 2 |

Figure v1—Triggered Reporting Field for STA Counters

STA Counter Trigger Condition is a bit-field that specifies reporting triggers when requesting triggered STA Statistics reporting. The format of STA Counter Trigger Condition is shown in Figure v2.

|  | B0 | B1 | B2 | B3 | B4 | B5 | B6 | B7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dot11Failed | dot11Retry | dot11Multiple Retry | dot11Frame Duplicate | dot11RTS Failure | dot11ACK Failure | dot11FCS Error | Reserved |
| Bits | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Figure v2 — STA Counter Trigger Condition field

- Each bit in the STA Counter Trigger Condition field is set to 1 to request that a STA Statistics Report be generated when the corresponding STA counter defined in Figure k25 (in 7.3.2.22.8) increases by a value greater than or equal to the value of the corresponding STA counter threshold in Figure v1, within the total number of MSDUs given in Measurement Count. One or more trigger conditions may be set with specified thresholds.

The Trigger Timeout field contains a value in units of 100 TU during which a measuring STA shall not generate further triggered STA Statistics Reports after a trigger condition has been met.

The Measurement Count field contains a number of MSDUs used in determining trigger thresholds as described above.

The format of Triggered Reporting field for RSNA Counters is shown in Figure v3.

|  | RSNA Trigger Condition | dot11RSNAStats TKIPICVErrors Threshold | dot11RSNA StatsTKIPLocal MICFailures Threshold | dot11RSNA StatsTKIPRemote MICFailures Threshold | dot11RSNAStats CCMPReplays Threshold | dot11RSNA StatsCCMP DecryptErrors Threshold |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 2 | 2 | 2 | 2 |
|  | dot11RSNAStats TKIPReplays Threshold | dot11RSNAStats Broadcast MICErrors Threshold | Measurement Count | Trigger Timeout |  |  |
| Octets: | 2 | 2 | 2 | 2 |  |  |

Figure v3—Triggered Reporting Field for RSNA Counters

RSNA Trigger Condition is a bit-field that specifies reporting triggers when requesting triggered STA Statistics reporting. The format of Trigger Condition is shown in Figure v4.

|  | B0 | B1 | B2 | B3 | B4 |
| --- | --- | --- | --- | --- | --- |
|  | RSNAStats TKIPICV | RSNA StatsTKIP LocalMIC | RSNA StatsTKIP RemoteMIC | RSNAStats CCMPReplays | RSNA StatsCCMP Decrypt |
| Bits: | 1 | 1 | 1 | 1 | 1 |
|  | B5 | B6 | B7-B15 |  |  |
|  | RSNAStats TKIPReplays | RSNAStats BroadcastMIC | Reserved |  |  |
| Bits: | 1 | 1 | 9 |  |  |

Figure v4— RSNA Trigger Condition field

- Each bit in the Trigger Condition field is set to 1 to request that a STA Statistics Report be generated when the corresponding RSNA Error or Failure counter defined in Figure v8 (see 7.3.2.22.8) increases by a value grater than or equal to the value of the corresponding RSNA Failure Threshold in Figure v3 within the total number of MSDUs given in Measurement Count. One or more trigger conditions may be set with specified thresholds.

The Trigger Timeout field contains a value in units of 100 TU during which a measuring STA shall not generate further triggered STA Statistics Reports after a trigger condition has been met.

The Measurement Count field contains a number of MSDUs to be used in determining trigger thresholds as described above.

Insert the following clause after 7.3.2.21.10 renumbering subsequent clauses

#### 7.3.2.21.11 Multicast Diagnostics Request

The Measurement Request field corresponding to a Multicast Diagnostics Request is shown in Figure v5. The response to a Multicast Diagnostics Request is a Multicast Diagnostics Report.

|  | Randomization Interval | Measurement Duration | Multicast MAC Address | Multicast Triggered Reporting (optional) |
| --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 6 | 3 |

Figure v5—Measurement Request field format for a 
Multicast Diagnostics Request

Randomization Interval shall be set to the desired maximum random delay in the measurement start time, expressed in TUs. The use of Randomization Interval is described in 11.11.3. Randomization Interval is not used and shall be set to 0 when requesting triggered Multicast diagnostics.

Measurement Duration shall be set equal to the duration of the requested measurement, expressed in TUs. Requested Measurement Duration for a Multicast Diagnostics Report shall always be an integral number of beacon intervals. When setting up a triggered multicast diagnostics alert, Measurement Duration is not used and shall be set to 0.

Multicast MAC Address contains the MAC address of the multicast traffic (the multicast group) to which the request relates. Multicast MAC address may also be set to the broadcast address to request diagnostic reporting on broadcast traffic.

The Multicast Triggered Reporting field is used to specify trigger conditions and thresholds. It is only present when setting up triggered management diagnostic reporting. The Multicast Triggered Reporting field is as shown in Figure v6.

|  | Multicast Trigger Condition | Report Timeout | Trigger Timeout |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |

Figure v6—Multicast Triggered Reporting field

Multicast Trigger Condition is a bit-field that specifies reporting triggers for triggered management diagnostic reporting. The format of Multicast Trigger Condition is shown in Figure v7.

|  | B0 | B1 | B7 |
| --- | --- | --- | --- |
|  | Report Timeout | Reserved |  |
| Bits: | 1 | 6 |  |

Figure v7 —Multicast Trigger Condition field

- Report Timeout is set to 1 to request that a Multicast Triggered Report be generated when no multicast frames with the monitored multicast or broadcast address are received in a time that equals to value given in Report Timeout Field.

The Report Timeout field contains a time value in units of 10 TU to be used as the threshold value for the Report Timeout trigger condition.

The Trigger Timeout field contains a value in units of 100 TU during which a measuring STA shall not generate further Multicast Triggered Reports after a trigger condition has been met.

#### 7.3.2.22 Measurement Report element

Change Table 30 in 7.3.2.22 as follows:

Table 30—Measurement Type definitions for measurement reports

| Name | Measurement Type | Measurement Use |
| --- | --- | --- |
| Basic report | 0 | Spectrum Management |
| Clear Channel Assessment (CCA) report | 1 |  |
| Receive power indication (RPI) histogram report | 2 |  |
| Channel load report | 3 | Radio Resource Measurement |
| Noise histogram report | 4 |  |
| Beacon report | 5 |  |
| Frame report | 6 |  |
| STA statistics report | 7 | Radio Resource Measurement & Wireless Network Management |
| LCI report | 8 | Radio Resource Measurement |
| QoS metrics report | 9 | Radio Resource Measurement & Wireless Network Management |
| Multicast Diagnostics | 10 | Wireless Network Management |
| Reserved | 11-255 | N/A |

##### 7.3.2.22.8 STA Statistics Report

Change Figure k24 in 7.3.2.22.8 as follows:

|  | Measurement Duration | Statistics Group Data | Reporting Reason |
| --- | --- | --- | --- |
| Octets: | 2 | Variable | Variable |

Figure k24—Measurement Report field format for a STA Statistics Report

Insert a new row into table k8 as shown below:

Table k8—Group Identity for a STA Statistics Report

| Group Identity Requested | Statistics Returned |
| --- | --- |
| 2 | dot11RSNAStatsTKIPICVErrors (Counter32) dot11RSNAStatsTKIPLocalMICFailures (Counter32) dot11RSNAStatsTKIPRemoteMICFailures (Counter32) dot11RSNAStatsCCMPReplays (Counter32) dot11RSNAStatsCCMPDecryptErrors (Counter32) dot11RSNAStatsTKIPReplays (Counter32) dot11RSNAStatsBroadcastMICErrors (Counter32) |
| 3– 255 | None |

Insert the following text at the end of 7.3.2.22.8:

The format of the Measurement Report field for RSNA Counters Group is shown in Figure v8

|  | dot11RSNAStats TKIPICVErrors | dot11RSNA StatsTKIPLocal MICFailures | dot11RSNA StatsTKIPRemote MICFailures | dot11RSNAStats CCMPReplays | dot11RSNA StatsCCMP DecryptErrors |
| --- | --- | --- | --- | --- | --- |
| Octets: | 4 | 4 | 4 | 4 | 4 |
|  | dot11RSNAStats TKIPReplays | dot11RSNAStats Broadcast MICErrors |  |  |  |
| Octets: | 4 | 4 |  |  |  |

Figure v8— Measurement Report field format for RSNA Counters Group

The Reporting Reason field is a bit field indicating the reason that the measuring STA sent the STA Statistics report. It is only present if Statistics Group Name is STA Counters from dot11CountersTable or RSNA Counters.

The Reporting Reason field for STA Counters is shown in Figure v9.

|  | B0 | B1 | B2 | B3 | B4 | B5 | B6 | B7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dot11Failed | dot11Retry | dot11Multiple Retry | dot11Frame Duplicate | dot11RTS Failure | dot11ACK Failure | dot11FCS Error | Reserved |
| Bits | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Figure v9—Reporting Reason field for STA Counters

The Reporting Reason field for RSNA Counters is shown in Figure v10.

|  | B0 | B1 | B2 | B3 | B4 |
| --- | --- | --- | --- | --- | --- |
|  | RSNAStats TKIPICV | RSNA StatsTKIP LocalMIC | RSNA StatsTKIP RemoteMIC | RSNAStats CCMPReplays | RSNA StatsCCMP Decrypt |
| Bits: | 1 | 1 | 1 | 1 | 1 |
|  | B5 | B6 | B7-B15 |  |  |
|  | RSNAStats TKIPReplays | RSNAStats BroadcastMIC | Reserved |  |  |
| Bits: | 1 | 1 | 9 |  |  |

Figure v10—Reporting Reason field for RSNA Counters

In a requested STA Statistics Report, all bit fields in the Reporting Reason field are set to 0.

Insert the following clause after 7.3.2.22.10

#### 7.3.2.22.11 Multicast Diagnostics Report

The format of the Measurement Report field of a Multicast Diagnostics report is shown in Figure v11.

|  | Measurement Start Time | Measurement Duration | Multicast MAC Address | Multicast Reporting Reason | Multicast Received MSDU Count |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 2 | 6 | 1 | 4 |

Figure v11—Measurement Report field 
format for a Multicast Diagnostics Report

The Measurement Start Time shall be set to the value of the STA TSF timer at the time the measurement started. For a triggered Multicast Diagnostics report, this shall be the TSF value at the reporting STA when the trigger condition was met.

Measurement Duration shall be set equal to the duration over which the Multicast Diagnostics Report was measured, expressed in TUs. Reported Measurement Duration for a Multicast Diagnostics Report shall always be an integral number of beacon intervals due to the measurement process. Measurement Duration is not used in triggered diagnostics reporting and shall be set to 0.

Multicast MAC Address contains the MAC address of the multicast traffic (the multicast group) to which the report relates.

The Multicast Reporting Reason field is a bit field indicating the reason that the measuring STA sent the Multicast Diagnostics report. The Multicast Reporting Reason field is shown in Figure v12.

|  | B0 | B1 | B7 |
| --- | --- | --- | --- |
|  | Report Timeout Trigger | Reserved |  |
| Bits: | 1 | 6 |  |

Figure v12—Multicast Reporting Reason field

- The Report Timeout Trigger bit set to 1 indicates that Multicast Diagnostics Report was generated as a triggered report due to the Report Timeout trigger.

The Multicast Received MSDU Count contains the total number of multicast MSDUs for the Multicast MAC Address that were received during the Measurement Duration. For a triggered multicast diagnostics measurement this shall be the total number of frames received with the indicated Multicast MAC Address.

In a requested Multicast STA Performance Report, all bit fields in the Reporting Reason field are set to 0.

Chnage 7.3.2.35 as follows

### 7.3.2.35 Wireless Network Management Capability Information element

The Wireless Network Management Capability Information element contains information about the wireless network management capabilities of a STA as shown in Figure v1.

|  | Element ID | Length | Wireless Network Management Capabilities |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 |

Figure v1—Wireless Network Management Capability Information element format

The Element ID field is equal to the Wireless Network Management Capability value in Table 26.

The value of the length field is variable and depends on the length of the Wireless Network Management Capabilities field. The minimum value of the length field is 2.

The Wireless Network Management Capabilities field is a bit field indicating the advertised management capabilities of the STA. The Wireless Network Management Capabilities field is shown in Figure v2.

|  | B0 | B1 | B2 | B23 | B15 |
| --- | --- | --- | --- | --- | --- |
|  | Event Log | Diagnostics Channel | Multicast Alert | Reserved |  |
| Bits: | 1 | 1 | 1 | 143 |  |

Figure v2—Wireless Network Management Capabilities

- The Event Log bit set to 1 indicates the STA supports Event Log diagnostics as described in 11.15.2. The Event Log bit set to 0 indicates that the STA does not support this service.

- The Diagnostics Channel bit set to 1 indicates the STA supports Diagnostics Channel as described in 11.15.3. The Diagnostics Channel bit set to 0 indicates that the STA does not support this service.

- The Multicast Alert bit set to 1 indicates the STA supports Multicast diagnostics as described in 11.15.1.1. The Multicast Alert bit set to 0 indicates that the STA does not support this service.

- All other bits are reserved and shall be set to 0 on transmission and ignored on reception

The lack of a Wireless Network Management Capability element shall be interpreted as the STA having no advertised Wireless Network Management Capabilities.

# 11. MAC sublayer management entity

## 11.11 Radio Measurement Procedures

### 11.11.9 Specific Measurement Usage

#### 11.11.9.5 STA Statistics Report

Insert the following at the end of 11.11.9.5:

A STA may request that a STA Statistics report be sent when statistics of interest reach a defined threshold. This is termed a triggered STA Statistics measurement and shall be requested by setting the Enable and Report bits to 1 within a Measurement Request Element containing the STA Statistics Measurement Type. The Measurement Request field shall contain a STA Statistics Request with the trigger conditions specified in the Triggered Reporting field. One or more trigger conditions may be set with specified thresholds. See 11.11.8.

A STA accepting a triggered STA Statistics measurement shall measure the requested statistics. If a trigger condition occurs, the measuring STA shall send a STA Statistics measurement report to the requesting STA. The measuring STA shall not send further triggered STA Statistics reports until the Trigger Timeout period specified in the request has expired, or new trigger conditions have been requested. Measurement of STA Statistics shall continue during the reporting timeout period.

If a STA receives a requested STA Statistics measurement when triggered STA Statistics measurement is in progress, the triggered measurement shall be suspended for the duration of the requested measurement. When triggered measurement resumes the counted statistics shall be reset.

STA Statistics reported in a triggered STA Statistics report shall be the values accumulated over the number of transmitted MSDUs prior to the trigger event given in the Measurement Count field of the measurement request that established the trigger condition. Measurement duration shall not be used in triggered STA statistics measurement and the Measurement Duration field in both the Measurement Request and any Measurement Report shall be set to 0.

Once accepted by a measuring STA, a triggered STA Statistics measurement continues to be active until:

- The measuring STA disassociates or successfully reassociates, or

- The measurement is terminated by the requesting QSTA.

All triggered STA Statistics measurements shall be terminated at a measuring STA by receiving a triggered STA Statistics measurement request with the Enable bit set to 1 and the Report bit set to 0. A STA requesting a triggered STA Statistics measurement may update the trigger conditions by sending a triggered STA Statistics measurement request specifying the new trigger conditions.

Insert 11.151.1 as follows:

## 11.15 Wireless Network Management Procedures

### 11.15.1 Diagnostic Alerts

#### 11.15.1.1 Multicast Diagnostics Reporting

Multicast Diagnostics reporting allows an AP to receive information concerning the receive statistics of multicast and broadcast traffic at associated STAs. A non-AP STA that supports Multicast Diagnostics Reporting shall set the Multicast capability bit in the Radio Management Capabilities element to 1.

An AP may send a Multicast Diagnostics request to a non-AP STA that has indicated multicast diagnostics capability. If the STA accepts the request it shall count the number of received MSDUs with the specified multicast address during the requested Measurement Duration. This value shall be returned in a Multicast Diagnostics report.

An AP may request that triggered Multicast Diagnostic reporting is enabled at associated non-AP STAs that have indicated multicast diagnostics capability. To enable Multicast Diagnostic reporting, the AP shall send a Measurement Request element containing a Multicast Diagnostics Request Type and with Enable and Report bits set to 1 within a Measurement Request frame. See 11.11.8. The Multicast MAC Address and trigger conditions for diagnostic reporting shall be specified in the request.

Multicast Diagnostic reporting may be requested for broadcast traffic by setting Multicast MAC address to the broadcast address.

A non-AP STA accepting a request for multicast diagnostic reporting shall send a Multicast Diagnostics Report to the requesting STA if the specified trigger condition occurs. The measuring non-AP STA shall not send further Multicast Diagnostics Reports until the Trigger Timeout period specific in the request has expired, or revised trigger conditions have been requested.

Once accepted, Multicast Diagnostics reporting continues to be active for the specified Multicast MAC address until any of the following occurs:

- Receipt of a Measurement Request element containing a Multicast Diagnostics Request Type and with Enable bit set to 1 and Report bits set to 0 within a Measurement Request frame.

- Receipt of a Measurement Request element containing a Multicast Diagnostics Request Type, with Enable and Report bits set to 1, but with no trigger conditions.

- The STA leaves the Multicast Group or disassociates

Insert A.4.14 into Annex A as follows:

# Annex A

## A.4 PICS proforma–IEEE Std. 802.11, 2006 Edition

Insert new items RME2.4, RME2.5 and RME2.6 after RME2.3 in A4.14:

### A.4.14 Wireless Network Management extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
|  | Are the following Wireless Network Management capabilities supported? |  |  |  |
| RME2.4 | Triggered STA Statistics Reporting | 7.3.2.21.8, 7.3.2.22.8, 11.11.9.5 | CFv:M | Yes, No, N/A |
| RME2.5 | Triggered QoS Metrics Reporting | 7.3.2.21.10, 7.3.2.22.10, 11.11.9.8 | (CFv AND CF12):O | Yes, No, N/A |
| RME2.6 | Multicast Diagnostics Reporting | 7.3.2.21.11 7.3.2.22.11, 11.15.1.1 | CFv:M | Yes, No, N/A |

References

11-05-1070-03-000v-normative-text-proposal-diagnostics-and-troubleshooting, Qi et al, January 2006