IEEE P802.11
Wireless LANs

| Changes to FTIE Advertisements |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date March 6, 2006 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Kapil Sood | Intel Corp. | 2111 NE 25th Ave JF3-206 Hillsboro OR 97124 | +1-503-264-3759 | Kapil.Sood@intel.com |
| Frank Ciotti | Motorola | 7700 W. Parmer Ln Austin, TX 78726 | 512-996-5753 | frank.ciotti@motorola.com |
| Bill Marshall | AT&T | 180 Park Ave, Florham Park, NJ 07932 | 973-360-8718 | wtm@research.att.com |
| Mike Montemurro | Chantry Networks | 1900 Minnesotta Court Ste 125, Mississauga ON L5N3C9 | 905-363-6413 | Michael.montemurro@siemens.com |

Abstract

The document provides the text updates to TGr draft D1.1.  The text addresses the Beacon Bloat comments to provide mechanisms that reduce the size of beacon and probes for Fast BSS Transitions.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Updates based on TGr Draft D1.1

Section 7.3.2.43, Page 24, line 26

Replace: “When sent by an AP, R0KH-ID indicates the primary (or default) AP Authenticator, and is the value of the MIB variable dot11FTR0KeyHolderID. When sent by a STA, R0KH-ID indicates the PMK-R0 holder with

which the STA negotiated its current security association.

R1KH-ID indicates the PMK-R1 Key Holder that will be utilized by the AP for key derivation.”

With: “When sent by a STA, R0KH-ID indicates the PMK-R0 holder with which the STA negotiated its current security association.  When sent by AP, R0KH-ID shall be the same as the value received from the STA.  When sent by the AP, the R1KH-ID indicates identity of the key holder of the AP.  When sent by the STA, R1KH-ID shall be same as the value received from the AP.

R0KH-ID indicates the PMK-R0 Key Holder that will be utilized by the AP for PMK-R1 key procurement.  R1KH-ID indicates the AP’s key holder identity which will be used by the STA for deriving the PMK-R1 keys.”

Section 7.3.2.43, Page 24, Remove line 33-36

Section 7.2.3.1, delete order 29 “Fast BSS Transition Information”

Section 7.2.3.4, Page 14, Table 7, Order 9 and 10, and all other places in 7.2 where the same text appears:

Replace: “advertised an FTIE in its Beacon…”

With: “advertised its Fast BSS Transition Capability in the MDIE in its Beacon…”

Section 8.5A.4, Page 39, Line 37

Replace: “Each AP shall advertise its R0KH-ID in the FTIE”

With: “Each AP shall deliver its R0KH-ID to the STA in the FTIE of the (Re)Association Response message during First Contact .”

Section 8.5A.5, Page 40, Line 10

Insert at the end of paragraph: “Each AP shall deliver its R1KH-ID to the STA in the FTIE”

Section 8.5A.8.1, Page 43, Line 30

Replace: “The STA shall include the Key Holder IDs (R0KH-ID and R1KH-ID in the FTIE), and the Key Name (PMKR1Name in the RSNIE) that it is using to generate the PTK. By including the key names and their respective key holders the target AP can use this information to ensure”

With: “The STA shall include the R0KH-ID in the FTIE.  The target AP can use the R0KH-ID and the STA MAC address to ensure”

Section 8.5A.8.1, Page 43

Remove line 61: “—  R1KH-ID shall be the value advertised by the target AP in its Beacons and Probe Responses.”

Remove line 57: “— Resource mechanisms shall be the value advertised by the target AP in its Beacons and Probe Responses.”

Replace Line 64: “The MDIE shall contain the Mobility Domain Identifier obtained from the target AP.”

With: “The MDIE shall contain the Mobility Domain Identifier, Fast BSS Transition Capability, and Resource Policy obtained from the target AP.  The Target AP advertises the MDIE in beacons and probe responses.”

Section 8.5A.8.1, Page 44

Change line 5-6 “PMKID count shall be 1” and “PMKID field shall contain the PMKR1Name…..”

With “PMKID count shall be 0” and “PMKID field shall be empty.”

Section 8.5A.8.2, Page 44

Replace Line 64: “The contents of the Fast BSS Transition Information Element (FTIE) shall be identical to the contents of the FTIE advertised by the AP in Beacons and Probe Responses, and shall be set as follows: “

With: “The contents of the Fast BSS Transition Information Element (FTIE) shall be set as follows:”

Section 8.5A.8.2, Page 45

Replace Line 1-8: “— Resource mechanisms shall indicate the capabilities of the target AP.

- Key holder IDs (R0KH-ID, and R1KH-ID) shall be identical to the key holder identification provided by the STA in the first message.

The MDIE shall contain the Mobility Domain Identifier. This shall be the same as the MDIE advertised by the AP in Beacons and Probe Responses.”

With: “ - R0KH-ID shall be identical to the R0 Key Holder Identifier provided by the STA in the first message:

- R1KH-ID shall be set to the R1 Key Holder Identifier of the AP

The MDIE shall contain the Mobility Domain Identifier, Fast BSS Transition Capability, and the Resource Policy. This shall be the same as the MDIE advertised by the AP in Beacons and Probe Responses.”

Section 8.5A.8.3, Page 45

Replace Line 64: “The Fast BSS Transition Information Element (FTIE) shall be identical to the FTIE presented in the first message of this sequence.”

With: “The Fast BSS Transition Information Element (FTIE) shall be identical to the FTIE presented in the second message of this sequence.”

Section 8.5A.8.3, Page 46, Line 1

Replace: “The MDIE shall contain the Mobility Domain Identifier. This shall be identical to the MDIE contained in the first message of this sequence.”

With: “The MDIE shall contain the Mobility Domain Identifier, Fast BSS Transition Capability, and the Resource Policy. This shall be identical to the MDIE contained in the first message of this sequence.”

Section 8.5A.8.4, Page 47, Lines 16-19

Replace: “The MDIE shall contain the Mobility Domain Identifier. This shall be identical to the MDIE contained in the second message of this sequence.”

With: “The MDIE shall contain the Mobility Domain Identifier, Fast BSS Transition Capability, and the Resource Policy. This shall be identical to the MDIE contained in the second message of this sequence.”

Section 8.A.1.1, Page 49, Lines 41-55

Replace Lines 41-55: “Fast BSS Transition capability is advertised in the Beacon and Probe Response frames by inclusion of the FTIE and MDIE. The Mobility Domain Identifier shall be the value of dot11FTMobilityDomainID. The reservation limit advertised in the FTIE shall be the value of dot11FTReservationLimit. Note: the mobility domain identifier IE may be present without FTIE since it may be used in other mecha-nisms beyond the Fast Transition mechanisms.”

With: “Fast BSS Transition capability is advertised in the Beacon and Probe Response frames by setting the Fast BSS Capability bit in the MDIE.  The MDIE is advertised in the Beacons and Probe Responses to indicate the MD-ID, Fast BSS Transition Capability, and the Resource Policy.  When the MDIE is used without Fast BSS Transition Capability (Fast BSS Transition Capability bit set to zsero), then all bits of the Fast BSS Capability and Resource Policy field shall be ignored.

The Mobility Domain Identifier shall be the value of dot11FTMobilityDomainID. The reservation limit advertised in the MDIE shall be the value of dot11FTReservationLimit. ”

Section 8.A.2, Page53, Lines57

Replace: “FTIE and MDIE in the (re)association request,”

With: “MDIE in the (re)association request,”

Section 8.A.2, Page54, Lines50-54

Replace: “Upon successful IEEE 802.11 open authentication, STA shall send a (re)association request to the AP and shall include the FTIE and MDIE. The contents of the FTIE and MDIE shall be the values advertised by the AP in its Beacons or Probe Responses. The presence of the FTIE and MDIE indicates to the AP a desire to enable future Fast BSS Transitions.”

With: “Upon successful IEEE 802.11 open authentication, STA shall send a (re)association request to the AP and shall include the MDIE. The contents of the MDIE shall be the values advertised by the AP in its Beacons or Probe Responses. The presence of the Fast BSS Transition Capability bit set to one in Fast BSS Transition Capability and Resource Policy field indicates to the AP a desire to enable future Fast BSS Transitions.”

Section 8.A.2, Page54, Lines57-65

Replace: “The (re)association response from the AP shall contain a FTIE and MDIE, with contents as presented in Beacons and Probe Responses. The AP shall include its security capabilities in the RSN Information Element. The PMKID list field shall be empty.

STA->AP: (Re)association Request (FTIESTA, MDIESTA, RSNIESTA)

AP->STA: (Re)association Response (FTIEAP, MDIEAP, RSNIEAP).”

With: “The (re)association response from the AP shall contain the MDIE with contents as presented in Beacons and Probe Responses.  The FTIE in the (re)association response shall include the key holder (R0KH-ID) of the AP.  The AP shall include its security capabilities in the RSN Information Element. The PMKID list field shall be empty.

STA->AP: (Re)association Request (MDIESTA, RSNIESTA)

AP->STA: (Re)association Response (FTIEAP, MDIEAP, RSNIEAP)”

Section 8.A.2, Page55, Lines 13

Insert: “- R0KH-ID”

Section 8.A.2, Page55, Lines 41

Replace: “the STA’s (re)association request.”

With: “the AP’s (re)association response”

Section 8.A.2, Page55, Lines 43-50

Replace: “Message 3: the AP shall include the PMKR1Name in the PMKID field of the RSNIEAP. The AP shall also include the FTIEAP, MDIEAP and PTK key lifetime in the TIE[KeyLifetime]. The FTIEAP, and MDIEAP shall be the same as that advertised by the AP’s Beacon and Probe Responses. The PMKR1Name shall be as calculated by the R1KH according to the procedures of 8.5A.5; in order for future Fast BSS Transitions to be successful it needs to be the same as the PMKR1Name in Message #2.”

With: “Message 3: the AP shall include the PMKR1Name in the PMKID field of the RSNIEAP. The AP shall also include the FTIEAP, MDIEAP, and PTK key lifetime in the TIE[KeyLifetime]. The FTIEAP, and MDIEAP shall be the same as in (re)association response.  The PMKR1Name shall be as calculated by the R1KH according to the procedures of 8.5A.5; in order for future Fast BSS Transitions to be successful it needs to be the same as the PMKR1Name in Message #2”

Section 8.A.3.3, Page57, Lines 22, 27

Replace: “FTIE”

With: “MDIE”

Section 8.A.4, Page58, Lines 1, 3

Replace: “FTIE”

With: “MDIE”

Figures 121F

Remove: FTIESTA from (Re)association Request

Table 35A, Order 4

Replace: Mobility Domain Identifier

With: Mobility Domain Information

Tables 32P, 32Q, 32R, 32S, and everywhere it appears in the “Information” column of tables in 7.2

Replace: “Mobility Domain Identifier”

With:  “Mobility Domain Information”