IEEE P802.11
Wireless LANs

| Normative Text Proposal for TGv |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-03-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Arnaud Meylan | Qualcomm |  |  | ameylan@qualcomm.com |
| Sanjiv Nanda | Qualcomm |  |  |  |

Abstract

This document contains a normative text proposal in support of a framework in accordance with REQ2010

## Req 2010: Power Saving

TGv shall support one or more power saving schemes to optimize the STA’s power consumption during prolonged time periods of no transmission or reception activity in the order of several hundred’s of ms’s or above.

This document is based largely on P802.11-REVma-D5.1 and P802.11k-D3.0 and P802.11-vD0.02

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Contents

7. Frame formats	3

7.1 MAC frame formats	3

7.2.3.1	Beacon frame format	3

7.2.3.9	Probe Response frame format	3

7.3.2 Information Elements	3

7.3.2.36 Standby Support Element	4

10. Layer management	5

10.3 MLME SAP Interface	5

10.3.6.1 MLME-ASSOCIATE.request	5

10.3.6.1.2 Semantics of the service primitive	5

10.3.7 Reassociate	5

10.3.7.1 MLME-REASSOCIATE.request	5

10.3.7.1.2 Semantics of the service primitive	5

10.3.10 Start	6

10.3.10.1 MLME-START.request	6

10.3.10.1.1 Function	6

10.3.10.1.2 Semantics of the service primitive	6

11. MAC sublayer management entity	8

11.2.1.11 AP aging function	8

11.2.1.12 AP dissacociation function	8

# 7. Frame formats

#### 7.1 MAC frame formats

      - Beacon frame format

Insert a new row into table8 8—Beacon frame body as shown below:

| Order | Information | Notes |
| --- | --- | --- |
| X | Standby Support element | The Standby support element shall be present if dot11WirelessManagementImplemented is true |

      - Probe Response frame format

Add to Table 15—Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| X | Standby Support Element | Standby Support Element shall be present if dot11WirelessManagementImplemented is true. |

### 7.3.2 Information Elements

Insert Element ID x  into Table 26—Element IDs and change the Reserved row accordingly:

Table 20—Element IDs

| Information Element | Element ID |
| --- | --- |
| Standby Support | X |
| Reserved | X+1, 221 |

Insert paragraph after 7.3.2.35

#### 7.3.2.36 Standby Support Element

| Element ID (1) | Length (1) | MaximumListenInterval (1) | InactivityTimeout (2) |  |
| --- | --- | --- | --- | --- |

The Standby support element is present in Beacons and Probe responses of an AP when dot11WirelessManagementImplemented is true

The MaximumListenInterval is one octet and specifies the maximum listen interval supported by the AP for new associations.

The inactivity timeout is specified in seconds. It informs a STA of the inactivity timeout used by the AP.

# 10. Layer management

#### 10.3 MLME SAP Interface

Change 10.3.6 as follows:

#### 10.3.6.1 MLME-ASSOCIATE.request

##### 10.3.6.1.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-ASSOCIATE.request	(
PeerSTAAddress,
AssociateFailureTimeout,
CapabilityInformation,
ListenInterval,
Supported Channels,
RSN Information Element,
QoSCapability,
WirelessManagementCapabilities
)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ListenInterval | Integer | >=0 <=MaximumListenInterval | Specifies the number of beacon intervals that can pass before the STA awakens and listens for the next beacon. When MaximumListenInterval is available ListenInterval shall be <= MaximumListenInterval |

Change 10.3.7 as follows:

### 10.3.7 Reassociate

#### 10.3.7.1 MLME-REASSOCIATE.request

##### 10.3.7.1.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-REASSOCIATE.request	(
PeerSTAAddress,
AssociateFailureTimeout,
CapabilityInformation,
ListenInterval,
Supported Channels,
RSN Information Element,
QoSCapability,
WirelessManagementCapabilities
)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ListenInterval | Integer | >=0 <=MaximumListenInterval | Specifies the number of beacon intervals that can pass before the STA awakens and listens for the next beacon. When MaximumListenInterval is available ListenInterval shall be <= MaximumListenInterval |
| WirelessManagementCapabilities | As defined in the Wireless Network Management Capabilities element | As defined in the Wireless Network Management Capabilities element | Specifies the Wireless Network Management capabilities of the non-AP STA. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |

### 10.3.10 Start

#### 10.3.10.1 MLME-START.request

##### 10.3.10.1.1 Function

This primitive requests that the MAC entity start a new BSS.

##### 10.3.10.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-START.request(

SSID,

BSSType,

BeaconPeriod,

DTIMPeriod,

InactivityTimeout,

CF parameter set,

PHY parameter set,

IBSS parameter set,

ProbeDelay.

CapabilityInformation,

BSSBasicRateSet,

OperationalRateSet,

Country,

IBSS DFS Recovery Interval,

EDCAParameterSet

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| InactivityTimeout | Integer | As defined in 7.3.2.36 | Indicates to STA in the BSS when the AP may disassociate them upon inactivity |

# 11. MAC sublayer management entity

##### 11.2.1.11 AP aging function

Insert at the end of paragraph 11.2.1.11

The AP advertises the MaximumListenInterval in the Beacon and Probe Responses, that is the largest ListenInterval that the AP shall support for a new association request. Existing associations shall not be affected by the MaximumListenInterval advertised by the AP.

Insert after paragrapm 11.2.1.11

##### 11.2.1.12 AP dissacociation function

STA in power save mode may not exhibit traffic for extended periods of time. As long as the relevant security keys are up to date, the AP shall not disassociate a power save STA that transmitted one frame or more in the past InactivityTimeout interval. AP may disassociate a STA that transmitted no frame in the past InactivityTimeout interval. The InactivityTimeout interval is included in the Standby Support element in beacon and probe response frames.

11.8.1 STA association procedures

Insert at the beginning of first paragraph

When the Standby Support Information element is present in Beacon and Probe response frames the STA shall not request a ListenInterval larger than that specified in the MaximumListenInterval.