|  |  |  |  | LB80 Comment Resolution |  |
| --- | --- | --- | --- | --- | --- |
| CID | Commenter: | Clause: | Addressed By: |  | Original Date Prepared |
| 59 78 30 48 42 60 115 | Reuss Stanley Karcz Brasier Jokela Reuss Wells | 7.3.2.22.11 | Susan Dickey |  | March 8, 2006 |

- COMMENT:

59 The first sentence is fuzzy and redundant.

78 Missing regulatory Domain

30 Is Measurement Time the station's local TSF timer?

48 Data format of the Measurement Time field is not specified.

42 It would be goog to mention what is actually measured, i.e., absolute power level of what?

60 What is the format of the Measurement Time field.

115 Properties of "local time" should be referenced for clarity.

- Suggested Remedy (If appropriate):

59 Remove the first sentence.

78 Add regulatory domain.

30 clarify.

48 Specify how the field is coded.

42 Fix

60 Specify how the field is coded. I presume that this refers to the timestamp field derived from the TSF, as described in 7.3.1.10. If so, this section should be referenced.

115 Change sentence to read:"The Measurement Time shall be the local time (see 10.3.33.3.2) at the station when the WRSS measurement is made."

- Background, Explanation, Discussion, etc.:

Most of the comments above seem to be a result of not reading this clause in the context of the other clauses in 7.3.2.22 in the base 802.11 standard. The first sentence of the 7.3.22.2.11 reads in exactly the same way as the first sentences in the other clauses. None of the other subclauses of 7.3.22.2 feel the need to reference 7.3.1.10 for the coding of the TSF timer, since this is fundamental to this section. However, we did neglect to mention the TSF timer specifically in our description of the Measurement Time field. In addition, the language describing the WRSS as an absolute power level is unclear in this context and should be replaced with a simple reference to 20.2.3.3.

The comment about adding a regulatory domain is valid and important, since it must be known for channel number to be meaningful. We need to add the WAVE band to Annex J.

- Recommended Resolution of the Comment:

Reject Comment 59, since language matches other Measurement Type clauses.

Resolve Comments 30, 48, 60 and 115 by changing page 9, line 7 to:

The Measurement Time shall be the value of the local TSF timer at the station where the WRSS measurement was made.

Resolve Comment 42 by changing page 9, lines 8 and 9 to:

The WRSS Value shall be an integer from 0 to 255. See 20.2.3.3 for mapping and detailed description.

Leave Comment 78 open until the additions to Annex J have been added to the amendment.

- Motion (if technical and/or significant):

(And instructions to the editor.)

Move to ask the editor to make the changes to resolve comments as above and for the group to take an action item for someone to write the amendment describing the WAVE band to Annex J.

Motion by: ____________________

Second:  ______________________

| Approve: | Disapprove: | Abstain: |
| --- | --- | --- |