IEEE P802.11
Wireless LANs

| Table of Resource Types |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-04-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bill Marshall | TGr Editor | 180 Park Ave, Florham Park, NJ 07932 | 973-360-8718 | wtm@research.att.com |

Abstract

TGr D2.0 defines a general purpose mechamism for negotiating resources prior to a transition. It defines a Resource Information Container, consisting of a sequence of IEs that express the resource requirements and/or desires of the STA.

In order to make the draft more specific as to implementation requirements, this submission proposes the addition of a Table to the definition of the Resource Information Container, containing a list of the IEs that may appear in an RDIE.  Such a table could be extended by other amendments to add other resource types to this framework.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Change 7.3.2.46 as follows:

(Note – other submissions move the changed portion of 7.3.2.46 to 8A.7.  It is intended that these changes also be moved to 8A.7 if that submission is also accepted.)

7.3.2.46 Resource information container information elements

The Resource Information Container (RIC) refers to a collection of Information Elements that are used to express a resource request in a resource request message. A RIC is also used to convey responses to the corresponding requests.

Two IE types are used to define the RIC structure:

- RIC_ROOT (RRIE): used as the RIC header

- RIC_DATA (RDIE): used to describe each Resource Request. A simple Resource Request is an RDIE followed by single Resource IE (e.g., a TSPEC).

A Resource Information Container has one RRIE and a list of zero or more Resource Requests, as shown in Figure 113R.

When multiple Resource Requests appear in a RIC, the STA's intent is that all of them be allocated. Thus an "AND" relationship exists between the Resource Requests.

Each Resource Request consists of an RDIE followed by one or more alternative Resource descriptors. Each resource descriptor consists of one or more Information Elements.  The possible resource descriptors that may be appear in a RIC, and the information elements that they contain, are given in Table XX:

Table XX: Resource Types and Resource Descriptor definitions

| Resource Type | Resource Descriptor definition | Reservation Procedures | Notes |
| --- | --- | --- | --- |
| QoS | In a request: TSPEC (see 7.3.2.30), followed by zero or more TCLAS (7.3.2.31), followed by zero or one TCLAS Processing (7.3.2.33) In a response: TSPEC (see 7.3.2.30), followed by zero or one Schedule (7.3.2.34) | See 11.4 | May only be sent by a QSTA to a QAP. |

An example of a Resource Request, consisting of two alternative resource descriptors, is shown in Figure 113S.

(in above figure, change to RDIE, Resource Descriptor, Resource Descriptor)

For example, when the resource being requested is QoS for downstream traffic, a TSPEC information element may be followed by one or more TCLAS information elements and, when multiple TCLAS information elements are present, a TCLAS Processing element. Such an example Resource Request with two alternative TSPECs is shown in Figure 113T.

(in above figure, dup the TCLAS-TCLAS-TCLAS Processing after each TSPEC)

If there are multiple Resource Descriptors then these are treated as "choices" by the target AP. The AP shall attempt to allocate whatever is specified in the first Resource Descriptor; if this fails the AP shall attempt to allocate whatever is specified in the next Resource Descriptor instead, and so on until a successful allocation or the AP reaches the end of the Resource Descriptor list. Thus, an "OR" relationship exists between Resource Descriptors that follow an RDIE, with the Resource Descriptors appearing in order of preference.

An example of a RIC with two resource requests, each with a single TSPEC, is given in Figure 113U.

An example of a RIC with one resource request, with a choice of two TSPECs, is given in Figure 113V. This indicates that the target AP can select one of the two TSPECs

.

7.3.2.46.1 RIC root information element (RRIE)

The Resource Information Container Root Information Element (RRIE) is defined in Figure 113W.

The length field shall contain a value 2.

The RRIE Identifier contains an arbitrary value that shall be unique among outstanding resource allocation requests at a STA to enable the matching of responses to requests.

The RDIE Count contains the number of RDIEs in this RIC.

7.3.2.46.2 RIC data information element (RDIE)

The Resource Information Container Data Information Element (RDIE) is defined in Figure 113X.

The length field shall contain a value 2.

The RDIE Identifier is an arbitrary value, chosen by the resource requestor, which uniquely identifies the RDIE within the RIC. It is used in the response to refer to a specific resource allocation request.

The Control Options field is defined in Figure 113Y.

(in above figure, change “Resource Count” to “Resource Descriptor Count”)

The Mandatory bit set to 1 indicates that one of the Resource IEs that follow the RDIE needs to be allocatable/allocated for the resource request to be considered successful. If the Mandatory bit is set to 0, and none of the Resource IEs is allocatable/allocated, the AP continues processing the remainder of the RIC.

The Confirm bit is used only in response messages to indicate success or failure of the allocation. It is ignored in request messages, and shall be set to zero.

The Resource Descriptor Count indicates the number of alternative Resource Descriptors that follow this RDIE.

In clause 8A.7, change “Resource inforation element” to “Resource Descriptor” throughout.