IEEE P802.11
Wireless LANs

| Normative Text for DLS Teardown Proposal |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Date: 2006-05-17 |  |  |  |  |  |
| Author(s): |  |  |  |  |  |
| Name | Company | Address | Phone |  | email |
| Shlomo Ovadia | Intel Corp | 2200 Mission College Blvd, Santa Clara, CA 95054 | 1-408-765-1844 | Shlomo.ovadia@intel.com |  |
| Adrian P. Stephens | Intel Corp | 15 JJ Thompson Avenue, Cambridge, CB3 0FD, United Kingdom | +44 1223 763457 | Adrian.p.stephens@intel.com |  |
| Boris Ginzburg | Intel Corp | P. O. Box 1659, Matam Industrial Park, Haifa 31015 ISRAEL | 972-4656739 | Boris.ginzburg@intel.com |  |
| Simon Barber | Devicescape | 900 Cherry Ave., San Bruno, CA 94066 | 1-650-829-2618 | Simon@devicescape.com |  |
| Kevin Hayes | Atheros Communications | 5480 Great America Parkway, Santa Clara, CA 95054 | 1-408-773-5275 | Kevin@athros.com |  |

Abstract

This submission proposes normative text suitable for incorporation into 802.11REVma. This submission addresses DLS teardown conditions that are not included in IEEE 802.11ma/D6.0 Standard draft.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

11.7.3 DLS Teardown

The DLS teardown procedure is divided into QSTA-initiated and QAP-initiated DLS teardown. The QSTA-initiated DLS teardown message flow is illustrated in Figure 205.

- 11.7.3.1 QSTA-initiated DLS Teardown procedure Teardown procedure at the QSTA

Upon receipt of MLME-DLSTeardown.request primitive from the SME, the QSTA shall initiate the teardown of the direct link by sending the DLS Teardown frame to the QAP. The applicable values of ReasonCode and their encoding to the Reason Code field (see 7.3.1.7) values are defined in Table 81.

If the transmission of the frame is successful, it shall issue an MLME-DLSTeardown.confirm primitive with

a result code of SUCCESS. If the frame could not be transmitted, it shall issue an MLME-DLSTeardown.confirm primitive with a result code of FAILURE.

Upon receipt of the DLS Teardown frame (from the QAP), the QSTA shall issue an MLME-DLSTeardown.indication primitive to the SME and shall delete the QSTA from the list of the QSTAs with which direct link has been established.

NOTEthe DLS teardown procedure applies to a specific DLS session as each QSTA may have multiple simultaneous DLS sessions with other QSTAs.

Prior to disassociation/deauthentication from the QAP, QSTAs shall initiate the teardown of all direct links they have by sending DLS Teardown frames to the QAP. If the DLS Teardown frame Max Retry Limit was reached with no response from the QAP the QSTA shall send DLS Teardown frames to its peer DLS QSTAs with ReasonCode PEER_INITIATED.

On receipt of a DLS Teardown frame from a peer QSTA, a QSTA shall send a DLS Teardown frame to the QAP with ReasonCode set to PEER_INITIATED.

A QAP that receives a DLS Teardown frame with ReasonCode set to PEER_INITIATED should send a DLS Teardown frame to any other QSTAs that have a DLS link established with QSTA1.

NOTEThe failed QSTA may re-establish its DLS link according to Section 11.7.

- Table 81 Encoding of ReasonCode to reason Code field value for DLS Teardown

| ReasonCode | Reason Code Field | Applied to |
| --- | --- | --- |
| QSTA_LEAVING | 36 | Non-AP QSTA |
| END_DLS | 37 | Non-AP QSTA |
| UNKNOWN_DLS | 38 | Non-AP QSTA |
| TIMEOUT | 39 | Non-AP QSTA |
| STAKEY_MISMATCH | 45 | Non-AP QSTA |
| PEER_INITIATED | 46 | AP and Non-AP QSTA |
| AP_INITIATED | 47 | Non-AP QSTA |

11.7.3.2 Teardown procedure at the QAP

Upon receipt of the DLS Teardown frame from a QSTA, the QAP shall send a DLS Teardown frame to the destination QSTA.

Upon receipt of MLME-DLSTeardown.request primitive from the SME, the QAP shall announce the tearing down of the direct link by sending the DLS Teardown frame to the two QSTAs using the direct link. The only applicable value of the ReasonCode is STAKEY_MISMATCH and its encoding to the Reason Code field value is defined in Table 81.

If the transmission of the frame is successful, it shall issue an MLME-DLSTeardown.confirm primitive with a result code of SUCCESS. If the frame could not be transmitted, it shall issue an MLME-DLSTeardown.confirm primitive with a result code of FAILURE.

- 11.7.3.3 QAP-initiated DLS Teardown Procedure

The QAP-initiated DLS teardown procedure may be used in cases where the QSTA is unable to initiate the DLS teardown.  The QAP should initiate a DLS teardown procedure when the QAP detects that either end of a DLS link has left the BSS without a teardown of the DLS link.

NOTEA QSTA is considered to have left the BSS upon:

- Receipt of a deauthentication frame,

- Receipt of a deassociation frame, or

- Detected loss of connectivity.

If there are one or more QSTAs with open DLS connections with the QSTA being removed, the QAP shall send a DLS Teardown frame to each such QSTA with ReasonCode AP_INITIATED.

NOTEThe QAP may also initiate DLS teardown for implementation-dependent reason.