IEEE P802.11
Wireless LANs

| Key lifetime and Reassociation Deadline Timeouts |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-05-03 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bill Marshall | TGr Editor | 180 Park Ave, Florham Park, NJ 07932 | 973-360-8718 | wtm@research.att.com |

Abstract

Decision was made at the TGr ad-hoc meeting in April to remove the KeyLifetime and ReassociationDeadline from msg#2 of the FT 4-way handshake, and instead include them in msg#3 of the FT Initial Association 4-way handshake.  This removes the problem of them not being covered by a MIC in an RSN.  This submission provides the text changes to D2.0 to implement this change.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Decision was made at the TGr ad-hoc meeting in April to remove the KeyLifetime and ReassociationDeadline from msg#2 of the FT 4-way handshake, and instead include them in msg#3 of the FT Initial Association 4-way handshake.  This removes the problem of them not being covered by a MIC in an RSN.  The Reassociation Deadline is being kept in the Reservation exchange, so that the target AP can provide a tighter deadline if resources are reserved. This submission provides the text changes to D2.0 to implement this change.

Delete “Timeout Interval (Key Lifetime)” from Table 11 – Association response

Delete “Timeout Interval (Key Lifetime)” from Table 13 – Reassociation response

Delete “Timeout Interval (Key Lifetime)” from Table 16 – Authentiation frame body

Delete “Timeut Interval (Key Lifetime)… and Timeout Interval(Reassociation Deadline) from entry 2 of Table 17 – Presence of information elements

Delete “Timeout Interval (Key Lifetime)” from entry 4 of Table 17 – Presence of information elements

Change “Timeout Interval (Reassociation Deadline)” in entry 4 as follows:

RIC may be present. Timeout Interval (Reassociation Deadline) may be present if a RIC is present.

Delete “Timeout Interval (Reassociation Deadline)” from Table 57D – Fast BSS Transition Response action frame body

Delete “Timeout Interval (Key Lifetime)” from Table 57D – Fast BSS Transition Response action frame body

Change “Timeout Interval (Reassociation Deadline)” in Table 57F – Fast BSS Transition Acknowledgement action frame body, as follows:

A Timeout Interval information element containing the Reassociation deadline interval may be present if resources were rquested in the Fast BSS Transition Action Confirm action frame.

Delete “Timeout Interval (Key Lifetime): from Table 57F – Fast BSS Transition Acknowledgement action frame body

Delete the paragraph in 8.5A.1 that starts “The remaining lifetime of the PTK is indicated to the STA by the TIE…”

Change Figures 154B, 154C, 154F, and 154G to remove “TIE(KeyLifetime)” everywhere, and remove “TIE(ReassociationDeadline)” from msg#2.

Change the contents of msg#3 of the 4-way handshake in 8A.2 in Figure 154J to include “TIE(ReassociationDeadline)”

Change the contents of msg#3 in the text of 8A.2 to include “TIE(ReassociationDeadline)”

Change the dash list item describing msg#3 as follows:

Message 3: the AP shall include the PMKR1Name in the PMKID field of the RSNIEAP. The AP shall also include the FTIEAP, MDIEAP, the Reassociation Deadline timeout in the TIE(ReassociationDeadline), and PTK key lifetime in the TIE[KeyLifetime]. The FTIEAP, and MDIEAP shall be the same as in the (re)association response. The PMKR1Name shall be as calculated by the R1KH according to the procedures of 8.5A.5; in order for future Fast BSS Transitions to be successful it needs to be the same as the PMKR1Name in Message #2.

Insert after the dash list describing msg#2 and msg#3:

NOTE – It is assumed by this standard that the Reassociation Deadline will be administered consistently across the Mobility Domain.

Change the first paragraph of 8A.3.3 as follows:

If the STA does not send a (re)association request to the target AP within the Reassociation Deadline interval received during the FT Initial Association or during the reservation mechanism, the target AP may delete the PTKSA and the STA shall abandon this transition attempt.

Change the first paragraph of 8A.3.6 as follows:

If a Timeout Interval information element with a Reassociation Deadline was included in the response received during a reservation, and the STA does not send a (re)association request to the target AP within that interval, the STA shall abandon this transition attempt.

Change the entry in Table 62A for “Timeout Interval (Reassociation Deadline)” as follows:

May optionally appear in the fourth message of the sequence.

Delete the entry in Table 62A for “Timeout Interval (Key Lifetime”)

Delete two paragraphs from 8A.5.2 that start “A Timeout Interval information element…”

In 8A.5.4, in the paragraph starting “A Timeout Interval information element may appear” delete the phrase in parentheses (“if resources are being requested than a Timeout Interval information element shall appear”), and delete the second sentence in the second dash list item (“If this is the first reassociation deadline being sent…”).

Delete the paragraph in 8A.5.4 starting “A Timeout Interval information element of type KeyLifetime”.

Change the nineth paragraph of 8A.7.2.1 (starting “A response to a successful resource request…”) from “contains the reassociation deadline” to “may contain a reassociation deadline”.

Change the eighth paragraph of 8A.7.2.2 (starting “If the request is other than a Reassociation Request…”) from “shall include a reassociation deadline” to “may include a reassociation deadline”.

Change the MIB entry dot11FTReassociationDeadline to have a range of 1000..65535, and a default value of 1000.  Add to the description “It is assumed that this value will be administered consistently across the Mobility Domain.”