IEEE P802.11
Wireless LANs

| Clause 5,6,7 Edits |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-05-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Dorothy Stanley | Aruba Networks | 1322 Crossman Ave Sunnyvale, CA 94089 | +1-630-363-1389 | dstanley@arubanetworks.com |
| Kapil Sood | Intel Corporation | 2111 NE 25th Ave JF3-206 Hillsboro OR 97124 | +1-503-264-3759 | kapil.sood@intel.com |

Abstract

This document contains proposed changes to the internal TGw draft, addressing comments from the internal TG review.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

5.2.3.2

Change the inserted list item from

- Protection mechanisms for unicast and broadcast management frames

To

- Protection mechanisms for robust unicast and broadcast management frames

5.4.3.5 Data origin authenticity

Change the text of Clause 5.4.3.5 as follows:

The data origin authenticity mechanism defines a means by which a STA that receives a data or robust management frame can determine which STA transmitted the MAC protocol data unit (MPDU) or MAC management protocol data unit (MMPDU). This feature is required in an RSNA to prevent one STA from masquerading as a different STA. This mechanism is provided for STAs that use CCMP or TKIP.

Data origin authenticity is only applicable to unicast data frames or unicast robust management frames. The protocols do not guarantee data origin authenticity for broadcast/multicast data frames, as this cannot be accomplished using symmetric keys and public key methods are too computationally expensive.

5.4.3.6 Replay detection

Change the text of Clause 5.4.3.6 as follows:

The replay detection mechanism defines a means by which a STA that receives a data or robust management  frame from another STA can detect whether the received frame is an unauthorized retransmission. This mechanism is provided for STAs that use CCMP or TKIP.

5.4.3.7 Robust Management Frame Protection

Management frame protection is required in an RSNA to protect against forgery, eavesdropping, and unauthorized disclosure attacks on robust unicast management frames, and to protect against forgery attacks on robust broadcast management frames.

Management frame protection uniformly extends the negotiated RSNA data frame protection protocol (CCMP or TKIP) to provide data confidentiality, replay protection, and data origin authenticity for unicast robust management frames, including action frames, disassociate and deauthenticate frames.

Forgery protection for robust broadcast management action frames is provided through the Broadcast Integrity Protocol (BIP), using AES-128-CMAC for message integrity. For robust broadcast management action frames the protocol provides replay protection, and specifies a mechanism for providing protection against forgery by an authenticated station (insider attacks). Robust broadcast disassociate and deauthenticate management frames are protected against insider attacks.

Management frame protection protocols apply to robust management frames after the RSNA PTK key establishment for protection of unicast frames is completed and after the GTKs  to protect  broadcast frames have been delivered. All management frames sent or received by a STA before keys are derived are unprotected.

5.8.6 Protection of robust broadcast management action frames

If protected management frame support has been negotiated then all robust broadcast management action frames that are transmitted are protected by BIP.

The AP may be configured to convert robust broadcast management action frames to unicast frames using the MUP mechanism. When MUP is used, Robust management action frames are never transmitted as broadcast frames. MUP is selected by the MIB variable:

Dot11RSNAProtectedManagementBroadcastPolicy.

When protected management frame support has been negotiated all robust broadcast management action frames are submitted for encapsulation to the action frame broadcast protection service as described in Clause 11.7. This service either converts the frame to unicast using the MUP mechanism or protects the frame using BIP

6.1.2 Security services

Change the text of Clause 6.1.2 as follows:

Security services in IEEE 802.11 are provided by the authentication service and by WEP, TKIP, and CCMP, and BIP.. The scope of the security services provided is limited to station-to-station data exchange and robust management frame exchanges. The data confidentiality service offered by an IEEE 802.11 WEP, TKIP, and CCMP implementation is the protection of the MSDU or MMPDU. For the purposes of this standard, WEP, TKIP, and CCMP, and BIP are viewed as logical services located within the MAC sublayer as shown in the reference model, Figure 11 (in 5.9). Actual implementations of the WEP, TKIP, and CCMP, and BIP services are transparent to the LLC and other layers above the MAC sublayer.

The security services provided by WEP, TKIP, and CCMP, and BIP in IEEE 802.11 are as follows:

- a) Data Confidentiality;

- b) Robust unicast  management frame confidentiality

- c) Authentication; and

- d) Access control in conjunction with layer management.

- e) Robust broadcast management frame integrity

During the authentication exchange, both parties exchange authentication information as described in Clause 8.

The MAC sublayer security services provided by WEP, TKIP, and CCMP, and BIP rely on information from nonlayer-2 management or system entities. Management entities communicate information to WEP through a set of MIB attributes. Management entities communicate information to TKIP and CCMP and BIP through a set of MAC sublayer management entity (MLME) interfaces and MIB attributes; in particular, the decision tree for TKIP and CCMP TKIP, CCMP and BIP defined in Clause 8.7 is driven by MIB attributes.

7.3.2.25 RSN Information element

Insert the following text the end of Clause 7.3.2.25:

802.1X authentication, CCMP pairwise and group key cipher suites (WEP-40, WEP-104, and TKIP not allowed), robust management frame protection allowed with AES-128-CMAC as the broadcast management suite selector.

30, // information element id, 48 expressed as Hex value

14, // length in octets, 20 expressed as Hex value

01 00, // Version 1

00 0F AC 04, // CCMP as the data group key cipher suite

01 00, // pairwise key cipher suite count

00 0F AC 04, // CCMP as pairwise key cipher suite

01 00, // authentication count

00 00 00 01 // 802.1X authentication

03 40 // Management frame protection is enabled and enforced

00 00 // No PMKIDs

00 0F AC 06, // AES-128-CMAC as the broadcast management cipher suite

References: