IEEE P802.11
Wireless LANs

| Interference Diagnostics |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-05-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jari Jokela | Nokia |  | +358504860445 | jari.jokela @nokia.com |
| Paivi Ruuska | Nokia |  | +358504835433 | paivi.m.ruuska @nokia.com |
| Mika Kasslin | Nokia |  |  | Mika.kasslin@nokia.com |

Abstract

This proposal is based on part 2 of 05/1076r0 – Triggered Measurements as a framework for 11v Diagnostic Alerts (Black et al, November 2005) and 06/513r0 Interference Diagnostics (Black et al, March 2006)

This proposal is simplified version of 06/513r0. It covers only co-located radio interference case and measurements in non-serving channels are not supported.

This proposal is compatible with the D0.02 draft.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Change 7.3.1.23 as follows:

#### 7.3.1.23 Transceiver Noise Floor field

The Transceiver Noise Floor field is a signed integer and is 1 octet in length. It shall indicate the noise floor of the receiver used by the STA transmitting the measurement pilot frame in which it appears in units of dBm with a tolerance of +/-5dB. The transceiver noise floor is referenced to the connector of the antenna transmitting the frame that contains the Transceiver Noise Floor field. If the frame is being sent on multiple antennas then the Transceiver Noise Floor value is the lowest value from all of the transmitting antennas. The Transceiver Noise Floor field is illustrated in Figure 54F.

Change Table 29 and the immediately following paragraph in 7.3.2.21 as follows:

#### 7.3.2.21 Measurement Request element

Table 29—Measurement Type definitions for measurement requests

| Name | Measurement Type | Measurement Use |
| --- | --- | --- |
| Basic Request | 0 | Spectrum Management |
| Clear Channel Assessment (CCA) request | 1 |  |
| Receive power indication (RPI) histogram request | 2 |  |
| Channel load request | 3 | Radio Resource Measurement |
| Noise histogram request | 4 |  |
| Beacon request | 5 |  |
| Frame request | 6 |  |
| STA statistics request | 7 |  |
| LCI request | 8 |  |
| QoS metrics request | 9 | Radio Resource Measurement & Wireless Network Management |
| Multicast Diagnostics | - 10 | - Wireless Network Management |
| Interference Diagnostics Request | 11 | Wireless Network Management |
| Reserved | 12-254 | N/A |
| Measurement Pause request | 255 | Radio Resource Measurement |

When the Enable bit is set to 0 the Measurement Request field contains the specification of the measurement request corresponding to the Measurement Type as described in 7.3.2.21.1 through 7.3.2.21.117.3.2.21.12. When the Enable bit is set to 1, the Measurement Request field is only present when requesting a triggered QoS Metrics, or Interference Diagnostics measurement.

Insert the following clause after 7.3.2.21.11

#### 7.3.2.21.12 Interference Diagnostics Request

The Measurement Request field corresponding to an Interference Diagnostics Request is shown in Figure v1. The response to an Interference Diagnostics Request is an Interference Diagnostics Report.

|  | Randomization Interval | Interference Triggered Reporting (optional) |
| --- | --- | --- |
| Octets: | 1 | 4 |

Figure v1—Measurement Request field format for a 
Interference Diagnostics Request

Randomization Interval is not used and shall be set to 0 when requesting triggered Interference diagnostics.

The Interference Triggered Reporting field is used to specify trigger conditions for management diagnostic reporting. This field shall be omitted in case of requested measurement. The Interference Triggered Reporting field is as shown in Figure v2.

|  | Interference Trigger Condition | Continuous Interference Threshold | Periodic Interference Threshold | Trigger Timeout |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 |

Figure v2—Interference Triggered Reporting field

Interference Trigger Condition is a bit-field that specifies reporting triggers for interference diagnostic reporting. The format of Interference Trigger Condition is shown in Figure v3.

|  | B0 | B1 | B2 | B7 |
| --- | --- | --- | --- | --- |
|  | Continuous | Periodic | Reserved |  |
| Bits: | 1 | 1 | 6 |  |

Figure v3—Interference Trigger Condition field

- Continuous is set to 1 to request that an Interference Diagnostics Report be generated when continuous interference is present that exceeds the value in Continuous Interference Threshold.

- Periodic is set to 1 to request that an Interference Diagnostics Report be generated when interference is present that is periodic in nature with a peak level exceeding the value in Periodic Interference Threshold.

The Continuous Interference Threshold field contains a value in dBm representing the interference level to be used as the threshold value for the continuous trigger condition. This field shall be set to zeros if Continuous field in Interference Trigger Condition field is set to 0.

The Periodic Interference Threshold field contains a value in dBm representing the interference level to be used as the threshold value for the periodic trigger condition. This field shall be set to zeros if Periodic field in Interference Trigger Condition field is set to 0.

The Trigger Timeout field contains a value in units of 100TU during which a measuring STA shall not generate further triggered interference reports after a trigger condition has been met. See 11.11.8.

Change Table 30 and the immediately preceding paragraph in 7.3.2.22 as follows:

#### 7.3.2.22 Measurement Report element

The Measurement Report field shall not be present when the Late bit is set to 1, the Incapable bit is set to 1 or the Refused bit is set to 1. Otherwise, it shall contain a measurement report, as described in 7.3.2.22.1 through 7.3.2.22.107.3.2.22.11.

Table 30—Measurement Type definitions for measurement reports

| Name | Measurement Type | Measurement Use |
| --- | --- | --- |
| Basic report | 0 | Spectrum Management |
| Clear Channel Assessment (CCA) report | 1 |  |
| Receive power indication (CNPI) histogram report | 2 |  |
| Channel load report | 3 | Radio Resource Measurement |
| Noise histogram report | 4 |  |
| Beacon report | 5 |  |
| Frame report | 6 |  |
| STA statistics report | 7 |  |
| LCI report | 8 |  |
| QoS metrics report | 9 | Radio Resource Measurement & Wireless Network Management |
| Multicast Diagnostic | 10 | Wireless Network Management |
| Interference Diagnostics Report | 11 | Wireless Network Management |
| Reserved | 310-255 | N/A |

Insert the following clause after 7.3.2.22.11

#### 7.3.2.22.12 Interference Diagnostics Report

The format of the Measurement Report field of an Interference Diagnostics report is shown in Figure v4.

|  | Interference Reporting Reason | Transceiver Noise Floor | Interference Characteristic |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 or 9 |

Figure v4—Measurement Report field format for an 
Interference Diagnostics Report

The Interference Reporting Reason field is a bit field indicating the reason that the measuring STA sent the Interference Diagnostics report. The Interference Reporting Reason field is shown in Figure v5.

|  | B0 | B1 | B2 | B7 |
| --- | --- | --- | --- | --- |
|  | Continuous Trigger | Periodic Trigger | Reserved |  |
| Bits: | 1 | 1 | 6 |  |

Figure v5—Interference Reporting Reason field

- The Continuous Trigger bit set to 1 indicates that the Interference Diagnostics Report was generated as a triggered report due to the Level trigger.

- The Periodic Trigger bit set to 1 indicates that Interference Diagnostics Report was generated as a triggered report due to the Periodic trigger.

In a requested Interference Report, all bit fields in the Reporting Reason field are set to 0. More than one bit field in the Interference Reporting Reason field may be set if more than one trigger condition was met.

The Transceiver Noise floor field is as defined in 7.3.1.23.

The Interference Characteristic field enables the STA to report the type, level and timing of the local interference. The Interference Characteristic field is defined in Figure v6.

|  | Interference Level | Interference Interval | Interference Burst Length | Interference Start Time |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 2 | 2 | 4 |

Figure v6—Interference Characteristic field format

The Interference Level field is a signed integer indicating the level of the detected interference in units of dBm. The interference level is referenced to the connector of the currently in-use receiving antenna.

The Interference Interval field is present if the interference is periodic and indicates the interval between two successive periods of interference in microseconds. The Interference Interval field shall be present if Periodic Trigger bit is set to 1.

The Interference Burst Length field is present if the interference is periodic and indicates the duration of each period of interference in microseconds. The Interference Burst Length field shall be present if Periodic Trigger bit is set to 1.

The Interference Start Time field is present if the interference is periodic and contains the least significant 4 bytes of the TSF timer at the start of the next interference burst. The Interference Start Time field shall be present if Periodic Trigger bit is set to 1.

If the reported interference is periodic in nature but complete timing information is not available a reporting STA may set Interference Interval, Interference Burst Length or Interference Start Time to 0 to indicate an unknown parameter.

Change 7.3.2.35 as follows

### 7.3.2.35 Wireless Network Management Capability Information element

The Wireless Network Management Capability Information element contains information about the wireless network management capabilities of a STA as shown in Figure v1.

|  | Element ID | Length | Wireless Network Management Capabilities |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 |

Figure v1—Wireless Network Management Capability Information element format

The Element ID field is equal to the Wireless Network Management Capability value in Table 26.

The value of the length field is variable and depends on the length of the Wireless Network Management Capabilities field. The minimum value of the length field is 2.

The Wireless Network Management Capabilities field is a bit field indicating the advertised management capabilities of the STA. The Wireless Network Management Capabilities field is shown in Figure v2.

|  | B0 | B1 | B2 | B3 | B4 | B25 | B15 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Event Log | Diagnostics | Multicast Alert | Presence | Interference Diagnostics | Reserved |  |
| Bits: | 1 | 1 | 1 | 1 | 1 | 121 |  |

Figure v2—Wireless Network Management Capabilities

- The Event Log bit set to 1 indicates the STA supports Event Log diagnostics as described in 11.15.2. The Event Log bit set to 0 indicates that the STA does not support this service.

- The Diagnostics Channel bit set to 1 indicates the STA supports Diagnostics Channel as described in 11.15.3. The Diagnostics Channel bit set to 0 indicates that the STA does not support this service.

- The Multicast Alert bit set to 1 indicates the STA supports Multicast diagnostics as described in 11.15.1.1. The Multicast Alert bit set to 0 indicates that the STA does not support this service.

- The Presence bit set to 1 indicates that the STA supports Presence as described in clause 11.15.4. The Presence bit set to 0 indicates that the STA does not support this service.

- The Interference Diagnostics bit set to 1 indicates the STA supports Interference diagnostics as described in 11.15.1.2. The Interference bit set to 0 indicates that the STA does not support this service.

- All other bits are reserved and shall be set to 0 on transmission and ignored on reception

The lack of a Wireless Network Management Capability element shall be interpreted as the STA having no advertised Wireless Network Management Capabilities.

Insert 11.15.1.2 as follows:

### 11.15.1.2 Interference Diagnostics Reporting

Interference diagnostics reporting allows an AP to receive information concerning the interference being experienced by an associated STA on the serving channel. Such interference may be due to an interaction between radios where a STA is co-located with another radio device. Interference diagnostic information can be used by the AP to manage communication to that STA such that the effect of the interference is limited.

A non-AP STA that supports interference diagnostics reporting shall set the Interference Diagnostics capability bit in the Wireless Network Management Capabilities element to 1.

An AP may request an Interference Diagnostic Report from an associated non-AP STA by sending an Interference Diagnostics Request without trigger conditions. The non-AP STA, if it accepts the request, shall return an Interference Diagnostics Report. Alternatively, an AP may request that triggered interference diagnostic reporting is enabled at associated non-AP STAs that have indicated interference diagnostics capability. To enable STA interference diagnostic reporting, the AP shall send a Measurement Request element containing an Interference Diagnostics Request Type and with Enable and Report bits set to 1 within a Measurement Request frame. See 11.11.8. The trigger conditions for diagnostic reporting shall be specified in the request, e.g. the AP may request that a Report be sent when the continuous interference level caused by the co-located radio exceeds a defined threshold.

A non-AP STA accepting a request for triggered interference diagnostic reporting shall send an Interference Diagnostics Report to the requesting STA if one or more of the specified trigger conditions occurs. The measuring non-AP STA shall not send further interference reports until the Trigger Timeout period specific in the request has expired, or revised trigger conditions have been requested.

Interference diagnostics reporting shall be terminated on receipt of a Measurement Request element containing an Interference Diagnostics Request Type and with Enable bit set to 1 and Report bits set to 0 within a Measurement Request frame. Receipt of a Measurement Request element containing an Interference Diagnostics Request Type, with Enable and Report bits set to 1, but with no trigger conditions specified shall also terminate reporting.

A reporting STA may report any interference characteristics that it is able to determine. The method used by a STA to determine the characteristics and level of interference is outside the scope of this standard.

Insert A.4.14 into Annex A as follows:

# Annex A

## A.4 PICS proforma–IEEE Std. 802.11, 1999 Edition

Insert new item RME2.4 after RME2.3 in A4.14:

### A.4.14 Wireless Network Management extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RME2.4 | Interference Diagnostics Reporting | 7.3.2.22.12, 11.15.1.2 | CFv:O | Yes, No, N/A |

References: