IEEE P802.11
Wireless LANs

| WiNOT TGu Proposal for E911 support – Normative text proposal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-05-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Marian Rudolf (Editor) | InterDigital | 1000 Sherbrooke W. Montreal, QC, Canada H3A 3G4 | +1 514 904 6258 | marian.rudolf@interdigital.com |
| Joe Kwak | InterDigital | 482 Degas Bolingbrook, IL 60440 | +1 630 739 4159 | joekwak@sbcglobal.net |
| Eleanor Hepworth | Siemens Roke Manor Research | Roke Manor, Old Salisbury Lane, Romsey, Hampshire, SO51 0ZN, UK | +44 1794 833146 | eleanor.hepworth@roke.co.uk |
| Andrew McDonald | Siemens Roke Manor Research | Roke Manor, Old Salisbury Lane, Romsey, Hampshire, SO51 0ZN, UK | +44 1794 833146 | andrew.mcdonlad@roke.co.uk |
| Srinivas Sreemanthula | Nokia | 6000 Connection Drive, Irving, TX 75039 U.S. | +1 972 8945000 | Srinivas.Sreemanthula@nokia.com |
| Jon Edney | Nokia | Cambridge, UK; | +44 1353648567 | EXT-jon.1.edney@nokia.com |

Abstract

This document contains a normative text proposal in support of requirement R9I1 (Individual Requirements) in 05/822r9. This proposal has been produced by the WiNOT (Wireless NetwOrking Technology) consortium.

R9I1: “Define IEEE 802.11TM functionality which would be required to support an Emergency Call (e.g. E911) service as part of an overall, multi-layer solution. Specifically: Capability Advertisement and Authentication issues”

This text proposal is based on P802.11-REVma-D5.1.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Contents

7. Frame formats	3

7.2 Format of individual frame types	3

7.2.3 Management frames	3

7.2.3.1 Beacon frame format	3

7.2.3.4 Association Request frame format	3

7.2.3.5 Association Response frame format	3

7.2.3.6 Reassociation Request frame format	4

7.2.3.7 Reassociation Response frame format	4

7.2.3.9 Probe Response frame format	4

7.3 Management frame body components	4

7.3.2 Information Elements	5

7.3.2.29 TSPEC element	5

7.3.2.35 Emergency Service Information element	5

11. Layer Management	7

11. MLME	8

11.11 Interworking Procedures	8

11.11.1 Emergency call support	8

Annex A	9

A.4 PICS proforma–IEEE Std. 802.11, 2006 Edition	9

A.4.3 IUT configuration	9

A.4.16 Interworking extensions	9

Annex D	9

# 7. Frame formats

## 7.2 Format of individual frame types

### 7.2.3 Management frames

#### 7.2.3.1 Beacon frame format

Insert a new row into table 8 as shown below:

Table 8—Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 26 | Emergency Service | Emergency Service shall be present if dot11EmergencyServicesEnabled is true. |

#### 7.2.3.4 Association Request frame format

Insert a new row into table 10 as shown below:

Table 10—Association Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 11 | Emergency Service | Emergency Service shall be present if dot11EmergencyServicesEnabled is true. |

#### 7.2.3.5 Association Response frame format

Insert a new row into table 11 as shown below:

Table 11—Association Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 8 | Emergency Service | Emergency Service shall be present if dot11EmergencyServicesEnabled is true. |

#### 7.2.3.6 Reassociation Request frame format

Insert a new row into table 12 as shown below:

Table 12—Reassociation Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 12 | Emergency Service | Emergency Service shall be present if dot11EmergencyServicesEnabled is true. |

#### 7.2.3.7 Reassociation Response frame format

Insert new row into table 13 as follows:

Table 13—Reassociation Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 8 | Emergency Service | Emergency Service shall be present if dot11EmergencyServicesEnabled is true. |

#### 7.2.3.9 Probe Response frame format

Insert a new row before the last line into table 15 and modify the last row as shown below:

Table 15—Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 24 | Emergency Service | Emergency Service shall be present if dot11EmergencyServicesEnabled is true. |
| 25-255 | Requested information elements | Elements requested by the Request information element of the Probe Request frame. |

[Editor’s note: Will also need to incorporate Interworking IE into 11k Neighbor Report action frames]

## 7.3 Management frame body components

### 7.3.2 Information Elements

Insert Element ID x into Table 26 and change the Reserved row accordingly:

Table 26—Element IDs

| Information Element | Element ID |
| --- | --- |
| Emergency Service | 51 |
| Reserved | 52-221 |

#### 7.3.2.29 TSPEC element

Modify Figure 100 in section 7.3.2.29 as shown

|  | B0 | B1 B4 | B5 B6 | B7 B8 | B9 | B10 | B11 B13 | B14 15 | B16 | B17 | B18 B23 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Traffic type | TSID | Direction | Access Policy | Aggregation | APSD | User Priority | TSInfo Ack Policy | Schedule | Emergency service | Reserved |
| Bits | 1 | 4 | 2 | 2 | 1 | 1 | 3 | 2 | 1 | 1 | 6 |

Figure 100—TS Info field

Insert the following new clauses at the end of section 7.3.2.29

- The Emergency service subfield is 1 bit in length and shall be used to indicate access priority in case the initiating STA wishes to setup an emergency call such as represented by applicable regulatory requirements in a regulatory domain.

[Editor’s note: Could consider to add additional IE to ADDTS to indicate that there is an emergency call pending. FFS]

Insert the following new clauses after 7.3.2.34

#### 7.3.2.35 Emergency Service Information element

The Emergency Service Information element contains information about the capabilities of a STA to support emergency-type sessions as shown in Figure u1.

|  | Element ID | Length | Emergency Service field | BSSID | SSPN |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 6 | TBD |

Figure u1—Emergency Service Information element format

The Length field is variable. The minimum value of the Length field is 1.

The Emergency Service field is a bit field indicating the advertised capabilities and requests of the STA. The Emergency Service field is shown in Figure u2.

|  | B0 | B1 | B2 | B15 |
| --- | --- | --- | --- | --- |
|  | Emergency call supported | Emergency call requested | Reserved |  |
| Bits: | 1 | 1 | 14 |  |

Figure u2—Emergency Service field

- The Emergency call support bit set to 1 indicates the STA supports emergency call support functionality such as represented by applicable regulatory requirements in a regulatory domain. The Emergency call support bit set to 0 indicates that the STA does not support this service.

- The Emergency call requested bit set to 1 indicates the STA requests an emergency call type association to establish a session with the DS. The Emergency call requested bit shall be set to 0 otherwise.

- All other bits are reserved and shall be set to 0 on transmission and ignored on reception

The optional BSSID field may be advertised in case a dedicated BSS is set up to support emergency-type sessions.

The optional SSPN field may be advertised to facilitate emergency-type session establishment.

Both the BSSID field and the SSPN field shall not be included by non-AP STAs into the Emergency Service Information Element.

[Editor’s note: may sort out terminology to avoid confusion with TSPEC field, maybe consider to split the IE for STA and AP]

# 11. Layer Management

[Editor’s note: some MLME primitives may need update to indicate emergency-type support in reported BSS, also authentication indications need to be modified]

# 11. MLME

Insert the following new clauses after section 11.10

## 11.11 Interworking Procedures

This clause describes the actions and the procedures to enable seamless interworking of a multi-access capable STA with other non-802.11 access technologies or when other non-802.11 specific network protocols are involved.

### 11.11.1 Emergency call support

Support for emergency calls such as represented by applicable regulatory requirements in a regulatory domain is provided through capability advertisements in the BSS and non-AP STAs.

It is assumed that a high layer standardized protocol or protocol suite to establish an emergency call or using any other emergency services is implemented on the STA and part of the capability negotiation process when such a call is set-up. In particular, capability exchanges pertaining to this higher layer functionality such as implemented voice codecs are assumed to be negotiated through appropriate higher layer signalling.

Routing and Access Control Procedures in support of emergency calls are out-of-scope for this amendment.

If needed, several approaches could be used in the DS to separate the backhaul of emergency services traffic from other traffic. For example, this may be realized in the form of a separate physical link, a dedicated VLAN, a tunnelling protocol or other methods.

It is assumed that proper admission control at setup and ensuring call priority during an ongoing emergency calls can be realized through a combination of QoS support features or by other methods. For example, maintenance of existing sessions either by the initiating or other STAs in the BSS or maintenance of associations in a BSS may not be required under all circumstances and could be given less priority. Another possible approach left to the discretion of the BSS implementation is the use of a Virtual BSS exclusively reserved for the purpose to set-up and maintain call priority for emergency calls.

The presence of 802.11 emergency call support in the BSS shall be advertised to interworking capable STAs by signalling this capability through the corresponding Emergency Service Information Element in Beacon, Probe Response, Association Response and Reassociation Response frames when dot11EmergencyServicesEnabled is set to true. Optionally, the Emergency Service Information Elements in these frames may contain an advertised BSSID to be used exclusively for the purpose of carrying emergency-type traffic.

In the case when STA credentials are used to request access to DS services for emergency-type calls, the STA may initiate an emergency call by indicating this special case of traffic stream setup by setting the Emergency Service bit in the corresponding TSPEC request. How to determine if a BSS supports access based on the implemented STA credentials is out-of-scope of the present amendment.

In the case when a STA requests un-authenticated access to emergency services in the BSS, the STA shall indicate its intent to establish an emergency-type session by setting the Emergency call requested bit in the Emergency Service Information Element in its Association Request frames. This association shall be admitted without autnetication required when the support of this capability is advertised in the BSS Emergency Service Information Element. When an Emergency Service Information Elements advertised in Beacon, Probe Response, Association Response and Reassociation Response frames contains an advertised BSSID, the STA shall use this advertised BSSID to associate for the purpose of establishing an emergency-type session. When STA is already associated to a BSS when the emergency-type call is requested, it shall disassociate first and re-associate using the Emrgency Service Infformation Elements in its Re-association Request frames. In addition, an explicit emergency call indication can be used any time when allocating traffic streams through ADDTS frame exchanges by the initiating STA.

[Editor’s note: May want to clean- this up a bit, but this is already the idea].

# Annex A

## A.4 PICS proforma–IEEE Std. 802.11, 2006 Edition

### A.4.3 IUT configuration

Append this entry to the end of the IUT configuration table:

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| *CFu | Interworking supported | 11.11 | O | Yes, No, N/A |

Insert this new clause after clause A.4.16:

### A.4.16 Interworking extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
|  | Are the following Interworking capabilities supported? |  |  |  |
| IW1 | Emergency call support | 11.11.1 | CFu:M | Yes, No, N/A |

Add the following MIB attributes to Annex D:

# Annex D

In the dot11StationConfig table of Annex D, insert the following entries at the end of the dot11StationConfigEntry sequence list:

Dot11StationConfigEntry ::=

SEQUENCE {

…

dot11EmergencyServicesEnabled TruthValue }

Insert the following elements at the end of the dot11StationConfigTable element definitions:

dot11EmergencyServicesEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station

implementation is capable of supporting either non-802.11 Network

Access services or non-802.11 specific protocol functionalities in support of seamless interworking.

The default value of this attribute is FALSE."

::= { dot11StationConfigEntry TBD }