IEEE P802.11
Wireless LANs

| Normative Text Proposal for resolving LB83 comments related to Access Category Service Load |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-5-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Ganesh Venkatesan | Intel Corporation | 2111 NE, 25th Ave, Hillsboro, OR 97124. | 503264 0637 | ganesh.venkatesan@intel.com |
| Emily H. Qi | Intel Corporation | 2111 N.E 25th Ave. Hillsboro, OR 97124 | 503-264-7799 | Emily.h.qi@intel.com |

- Abstract

This document is a normative text proposal with resolutions for LB83 comments 78, 79, 81, 142,179, 266, 285, 287, 288, 290, 355, 356, 370, 428, 462, 514, 525, 550, 638, 639, 640, 642, 717 and 718 listed in document 06/584r5.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Abstract	1

7. Frame formats	2

7.1 MAC frame formats	2

7.3.2 Information Elements	2

Change Table 26 in in 802.11k draft 4.0:	2

Delete the following instruction to the editor from 802.11k Draft 4.0:	3

Add the following after 7.3.2.40 in  802.11k Draft 4.0:	3

Annex A	4

A.4 PICS proforma–IEEE Std 802.11, 1999 Edition9	4

A.4.13 Radio Resource Measurement extensions	4

# 7. Frame formats

## 7.1 MAC frame formats

#### 7.2.3.1 Beacon frame format

Addt the following row at the end of Table 8—Beacon frame body in  802.11k Draft 4.0:

| 29 | QBSS Available Admission Capacity | The QBSS AAC information element shall be present only in a QAP and only if dot11RadioMeasurementEnabled is true. |
| --- | --- | --- |

#### 7.2.3.9 Probe Response frame format

Insert the following row at the end of Table 15—Probe Response frame body in  802.11k Draft 4.0:

| 27 | QBSS Available Admission Capacity | The QBSS AAC information element shall be present only in a QAP and only if dot11RadioMeasurementEnabled is true. |
| --- | --- | --- |

Change the following row at the end of Table 15—Probe Response frame body in  802.11k Draft 4.0:

| 24278-n | Requested information elements | Elements requested by the Request information element of the Probe Request frame. |
| --- | --- | --- |

### 7.3.2 Information Elements

- Change Table 26 in in 802.11k draft 4.0:

Table 26—Element IDs

| Information Element | Element ID |
| --- | --- |
| AP Channel Report | 51 |
| Neighbor Report | 52 |
| RCPI | 53 |
| RSNI | 54 |
| BSS Load | 55 |
| Antenna Information | 56 |
| QBSS Available Admission Capacity | 57 |
| Reserved | 5158-220 |

- Delete the following instruction to the editor from 802.11k Draft 4.0:

- Add the following after 7.3.2.40 in  802.11k Draft 4.0:

##### 7.3.2.41 QBSS Available Admission Capacity element

QBSS Available Admission Capacity element contains a list of Available Admission Capacity at different User Priorities  as shown in Figure 112K.

|  | Element ID | Available Admission Capacity Bitmask | Available Admission Capacity List |
| --- | --- | --- | --- |
| Octets: | 1 | 2 | 2 x (total number of non-zero bits in Available Admission Capacity Bitmask) |

Figure 112K — QBSS Available Admission Capacity element Data field format

The value of the QBSS Available Admission Capacity element Length field is set to 2 + 2 x (total number of non-zero bits in Available Admission Capacity Bitmask).

Available Admission Capacity Bitmask field indicates the UP traffics that have Available Admission Capacity specified in the following Available Admission Capacity List. The format of Available Admission Capacity Bitmask is defined in Table 43C.

Table 43C: Available Admission Capacity Bitmask Definition

| Bit (s) | Description |
| --- | --- |
| 0 | UP 0 Traffic |
| 1 | UP 1 Traffic |
| 2 | UP 2 Traffic |
| 3 | UP 3 Traffic |
| 4 | UP 4 Traffic |
| 5 | UP 5 Traffic |
| 6 | UP 6 Traffic |
| 7 | UP 7 Traffic |
| 8-15 | Reserved |

- The bit is set to 1 to indicate that the Available Admission Capacity for the corresponding traffic type is present in the Available Admission Capacity List field. The bit is set to 0 to indicate that the Available Admission Capacity for the corresponding traffic type is not present in the Available Admission Capacity List field.

The Available Admission Capacity List comprises a sequence of Available Admission Capacity fields corresponding respectively to the non-zero bits in the Available Admission Capacity Bitmask field.

The Available Admission Capacity field is 2 octets long and contains an unsigned integer that specifies the remaining amount of medium time available via explicit admission control for the corresponding UP traffic, in units of 32 microsecond periods per 1 second. The field is helpful for roaming non-AP QSTAs to select a QAP that is likely to accept future admission control requests, but it does not represent a guarantee that the HC will admit these requests.

10.3.2.2.2 Semantics of the service primitive

Insert the following row at the end of the BSSDescription table in  802.11k Draft 4.0:

| QBSS Admission Available Capacity | As defined in frame format | As defined in frame format | The values from the QBSS Admission Available Capacity information element if such an element was present in the probe response or beacon, else null. Present only when the MIB attribute dot11RadioMeasurementEnabled is true. |
| --- | --- | --- | --- |

# Annex A

## A.4 PICS proforma–IEEE Std 802.11, 1999 Edition9

### A.4.13 Radio Resource Measurement extensions

Add the following row to the table in 802.11k Draft 4.0:

| RRM22 | QBSS Available Admission Capacity | 7.3.2.41 | (CFk AND CF! AND CF12):M | Yes, No, N/A |
| --- | --- | --- | --- | --- |
| RRM22.1 | Generate and transmit AP Channel Report | 7.2.3.1, 7.2.3.9, 7.3.2.41 | (CFk AND RRM22):M | Yes, No, N/A |
| RRM22.2 | Receive and process AP Channel Report | 7.2.3.1, 7.2.3.9, 7.3.2.41 | (CFk AND RRM22):M | Yes, No, N/A |