IEEE P802.11
Wireless LANs

| LB84-CID-6780-text-proposal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-06-01 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Matthew Fischer | Broadcom | 190 Mathilda Place, Sunnyvale, CA 94086 | 1 408 543 3370 | mfischer@broadcom.com |
| Amit Bansal | Wipro-Newlogic | Electronic City, Bangalore, India | +91 80 30295128 | amit.bansal@wipro.com |
| Yuichi Morioka | Sony Corporation | 2-17-1 Higashigotanda, Shinagawa, Tokyo, Japan | +81 3 6409 3201 | morioka@wcs.sony.co.jp |

Abstract

This document includes proposed text for modification of the TGn 1.0 draft. The text is intended to provide a solution to the issue raised by LB84 Comment ID (CID) 6780 regarding the exact statement of restrictions regarding the use of the L-SIG TXOP function as described in subclause 9.15 of TGn draft 1.0. In addition to CID 6780, the following CIDs should also be resolved with the adoption of this text: 1302, 3838, 3839, 7364, 7335, 5152, 1301.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Replace lines 22-30 in clause 9.15, on page 105 of TGn draft 1.0 with the following lines:

“A STA shall not transmit an L-SIG TXOP Protected frame to a recipient that does not support L-SIG TXOP Protection. When the STA is associated with an access point, then support at a recipient which is associated with the same access point is indicated if the L-SIG TXOP Protection Full Support bit is set to 1 in the Additional HT Information Element broadcast in Beacons transmitted by the access point with which the STAs are associated. L-SIG TXOP support at the recipient may additionally be determined through examination of HT Capability Elements exchanged during association or exchanged during DLS setup."

Replace line 22-31 in clause 9.6, on page 87 of TGn draft 1.0 with the following lines:

“Control frames that initiate a frame exchange shall be transmitted at one of the rates in the BSSBasicRateSet parameter, except in the following cases when other rates may also be used as described below:

- When the transmitting STA’s protection mechanism is enabled, and the control frame is a 	 protection mechanism frame

- When the control frame is a BlockAckReq or BlockAck frame

- When L-SIG TXOP protection is used

- When the control frame is a STBC control frame

In the formerfirst case, the control frame shall be transmitted at a rate according to the separate rules for determining the rates of transmission of protection frames in 9.13. In the lattersecond and third cases, the control frame shall be transmitted in accordance with this subclause. In the last case, the control frame shall be transmitted using the STBC basic MCS."

Insert the following lines at the end of clause 9.6 of TGn draft 1.0:

“When using L-SIG TXOP Protection, a STA shall send control frames using one of the rates in the Basic MCS Set if it receives from the AP with which it is associated an Additional HT Information element with the L-SIG TXOP Protection Full Support bit set to 0.

When using L-SIG TXOP Protection, a STA shall send control frames using any rate acceptable to the intended recipient if it receives from the AP with which it is associated an Additional HT Information element with the L-SIG TXOP Protection Full Support bit set to 1.”

References: