IEEE P802.11
Wireless LANs

| RDIE without count |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-07-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bill Marshall | TGr Editor | 180 Park Ave, Florham Park, NJ 07932 | 973-360-8718 | wtm@research.att.com |
| Kapil Sood | Intel Corp. | 2111 NE 25th Ave JF3-206 Hillsboro OR 97124 | +1-503-264-3759 | Kapil.Sood@intel.com |
| Rajneesh Kumar | Cisco Systems | 170 W Tasman Drive San Jose, Ca 95124 | +1-408-527-6148 | rajneesh@cisco.com |
| Michael Montemurro | Research in Motion | 5090 Commerce Blvd, Mississauga, ON L4W 5W4, Canada | +1-905-629-4746 | mmontemurro@rim.com |

Abstract

Submission 11-06-0650-00-000r-qos-resource-control-flows.ppt proposed several changes to the TGr reservation mechanism.  This submission proposes the text changes to implement the removal of the Resource Descriptor Count field from the RDIE.

Without the Resource Descriptor Count, the STA and AP use the IE Type to determine the number of Resources that are contained in the RDIE – if the next IE in the frame is one of those listed in Table 62C, then another Resource Descriptor follows, else the end of the RDIE has been reached. This is similar to the procedure that the STA and AP use to determine the end of a Resource Descriptor.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

7.3.2.45.2, Figure 113S, change “Resource Descriptor Count” to “Reserved”

Note – if 11-06-0814 is also accepted, then the above instructions are to remove the “Resource Descriptor Count” from Figure 113R.

7.3.2.45.2, delete paragraph starting “The Resource Descriptor Count…”

8A.7.2 Resource Information Container

In the 4th paragraph, remove “or more alternative Resource Descriptors”

Replace the 6th paragraph with the following:

The AP shall attempt to allocate whatever is specified in the Resource Descriptor; if this fails the AP shall begin processing the next RDIE, if one exists.

Delete the paragraph beginning with “An example of a Resource Request consisting of two alternative ….”

Delete Figure 154O

Replace the last sentence of the paragraph beginning with “ For example, when the resource being…” with:

Such an example Resource Request is shown in Figure 154P.

Delete the second sequence of TSPEC-TCLAS-TCLAS-TCLAS Processing in figure 154P.

Delete figure 154R and the paragraph above it.

8A.7.3.1, third paragraph, change as follows:

The STA shall consider the resources that it desires at the next AP. For QoS resources, each traffic stream shall be requested by a separate RDIE and associated TSPEC(s). The RDIE Identifier in the RDIE shall be an arbitrary value chosen by the STA that uniquely identifies the RDIE within the RIC. In the Control Options field, the [0791]Status[#1401, #1412] bit shall be set to zero.

Replace clause8A.7.3.1, fourth paragraph with the following:

Following each RDIE, the STA shall include one Resource Descriptor that defines the resources required for STA after transition. For instance, a STA would construct an RDIE and associated TSPEC IE’s to define a traffic stream required at the BSS-Transition candidate.

Modify the flowchart in figure 154U as follows:

- Change the “select first Resource Descriptor” to “select Resource Descriptor”

- Remove the “More Resource Descriptor” box and associated logical processing for “YES”

8A.7.3.2, fifth paragraph, change as follows:

In response to a request RIC, the AP shall construct a response RIC and include it in the response message. [#716]The response RIC shall comprise one RDIE for each resource that the AP has assigned or attempted to assign. The RDIEs shall be in the same order as in the request and the RDIE Identifier in each RDIE shall be the value of the RDIE Identifier in the corresponding RDIE in the request. The "status[#1401, #1412]" bit in the Control Options in the RDIE shall be set according to the result of the allocation request as follows:

-- Status = '0'[#1401, #1412] indicates that the resources are available. If the response has the status SUCCESS the resources have been [#1413]reserved. The RDIE shall also be followed by the Resource [#492]Descriptor[0560] that was accepted.

-- Status = '1'[#1401, #1412] indicates that the resources could not be allocated. In this case the AP may include a single Resource [#492]Descriptor[0560] following the RDIE indicating a suggested resource that could have been allocated.