IEEE P802.11
Wireless LANs

| LB84-CID-11025-text-proposal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 11-08-2006 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Amit Bansal | Wipro-Newlogic | Electronic City, Bangalore, India | +91 80 30295128 | amit.bansal@wipro.com |
| Matthew Fischer | Broadcom | 190 Mathilda Place, Sunnyvale, CA 94086 | 1 408 543 3370 | mfischer@broadcom.com |

Abstract

This document includes proposed text for modification of the TGn 1.01 (insertions marked) draft. The text is intended to provide a solution to the issue raised by LB84 Comment ID (CID) 11025 regarding the duration field of an A-MPDU. In addition to CID 11025, the following CIDs should also be resolved with the adoption of this text: 1117, 1118, 1119, 1734, 1946.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

TGn Editor: Delete the following text in subclause 7.1.3.2, on page 14, lines 12-13 of TGn Draft 1.01:

Insert the following at the end of the subclause:

The Duration fields in the MAC Headers of MPDUs in an aggregate shall all carry the same value.

TGn Editor: Insert the following text in subclause 7.1.4, on page 19, line 59 of TGn draft 1.01:

Change subclause 7.1.4 as follows:

Within all data frames containing QoS CF-Poll, the Duration/ID field value is set to one of the following:

- One SIFS duration plus the TXOP limit, if the TXOP limit is nonzero, or

- The time required for the transmission of one MPDU of nominal MSDU size, or nominal A-MSDU size if the recipient is a HT device, and the associated ACK frame plus two SIFS intervals, if the TXOP limit is zero.

TGnEditor: Change the following text in subclause 7.1.4, on page 19, line 62 – 65 of TGn Draft 1.01:

Within all data or management frames sent in a CP by the QSTAs outside of a controlled access phase (CAP) and outside of a PSMP sequence, following a contention access of the channel using a given AC, the Duration/ID field is set to one of the following values:

TGn Editor: Delete the following text in subclause 7.1.4, on page 20, lines 18 - 28 of TGn Draft 1.01:

Change  item c) of 7.1.4  as follows:

c) The minimumEither of

1) The time required for the transmission the pending MPDUs of the AC and the associated ACKs, if any, and applicable SIFS durations, and

2) The time limit imposed by the MIB attribute dot11EDCATableTXOPLimit (dot11EDCAQAPTableTXOPLimit for the QAP) for that AC minus the already used time within the TXOP.

TGnEditor: Insert the following text in subclause 7.1.4, on page 20, line 18 of TGn Draft 1.01:

Change subclause 7.1.4 as follows:

a) For management frames, frames with QoS Data subfield set to 0, and unicast data frames with Ack Policy subfield set to Normal Ack,

1) The time required for the transmission of one ACK frameresponse PPDU (ACK or BA, as appropriate, including appropriate IFS values), if the frame is contained in the final PPDUfragment of the TXOP, or

2) The time required for the transmission of one ACK frameresponse PPDU (ACK or BA, as appropriate, including appropriate IFS values) plus the time required for the transmission of the following MPDUPPDU and its response if required (including appropriate IFS values), or

3) Either of, but not exceeding (ii):

i) The time required for the transmission of the pending PPDUs of the given AC and the associated response PPDU (ACKs or BAs, as appropriate), if any, and applicable SIFS durations, or

ii) The time limit imposed by the MIB attribute dot11EDCATableTXOPLimit (dot11EDCAQAP-TableTXOPLimit for the QAP) for the given AC minus the already used time within the TXOP, as measured from the end of the PPDU containing this frame.

b) For unicast data frames with the Ack Policy subfield set to No Ack or Block Ack and for multicast/broadcast frames,

1) Zero, if the frame is contained in the final fragmentPPDU of the TXOP, or

2) The time required for the transmission of the following MPDUPPDU and its response framePPDU, if required (including appropriate IFS values), or

3) Either of, but not exceeding (ii):

i) The time required for the transmission of the pending PPDUs of the given AC and the associated response PPDUs (ACKs or BAs, as appropriate), if any, and applicable SIFS durations, or

ii) The time limit imposed by the MIB attribute dot11EDCATableTXOPLimit (dot11EDCAQAP-TableTXOPLimit for the QAP) for the given AC minus the already used time within the TXOP, as measured from the end of the PPDU containing this frame.

c) The minimum of

1) The time required for the transmission the pending MPDUs of the AC and the associated ACKs, if any, and applicable SIFS durations, and

2) The time limit imposed by the MIB attribute dot11EDCATableTXOPLimit (dot11EDCAQAP-TableTXOPLimit for the QAP) for that AC minus the already used time within the TXOP.

The Duration fields in the MAC Headers of MPDUs within an A-MPDU all carry the same value as calculated based on the rules mentioned in this subclause where the timing reference for the duration information is the end of the A-MPDU, not the ends of the individual MPDUs carried within the A-MPDU.

References:

| CID | Name of Commenter(Ed) | Clause Number(C) | Page(Ed) | Line(Ed) | Clause Title(Ed) | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1117 | Fischer, Matthew | 7.1.3.2 | 15 | 20 | Duration/ID field | The use of the term "shall" has been deemed inappropriate for this clause and the sentence indicated here should really be in 7.1.4, since all QSTA duration value settings have already been redirected away from 7.1.3.2 and into 7.1.4. | Strike the word "shall" from the sentence "The Duration fields in the MAC Headers of MPDUs in an aggregate shall all carry the same value." and move the sentence to be placed at the end of 7.1.4 |
| 1118 | Fischer, Matthew | 7.1.3.2 | 15 | 20 | Duration/ID field | "in an aggregate" is unclear. Should it be "in an A-MPDU"? | clarify and consistant |
| 1119 | Fischer, Matthew | 7.1.3.2 | 15 | 20 | Duration/ID field | "shall all carry the same value" - What value should that be? | clarify |
| 11025 | Marshall, Bill | 7.1.3.2 | 15 | 20 | Duration/ID field | duration value of MPDUs is unclear | specify the value of duration |
| 1734 | Inoue, Yasuhiko | 7.1.4 | 22 | 3 | Duration/ID field in data and management frames | Do we need to change text for HCCA TXOP? |  |
| 1946 | Ji, Lusheng | 7.1.4 | 22 | 2 | Duration/ID field in data and management frames | Changing "minimum" to "either" means the transmitter algorithm needs to be specified. When is the transmitter to use one versus the other? | Make the sub-items 1) and 2) into c) and d) in the main list. Main list is already a choice of the various alternatives. Add some explanation for when each of these choices is to be used. |