IEEE P802.11
Wireless LANs

| LB84 Replacement text for 802.11n Draft 1.0 Clause 20.3.2 PLCP Frame Format |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-06-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bjorn A. Bjerke | Qualcomm, Inc. | 9 Damonmill Sq. Suite 2A, Concord, MA 01742, USA | 781-276-0912 | bbjerke@qualcomm.com |

Abstract

This document offers replacement text for Clause 20.3.2 PLCP Frame Format that addresses LB84 comments CID 4000, 182, and 7382. New text is underlined (Figure n49 is also new).

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

PLCP frame format

(Ed: hanging subclause)

Two new formats are defined for the PLCP (PHY Layer Convergence Protocol):  Mixed Mode mode and Greenfield Green Field. These two formats are called HT formats.  Figure n1 shows the non-HT format and the HT formats.  In addition to the HT formats, there is a non-HT duplicate format (specified in Error: Reference source not found (Ed: Error: Reference source not found )) that duplicates the 20MHz non-HT packet in two 20MHz halves of a 40MHz channel.

Figure n1- PLCP packet format

The elements of the PCLP packet are:

- L-STF: Non-HT Short Training Field

- L-LTF: Non-HT Long Training Field

- L-SIG: Non-HT Signal Field

- HT-SIG: High Throughput Signal Field

- HT-STF: High Throughput Short Training Field

- HT-LTF1:  First High Throughput Long Training Field (Data HT-LTF)

- HT-LTF’s: Additional High Throughput Long Training Fields (Data HT-LTFs and Extension HT-LTFs)

- Data – The data field includes the PSDU (PHY sub-layer Service Data Unit)

(Ed: Consider turning into a table)

The HT-SIG, HT-STF and HT-LTF’s exist only in HT packets.  In non-HT and non-HT duplicate formats only the L-STF, L-LTF, L-SIG and Data fields exist.

In both the Mixed Mode and Green Field frame formats, there are two types of HT-LTFs: Data HT-LTFs (DLTFs) and Extension HT-LTFs (ELTFs).  DLTFs provide the necessary reference for the receiver to form a channel estimate that allows it to demodulate the data portion of the frame.  DLTFs are always included in High Throughput frames, and the number is determined by the number of space time streams being transmitted in the frame (see Error: Reference source not found). ELTFs provide additional reference in sounding PPDUs so that the receiver can form an estimate of additional dimensions of the channel beyond those that are used by the data portion of the frame.  PLCP preambles in High Throughput frames that include both DLTFs and ELTFs are referred to as staggered preambles.

In both Mixed Mode and Greenfield frame formats, there are two types of HT-LTFs: Data HT-LTFs (DLTFs) and Extension HT-LTFs (ELTFs).  DLTFs are always included in High Throughput frames to provide the necessary reference for the receiver to form a channel estimate that allows it to demodulate the data portion of the frame.  The number of DLTFs, NDLTF, may be either 1, 2, or 4, and is determined by the number of space time streams being transmitted in the frame (see Error: Reference source not found).  ELTFs provide additional reference in sounding PPDUs so that the receiver can form an estimate of additional dimensions of the channel beyond those that are used by the data portion of the frame.  The number of ELTFs, NELTF, may be either 0, 1, 2, or 4 (see Error: Reference source not found).  PLCP preambles in which DLTFs are followed by ELTFs are referred to as staggered preambles. The Mixed Mode and Greenfield frame formats shown in Figure n1 both contain staggered preambles for illustrative purposes.