IEEE P802.11
Wireless LANs

| Compressed Steering Matrices Order |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-07-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Assaf Kasher | Intel | POB 1659, Haifa 31015, Israel | 97248651547 | assaf.kasher@intel.com |
| Gal Basson | Intel | POB 1659, Haifa 31015, Israel | 972547885404 | gal.basson@intel.com |
| Joonsuk Kim | Broadcom | 190 Mathilda place, Sunnyvale, CA94086 | 1-408-543-3455 | joonsuk@broadcom.com |
| Vinko Erceg | Broadcom | 15435 Innovation Dr. San Diego, CA 92128 | 1-858-521-5885 | verceg@broadcom.com |

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <Error! Hyperlink reference not valid.>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Abstract

This document contains 802.11n D1.0 submission to solve the following comments: CID 3986

This submission addresses a comment referring to the fact that the order of the steering matrices feedback (by frequency and then by element) is different from the order of matrix sending in other sections (all the elements in one matrix and then the elements of the matrix in the next frequency).

Replace the following paragraphs in page 73, section 7.4.7.8 (Compressed Steering Matrices Feedback frame)

Quantized CSI Feedback Information field is defined as: Data: Channel matrix elements indexed in order by data subcarrier index and angle. This simplifies the interpolation angles between subcarriers, if grouping other than 1 is employed.  The explanation on how these angles are generated from the steering matrix V is given in 20.3.5.2.3

The angles are sent in order according to Table n46.

with

Quantized CSI Feedback Information field is defined as: Data: Channel matrix elements indexed in order by matrix angles and data subcarrier index.  The explanation on how these angles are generated from the steering matrix V is given in 20.3.5.2.3.

The angles in a compressed steering matrix per tone are sent in order according to Table n46.

Replace the two examples in page 75, end of section 7.4.7.8 (Compressed Steering Matrices Feedback frame)

Example:  2x2, 20MHz, no grouping, bits for each  = 3, bits for each =5. Each ij over all tone indices has 168 bits.  Each ij over all tone indices has 280 bits.

| bits | b1..b5 | b6..b10 | … | b276..b280 | b281..b283 | b284..b286 | … | b446..b448 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| data | 11(f-28) | 11(f-27) | … | 11(f28) | 21(f-28) | 21(f-27) | … | 21(f28) |

Example: 4x2, 40MHz, 4 tone grouping, ,  = 4. Each ij over all tone indices has 120 bits.  Each ij over all tone indices has 60 bits.

| bits | b1..b4 | b5..b8 | … | b117..b120 | b121..b124 | … | b361..b362 | … | b661..b664 | … | b899..b900 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| data | 11(f-58) | 11(f-54) | … | 11(f58) | 21(f-58) | … | 21(f-58) | … | 32(f-58) | … | 42(f58) |

with

Example:  2x2, 20MHz, no grouping, bits for each  = 3, bits for each =5. Each matrix is encoded in 8 bits.

| bits | b1..b5 | b6..b8 | b9..b13 | b14..b16 | … | b441..b445 | b446..b448 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| data | 11(f-28) | 21(f--28) | 11(f-27) | 21(f--27) | … | 11(f28) | 21(f28) |

Example: 4x2, 40MHz, 4 tone grouping, ,  = 4. Each matrix is encoded in 30 bits.

| bits | b1..b4 | b5..b8 | … | b27..b28 | b29..b30 | b31..b34 | … | b59..b60 | … | b871..b874 | … | b899..b900 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| data | 11(f-58) | 21(f-58) | … | 32(f-58) | 42(f-58) | 11(f-54) | … | 42(f-54) | … | 11(f58) | … | 42(f58) |