IEEE P802.11
Wireless LANs

| Frequent Handover |  |  |  |
| --- | --- | --- | --- |
| Date: 2006-Jan-3 |  |  |  |
| Author(s): |  |  |  |
| Name | Company | Phone | email |
| Wang Bin | ZTE |  | Wang.binxa@zte.com.cn |
| Li Feng | ZTE |  | Li.feng8@zte.com.cn |

Abstract

This proposal is a mechanism to enable frequent handover detection/avoid/control in a distributed manner across cooperative clients and access points.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Frequent handover Normative Text

- Frame formats

      - Beacon frame format

- Insert the following row at the appropriate location in Table 5:

| 25 | Frequent handover Capability Information | The Frequent handover Capability indicate wheather the AP support Frequent Handover avoid or not. |
| --- | --- | --- |

- Probe Response frame format

- Insert the following row at the appropriate location in Table 12:

| 25 | Frequent handover Capability Information | The Frequent handover Capability indicate wheather the AP support Frequent Handover avoid or not. |
| --- | --- | --- |

7.3.1.11 Action field

Insert the following rows into Table 19a:

| Code | Meaning | See subclause |
| --- | --- | --- |
| 10 | Frequent handover | 7.4.2 |

7.4 Action frame format details

Insert the following subclauses after subclause 7.4.1.

7.4.2 Frequent Handover action frame details

Several Action frame formats are defined for Frequent handover purposes. The Action field values, associated with each frame format within theFrequent Handover Category, are defined in Table 20.9.

Table 20.9—frequent handover action field values

| Action field value | Meaning |
| --- | --- |
| 1 | HO request |
| 2 | HO response |
| 3 | FHAU |
| 4-255 |  |

7.4.2.1 HO request frame format

The HO request frames are used by STA to request a handover to the current AP. When STA want to init a handover and have selected a target AP, then the STA will transmit a HO request frame. The frequent handovcr detection set will be attach to this frame and transmit to the current AP simultaneously.

The frame body of the HO request frame contains the information shown in Table.

| Category | Action | STA Address | Target AP address | Frequent Handover Detection Set |
| --- | --- | --- | --- | --- |
| Octets:1 | 1 | 6 | 6 | variable |

7.4.2.2 HO response frame format

The HO response frames are used to indicate the result to the STA that have sent the HO request.

| Category | Action | STA Address | Target AP address | Status code | Other information elements |  |
| --- | --- | --- | --- | --- | --- | --- |
| Octets:1 | 1 | 6 | 6 | 2 | variable |  |

7.4.2.3 FHAU frame format

The FHAU frames are used to update the frequent handover avoid policies in the STA which have been detected in frequent handover status. Meanwhile, by transport this frame through DS, this action frame can also update the frequent handover avoid policy of the APs in ESS.

The frame body of the FHAU frame contains the information show in table.

| Category | Action | STA Address | Count IE | Frequent Handover Avoid Set |
| --- | --- | --- | --- | --- |
| Octets:1 | 1 | 6 | 1 | variable |

- MLME

- Insert the following subclauses with the tables included therein, renumbering as necessary:

- Basic STA frequent handover

- STAs in an infrastructure network typically arosing frequent handover by

- In a overlapped area where average RSSI of different AP wave up and down;

- In a variety radio circumstance, STA will try to connect the best signal AP;

- Load balancing scheme make the STA to release resources ;

- STA change the QoS requirement;

- Because of transmit power adjust or advanced power control;

- Because of dynamic channel selection, the STA may be isolated and association/reassociation to a new AP;

- The roaming of STA;

- Handle frequent handover in an infrastructure network

In a distributed infrastructure network, FHAAP should monitor and supervise every STA’s handover behavior. When the STA init a HO request (whether fast transition or not), some measurement report will be transmit to the FHAAP. FHAAP memorize the measurement report and perform the frequent handover detection. If FHAAP can confirm the STA has been in frequent handover status, FHAAP will enable or update the FHAP (frequent handover avoid policy) of the STA and the other FHAAP in the same ESS, so as to avoid the STA’s frequent handover.

Optional: the HO request and HO response could be replaced by 11r and 11k frames. After FT request bas been send to the AP, the AP should init a measurement report request to the STA, after the STA report the measurement report, AP would feedback FT response.

Optional:In the FH status, the STA who want to handover will request a FH request to the FHAAP it connected, and the FHAAP will control the request according to the FHAP.

The measurement report that a STA pass to the ESS should contain:

- average RSSI list of candidate APs;

- neighbor report (11k);

- handover threshold status;

- transmit power;

- channel quality;

- the load level of candidate APs;

The FHAP will contain following parameter:

- adaptive handover threshold value;

- frequent handover status;

- transmit power control;

- dynamic channel control;

- load scheme control;

Motion: “The substantive text IEEE 802/11_06_ 1279r0 Load Balancing addresses objectives: load balancing cooperative approach, advertisement, and access point coordination is worthy of inclusion in the base draft”

- Merge=

- No=

- Abstain=