IEEE P802.11
Wireless LANs

| LB84-Terminology-Confusion-text-proposal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-08-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Yuichi Morioka | Sony Corporation | 6-7-35 Kitashinagawa, Shinagawa-ku Tokyo, 1410001 Japan | +81-3-6409-3201 | Yuichi.Morioka@jp.sony.com |
| Tomoya Yamaura | Sony Corporation | 6-7-35 Kitashinagawa, Shinagawa-ku Tokyo, 1410001 Japan | +81-3-6409-3201 | yamauratomoya@yahoo.co.jp |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB84 comments:

- 472 (Reject. Actually comment resolution to reject this was approved by TGn at San Diego)

- 878 (Counter. “MFB Requester/MFB Responder” are proposed.),

- 3992 (Counter, though conceptually accepted. Unified usage of “Beamformer/Beamformee” is proposed.)

- 4776 (Counter. “MFB Requester/MFB Responder” are proposed.)

- 6790 (Counter, though conceptually accepted. “MFB Requester/MFB Responder” are proposed for LA, and “Beamformer/Beamformee” are proposed for TxBF)

regarding inconsistency of the use of terminology in the draft.

The changes marked in this document are based on TGn Draft version D1.02

** Note: corrected the heading in rev.01

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

## Received comments related to terminology confusion in LB84

| CID | Page(Ed) | Line(Ed) | Clause(Ed) | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 472 | 49 |  | 7.3.2.47.6 | "beamformee"? We need a better term here, and in the other 7 places this term is being used. | Find a better term. |
| 878 | 118 | 5 | 9.19.1 | The terms "initiator" and "sender" seem to be used interchangeably in this clause and in 9.19.1 (p 118, line 37). Given that the note on the bottom of page 118 says "bidirectional request/responses are permitted", it would be more clear to consistently use the term "sender" | Replace occurrences of the term "Initiator" with "sender" in both 9.19.1 and 9.19.2 |
| 3992 | 0 |  | General | In various sections in the discussion of beamforming stations the terms transmitter/receiver, requester/responder, beamformer/beamformee, are used. | One term should be used in all cases. Transmitter/Receiver should especially be avoided because they change through the flow. |
| 4776 | 118 | 6 | 9.19.1 | The term 'sender' has been used for the first time. I think this should be 'initiator STA' or another previously used definition | initiator STA?? |
| 6790 | 117 |  | 9.19.1 | Terminology confusion. Initiator and responder would be defined as a relationship of TxOP operation. TxOP holder is originally called as "initiator" and other responding STAs are called as "responder(s)". However, in clauses for FLA, TxBF, "initiator" is used as "initiating STA of FLA (and/or TxBF), and "responder" is used for "responding STA of FLA (and/or TxBF)" sometimes. | Use consistent terminology. |

## Background of proposal in this submission

This submission is aimed to sort out the terminology confusion found in 802.11n D1.02 (ref [1]).

There are instances where terms such as sender/responder, and transmitter/receiver are used without clear definition.  Also, in many occasions, the same terms like initiator/responder are used to explain different entity characteristics.

This document will provide a unified definition to each terminology and will give the editor guideline to use consistent terminology.

## Issues

Here I list the problems with the current use of terminology.

- Sender/Responder

In subclause 9.21 “Link Adaptation”, terms “Sender/Responder” are used to identify entities that send MRQ/MFB.  The term “Sender” is unclear and the term “Responder” is already used in REVma-D7.0 (ref [2]), clause 8.2.2.2 “Open System authentication”, p.164, line48, to define an authentication entity.

Suggest changing “Sender” to “MFB Requester”, “Responder” to “MFB Responder” in subclause 9.19.

- Initiator/Responder

Throughout the draft, terms “Initiator/Responder” are used to identify ownership of the TXOP.  However, as stated above, the term “Responder” is already used in REVma-D7.0 and the term “Initiator” is also used in sections such as subclause 8.1.4 “RSNA PeerKey Support”, p.159, line 52.

Suggest changing “Initiator” to “TXOP Initiator”, “Responder” to “TXOP Responder” when used in relation with the ownership of the TXOP.

- Initiator/Responder 2

In subclause 9.22.2 “Transmit Beamforming with implicit feedback” terms “Initiator/Responder” are also used to identify STA that sends steered PPDU and STA that receives it.  However, as stated above, terms “Initiator/Responder” are already used elsewhere.

Suggest changing “Initiator” to “Beamformer”, “Responder” to “Beamforemee” in subclause 9.22.2.

- Transmitter/Receiver

In subclause 9.22.3 “Explicit feedback beamforming”, terms “Transmitter/Receiver” are used to identify STA that sends steered PPDU and STA that receives it.  However, the STA that sends steered PPDU is also the STA that receives feedback information, and vise versa, creating confusion.

Suggest changing “Transmitter” to “Beamformer”, “Receiver” to “Beamformee” in subclause 9.22.3.

- Beamformer/Beamformee

“Beamformer/Beamformee” are unique terms defined in subclause 3.

"Beamformer" is a STA that transmits a PPDU using a beamforming steering matrix.  "Beamformee" is a STA that receives a PPDU that was transmitted using a beamforming steering matrix.

These definitions would be correct through the required frame exchange and creates no confusion with other terms.

- Originator/Recipient

“Originator/Recipient” are also defined to identify STA that sends BAR and STA that sends BA. So we shall not use these terms for other purpose.

## Proposal

TGn Editor:  remove the following definitions in clause 3, “Definitions”, page5 of P802.11n D1.02 (ref [1]).

3.n27 responder: A STA that transmits a frame in response to a frame received from an initiator during a frame exchange sequence that includes A-MPDUs.

TGn Editor: add the following definitions in clause 3, “Definitions”.

3.x TXOP Initiator : A STA that holds a TXOP and transmits the first frame in a frame exchange sequence including aggregate PPDUs

3.x TXOP Responder : A STA that transmits a frame in response to a frame received from an TXOP initiator during a frame exchange sequence that includes A-MPDUs.

3.x MFB Requester: A STA that transmits a PPDU containing +HTC with MRQ bit set to ‘1’.

3.x MFB Responder: A STA that responds to a PPDU containing +HTC with MRQ bit set to ‘1’ with a PPDU containing +HTC with setting MFB using MCS indices or all “1”

TGn Editor: replace all occurrences of “Initiator” to “TXOP Initiator”, “Responder” to “TXOP Responder” when used in relation with the ownership of the TXOP.

TGn Editor: replace all occurrences of “sender” in clause 9.21 “Link adaptation” and its subclause, to “MFB Requester”

TGn Editor: replace all occurrences of “responder” in clause 9.21 “Link adaptation” and its subclause, to “MFB Responder”

TGn Editor: replace all occurrences of “initiator” in clause 9.22.2 “Transmit beamforming” and its subclause, to “Beamformer”

TGn Editor: replace all occurrences of “responder” in clause 9.22.2 “Transmit beamforming” and its subclause, to “Beamformee”

TGn Editor: replace all occurrences of “transmitter” in clause 9.22.3 “Transmit beamforming” and its subclause, to “Beamformer”

TGn Editor: replace all occurrences of “receiver” in clause 9.22.3 “Transmit beamforming” and its subclause, to “Beamformee”

TGn Editor:  replace all occurrences of “beamformer” with “Beamformer in P802.11n D1.02 (ref [1]).

TGn Editor:  replace all occurrences of “beamformee” with “Beamformee in P802.11n D1.02 (ref [1]).

## References:

[1] IEEE P802.11n-D1.02 (insertions and deletions marked).pdf

[2] IEEE P802.11-REVma-D7.0-redline.pdf