IEEE P802.11
Wireless LANs

| LB84-L-SIGTXOP-Diagram |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-08-28 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Yuichi Morioka | Sony Corporation | 2-17-1 Higashigotanda, Shinagawa, Tokyo, Japan | +81-3-6409-3201 | Yuichi.Morioka@jp.sony.com |
| Huai-Rong Shao | Samsung Electronics | 75 West Plumeria Dr, San Jose, CA 95134, USA | +1-408-544-5552 | Hr.shao@samsung.com |
| Ariton Xhafa | Texas Instruments Inc. | 12500 TI Blvd., Dallas, TX 75243 USA | +1-214-480-6556 | axhafa@ti.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB84 comments:

- CID 870

- CID 6784

This document includes a proposed diagram to be added to the TGn Draft 1.03 [1].

The document is intended to provide a solution to the issue raised by CID 870 regarding the lack of explanatory diagram in subclause 9.13.4.  In addition, the document provides solution to CID 6784, which asks for a detailed explanation on how to set the L-SIG Duration value.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

TGn Editor: Add the following diagram at the beginning of clause 9.13.4.1, line40 of page 93;

Figure a - Basic Concept of L-SIG TXOP Protection

TGn Editor: Add the following diagram at the beginning of clause 9.13.4.2;

Figure b – Example of L-SIG Duration Setting

TGn Editor: Insert the following sentence at the end of line 16 of page 94;

The TXVECTOR L_LENGTH(defined in section 21.2.2.1), when L-SIG TXOP Protection is used, shall contain a value of;

The symbol  denotes the lowest integer greater than or equal to .

is a symbol interval of OFDM, and is defined in 21.3.7.

TGn Editor: Replace line 44 through 46 of page 94 with the following;

Under L-SIG TXOP Protection, the L-SIG Duration of the initial PPDU that establishes protection shall be;

where  is the length in time[usec] of the entire initial PPDU, and  is the length in time[usec] of the expected response PPDU.

is the length in time[usec] of the non-HT PCLP header where

Parameters , , and  are defined in Section 21.3.7.

When the initial PPDU that establishes protection requires no response (e.g. CTS_to_Self), the L-SIG Duration shall contain a value;

TGn Editor: Replace line 6 through 10 of page 95 with the following;

On receiving a PPDU containing an L-SIG Duration, the responder that asserted the L-SIG TXOP Protection support bit upon association, shall generate a response frame with the L-SIG Duration equivalent to;

under L-SIG TXOP Protection.

References:

[1] IEEE P802.11n-D1 03.pdf