IEEE P802.11
Wireless LANs

| Proposal to change tone index for grouping in the explicit feedback |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-08-30 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Joonsuk Kim | Broadcom | 190 Mathilda place Sunnyvale, CA 94086 | 1-408-543-3455 | joonsuk@broadcom.com |
| Gal Basson | Intel | POB 1659, Haifa 31015, Israel | 972547885404 | gal.basson@intel.com |
| Assaf Kasher | Intel | POB 1659, Haifa 31015, Israel | +972-4-8651547 | assaf.kasher@intel.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB84 comments:

- CID 2291, CID 2292, CID 12098, CID 12100, CID 984, CID 985, CID 11460

The changes marked in this document are based on TGn Draft version D1.03

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

## Introduction

There is still an ambiguity in tone index when grouping applies for the explicit feedback. To avoid the confusion, we propose to change the tone index to be symmetric over DC tone.

This document also includes a couple of comments to unify the format over CSI, Non-compressed and Compressed explicit feedback options from 7.4.8.8 to 7.4.8.10.

## CIDs to cover

### CID 984, 985

These comments are accepted unanimously, but text to instruct the editor is missing. This document provides the resolution.

### CID 11460

We propose to reject this comment because the example is already included in 7.4.8.9.

### CID 2291, 2292, 12098, 12100

When grouping applies, the tone indices are different for positive tones and negative tones as follows, which causes confusion.

Subcarriers are sent in order according to the following sequence. For 20 MHz bandwidth operation:

−28;−28+ Ng;−28+2Ng.........−1;1;1+Ng;1+2Ng....28

For 40 MHz bandwidth operation:

−58;−58+ Ng;−58+2Ng.........−2;2;2+ Ng;2+2Ng....58

What we propose is

Subcarriers are sent in order according to the following sequence. For 20 MHz bandwidth operation:

−28;−28+ Ng;−28+2Ng;... ;−28+mNg;−1;1;28-mNg;…; ;28-Ng;28

where m=28−2Ng, i.e., m is the maximum integer not exceeding (28-2)/Ng.

For 40 MHz bandwidth operation:

−58;−58+ Ng;−58+2Ng;... ;−58+nNg;−2;2;58-nNg;…; ;58-Ng;58

where n=58−3Ng, i.e., n is the maximum integer not exceeding (58-3)/Ng.

## Proposal to change

### In 7.4.8.8

TGn Editor:  Add the following text at the line 23 in page 56.

Grouping is a method to group adjacent tones to reduce the network overhead. The tone index to represent the group is defined in Table n35 and in Table n36.

TGn Editor:  Change the Table n35and Table n36 in page 56.

Table 35 – CSI Matrices Report for 20MHz

| Field | Size | Meaning |
| --- | --- | --- |
| SNR in Rx channel 1 | 8 bits | Signal to Noise Ratio in the first Rx chain of the STA sending the report. |
|  |  |  |
| SNR in Rx channel Nr | 8bits | Signal to Noise Ratio in the Nr'th Rx chain of the STA sending the report. |
| CSI Matrix for Carrier -28 | 3+2× Nb×Nc×Nr bits | CSI Matrix |
| CSI Matrix for Carrier -28+Ng | 3+2× Nb×Nc×Nr bits | CSI Matrix |
| … |  |  |
| CSI Matrix for Carrier -28+mNg | 3+2× Nb×Nc×Nr bits | CSI Matrix |
| CSI Matrix for carrier -1 | 3+2× Nb×Nc×Nr bits | CSI Matrix |
| CSI Matrix for carrier 1 | 3+2× Nb×Nc×Nr bits | CSI Matrix |
| CSI Matrix for carrier 1+Ng 28-mNg | 3+2× Nb×Nc×Nr bits | CSI Matrix |
| … |  |  |
| CSI Matrix for Carrier 28-Ng | 3+2× Nb×Nc×Nr bits | CSI Matrix |
| CSI Matrix for carrier 28 | 3+2× Nb×Nc×Nr bits | CSI Matrix |

where m=26Ng, i.e., m is the maximum integer not exceeding 26/Ng.

Table 36 – CSI Matrices Report for 40MHz

| Field | Size | Meaning |
| --- | --- | --- |
| SNR in Rx channel 1 | 8 bits | Signal to Noise Ratio in the first Rx chain of the STA sending the report. |
|  |  |  |
| SNR in Rx channel Nr | 8bits | Signal to Noise Ratio in the Nr'th Rx chain of the STA sending the report. |
| CSI Matrix for Carrier -58 | 3+2× Nb×Nc×Nr bits | CSI Matrix |
| CSI Matrix for Carrier -58 + Ng | 3+2× Nb×Nc×Nr bits | CSI Matrix |
| … |  |  |
| CSI Matrix for Carrier -58 + nNg | 3+2× Nb×Nc×Nr bits | CSI Matrix |
| CSI Matrix for carrier -2 | 3+2× Nb×Nc×Nr bits | CSI Matrix |
| CSI Matrix for carrier 2 | 3+2× Nb×Nc×Nr bits | CSI Matrix |
| CSI Matrix for carrier 2+ Ng 58 - nNg | 3+2× Nb×Nc×Nr bits | CSI Matrix |
| … |  |  |
| CSI Matrix for Carrier 58 - Ng | 3+2× Nb×Nc×Nr bits | CSI Matrix |
| CSI Matrix for carrier 58 | 3+2× Nb×Nc×Nr bits | CSI Matrix |

where n=55Ng, i.e., n is the maximum integer not exceeding 55/Ng.

### In 7.4.8.9

TGn Editor:  Add the following text at the line 44 in page 58.

Grouping is a method to group adjacent tones to reduce the network overhead. The tone index to represent the group is defined in Table n42 and in Table n43.

TGn Editor:  Change the following text in 7.4.8.9 in line 45, page 58.

For example in a 20 MHz channel and Ng=4 steering beamforming matrices are sent on carrier -26, -22, -18, -14, -10, -6, -2, -1, 1, 5, 9, 13, 17, 21, 25, 26.  -28, -24, -20, -16, -12, -8, -4, -1, 1, 4, 8, 12, 16, 20, 24, 28.

TGn Editor:  Change the Table n42and Table n43 in page 59 and page 60.

Table 42 – Non-compressed Beamforming Matrices Report for 20MHz

| Field | Size | Meaning |
| --- | --- | --- |
| SNR in stream 1 | 8 bits | Average Signal to Noise Ratio in the STA sending the report for stream 1 |
|  |  |  |
| SNR in stream Nc | 8bits | Average Signal to Noise Ratio in the STA sending the report for stream Nc. |
| Beamforming Matrix V for Carrier -28 | 2× Nb×Nc×Nr bits | Beamforming Matrix V |
| Beamforming Matrix V for Carrier -28+Ng | 2× Nb×Nc×Nr bits | Beamforming Matrix V |
| … |  |  |
| Beamforming Matrix V for Carrier -28+mNg | 2× Nb×Nc×Nr bits | Beamforming Matrix V |
| Beamforming Matrix V for Carrier -1 | 2× Nb×Nc×Nr bits | Beamforming Matrix V |
| Beamforming Matrix V for Carrier 1 | 2× Nb×Nc×Nr bits | Beamforming Matrix V |
| Beamforming Matrix V for Carrier 1+ Ng 28-mNg | 2× Nb×Nc×Nr bits | Beamforming Matrix V |
| … |  |  |
| Beamforming Matrix V for Carrier 28-Ng | 2× Nb×Nc×Nr bits | Beamforming Matrix V |
| Beamforming Matrix V for Carrier 28 | 2× Nb×Nc×Nr bits | Beamforming Matrix V |

where m=26Ng, i.e., m is the maximum integer not exceeding 26/Ng.

Table 1 – Non-compressed Beamforming Matrices Report for 40MHz

| Field | Size | Meaning |
| --- | --- | --- |
| SNR in stream 1 | 8 bits | Average Signal to Noise Ratio in the STA sending the report for stream 1 |
|  |  |  |
| SNR in stream Nc | 8bits | Average Signal to Noise Ratio in the STA sending the report for stream Nc. |
| Beamforming Matrix V for Carrier -58 | 2× Nb×Nc×Nr bits | Beamforming Matrix V |
| Beamforming Matrix V for Carrier -58+Ng | 2× Nb×Nc×Nr bits | Beamforming Matrix V |
| … |  |  |
| Beamforming Matrix V for Carrier -58+nNg | 2× Nb×Nc×Nr bits | Beamforming Matrix V |
| Beamforming Matrix V for carrier -2 | 2× Nb×Nc×Nr bits | Beamforming Matrix V |
| Beamforming Matrix V for carrier 2 | 2× Nb×Nc×Nr bits | Beamforming Matrix V |
| Beamforming Matrix V for Carrier 2+ Ng 58-nNg | 2× Nb×Nc×Nr bits | Beamforming Matrix V |
| … |  |  |
| Beamforming Matrix V for Carrier 58-Ng | 2× Nb×Nc×Nr bits | Beamforming Matrix V |
| Beamforming Matrix V for carrier 58 | 2× Nb×Nc×Nr bits | Beamforming Matrix V |

where n=55Ng, i.e., n is the maximum integer not exceeding 55/Ng.

### In 7.4.8.10

TGn Editor:  Add the following text at the second column of Table n48 in page 62.

Replace “Number of angles” with “Number of angles (Na)”.

TGn Editor:  Replace the following text from the line 30 to line 40 in page 63.

Subcarriers are sent in order according to the following sequence. For 20 MHz bandwidth operation:

−28;−28+ Ng;−28+2Ng.........−1;1;1+Ng;1+2Ng....28

For 40 MHz bandwidth operation:

−58;−58+ Ng;−58+2Ng.........−2;2;2+ Ng;2+2Ng....58

The Compressed beamforming matrices report for 20MHz has the structure defined in Table n<xx>:

The Compressed beamforming matrices report for 40MHz has the structure defined in Table n<yy>:

Table <xx> – Compressed Beamforming Matrices Report for 20MHz

| Field | Size | Meaning |
| --- | --- | --- |
| SNR in stream 1 | 8 bits | Average Signal to Noise Ratio in the STA sending the report for stream 1 |
|  |  |  |
| SNR in stream Nc | 8bits | Average Signal to Noise Ratio in the STA sending the report for stream Nc. |
| Beamforming Matrix V for Carrier -28 | Na×(b+b )/2 bits | Beamforming Matrix V |
| Beamforming Matrix V for Carrier -28+Ng | Na×(b+b )/2 bits | Beamforming Matrix V |
| … |  |  |
| Beamforming Matrix V for Carrier -28+mNg | Na×(b+b )/2 bits | Beamforming Matrix V |
| Beamforming Matrix V for Carrier -1 | Na×(b+b )/2 bits | Beamforming Matrix V |
| Beamforming Matrix V for Carrier 1 | Na×(b+b )/2 bits | Beamforming Matrix V |
| Beamforming Matrix V for Carrier 28-mNg | Na×(b+b )/2 bits | Beamforming Matrix V |
| … |  |  |
| Beamforming Matrix V for Carrier 28-Ng | Na×(b+b )/2 bits | Beamforming Matrix V |
| Beamforming Matrix V for Carrier 28 | Na×(b+b )/2 bits | Beamforming Matrix V |

where m=26Ng, i.e., m is the maximum integer not exceeding 26/Ng, and Na is the number of angles used for beamforming matrix V (see Table n48).

Table <yy> – Compressed Beamforming Matrices Report for 40MHz

| Field | Size | Meaning |
| --- | --- | --- |
| SNR in stream 1 | 8 bits | Average Signal to Noise Ratio in the STA sending the report for stream 1 |
|  |  |  |
| SNR in stream Nc | 8bits | Average Signal to Noise Ratio in the STA sending the report for stream Nc. |
| Beamforming Matrix V for Carrier -58 | Na×(b+b )/2 bits | Beamforming Matrix V |
| Beamforming Matrix V for Carrier -58+Ng | Na×(b+b )/2 bits | Beamforming Matrix V |
| … |  |  |
| Beamforming Matrix V for Carrier -58+nNg | Na×(b+b )/2 bits | Beamforming Matrix V |
| Beamforming Matrix V for carrier -2 | Na×(b+b )/2 bits | Beamforming Matrix V |
| Beamforming Matrix V for carrier 2 | Na×(b+b )/2 bits | Beamforming Matrix V |
| Beamforming Matrix V for Carrier 58-nNg | Na×(b+b )/2 bits | Beamforming Matrix V |
| … |  |  |
| Beamforming Matrix V for Carrier 58-Ng | Na×(b+b )/2 bits | Beamforming Matrix V |
| Beamforming Matrix V for carrier 58 | Na×(b+b )/2 bits | Beamforming Matrix V |

where n=55Ng, i.e., n is the maximum integer not exceeding 55/Ng, and Na is the number of angles used for beamforming matrix V (see Table n48).

TGn Editor:  Replace the following text from the line 44 to line 46 in page 63.

For example, 40 MHz operation with Ng=2 would have subcarriers in the following order:

[-58,-56….-4,-2,2,4…….56,58].

Grouping is a method to group adjacent tones to reduce the network overhead. The tone index to represent the group is defined in Table <xx> and in Table <yy>.

For example in a 40Mhz channel and Ng=4 beamforming matices are sent on carrier -58, -54, -50, …, -6, -2, 2, 6, …, 50, 54, 58.