IEEE P802.11
Wireless LANs

| TGn LB84 Submission on explicit FB matrix grouping |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-08-28 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Assaf Kasher | Intel | POB 1659, Haifa 31015, Israel | 97248651547 | assaf.kasher@intel.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB84 comments:

- 2292, 2316, 1210

The changes marked in this document are based on TGn Draft version D1.03.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Submission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

## Introduction to explicit FB matrix grouping comments

A few comments asked for clarification of the matrix grouping definition.   Specifically it requested to add examples for the grouping in both 20MHz and 40MHz.

## Discussion of each Comment:

| 2292 | N | 68 | 6 | 7.4.7.6 | T |  | Since Ng=2 and Ng=4 for 20MHz is so non-obvious, an example showing the selected carriers would be very useful | Make change indicated in comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal:  Counter

See below.

| 2316 | N | 72 | 1 | 7.4.7.7 | T |  | very useful example. | Please repeat this example for the other frames, too |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal:  Counter

See below:

| 12100 | Y | 65 | 1 | 7.4.7.6 | T |  | "Grouping" is under-described | Identify, perhaps here or elsewhere or both, that this grouping is of adjacent subcarriers |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Counter

See below

TGn Editor: Insert the following table after table n37 in draft 1.03

Table n37a – Carrier Grouping for Ng=2 and Ng=4

| BW and Grouping | Carriers in for which matrices are sent |
| --- | --- |
| 20MHz Ng=1 | All data and pilot carriers: -28 − -1, 1 − 28 |
| 20MHz Ng=2 | -28,-26,-24,-22,-20,-18,-16,-14,-12,-10,-8,-6,-4,-2,-1,1,3,5,7,9,11,13,15,17,19,21,23,25,27,28 |
| 20MHz Ng=4 | -28,-24,-20,-16,-12,-8,-4,-1,1,5,9,13,17,21,25,28 |
| 40MHz Ng=1 | All data and pilot carriers: -58 − -1, 1 − 58 |
| 40MHz Ng=2 | -58,-56,-54,-52,-50,-48,-46,-44,-42,-40,-38,-36,-34,-32,-30,-28,-26,-24,-22,-20,-18,-16,-14,-12,-10,-8,-6,-4, -2,2,4,6,8,10,12,14,16,18,20,22,24,26,28,30,32,34,36,38,40,42,44,46,48,50,52,54,56,58 |
| 40MHz Ng=4 | -58,-54,-50,-46,-42,-38,-34,-30,-26,-22,-18,-14,-10,-6, -2,2,6,10,14,18,22,26,30,34,38,42,46,50,54,58 |

TGn Editor: replace lines 44,45 in page 58 (section 7.4.8.9)

For example in a 20 MHz channel and Ng=4 steering matrices are sent on carrier -26, -22, -18, -14, -10, -6,

-2, -1, 1, 5, 9, 13, 17, 21, 25, 26.

with

For examples on which steering matrices are sent for a specific bandwidth and grouping see table n37a.

TGn Editor: replace lines 30-46 in page 63 (section 7.4.8.10)

Subcarriers are sent in order according to the following sequence. For 20 MHz bandwidth operation:

−28;−28+ Ng;−28+2Ng.........−1;1;1+Ng;1+2Ng....28

For 40 MHz bandwidth operation:

−58;−58+ Ng;−58+2Ng.........−2;2;2+ Ng;2+2Ng....58

For example, 40 MHz operation with Ng=2 would have subcarriers in the following order:

[-58,-56….-4,-2,2,4…….56,58].

with

Subcarriers are sent in order according to the grouping examples in table n37a.