IEEE P802.11
Wireless LANs

| TGn LB84 Submission on explicit FB format field |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-08-28 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Assaf Kasher | Intel | POB 1659, Haifa 31015, Israel | 97248651547 | assaf.kasher@intel.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB84 comments:

- 11405, 11431, 11456

The changes marked in this document are based on TGn Draft version D1.03.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Submission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

## Introduction to explicit FB matrix format field comments

A few comments requested that the explicit feedback format field which appears (under different names) in subclauses 7.4.8.8, 7.4.8.9, 7.4.8.10 be broken into its elements.

## Discussion of each Comment:

| 11405 | N | 64 | 6 | 7.4.7.6 | HE |  | Explicit feedback format would be better expressed as two fields of two bits each | As in comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Counter

| 11431 | N | 68 | 4 | 7.4.7.7 | HE |  | Explicit feedback format would be better expressed as two fields of two bits each | As in comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Counter

| 11456 | N | 73 | 4 | 7.4.7.8 | HE |  | Explicit feedback format would be better expressed as two fields of two bits each | As in comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Counter

TGn Editor: Replace the six'th column of table n31 (CSI Matrices control field)

B8 B11

Explicit Feedback Format

with

| B8 B9 | B10 B11 |
| --- | --- |
| Grouping (Ng) | Coefficient Size (Nb) |

TGn Editor: Replacd the last line of table n32 with the following lines:

| Grouping | Number of Carriers grouped into one: Set to 0 for Ng=1 (No grouping) Set to 1 for Ng=2 Set to 2 for Ng=4 A value of 3 is reserved |
| --- | --- |
| Coefficients Size | Number of bits in the representation of the real and imaginary parts of each element in the matrix. Set to 0 for Nb=4 Set to 1 for Nb=5 Set to 2 for Nb=6 Set to 3 for Nb=8 |

TGn Editor: Remove table n33

TGn Editor: Remove line 49-51

TGn Editor: Replace the last line of table n39 with the following lines

| Grouping | Number of Carriers grouped into one: Set to 0 for Ng=1 (No grouping) Set to 1 for Ng=2 Set to 2 for Ng=4 A value of 3 is reserved |
| --- | --- |
| Coefficients Size | Number of bits in the representation of the real and imaginary parts of each element in the matrix. Set to 0 for Nb=4 Set to 1 for Nb=5 Set to 2 for Nb=6 Set to 3 for Nb=8 |

TGn Editor: Delete Table n40.

TGn Editor Replace the last line of table n46 with the following lines:

| Grouping | Number of Carriers grouped into one: Set to 0 for Ng=1 (No grouping) Set to 1 for Ng=2 Set to 2 for Ng=4 A value of 3 is reserved |
| --- | --- |
| Codebook information | Set to 0 for 1 bit for ψ, 3 bits for φ Set to 1 for 2 bits for ψ, 4 bits for φ Set to 2 for 3 bits for ψ, 5 bits for φ Set to 3 for 4 bits for ψ, 6 bits for φ |

TGn Editor: Remove table n47