IEEE P802.11
Wireless LANs

| TGn LB84 Submission on Channel Selection in 20/40 MHz Mode |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Assaf Kasher | Intel Corporation | POB 1659, Haifa 31015, Israel | 97248651547 | assaf.kasher@intel.com |
| Bjorn A. Bjerke | Qualcomm, Inc. | 9 Damonmill Square, Suite 2a, Concord, MA 01742 | +1-781-276-0912 | bbjerke@qualcomm.com |
| Sanjiv Nanda | Qualcomm, Inc | 5775 Morehouse Drive, San Diego, CA 92121 | +1-858-845-2375 | snanda@qualcomm.com |
| Matthew Fischer | Broadcom | 190 Mathilda Place, Sunnyvale, CA 94086 | +1-408-543- 3370 | mfischer@broadcom.com |
| Adrian Stephens | Intel Corporation | 15 JJ Thompson Avenue, Cambridge CB3 0FD, UK |  | adrian.p.stephens@intel.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following 21 LB84 comments:

- 11.9.5 (D1.05: 11.9.8):

3614

- 11.9.5.1 (D1.05: 11.9.8.1):

1061

- 11.9.5.2 (D1.05: 11.9.8.2):

12112, 1521, 12113, 12247, 12114, 12248, 1062

- 11.9.5.3 (D1.05: 11.9.8.3):

1065, 2820, 12115, 6880, 12116, 1066, 4533, 6881, 7511, 7512

- Miscellaneous:

3102, 12117

The changes marked in this document are based on P802.11-REVma D8.0 (redline) and TGn Draft version D1.05.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

CIDs:

| 3614 | 149 |  | 11.9.5 | The indicated "co-existence" is not sufficient. There is no guidance on how often the scanning should be done; how the scanning should be performed during large data transfer. All the text appears to say is "apple pie is good, be nice" | Mandate that all stations maintain a extension channel CCA and will combine it with control channel CCA for their backoff counter decrement and transmission decisions. Further mandate that the AP should monitor traffic periodically (by scanning the channel at least once every dot11ScanPeriod) on the extension channel and use DFS based on the amount of load on the extension channel. Of course, I would greatly appreciate if a more thorugh mechanism is put in, but the above is a minimum. | Counter: optional secondary channel activity measurements added; see 1413r3 |
| --- | --- | --- | --- | --- | --- | --- |
| 1061 | 150 | 2 | 11.9.5.1 | do you mean "BSSs" instead of "devices" | change "devices" to "BSSs" | Counter: new text avoids problem; see 1413r3 |
| 12112 | 150 | 7 | 11.9.5.2 | Manual configuration ends up as "users chooses 40 MHz on their lucky number channel", and thus is not backwards compatible with OBSSs on the extension channel | Manual configuration of 40 MHz must be disallowed without a prior gating scan of the overlapping channels, and subsequent in-service scanning of the overlapping channels, including the requirement to vacate busy channels whose CCA and virtual NAV cannot be respected. | Counter: optional secondary channel activity measurements added; see 1413r3 |
| 1521 | 150 | 9 | 11.9.5.2 | Optional scanning of an overlapped channel when the device cannot respect the CCA or virtual NAV of the overlapped channel is not backwards compatible with OBSSs on the extension channel. | A station should sense CCA (and virtual NAV) on every channel it is about to transmit on. | Counter: optional secondary channel activity measurements added; see 1413r3 |
| 12113 | 150 | 9 | 11.9.5.2 | Optional scanning of an overlapped channel when the device cannot respect the CCA or virtual NAV of the overlapped channel is not backwards compatible with OBSSs on the extension channel. | Prior and in-service scanning must be mandatory. Testable performance requirements for this scanning must be provided. | Counter: optional secondary channel activity measurements added; see 1413r3 |
| 12247 | 150 | 9 | 11.9.5.2 | Optional scanning of an overlapped channel when the device cannot respect the CCA or virtual NAV of the overlapped channel is not backwards compatible with OBSSs on the extension channel. | See previous | Counter: optional secondary channel activity measurements added; see 1413r3 |
| 12114 | 150 | 12 | 11.9.5.2 | "The AP should reselect new parameters if an HT BSS that does not have the same control channel, extension channel offset starts operating on an overlapped channel". This will cause thrashing and does not promote a good frequency pan | We should define preferred 40MHz bands, and preferred control channels. In the absence of radar, the HT AP that does not conform to the preferred channelization must be the AP that must reselect its parameters. | Reject. Too restrictive. Adequate scanning procedures proposed in 1413r3 |
| 12248 | 150 | 12 | 11.9.5.2 | "The AP should reselect new parameters if an HT BSS that does not have the same control channel, extension channel offset starts operating on an overlapped channel". Simple statements like this do not solve the very complex issue of channel management in a hererogenous channel environment | Remove all statements regarding how to solve channel management. Omly leave the requirement to avoid collisions on the extension channel. | Reject: Adequate scanning and channel management procedures proposed in 1413r3 |
| 1062 | 150 | 14 | 11.9.5.2 | I cannot make sense of the phrase: "Given a choice of 40MHz channel selections recommended transmission by the previous rules," | help! | Accept: new text avoids problem; see 1413r3 |
| 1065 | 150 | 20 | 11.9.5.3 | it is not clear that an AP "has" a channel when it powers up, but I think that I can see the intent here - but there is no period of time specified, so the statement is sort of pointless as written | change the sentence beginning with "An HT AP" to "An HT AP shall receive the beacons of other HT BSSs on its desired channel of operation for ProbeDelay time before starting a BSS." | Counter: see 1413r3 |
| 2820 | 150 | 20 | 11.9.5.3 | Can't mandate that a STA receive anything. Too many unknowns, like signal strength and interference | Change to "An HT AP shall listen for Beacons of other…" | Counter: see 1413r3 |
| 12115 | 150 | 20 | 11.9.5.3 | No performance requirements are mentioned | Prior and in-service scanning must be mandatory. Testable performance requirements for this scanning must be provided. | See resolution of 12113 (counter; see 1413r3 |
| 6880 | 150 | 22 | 11.9.5.3 | How can the AP do "background scanning"? This places a requirement for the AP to have multiple radios. | Delete this sentence | Counter: new text avoids problem; see 1413r3 |
| 12116 | 150 | 22 | 11.9.5.3 | No performance requirements are mentioned | Prior and in-service scanning must be mandatory. Testable performance requirements for this scanning must be provided. | See resolution of CID 12113 (counter; see 1413r3) |
| 1066 | 150 | 25 | 11.9.5.3 | seems like it is not necessary for a STA to dissociate - why is it not possible for the STA to simply also switch to the 20MHz mode? | change "The STA" to "A STA" change "de-associate" to "disassociate" and change the "shall" to "may" | Counter: new text avoids problem; see 1413r3 |
| 4533 | 150 | 27 | 11.9.5.3 | This needs to be much more specific | Specify rules that the AP must follow to ensure that power-save STAs are informed about the channel transition | Counter: guidelines given; see 1413r3 |
| 6881 | 150 | 27 | 11.9.5.3 | Too vague. | Specify rules that the AP must follow to ensure that power-save STAs are informed about the channel transition | Counter: guidelines given; see 1413r3 |
| 7511 | 150 |  | 11.9.5.3 | "An HT AP shall receive the beacons of other HT BSSs on its channel when it first powers up. " This doesn't say how long, or what "first powers up" means. | This is actually a requirement on an SME. It should be described in terms of the MLME primitives. | Counter: new text avoids problem; see 1413r3 |
| 7512 | 150 |  | 11.9.5.3 | "In any transition of channel width, the AP should take in consideration any STAs that asleep. " Considerate APs are nice. But what this means is opaque. Unlike the channel switch announcement, the channel width change takes place immediately. It is not clear how the AP can be considerate. | Remove the sentence. | Counter: new text uses the channel switch announcement; see 1413r3 |

| 12117 | 152 | 15 | 11.15.1 | Table n57 does not address devices on overlapping channels such as the extension channel. | Prior and in-service scanning must be mandatory. Testable performance requirements for this scanning must be provided. | Counter: secondary channel activity measurements added; see 1413r3 |
| --- | --- | --- | --- | --- | --- | --- |

Submission Note:

We propose adding a Secondary Channel Activity field to the HT Notify Channel Width frame, so that STAs that wish to switch from receiving 40 MHz frames to receiving 20 MHz frames may inform the AP of the reason for wanting the switch. The AP may take such information, possibly collected from multiple STAs, into account when making decisions about relocating the BSS to a different channel and/or switching the channel width of the BSS. We have kept the three categories of secondary channel activity the same as in the previously proposed SCA request/report (which is now withdrawn in favor of the current proposal).

The proposed text in 11.9.8.3 depends on the Extended Channel Switch Announcement being proposed in document 06/1775r0 Extended Channel Switch Announcement for 802.11n.

TGn Editor:  insert the following new definitions into the appropriately alphabetically determined position within subclause 3. Definitions:

3.nx 20MHz PPDU: either a clause 17 PPDU, or clause 19 ERP-OFDM PPDU, or a clause 20MHz HT-PPDU as defined in clause 21.

3.nx 40MHz PPDU: 40MHz HT PPDU as defined in clause 21.

3.nx 20 MHz BSS: A BSS in which only 20 MHz PPDUs may be transmitted by members of the BSS.

3.nx 20/40 MHz BSS: A BSS in which both 20 MHz PPDUs and 40MHz PPDUs may be transmitted by members of the BSS.

TGn Editor: insert the following new subclause:

7.3.1.24a Secondary Channel Activity field

The Secondary Channel Activity field is used in an HT Notify Channel Width frame (see 7.4.8.2 Notify Channel Width frame format) in conjunction with the Channel Width field (see 7.3.1.24 Channel Width field) to indicate whether interfering activity on the secondary channel is a reason for requesting that frames sent to the sending STA be of 20 MHz width. The field may also be used to report on secondary channel activity even if a change of channel width is not requested, or if secondary channel activity is not a reason for requesting a change of channel width,

The length of the field is 1 octet and, as illustrated in Figure n15a (Secondary Channel Activity fixed field), it contains the following bits:

- SCA Forcing bit, which shall be set to 1 if secondary channel activity is the reason for requesting that frames sent to the sending STA be of 20 MHz width. Otherwise, the SCA Forcing bit shall be set to 0.

- Reduced SNR bit, which may be set to 1 to indicate that the SNR on the secondary channel is significantly lower than the SNR on the primary channel when receiving a 40 MHz PPDU. Otherwise, the Reduced SNR bit is set to 0. The determination of when to set this bit to 1 is implementation dependent.

- Interference bit, which may be set to 1 when interference is detected on the secondary channel when receiving a 20 MHz PPDU on the primary channel with good SNR.  Otherwise, the Interference bit is set to 0.

- Energy Detect bit, which may be set to 1 when energy is detected on the secondary channel, where the threshold is -60 dBm (TGn Editor: replace the -60 dBm value with the receiver minimum sensitivity value of BPSK, R=1/2 from Table n86, including modifications from 06/1584, if approved, plus 20 dB), when the primary channel is idle and the STA is performing CCA on the primary channel as defined in 21.3.21.5. Otherwise, the Energy Detect bit is set to 0.

- No SCA Information bit, which may be set to 1 when the STA does not have any information to send regarding secondary channel activity. When the No SCA Information bit is set to 1, all of the other bit fields shall be set to 0.

| SCA Forcing | Reduced SNR | Interference | Energy Detect | No SCA Information | Reserved (0) |
| --- | --- | --- | --- | --- | --- |

Bits:	        	1	  1	        1	      	1	       1    	          5-7

Figure 15b-Secondary Channel Activity fixed field

When the SCA Forcing bit is set to 1, at least one of the Reduced SNR, Interference, and Energy Detect bits shall be set to 1. When the SCA Forcing bit is set to 0, secondary channel activity may still be reported using one or more of the aforementioned three bits.

When the Channel Width field is set to 1 (indicating that any channel width enabled under Supported channel width set may be used), the SCA Forcing bit shall be set to 0.

TGn Editor: make the following changes to subclause 7.4.8.2 (Notify Channel Width):

7.4.8.2 Notify Channel Width

The format of the Notify Channel Width management action frames is defined in Table n29 (Notify Channel

Width).

Table n29-Notify Channel Width

| Order | Information |
| --- | --- |
| 1 | Category |
| 2 | Action |
| 3 | Channel Width (see 7.3.1.24 Channel Width field) |
| 4 | Secondary Channel Activity (see 7.3.1.24a Secondary Channel Activity field) |

This frame can be sent by both non-AP STA and AP. If an AP wishes to receive 20 MHz packets, it broadcasts this management action frame to all STAs. In addition, the AP indicates its current STA Channel Width in the HT Information Element in the beacon.

If a 20/40 MHz capable non-AP STA that is currently receiving 40 MHz packets wishes to receive only 20 MHz packets, it sends this management action frame to the AP. It may indicate that interfering activity on the secondary channel is the reason for requesting that frames sent to it be of 20 MHz width.

The Category field is set to 7 (representing HT).

The Action field is set to 0 (representing Notify Channel Width).

TGn Editor: make the following changes to subclause 9.20.7 (STA switching from 40 MHz to 20 MHz in 20/40 MHz BSS):

9.20.7 STA switching from 40 MHz to 20 MHz in 20/40 MHz BSS

Once associated, a non-AP STA shall support the same channel width capabilities it declared in its Association

Request. This capability is asserted in the Supported Channel Width Set in the HT capabilities element.

When a STA is associated with an AP that declares a Supported Channel Width of 20/40 MHz, the STA may

transmit frames in the 20 MHz wide primary channel as well as the 40 MHz wide channel. A non-AP STA

that is a member of a 40MHz BSS shall not transmit 20MHz frames in the secondary channel.

The AP may communicate its STA Channel Width either by transmitting the HT Information Element (in beacons

and probe response frames) or by transmitting the Notify Channel Width management action frame.

A 20/40 MHz capable non-AP STA in a 20/40 MHz capable BSS may request that frames sent to it be of 20 MHz width or of either 20 MHz width or 40 MHz width. The STA shall use the Notify Channel Width management action frame to achieve this.

A 20/40 MHz capable non-AP STA requesting that frames sent to it be of 20 MHz width may indicate that interfering activity on the secondary channel is a reason for the request by setting the appropriate bit(s) in the Secondary Channel Activity field of the Notify Channel Width management action frame. The Secondary Channel Activity field allows a STA to indicate the results one or more of the following measurements:

- Detection of reduced SNR on the secondary channel when receiving a 40 MHz PPDU. Detect if the SNR on the secondary channel is significantly lower than the SNR on the primary channel.

- Detection of interference on the secondary channel when receiving a 20 MHz PPDU on the primary channel with good SNR.

- Detection of energy on the secondary channel, where the threshold is -60 dBm (TGn Editor: replace the -60 dBm value with the receiver minimum sensitivity value of BPSK, R=1/2 from Table n86, including modifications from 06/1584, if approved, plus 20 dB) for both 2.4GHz and 5 GHz bands, when the primary channel is idle and the STA is performing CCA on the primary channel as defined in 21.3.21.5

The STA may choose not to give a reason for its request, in which case the No SCA Information bit of the Secondary Channel Activity field is set to 1.

The reaction of both STA and AP to switch the channel width may be not immediate after receiving these

frames. The STA that has transmitted one of these frames shall decide that the related request has been adopted after having received an HT frame of requested specified channel width.

Recommending a transmission width of 20MHz has no effect on the BSS requirements regarding protection

of 40MHz transmissions. A STA should re-associate with its Supported Channel Width Set field set to 0 (indicating

only 20 MHz operation) to obtain this protection.

TGn Editor: make changes to subclause 11.9 DFS procedures as shown below.

11.9 DFS procedures

Insert the following new subclause heading:

11.9.8 Channel selection methods for 20/40 MHz operation

Insert the following new subclause:

11.9.8.1 Introduction

On detecting the arrival of a BSS operating on the secondary channel, the AP operating a 20/40 MHz BSS may choose to move to a different pair of channels or switch to 20 MHz BSS operation on any channel other than the secondary channel in order to avoid the arriving BSS.

Insert the following new subclause:

11.9.8.2 Scanning prior to starting a BSS

Before an AP starts a 20/40 MHz BSS it shall scan the environment for existing BSSs.  It may use passive scan for at least MinChannelTime or active scan using HT Mixed Format PPDUs or non-HT Format PPDU types.  If the scanning AP chooses to start a 20/40 MHz BSS that occupies the same two channels as an existing 20/40MHz BSS, then the scanning AP shall ensure that the primary channel of the new BSS is identical to the primary channel of the existing 20/40 MHz BSS and that the secondary channel of the new 20/40 MHz BSS is identical to the secondary channel of the existing 20/40 MHz BSS. The AP may also use passive or active scan after the BSS has been started, with the same PPDU format restrictions as given above.

The AP shall use the same technique to scan channels before making a transition from a 20 MHz BSS to a 20/40 MHz BSS.

Insert the following new subclause:

11.9.8.3 Channel management at the AP

An AP operating a 20/40 MHz BSS may decide to move the BSS and/or adjust its channel width if, for example, another BSS starts to operate in either or both of the primary or secondary channels, or radar is detected in either or both of the primary or secondary channels. Specifically, the AP may move its BSS to a different pair of channels, or change from a 20/40 MHz BSS to a 20 MHz BSS using either the primary channel of the previous channel pair or any other available 20 MHz channel, The AP may also change from a 20 MHz BSS to a 20/40 MHz BSS.

An AP that wishes to maintain existing associations while making a channel width change or while performing a channel pair relocation may inform associated STAs that it is making the change by including an Extended Channel Switch Announcement element and a Secondary Channel Offset element in Beacon, Probe Response, and Extended Channel Switch Announcement frame transmissions until the intended channel switch time.  The New Channel Number of the Extended Channel Switch Announcement element represents the new channel in the case that the BSS after relocation/width change will be a 20MHz BSS or the primary channel of the new pair of channels in the case that the BSS after relocation/width change will be a 20/40 MHz BSS. When changing to a new pair of channels, the Secondary Channel Offset element may have the value 1 or 3 depending on the position of the secondary channel relative to the primary channel in the new pair of channels. When the change will result in a 20MHz BSS, then the Secondary Channel Offset element shall have the value 0.

When changing from one pair of channels to a new pair of channels, the Supported Channel Width Set field of the HT capabilities element in the Beacon remains unchanged.

APs that perform a channel switch or channel width adjustment shall include in frames containing HT capabilities elements, Extended Channel Switch Announcement elements, and Secondary Channel Offset elements that announce the change, values for New Channel Number, Supported Channel Width Set and Secondary Channel Offset that comply with the values given in Table nx.

Table nx - Channel switching and channel width adjustment combinations

| Original channel | Destination channel |  |  |  |
| --- | --- | --- | --- | --- |
|  | 20 MHz channel | Different 20 MHz channel | Pair of 20 MHz channels with same primary channel as original | Pair of 20 MHz channels with different primary channel than original |
| 20 MHz channel | N/A | New Channel Number: any number different from current. Supported Channel Width Set: 0. | New Channel Number: same as current (must satisfy channel definitions in 21.3.14). Supported Channel Width Set: 1. Secondary Channel Offset: 1 or 3. | New Channel Number: any number different from current, satisfying channel definitions in 21.3.14. Supported Channel Width Set: 1. Secondary Channel Offset: 1 or 3. |
| Pair of 20 MHz channels | New Channel Number: same as current primary channel. Supported Channel Width Set: 0. Secondary Channel Offset: 0 (if present). | New Channel Number: any number different from current primary channel. Supported Channel Width Set: 0. Secondary Channel Offset: 0 (if present). | N/A | New Channel Number: any number different from current, satisfying channel definitions in 21.3.14. Supported Channel Width Set: 1. Secondary Channel Offset: 1 or 3. |

Movements of 20/40 MHz BSSs from one channel pair to a different channel pair and changes between 20 MHz BSS and 20/40 MHz BSS modes of operation should be scheduled so that all STAs in the BSS, including STAs in power save mode, have the opportunity to receive at least one Extended Channel Switch Announcement element and Secondary Channel offset element before the switch. The AP may send the Extended Channel Switch Announcement element and Secondary Channel Offset element in a Beacon without performing a backoff, after determining the WM is idle for one PIFS period.