IEEE P802.11
Wireless LANs

| Enhancement of the Compressed Steering Matrix performance with power control feedback. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-09-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Andrew Storm | Bandspeed | GPO Box 5147, Melbourne, Victoria 3001, Australia | 61-3-98196846 | astorm@bandspeed.com |
| Scott Leyonhjelm | Nanoradio | 39 Leveson St, P.O. Box 770, Nth Melbourne, 3051, Australia. | 61-431-594167 | scott.leyonhjelm@vu.edu.au |

Abstract

Note that the content of the document does not directly address any current CIDs, rather its immediate intention is to make a technical contribution for discussion and further consideration in the 802.11n working body.

Currently clauses relating to the compressed steering matrix, 21.3.11.2.3 and 7.4.8.10 in the draft standard D1.03, allow for the feedback of steering matrices with orthonormal columns only. Including power control, in the form of a column scaling of the steering matrix, offers additional performance gains for a small increase in feedback information. Also included is a proposal to modify the text in subclause 21.3.11.2.3 & 7.4.8.10 for compressed steering matrix feedback in TGn Draft D1.03

In the 2x2 case there is an approximate doubling in the feedback information with this proposal for a performance gain of 3dB at a PER of 10%. It should be noted that a smaller relative increase in feedback information is required for the larger configurations.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

## Introduction

Note that the content of the document does not directly address any current CIDs, rather its immediate intention is to make a technical contribution for discussion and further consideration in the 802.11n working body.

The document considers

- The proposals technical argument

- Simulation results illustrating the benefit of the proposal

- A proposal to modify the text in subclause 21.3.11.2.3 & 7.4.8.10 for compressed steering matrix feedback in TGn Draft D1.03.

- Feedback information length calculations and comparisions for different configurations.

## Technical Argument

Currently clauses relating to the compressed steering matrix, 21.3.11.2.3 and 7.4.8.10 in the draft standard D1.03, allow for the feedback of steering matrices with orthonormal columns only. Including power control, in the form of a column scaling of the steering matrix, offers additional performance gains for a small increase in feedback information.

Consider the usual MIMO channel model

r=HQs+ν.

The complex received signal vectorrand the noiseνhave dimensionsN×1, the complex channel matrixHhas dimensionsN×M, the optional steering matrixQhas dimensions M×K and the signalshas dimensionsK×1, whereK is the number of transmit streams. We assume thatN≥Mand that the total signal power is constrained, so thatE(sHs)≤P. Also the noise is assumed to have covarianceΣν=E(ννH)=σ2I.

It is claimed that when the number of transmit streams is equal to the number of transmit antennas,K=M that two reasonable cost functions are constant on the space of unitary steering matrices. Consequently an eigenvector matrix computed with full knowledge of the channel will perform no better than a fixed steering matrix such as a Walsh or Fourier matrix.

The first cost function is the channel capacity, which in the absence ofQis given by the formula [1]

C=log1+λmPmσ2.

The numbersλmare the eigenvalues of the matrixHHHand Pmis the water-filling power allocation (which depends only on Pandλm). When a unitary matrixQis inserted thenH becomesHQandHHH becomesHQQHHH=HHH. SinceHHHis unchanged the eigenvalues, and consequently the capacity, are unaffected by the insertion of the unitary matrixQ.

The second cost function is the Frobenius norm of the least squares error covariance matrix. In this case we can assume a more general noise covarianceΣν. Applying the pseudo-inverseH+=(HHΣν−1H)−1HHΣν−1 torwe obtains=s+ε. The residual error vectorε=(HHΣν−1H)−1HHΣν−1νhas covarianceΣε=(HHΣν−1H)−1. The squared Frobenius norm of this matrix isF=trace(ΣεHΣε)=trace((HHΣν−1H)−2). SinceHHΣν−1H is Hermitian and positive definite it may be expressed as HHΣν−1H=VDVHwhereVis aM×Munitary matrix and D=diag(dm)is a diagonal matrix of eigenvalues. Then the cost function may be expressed asF=dm−2. If a unitary steering matrixQis inserted thenHHΣν−1HbecomesQHHHΣν−1HQ=(QHV)D(VHQ). Note thatQHVis just another unitary matrix, and consequently the eigenvalues and the cost function are unchanged.

These arguments demonstrate that if a performance gain is to be achieved by the insertion of a square steering matrix then that matrix cannot be unitary. Additional arguments, such as optimizing either of the above cost functions under a total power constraint, show that performance improvements may be obtained by using a steering matrix consisting of an eigenvector matrix together with a column scaling:Q=VA whereA=diag(ak).

These remarks strictly apply to the case of a symmetric MCS with the same modulation on each stream. In the case of an asymmetric MCS, an eigenvector matrix will have an advantage over a Walsh or Fourier matrix in that it will allow the transmission of the stream with the most sensitive modulation over the best mode in the channel. However, the eigenvalues will vary between sub-carriers, and power control will allow an additional fine tuning of the eigenvalues on each sub-carrier.

When the number of transmit streams is less than the number of transmit antennas then a genuine performance gain is obtained by using a Qmatrix consisting of normalized eigenvectors, compared to a fixed steering matrix, because the eigenvector matrix allows for transmission on theK best modes in the channel, whereas the performance with a fixed steering matrix is influenced by the worst of theMeigenvalues. However, in this case power control in the form of a column scaling allows for additional fine-tuning with some performance gain.

## Simulation Results

The plots below show the packet error rate as a function of SNR for three types of steering matrix: a fixed steering matrix, a unitary eigenvector matrix, and an eigenvector matrix with power control. The fixed steering matrix is a Walsh matrix in the 2x2 case and a Fourier matrix in the 3x3 case. The first two plots are for channel model B and MCS 14 (two transmit streams) and are for un-quantized column scale factors. The simulation was with impairments as specified in CC67. The third plot is for an asymmetric MCS, MCS 35.

When the column scale factors were quantized we found very little change in performance when using four or six bits per scale factor.

The first plot is forN=M=2 (K =2). Packet error rate is not one of the cost functions considered above, but the plot shows little difference in performance between the unitary eigenvector matrix and the Walsh steering matrix. Power control (subcarrier grouping = 1) delivers an improvement of about 3dB at a PER of 10% in this case.

The second plot is for N=M=3 (andK=2). In this case the unitary eigenvector matrix offers a substantial improvement over the Fourier matrix. Power control delivers a small additional improvement of about 0.5 dB at a PER of 10%.

The next plot is for an asymmetric MCS, MCS 35 (64QAM & 16QAM R1/2). It shows that the eigenvector matrix is slightly better than the Walsh matrix in this case, by about 1.5dB at 10% PER. Power control offers a further improvement of about 1dB.

Quantisation of the scale coefficients to 4 or 6 bits show negligible performance degradation as illustrated below.

## Proposal for the change of subclause 21.3.11.2.3 & 7.4.8.10

In 21.3.11.2.3 modify the format of the feedback matrix Vto the following:

V=i=1min(M,N−1)Di1i−1ejφii…ejφN−1,i1l=i+1NGliT(ψli)AN×M.

The matrixAN×Mhas the formatAN×M=A0where A=diag(amm)is aM×Mdiagonal matrix with positive entries. In the case M=1 one assumesa11=1

In 7.4.8.10 make the following changes.

In Table n47, replace the phrase ‘n bits forφ’ with the phrase ‘n bits forφ and a’.

Replace Table n48 with the following table.

Table n48 – Order of feedback quantities in the Quantized Steering Matrices Feedback Information field.

| V(f) | Number of feedback quantities | The order of scale factors and angles in the Quantized Steering Matrices Feedback Information field |
| --- | --- | --- |
| 2x1 | 2 | f11, y21 |
| 2x2 | 4 | a11,a22,f11,y21 |
| 3x1 | 4 | f11,f21,y21,y31 |
| 3x2 | 8 | a11,a22,f11,f21,y21,y31,f22,y32 |
| 3x3 | 9 | a11,a22,a33,f11,f21,y21,y31,f22,y32 |
| 4x1 | 6 | f11,f21,f31,y21,y31,y41 |
| 4x2 | 12 | a11,a22,f11,f21,f31,y21,y31,y41,f22,f32,y32,y42 |
| 4x3 | 15 | a11,a22,a33,f11,f21,f31,y21,y31,y41,f22,f32,y32,y42,f33,y43 |
| 4x4 | 16 | a11,a22,a33,a44,f11,f21,f31,y21,y31,y41,f22,f32,y32,y42,f33,y43 |

The scale factors are quantized according to the formula

where.

ba (=b) is the number of bits used to quantize a

## Feedback Information Length Examples

The table below illustrates amount of feedback information (bits) (Bandwidth = 20MHz) required when using the different explicit feedback techniques. In the 2x2 configuration the amount of feedback information slightly more than doubles with the newly proposed compressed steering feedback mechanism, whilst in the 4x4 configuration the amount of information only increases by 50%.

|  | 2x2 |  | 4x4 |  | Comments |
| --- | --- | --- | --- | --- | --- |
|  | No Group | 4 Group | No Group | 4 Group |  |
| H Matrix (also approx. Uncompressed Steering | 471 | 136 | 1817 | 522 | Nb = 8bits |
| Compressed Steering | 56 | 16 | 80 | 280 | 3+5 bits |
| Newly Proposed Compressed Steering | 126 | 36 | 120 | 420 | 3+5bits, 5 bits for amplitude |

## Reference

[1] I. Emre Telatar, “Capacity of Multi-antenna Gaussian Channels”, AT&T Bell Laboratories, Internal Tech. Memo. June 1995.