IEEE P802.11
Wireless LANs

| Key distribution – General Comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-10-25 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bill Marshall | TGr Editor | 180 Park Ave, Florham Park, NJ 07932 | 973-360-8718 | wtm@research.att.com |

Abstract

This document proposes a resolution to several of the LB87 comments directed to the key distribution scheme added in D3.0. These were briefly discussed at the TGr ad-hoc, generally believed non-controversial, but in need of a submission to identify the changes.

NAS-ID is being removed from Beacons and Probe Responses. R1KH-ID is being included in those frames in its place.  The key distribution scheme in D3.0 8.5A.6 needs to know the value of R1KH-ID  in order to generate the R1DIST value for message #1 of the authentication sequence, so its appearance in Beacons and Neighbor reports is required.

R1DIST and R1DISTV are added to the FTIE, instead of separate information elements.

8.5A.6.1 and 8.5A.6.2 restored from D2.2, with Key Holder identity and key transfer requirements

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Overview

NAS-ID is being removed from Beacons and Probe Responses. R1KH-ID is being included in those frames in its place.  The key distribution scheme in D3.0 8.5A.6 needs to know the value of R1KH-ID  in order to generate the R1DIST value for message #1 of the authentication sequence, so its appearance in Beacons and Neighbor reports is required.

R1DIST and R1DISTV are added to the FTIE, instead of separate information elements.

8.5A.6.1 and 8.5A.6.2 restored from D2.2, with Key Holder identity and key transfer requirements

Make the following changes to D3.0:

3.122a

Add “R1DIST: Authentication information exchanged between the STA and the PMK-R0 key holder with which it performed the initial association, cryptographically protected, which authorizes a PMK-R1 key holder.”

3.122b

Add “R1DISTV: Authentication information exchanged between the PMK-R1 key holder and the STA that demonstrates the R1KH’s authorization to be a PMK-R1 key holder.”

7.2.3.1:

Change row 31 of Table 8, NAS-ID, to Fast Transition Information, with note stating “The FTIE shall be present when dot11FastBSSTransitionEnabled is set to true and dot11RSNAEnabled is set to true.”

7.2.3.9:

Change row 30 of Table 15, NAS-ID, to Fast Transition Information, with note stating “The FTIE shall be present when dot11FastBSSTransitionEnabled is set to true and dot11RSNAEnabled is set to true.”

7.2.3.10:

Delete rows 10 and 11 of Table 16, R1 Distribution Token, and R1 Distribution Valid. Delete mention of R1 distribution token and R1 distribution valid from Table 17.

7.3.2:

Delete Element ID 58 from Table 26, R1 Distribution Token.

Delete Element ID 59 from Table 26, R1 Distribution Valid.

7.3.2.37:

Insert a new row into Table 43b with Sub-Element ID=3, and Name= “PMK-R1 Key Holder ID”

Change the row marked “Reserved” to 4-220.

Before the final paragraph of 7.3.2.37 (which starts “The Vendor Specific sub-element…”) insert the following:

The PMK-R1 Key Holder ID sub-element contains the R1KH-ID subfield as shown in Figure 112ga. The R1KH-ID subfield shall not be included if the R1KH-ID of the reported AP is not known.

Add Figure 112ga, 1 octet Sub-Element ID, 1 octet Length, 6 octets R1KH-ID, and title “PMK-R1 Key Holder ID sub-element format”

The value of the PMK-R1 sub-element length field in octets is 6.

The R1KH-ID field is 6 octets long and contains the R1KH-ID of the reported AP.

7.3.2.46:

Change Figure 112U to show R1KH-ID as the first item. R1KH-ID length 6.

Add to the paragraph starting “The Length field…” “If the FTIE contains only the R1KH-ID, then the length field contains the value 6.”

Add paragraph below Figure 112U “R1KH-ID indicates the AP’s key holder identity, which is used by the STA for deriving the PMK-R1 keys. It is encoded following the conventions from 7.1.1.” (as updated by approved comments directed to this existing text).

Delete row “R1KH-ID” in Table 34A. Change row “GTK” to use value 1.

Add two new rows after “GTK”

Value 2, R1DIST, length variable.

Value 3, R1DISTV, length 32.

Delete paragraph below table “R1KH-ID”, which was moved to appear earlier in this subclause.

After paragraph “GTK”, add two new paragraphs.

R1DIST is used to carry an authentication and authorization token between the STA and the PMK-R0 key holder. Its contents are described in 8A.5.1 and 8A.5.2.

R1DISTV is used to convey an authentication and authorization token between the PMK-R1 key holder and the STA. Its contents are described in 8A.5.2.

7.3.2.49:

Delete this clause.

7.3.2.50:

Delete this clause.

7.3.2.51:

Delete this clause.

7.4.7.1:

Delete row 4 of Table 57C, R1DISTIE.

7.4.7.2:

Delete rows 4 and 5 of Table 57D, R1DISTIE and R1DISTVIE.

8.5A.1:

Change fifth paragraph as follows:

The FT key hierarchy derives its keys using a Key Derivation Function (KDF) as defined in 8.5A.3 with separate labels to further distinguish derivations. Each level of the hierarchy derives keys for the level below it. The protocol for distribution of keying material from the PMK-R0 key holder to the PMK-R1 key holders is outside the scope of this specification. However, to ensure security, this key hierarchy requires that keys only be distributed down the key hierarchy, and by the use of a secure transport protocol.

8.5A.6:

Insert the following before 8.5A.6, and renumber existing 8.5A.6 as 8.5A.7:

8.5A.6 Fast BSS Transition key holders

The PMK-R0 shall be stored in a component called the PMK-R0 Key Holder. The PMK-R0 Key Holder derives the PMK-R0 key for use in the Mobility Domain utilizing either the MSK (when the AKM negotiated is 00-0F-AC:3) or the PSK (when the AKM negotiated is 00-0F0AC:4). The PMK-R0 Key Holder shall be responsible for deriving a PMK-R1 key for each PMK-R1 Key Holder within the Mobility Domain.

The PMK-R1 shall be stored in a component called the PMK-R1 Key Holder. The PMK-R1 Key Holder shall derive the PTK mutually with the STA.

The R0KH and R1KH are responsible for the derivation of keys in the FT key hierarchy. For Fast BSS Transition, the functions of the IEEE 802.1X Authenticator are distributed among the R0KH, and R1KH in the AP. Each Key Holder is assumed to be addressable as a distinct entity. Each Key Holder name is assumed to be expressed as a unique identifier within the Mobility Domain. This identifier is communicated to the STA and other key holders, and is bound into the key derivation. Each key holder name shall be mapped to a physical entity in the DS where it resides.

The R0KeyHolder shall meet the following requirements:

— The R0KeyHolder shall be co-resident with the NAS Client functionality of the IEEE 802.1X Authenticator.

— The R0KeyHolder identifier (R0KH-ID) shall be set to the NAS ID as defined in RFC 2865.

— When the PMK-R0 lifetime expires, the R0KeyHolder shall delete the PMK-R0 SA and should revoke all PMK-R1s derived from the PMK-R0.

The R1KeyHolder shall meet the following requirements:

— The R1KeyHolder identity (R1KH-ID) shall be set to the MAC address of the physical entity that stores the PMK-R1 and uses it to generate the PTK. That same MAC address shall be used to advertise the PMK-R1 identity to the STA and the R0KH.

— The R1KeyHolder shall provide the IEEE 802.1X Authenticator function to derive and distribute the GTK to all connected STAs.

— When the PMK-R1 lifetime expires, the R1KeyHolder shall delete the PMK-R1 PMKSA and shall revoke all PTKSAs derived from the PMK-R1 using the MLME-DELETEKEYS primitive.

Change title of existing 8.5A.6 from “PMK-R1 distribution within a mobility domain” to the following:

8.5A.7 Assumptions for PMK-R1 distribution within a Mobility Domain

Delete the first five paragraphs of existing 8.5A.6 and insert in their place:

The PMK-R0 Key Holder and the PMK-R1 Key Holder are assumed to have a trustworthy channel between them that can be used to exchange cryptographic keys without exposure to any intermediate parties.  This standard assumes that the key transfer includes the PMK-R1, the PMK-R1 context, and the associated key authorizations.  The protocol for distribution of keying material from the PMK-R0 key holder to the PMK-R1 key holder is outside the scope of this specification.

In sixth paragraph of 8.5A.6, change “identity of the target AP as determined by the NASID Information Element in its beacons and probe responses” to “identity of the R1KH of the target AP”.

In seventh paragraph of 8.5A.6, change “The target AP shall extract the token from the R1DIST Information Element and send it along with the MAC address” to “The target AP shall send the R1DIST value from the FTIE, along with the MAC address…”

In eighth paragraph of 8.5A.6, change “The R0KH compares the NAS-Id from the unwrapped token with the identity of the target AP used during when establishing its shared security association” to “The R0KH compares the R1KH-ID from the unwrapped token with the identity of the R1KH of the target AP used when establishing its shared security association.”

In final paragraph of 8.5A.6, change “and compare the enclosed NAS-Id with the one it received from the taget AP” to “and compare the enclosed R1KH-ID with the one it received from the target AP”.

8A.1.1:

Change “The NAS-Id of the authenticator associated with the AP advertising Fast BSS Transition capability shall be the value of dot11FTNASID.” to: “The R1KH-ID in the FTIE shall be the value of dot11FTR1KeyHolderID.”

8A.3.1 and 8A.3.2:

Remove R1DIST and R1DISTV as separate IEs in the message exchanges (if added by other comments).

Add R1DIST and R1DISTV as parameters in the FTIE.

Change “If the STA has no record of authorizing the target AP to be a PMK-R1 holder it shall include an R1DIST IE in its Authentication-Request (see clause 8.5A.6). If an R1DIST IE was present in the Authentication-Request than an R1DIST IE and an R1DISTV IE must be in the accompanying Authentication-Response (and must be correctly processed according to clause 8.5A.6) or authentication shall be deemed to have failed.”

To “If the STA has no record of authorizing the target AP to be a PMK-R1 holder it shall include an R1DIST in the FTIE in its Authentication-Request (see clause 8.5A.6). If an R1DIST was present in the Authentication-Request then an R1DIST and an R1DISTV shall be in the accompanying Authentication-Response (and must be correctly processed according to clause 8.5A.6) or authentication shall be deemed to have failed.”

8A.5:

Delete rows “R1 Distribution” and “R1 Distribution Valid” from Table 64A.

8A.5.1:

Change “If the STA has no record of previously authorizing the NAS identified by the target AP’s NAS-Id it includes an R1DIST IE. The target AP uses the R0KH-ID, the STA MAC, and the token conveyed in the R1DIST IE to obtain the appropriate PMK-R1.”

To “If the STA has no record of previously authorizing the R1KH identified by the target AP’s R1KH-ID it includes an R1DIST optional parameter in the FTIE. The R1KH of the target AP uses the R0KH-ID, the STA MAC, and the R1DIST value conveyed in the FTIE to obtain the appropriate PMK-R1.”

8A.5.1:

Insert bullet 2.333 in the FTIE description, with text “R1KH-ID shall be the value of R1KH-ID for the target AP, obtained by the STA through either a Beacon, Probe Response, or Neighbor Report.”

Insert bullet 2.666 in the FTIE description, with text “If the STA has no record of having authorized this R1KH-ID to hold a PMK-R1, then an R1DIST optional parameter shall be included in the FTIE. The value of this optional parameter shall be the AES-KeyWrap of (Ns || R1KH-ID), where Ns is a sixteen octet random number chosen by the STA, and R1KH-ID is the identity of the R1KH of the AP advertised in its beacons and probe responses. The key used to perform this wrapping shall be R0KH-Distribution-Key described in 8.5a.4.

Delete paragraph starting “The R1DIST IE shall be present…”

8A.5.2:

Insert bullet 3.333 in the FTIE description, with text “If the STA included an R1DIST optional parameter in the fast BSS transition request, then an R1DIST optional parameter shall be present. The value of the R1DIST optional parameter shall be the AES-KeyWrap of (R1KH-ID || Na), where Na is a sixteen (16) octet random number chosen by the R0KH according to the recommendations of 8.5.8. The wrapping shall use R0KH-Distribution-Key described in 8.5a.4.

Insert bullet 3.666 in the FTIE description, with text “If the STA included an R1DIST optional parameter in the fast BSS transition request, then an R1DISTV optional parameter shall be present. The value of the optional parameter shall be calculated as SHA-256(Ns || Na).

Delete paragraph starting “The R1DIST IE shall be present…” and paragraph starting “The R1DISTV IE shall be present…”

Annex A:

Add a PICS entry: PC35.13 “R1KH Authentication procedures” referencing 8.5A.7, Status PC35:M, with support Y/N/N/A.

Annex D:

Delete dot11FastBSSTransitionConfigEntry dot11FTNASId.

Delete dot11FTNASID.

Resolve LB87 comments as follows:

| 14 | Rejected. There is a need for distinct names of R0KH-ID and R1KH-ID, and R0KH-ID is defined as the NAS-ID. |
| --- | --- |
| 72 | Counter. NAS-ID deleted. |
| 73 | Counter. NAS-ID deleted. |
| 88 | Rejected. There is a need for distinct names of R0KH-ID and R1KH-ID, and R0KH-ID is defined as the NAS-ID. |
| 92 | Counter. Added 3.122a R1DIST: Authentication information exchanged between the STA and the PMK-R0 key holder with which it performed the initial association, cryptographically protected, which authorizes a PMK-R1 key holder. Added 3.122b R1DISTV: Authentication information exchanged between the PMK-R1 key holder and the STA that demonstrates the R1KH’s authorization to be a PMK-R1 key holder. |
| 102 | Counter. Definitions added to clause 3. |
| 106 | Counter. NAS-ID deleted. |
| 107 | Counter. NAS-ID deleted. |
| 292 | Counter. NAS-ID deleted. Row changed to “Fast Transition Information” with note stating “The FTIE shall be present when dot11FastBSSTransitionEnabled is set to true and dot11RSNAEnabled is set to true.” |
| 293 | Counter. NAS-ID deleted. Row changed to “Fast Transition Information” with note stating “The FTIE shall be present when dot11FastBSSTransitionEnabled is set to true and dot11RSNAEnabled is set to true.” |
| 294 | Counter. NAS-ID deleted. Row changed to “Fast Transition Information” with note stating “The FTIE shall be present when dot11FastBSSTransitionEnabled is set to true and dot11RSNAEnabled is set to true.” |
| 295 | Counter. NAS-ID deleted. Row changed to “Fast Transition Information” with note stating “The FTIE shall be present when dot11FastBSSTransitionEnabled is set to true and dot11RSNAEnabled is set to true.” |
| 296 | Counter. NAS-ID deleted. Row changed to “Fast Transition Information” with note stating “The FTIE shall be present when dot11FastBSSTransitionEnabled is set to true and dot11RSNAEnabled is set to true.” |
| 297 | Counter. Suggested text, with NAS-ID changed to FTIE. |
| 298 | Counter. Changed to "The FTIE is present when dot11FastBSSTransitionEnabled is set to true and dot11RSNAEnabled is set to true." |
| 299 | Counter. NAS-ID deleted. Row changed to “Fast Transition Information” with note stating “The FTIE shall be present when dot11FastBSSTransitionEnabled is set to true and dot11RSNAEnabled is set to true.” |
| 300 | Counter. NAS-ID deleted. Row changed to “Fast Transition Information” with note stating “The FTIE shall be present when dot11FastBSSTransitionEnabled is set to true and dot11RSNAEnabled is set to true.” |
| 381 | Counter. NAS-ID deleted. Row changed to “Fast Transition Information” with note stating “The FTIE shall be present when dot11FastBSSTransitionEnabled is set to true and dot11RSNAEnabled is set to true.” |
| 382 | Counter. NAS-ID deleted. Row changed to “Fast Transition Information” with note stating “The FTIE shall be present when dot11FastBSSTransitionEnabled is set to true and dot11RSNAEnabled is set to true.” |
| 383 | Counter. NAS-ID deleted. Row changed to “Fast Transition Information” with note stating “The FTIE shall be present when dot11FastBSSTransitionEnabled is set to true and dot11RSNAEnabled is set to true.” |
| 385 | Accepted. NAS-ID deleted. Row changed to “Fast Transition Information” with note stating “The FTIE shall be present when dot11FastBSSTransitionEnabled is set to true and dot11RSNAEnabled is set to true.” |
| 394 | Counter. Row deleted from table |
| 395 | Counter. Row deleted from table |
| 396 | Counter. Row deleted from table |
| 397 | Counter. Row deleted from table |
| 398 | Counter. Row deleted from table |
| 403 | Counter. R1 distribution token incorporated into FTIE instead of a separate IE |
| 404 | Counter. R1 distribution token incorporated into FTIE instead of a separate IE |
| 405 | Counter. R1 distribution token incorporated into FTIE instead of a separate IE |
| 406 | Counter. R1 distribution token incorporated into FTIE instead of a separate IE |
| 407 | Counter. R1 distribution token incorporated into FTIE instead of a separate IE |
| 408 | Counter. R1 distribution token incorporated into FTIE instead of a separate IE |
| 409 | Counter. R1 distribution token incorporated into FTIE instead of a separate IE |
| 410 | Counter. R1 distribution token incorporated into FTIE instead of a separate IE |
| 411 | Counter. R1 distribution token incorporated into FTIE instead of a separate IE |
| 412 | Counter. R1 distribution token incorporated into FTIE instead of a separate IE |
| 413 | Counter. R1 distribution token incorporated into FTIE instead of a separate IE |
| 414 | Counter. R1 distribution token incorporated into FTIE instead of a separate IE |
| 415 | Counter. R1 distribution token incorporated into FTIE instead of a separate IE |
| 416 | Counter. R1 distribution token and R1 distribution valid incorporated into FTIE instead of separate IEs |
| 417 | Counter. R1 distribution token and R1 distribution valid incorporated into FTIE instead of separate IEs |
| 418 | Counter. R1 distribution token and R1 distribution valid incorporated into FTIE instead of separate IEs |
| 419 | Counter. R1 distribution token and R1 distribution valid incorporated into FTIE instead of separate IEs |
| 420 | Counter. R1 distribution token and R1 distribution valid incorporated into FTIE instead of separate IEs |
| 421 | Counter. R1 distribution token and R1 distribution valid incorporated into FTIE instead of separate IEs |
| 422 | Counter. R1 distribution token and R1 distribution valid incorporated into FTIE instead of separate IEs |
| 423 | Counter. R1 distribution token and R1 distribution valid incorporated into FTIE instead of separate IEs |
| 424 | Counter. R1 distribution token and R1 distribution valid incorporated into FTIE instead of separate IEs |
| 427 | Accepted. Procedures in 8.5A.6 specify the use of status code 38 for all errors. No new status codes are needed. |
| 429 | Counter. Procedures in 8.5A.6 specify the use of status code 38 for all errors. No new status codes are needed. |
| 433 | Counter. R1 distribution token and R1 distribution valid combined into FTIE |
| 436 | Counter. R1 distribution token and R1 distribution valid combined into FTIE |
| 441 | Counter. NAS-ID deleted. |
| 442 | Counter. NAS-ID deleted. |
| 443 | Counter. NAS-ID deleted. |
| 485 | Rejected. 3-party protocol is mandatory; no capability indication is needed. |
| 545 | Counter. R1KH-ID made first item in this IE, and present in every FTIE. Removed from table of Optional Parameters. |
| 551 | Accepted. Text added to 8.5A.6. |
| 552 | Counter. NAS-ID deleted. |
| 604 | Accepted |
| 605 | Counter. NAS-ID deleted. |
| 606 | Counter. NAS-ID deleted. |
| 607 | Accepted |
| 610 | Counter. NAS-ID deleted. |
| 611 | Accepted. Clause deleted. |
| 612 | Accepted |
| 615 | Accepted |
| 616 | Counter. Clause 7.3.2.50 deleted. |
| 617 | Counter. Clause 7.3.2.50 deleted. |
| 618 | Counter. Clause 7.3.2.50 deleted. |
| 620 | Accepted |
| 621 | Counter. Clause 7.3.2.50 deleted. |
| 622 | Accepted |
| 623 | Counter. Clause 7.3.2.51 deleted. |
| 624 | Accepted |
| 625 | Counter. Clause 7.3.2.51 deleted. |
| 626 | Counter. Clause 7.3.2.51 deleted. |
| 627 | Counter. Clause 7.3.2.51 deleted. |
| 628 | Counter. R1DISTV merged into FTIE. Clause 7.3.2.51 deleted. |
| 629 | Accepted |
| 630 | Counter. Clause 7.3.2.51 deleted |
| 631 | Counter. Clause 7.3.2.51 deleted. |
| 632 | Counter. Clause 7.3.2.51 deleted |
| 635 | Counter. Clause 7.3.2.51 deleted |
| 636 | Counter. Clause 7.3.2.51 deleted |
| 637 | Counter. Clause 7.3.2.51 deleted |
| 650 | Counter. R1DISTIE deleted |
| 651 | Counter. R1DISTIE deleted |
| 652 | Counter. R1DISTIE deleted |
| 653 | Counter. R1DISTIE deleted |
| 654 | Counter. R1DISTIE deleted |
| 655 | Counter. R1DISTIE deleted |
| 656 | Accepted |
| 657 | Counter. R1DISTIE deleted |
| 658 | Counter. R1DISTIE deleted |
| 663 | Counter. R1DISTIE deleted |
| 664 | Counter. R1DISTIE deleted |
| 665 | Counter. R1DISTIE deleted |
| 666 | Counter. R1DISTIE deleted |
| 667 | Counter. R1DISTIE deleted |
| 668 | Counter. R1DISTIE deleted |
| 669 | Counter. R1DISTIE deleted |
| 670 | Accepted |
| 671 | Counter. R1DISTIE deleted |
| 672 | Counter. R1DISTIE deleted |
| 673 | Counter. R1DISTVIE deleted |
| 674 | Counter. R1DISTVIE deleted |
| 675 | Counter. R1DISTVIE deleted |
| 676 | Counter. R1DISTVIE deleted |
| 789 | Counter. Clause 8.5A.6 restored from D2.2. Text changes given in 11-06-1596. |
| 855 | Accepted. Changed to KDF-640. R0KH-Distribution-Key=L(R0-Key-Data, 384,256) |
| 866 | Accepted |
| 868 | Counter. Removed "mk = …..¦¦ SPA)" sentence. Changed KDF-384 to KDF-640 in R0-Key-Data derivation, and create additional keying material. Then, insert R0KH-Distribution-Key = L(R0-Key-Data, 384, 256). Remove line 54 " - KDF-256 is the KDF … 256 bits". |
| 872 | Counter. Removed "mk = …..¦¦ SPA)" sentence. Changed KDF-384 to KDF-640 in R0-Key-Data derivation, and create additional keying material. Then, insert R0KH-Distribution-Key = L(R0-Key-Data, 384, 256). Remove line 54 " - KDF-256 is the KDF … 256 bits". |
| 873 | Rejected. Additional keying material is needed for securely pushing PMK-R1s to the R1KH |
| 874 | Counter. MK (or R0KH-Distribution-Key) can be derived just once, just like PMK-R0, or may be derived as often as the implementation desires (but will get the same result each time). No additional text is needed. |
| 876 | Counter. L(..256, 128), and definition of R0KH-Distribution-Key extracted from final 256 bits. |
| 888 | Counter. Line “Distribution of PMK-R1” deleted. |
| 889 | Counter. Line “Distribution of PMK-R1” deleted. |
| 890 | Accepted |
| 896 | Counter. Removed "mk = …..¦¦ SPA)" sentence. Changed KDF-384 to KDF-640 in R0-Key-Data derivation, and create additional keying material. Then, insert R0KH-Distribution-Key = L(R0-Key-Data, 384, 256). Remove line 54 " - KDF-256 is the KDF … 256 bits". |
| 920 | Counter. R1KH-ID consistently made 6 octets |
| 944 | Accepted. Text added to 8.5A.7 stating the assumptions being made. |
| 953 | Accepted. Text changes in 11-06-1596 |
| 955 | Accepted. Text changes in 11-06-1596 |
| 956 | Accepted. Text changes in 11-06-1596 |
| 958 | Accepted. Text changes in 11-06-1596 |
| 959 | Accepted. Text changes in 11-06-1596 |
| 974 | Accepted. Statement added to 8.5A.1. |
| 982 | Counter. Clause 8.5A.6 restored from D2.2. Text changes given in 11-06-1596. |
| 985 | Counter. Clause 8.5A.6 restored from D2.2. Text changes given in 11-06-1596. |
| 986 | Rejected. This protocol is out of scope, and stated as such in 8.5A.1. Assumptions about this protocol being made by TGr are contained in 8.5A.7 |
| 987 | Counter. Clause 8.5A.6 restored from D2.2. Text changes given in 11-06-1596. |
| 996 | Accepted. Statement added to 8.5A.1 |
| 999 | Accepted. Text changes in 11-06-1596 |
| 1000 | Accepted. Text changes in 11-06-1596 |
| 1001 | Rejected. STA calculation of R1DIST is required. |
| 1002 | Rejected. STA calculation of R1DIST is required. |
| 1003 | Rejected. STA calculation of R1DIST is required. |
| 1004 | Rejected. STA calculation of R1DIST is required. |
| 1005 | Rejected. STA calculation of R1DIST is required. |
| 1006 | Rejected. STA calculation of R1DIST is required. |
| 1007 | Rejected. STA calculation of R1DIST is required. |
| 1008 | Rejected. STA calculation of R1DIST is required. |
| 1009 | Rejected. STA calculation of R1DIST is required. |
| 1036 | Counter. Indicated text deleted |
| 1037 | Accepted. Text changes in 11-06-1596 |
| 1038 | Accepted. Text changes in 11-06-1596 |
| 1039 | Counter. Indicated text deleted |
| 1040 | Counter. Indicated text deleted |
| 1041 | Counter. Indicated text deleted |
| 1042 | Counter. Indicated text deleted. Other uses are in IEEE mandated boilerplate. |
| 1044 | Counter. All mention of a Mobility Domain Controller deleted from the draft. |
| 1045 | Counter. All mention of a Mobility Domain Controller deleted from the draft. |
| 1046 | Counter. All mention of a Mobility Domain Controller deleted from the draft. |
| 1049 | Accepted. All mention of a Mobility Domain Controller deleted from the draft. |
| 1050 | Counter. Paragraph on MDC deleted. Restored text from D2.2 gives assumptions about secure key transfers. |
| 1051 | Accepted. All mention of a Mobility Domain Controller deleted from the draft. |
| 1052 | Accepted. All mention of a Mobility Domain Controller deleted from the draft. |
| 1053 | Counter. All mention of a Mobility Domain Controller deleted from the draft. Clause 8.5A.6 restored from D2.2. Text changes given in 11-06-1596. |
| 1056 | Counter. All mention of a Mobility Domain Controller deleted from the draft. |
| 1057 | Accepted. All mention of a Mobility Domain Controller deleted from the draft. |
| 1058 | Counter. All mention of a Mobility Domain Controller deleted from the draft. |
| 1059 | Counter. All mention of a Mobility Domain Controller deleted from the draft. |
| 1060 | Counter. All mention of a Mobility Domain Controller deleted from the draft. |
| 1061 | Counter. All mention of a Mobility Domain Controller deleted from the draft. |
| 1062 | Counter. All mention of a Mobility Domain Controller deleted from the draft. |
| 1064 | Accepted. All mention of a Mobility Domain Controller deleted from the draft. |
| 1065 | Counter. All mention of a Mobility Domain Controller deleted from the draft. |
| 1066 | Counter. All mention of a Mobility Domain Controller deleted from the draft. |
| 1067 | Counter. All mention of a Mobility Domain Controller deleted from the draft. |
| 1068 | Counter. Indicated text deleted. |
| 1069 | Counter. Indicated text deleted. |
| 1070 | Accepted |
| 1071 | Counter. Indicated text deleted. |
| 1072 | Counter. Indicated text deleted. |
| 1075 | Accepted. Text changes in 11-06-1596 |
| 1076 | Counter. Indicated text deleted. |
| 1077 | Accepted. Text changes in 11-06-1596 |
| 1079 | Counter. Text in 8A.5.1 “The value of this optional parameter shall be the AES-KeyWrap of (Ns ¦¦ R1KH-ID), where Ns is a sixteen octet random number chosen by the STA, and R1KH-ID is the identity of the R1KH of the AP advertised in its beacons and probe responses.” |
| 1080 | Counter. NAS-ID deleted. |
| 1081 | Counter. NAS-ID deleted. |
| 1082 | Counter. NAS-ID deleted. |
| 1084 | Counter. NAS-ID deleted. |
| 1090 | Accepted. Statement added to 8.5A.7. Text changes in 11-06-1596 |
| 1094 | Counter. MK is not needed at the target AP. Text changed to “The target AP shall send the R1DIST value from the FTIE, along with the MAC address…” |
| 1095 | Counter. MK is not needed at the target AP. Text changed to “The target AP shall send the R1DIST value from the FTIE, along with the MAC address…” |
| 1096 | Rejected. This protocol is out of scope, and stated as such in 8.5A.1. Assumptions about this protocol being made by TGr are contained in 8.5A.7 |
| 1097 | Accepted. Text changes in 11-06-1596 |
| 1099 | Counter. NAS-ID deleted |
| 1106 | Accepted. Text changes in 11-06-1596 |
| 1109 | Rejected. This protocol is out of scope, and stated as such in 8.5A.1. Assumptions about this protocol being made by TGr are contained in 8.5A.7 |
| 1111 | Counter. Text changes in 11-06-1596 |
| 1113 | Accepted. Text changes in 11-06-1596 |
| 1115 | Counter. Text changes in 11-06-1596 |
| 1116 | Counter. Text changes in 11-06-1596 |
| 1131 | Counter. Text changes in 11-06-1596 |
| 1196 | Counter. NAS-ID deleted |
| 1197 | Accepted |
| 1353 | Accepted. Text changes in 11-06-1596 |
| 1354 | Accepted. Text changes in 11-06-1596 |
| 1364 | Accepted. Text changes in 11-06-1596 |
| 1379 | Accepted. Text changes in 11-06-1596 |
| 1380 | Accepted. Text changes in 11-06-1596 |
| 1453 | Accepted. Text changes in 11-06-1596 |
| 1454 | Counter. R1 distribution row deleted |
| 1456 | Counter. R1 distribution row deleted |
| 1459 | Counter. Text changes in 11-06-1596 |
| 1474 | Counter. Text in 8A.5.1 “The value of this optional parameter shall be the AES-KeyWrap of (Ns ¦¦ R1KH-ID), where Ns is a sixteen octet random number chosen by the STA, and R1KH-ID is the identity of the R1KH of the AP advertised in its beacons and probe responses.” |
| 1494 | Counter. Text changes in 11-06-1596 |
| 1497 | Counter. Text changes in 11-06-1596 |
| 1559 | Accepted |
| 1567 | Accepted |
| 1572 | Accepted |
| 1580 | Accepted |
| 1657 | Accepted |
| 2031 | Accepted. Text changes in 11-06-1596 |
| 2032 | Accepted. Text changes in 11-06-1596 |
| 2034 | Counter. IEs deleted. Text changes in 11-06-1596 |
| 2037 | Accepted. R1DIST and R1DISTV included in FTIE, and PC35.2. PC35.13 added. Text changes in 11-06-1596 |
| 2041 | Counter. NAS-ID deleted |
| 2042 | Counter. Key distribution protocol is mandatory. PICS entry 35.13 added. Text changes in 11.06-1596 |
| 2046 | Accepted. Text included in 8.5A.6. See 11-06-1596 |
| 2049 | Accepted. Text included in 8.5A.6. See 11-06-1596 |
| 2051 | Counter. NAS-ID deleted |
| 2052 | Counter. NAS-ID deleted |
| 2053 | Accepted. NAS-ID deleted |