IEEE P802.11
Wireless LANs

| Timing Measurement Enhancement for Synchronization of AV streams |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Kevin Stanton | Intel Corporation | JF2-53 2111 NE Elam Young Pkwy Hillsboro OR 97124 | (503) 712-1888 | kevin.b.stanton@ieee.org |
| Emily Qi | Intel Corporation | 2111 NE 25th Hillsboro OR 97124 | 503-264-7799 | Emily.h.qi@intel.com |

Abstract

AV media streaming requires accurate time synchronization when multiple networked devices render coupled streams (e.g. networked speakers) or when the media originates from a real-time source (e.g. source is from DVD player, tuner, or is multicast).  Such streaming requires that an accurate, common representation of time is distributed among the relevant STAs.

Today, existing 802.11v Presence Request/Response primitives cause timestamp difference measurements to be made when certain frames are received or transmitted.  This document describes enhancements to Presence Request/Response which make the timestamp capability usable for distributing the network clock between STAs and even non-WLAN stations (via p802.1AS).

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Background:

Synchronized and real-time media streaming applications require that link delays and clock offset be measured in such a way that time and frequency synchronization can be realized across LANs that include WLAN segments.  Example applications include wireless speakers, media “push-mode” and real-time/live performance streams.

It is intended that this submission add sufficient p802.11v capabilities to allow the synchronization of time and frequency across multiple 802 LANs by the application of p802.1AS, which is based on IEEE1588 Version 2.

While the current Presence Request/Response methods measure link delay, this submission proposes several additions which make the protocol suitable for measuring both the link delay and the clock offset between communicating STAs, primarily through the return of both timestamps captured by the responder rather than the difference between the timestamps.

# 4. Abbreviations and Acronyms

Insert the following new acronym in alphabetical order:

TAI		International Atomic Time

UTC		Coordinated Universal Time

Modify Clause 7.3.2.40.1 as indicated below:

        - Presence Indication Parameters field

The Presence Indication Parameters field contains STA presence reporting characteristics. The format of the Presence Indication Parameters field is shown in Figure v61

|  | Element ID (1) | Length (75) | Report Interval Units | Normal Report Interval | Normal number of frames per channel | In-Motion Report Interval | In-Motion number of frames per channel | Inter-frame Interval |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 2 | 1 | 2 | 1 | 1 |

- Figure v61 —Presence Reporting Parameters

The Report Interval Units field contains the units used for the Normal Report Interval field and In-Motion Report Interval field, as indicated in Table v-1.

- Table v-1—Report Interval Units Field

| Report Interval Units | Description |
| --- | --- |
| 0 | Seconds |
| 1 | Microseconds |
| 2 | Minutes |
| 3 | Hours |
| 4 - 255 | Reserved |

The Normal Report Interval is the time interval, expressed in the units indicated in the Report Interval Units field  seconds at which the STA reports or is expected to report its presence when the STA is stationary.

The Normal Number of Frames per Channel is the number of Presence Request frames per channel sent or expected to be sent by the STA at each Stationary Report Interval.

The In-Motion Report Interval is the time interval, expressed in the units indicated in the Report Interval Units field  seconds at which the STA reports or is expected to report its presence when the STA is in motion. If motion detection is not supported, this field is set to 0. The definition of motion and the means to determine motion are outside the scope of this standard.

The In-Motion Number of Frames per Channel is the number of Presence Report frames per channel sent or expected to be sent by the STA at each In-Motion Report Interval. If motion detection is not supported, this field is set to 0.

The Inter-frame Interval is the time interval, expressed in milliseconds between the transmissions of each of the Stationary or In-Motion frames per channel.

        - Presence Request Options Field

Add a new row to table v32 as indicated below:

- Table v32—Presence Request Option Bit description

| Bit | Included Field |
| --- | --- |
| 3 | Timing Offset Measurement |
| 4-15 | Reserved |

Modify Clause 7.3.2.40.7 as indicated below:

        - Timing Measurements field

The Timing Measurements field contains timing information. The format of the Timimg Measurements field is shown in Figure v67.

|  | Element ID (7) | Length (6 or 18) | Timestamp Difference | Timestamp Difference Units | Timestamp Difference Accuracy | Ingress Timestamp (optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 4 | 1 | 1 | 12 |

- Figure v67 —Timing Measurements Field

The Timestamp Difference field contains the time difference between the time that a unicast Presence Request frame was received from a STA, defined to occur at the PHY-RXEND.indication of the received Presence Request frame, and the time that the corresponding ACK frame was sent to the STA, defined to occur at the PHY-TXSTART.confirm of the ACK frame transmission.

The Timestamp Difference Units field contains the units for the timestamp difference field, as indicated in Table V35.

- Table v35—Timestamp Difference Units

| Timing Difference Units | Description |
| --- | --- |
| 0 | Microseconds |
| 1 | Hundreds of Nanoseconds |
| 2 | Tens of Nanoseconds |
| 3 | Nanoseconds |
| 4 | Tenths of Nanoseconds |
| 5 - 255 | Reserved |

The Timestamp Difference Accuracy field contains the expected standard deviation of the timestamp difference of the timestamp in the units indicated in the Timestamp Difference Units field.

The Ingress Timestamp field contains the time at which the unicast Presence Request frame was received by a STA, defined to occur at the PHY-RXEND.indication of the received Presence Request frame. The Ingress Timestamp field is composed of the time fields defined in Table v-2. The Ingress Timestamp field is present when the Options field in the Presence Requst Option element is set to the value of Timing Offset Measurenent, as defined in 7.3.2.40.3.

- Table v-2—Ingress Timestamp Field

| Octets | Type | Description |
| --- | --- | --- |
| 6 | Unsigned Integer | Seconds |
| 4 | Unsigned Integer | Nanoseconds (always less than 1e9) |
| 2 | Integer | Timescale Offset (used to compute UTC from TAI –these differ by the number of leap seconds applied to UTC since 1972) |

Insert the following text as Clause 11.15.4.5

11.15.4.5 Timing Offset Measurement Procedure

The Timing Offset Measurement is used to synchronize the time and/or the frequency of clocks as needed by Audio/Video and other applications.

A STA may request a Timing Offset Measurement from a peer STA that supports Timing Offset Measurement. To request a Timing Offset Measurement, the STA shall send a Presence Request frame with a Presence Parameters information element that includes the Presence Request Option to indicate that a Timing Offset Measurement is requred.

A STA that supports Timing Offset Measurement and receives Timing Offset Measurement request shall respond with Presence Response frames that include a Time Measurement and a Presence Status element indicating the result of the request.  If the responding STA is not able to provide Timing Offset Measurement, the STA shall send a response with a Presence Status element that indicates Incapable.