IEEE P802.11
Wireless LANs

| Shortening of R1KH-ID |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-10-25 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bill Marshall | TGr Editor | 180 Park Ave, Florham Park, NJ 07932 | 973-360-8718 | wtm@research.att.com |

Abstract

By other submissions, R1KH-ID is replacing NAS-ID in Beacons and Probe Responses.

This is a significant improvement to the problem of “Beacon Bloat”, in that R1KH-ID is only 6 bytes where the NAS-ID was 48.

R1KH-ID is stated in 8.5A.6 as being the MAC address of the physical entity that stores the PMK-R1. But the exact mechanism that produces a unique identifier is not critical; rather it is important that the same identifier be advertised both to the STA and to the R0KH.

To further reduce the bloat in Beacons, this submission proposes that the R1KH-ID be reduced to 4 octets. This submission proposes the changes needed to implement this change.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

By other submissions, R1KH-ID is replacing NAS-ID in Beacons and Probe Responses.

This is a significant improvement to the problem of “Beacon Bloat”, in that R1KH-ID is only 6 bytes where the NAS-ID was 48.

R1KH-ID is stated in 8.5A.6 as being the MAC address of the physical entity that stores the PMK-R1. But the exact mechanism that produces a unique identifier is not critical; rather it is important that the same identifier be advertised both to the STA and to the R0KH.

To further reduce the bloat in Beacons, this submission proposes that the R1KH-ID be reduced to 4 octets. This submission proposes the changes needed to implement this change.

Changes to D3.0 (as modified by either 11-06-1613, or 11-06-1596, or some other submission that made similar changes to the Beacon.)

3.97f:

In the definition of R1KH-ID, change 6 octets to 4 octets.

7.3.2.37:

In definition of sub-element for Neighbor Report, change length of R1KH-ID to 4. Three places.

7.3.2.46:

Change size of R1KH in Figure 112U to 4 octets.

Adjust the statement under Figure 112U for FTIE with only R1KH-ID being length 4.

8.5a.5:

Change description of R1KH-ID to be 4-octet identifier.

Final sentence, delete “and shall be MAC address of R1KH”

8.5a.6:

Change “The R1KeyHolder identity (R1KH-ID) shall be set to the MAC address of the physical entity that stores the PMK-R1 and uses it to generate the PTK. That same MAC address shall be used to advertise the PMK-R1 identity to the STA and the R0KH”

To “The R1KeyHolder identity (R1KH-ID) shall be set to a unique value within the Mobility Domain. That same value shall be used to advertise the PMK-R1 identity to the STA and the R0KH”

D:

In dot11FTR1KeyHolderID, change size (6) to (4).

Change resolution to comments #915,916, 917, 918, 919, 920, 921, 922, 923, 924, 935, 936, and 2049, from 6 octets to 4 octets.