IEEE P802.11
Wireless LANs

| General Encapsulation replacement for RRB |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-08-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bill Marshall | AT&T | 180 Park Ave, Florham Park, NJ 07932 | 973-360-8718 | wtm@research.att.com |

Abstract

LB87 Comment #1835 stated:

The communication between the STA and the target AP over the DS is broken into two phases - first the over the air-to-the-current-AP-using-action-frames, and then the RRB-over-the-DS phase. Why does the current AP have to be in the middle of this? It seems that the RRB function could be eliminated by introducing a more general encapsulation mechanism, such as that proposed in 06/832r3.

And had a proposed resolution as follows:

Incorporate the general encapsulation Ethertype proposed in 06/832r3, inserting a new clause 7.6, defining an FT Type field value. This is an extensible framework, enabling STA to DS communication and takes the current AP out of the middle.

This submission attempts to put some meat on this skeleton.

Overview

The intent of this submission is to enable encapsulation of Management frames inside Data frames, with the Destination Address being the BSSID of a target AP.  The currently association AP would see this as an ordinary Data frame and process it accordingly, sending it over the DS to the Destination Address in the payload.  The target AP would receive the frame in a manner similar to pre-authentication frames. The target AP extracts the embedded Management frame, processes it, and encapsulates the Management response frame in a similar way sending it along the reverse path back to the STA.

The following Management frames may be sent in this way: Authentication request/response, Probe request/response, and Action.

When sent over-the-air, Authentication frames, which are Management frames (type 00) with subtype 1011, contain a Source Address (SA), Destination Address (DA), BSSID, duration, and sequence control.  The frame body starts with the information contained in Table 7-16 (Authentication Algorithm, Authentication Transaction Seq num, etc).

Data frames sent over-the-air to the DS are type 10, with subtype dependent on QoS.  Like Management frames, they contain a Source Address (SA), Destination Address (DA), BSSID, duration, and sequence control. The frame body starts with the 14-octet 802 MAC header (6 octet DA, 6 octet SA, 2 octet type), followed by the payload (typically the IP header, TCP/UDP header, then data), and lastly by the CRC.

When a STA sends a Management frame encapsulated in a Data frame, it would contain the following:

MAC header:	To-DS = 1

Addr1 = BSSID

Addr2 = STA MAC address

Addr3 = AP MAC address

Duration/QoS Control as would appear in a normal data frame

Frame Body:	DA (6 octets) = Target AP BSSID

SA (6 octets) = STA MAC address

EtherType (2 octets) = new value assigned by IEEE

ContentType (2 octets) = 0001 (encapsulated management frame)

Mgmt hdr:	TargetAP = AP MAC address

STA MAC address = STA MAC address

FrameCtl = Mgmt frame type

Mgmt data:	Contents as given in Tables 8-19.

When the AP receives this over-the-air frame, and forwards it over-the-DS, it would contain:

MAC hdr:	DA (6 octets) = Target AP BSSID

SA (6 octets) = STA MAC address

Type (2 octets) = new value assigned by IEEE

ContentType (2 octets) = 0001 (encapsulated management frame)

Mgmt hdr:	TargetAP = AP MAC address

STA MAC address = STA MAC address

FrameCtl = Mgmt frame type

Mgmt data:	Contents as given in Tables 8-19.

When the target AP receives this over-the-DS frame, it extracts the Mgmt data and processes it. The response is sent over-the-DS containing:

MAC hdr:	DA (6 octets) = STA MAC address

SA (6 octets) = Target AP BSSID

Type (2 octets) = new value assigned by IEEE

Payload:	ContentType (2 octets) = 0001 (encapsulated management frame)

Mgmt hdr:	TargetAP = AP MAC address

STA MAC address = STA MAC address

FrameCtl = Mgmt frame type

Mgmt data:	Contents as given in Tables 8-19.

When the current AP receives this over-the-DS frame, it handles it as an ordinary data frame and sends it over-the-air to the STA:

MAC header:	From-DS = 1

Addr1 = STA MAC address

Addr2 = BSSID

Addr3 = Target AP BSSID

Duration/QoS Control as would appear in a normal data frame

Frame Body:	DA (6 octets) = STA MAC address

SA (6 octets) = Target AP BSSID

Type (2 octets) = new value assigned by IEEE

Payload:	ContentType (2 octets) = 0001 (encapsulated management frame)

Mgmt hdr:	TargetAP = AP MAC address

STA MAC address = STA MAC address

FrameCtl = Mgmt frame type

Mgmt data:	Contents as given in Tables 8-19.

Changes needed to 802.11 standard

The frame format for the generalized encapsulation mechanism needs to be defined. A new clause 7.6 seems to be a good place. This clause defines the frame format as Content-type (2 octets) and payload (variable length). A table in 7.6 will give possible content types, and for each will provide a pointer to a clause that describes its use.

Content-Type 0000 will be reserved.

Content-Type 0001 will be for encapsulated management frames, with pointer to 7.2.4.

Other type values reserved.

The frame format for the encapsulated management frames needs to be defined. A new clause 7.2.4 seems to be the best place.

Procededures for generating and receiving these encapsulated management frames need to be defined. A new clause 11.14 seems to be the best place.

The originating STA needs a few words about priority for these encapsulated frames in 11.14. Procedures that use them (like in 11A.3 and 11A.4) need to identify the mechanism they are using, with a cross reference to 11.14.

The current AP treats these just like data frames when received over-the-air from the STA. Nothing special. When receiving a response from the target AP over-the-DS, the current AP needs to give them the same priority as Management frames for the downlink. This text also in 11.14.

The target AP needs to recognize the DA matching their BSSID in the over-the-DS frames. The SME of the target AP would first get these frames, and examine the sub-type. This text also in 11.14.

For frames with sub-type of Encapsulated Management frame, the SME uses a MLME-REMOTE_MANAGEMENT.request interface to pass the frame down to the MAC. The MAC may respond itself (e.g., a Probe Response) and if so, provides the response via MLME-REMOTE_MANAGEMENT.confirm back to the SME, who encapsulates it and sends over-the-DS back to the STA.  If the MAC can’t respond itself (e.g., an Authentication request), the MAC uses the existing MLME-AUTHENTICATION.indication to pass the information to the SME; the SME responds with MLME-AUTHENTICATION.response, as it does presently in the current authentication procedures; the MAC turns that into MLME-REMOTE_MANAGEMENT.confirm; the SME encapsulates that and sends over-the-DS back to the STA. This text also in 11.14.

Change to TGr D7.0

Delete definition 3.125A, Remote Request Broker

Delete acronym RRB, Remote Request Broker

Insert a new clause 7.2.4 as follows:

7.2.4 Encapsulated management frames

The format of the frame body for an encapsulated management frame is as defined in Figure 7-18a.

|  | Target AP BSSID | STA MAC Address | Management Frame control | Management Frame contents |
| --- | --- | --- | --- | --- |
| octets | 6 | 6 | 2 | variable |

Figure 7-18a: Encapsulated management frame

The encapsulated Management frame is carried using the general encapsulation mechanism, defined in 7.6. The payload type field in the general encapsulation frame contains the value 0001.

Management frame encapsulation is used by a non-AP STA to exchange management frames (as defined in 7.3.2) with APs other than its currently associated AP.  The STA generates the management frame contents and encapsulates it in a Data frame (described in 7.2.2) with the frame body described by Figure 7-18a.  This data frame is sent over the WM to the AP with which the STA is currently associated, which forwards the data frame over the DS. The target AP sends the corresponding management response frame to the STA using the same encapsulation.

The Target AP BSSID is the BSSID of the AP involved in the exchange.

The STA MAC address is the MAC address of the STA involved in the exchange.

The Management Frame control is 2 octets, and is defined by Figure 19 in 7.1.3.1.  The Protocol Version field, Type field, and Subtype field shall be set according to 7.1.3.1. All remaining fields shall be set to zero.

The Management Frame content is one of the following:

- Probe request (see 7.2.3.8)

- Probe response (see 7.2.3.9)

- Authentication (see 7.2.3.10)

- Action.   (see 7.2.3.12 and 7.4)

Procedures for use of encapsulated management frames are given in 11.14.

Delete all changes to clause 7.3.1.11 from this amendment

Delete clause 7.4 and all its sub-clauses.

Insert a new clause 7.6 as follow:

7.6 General Encapsulation Mechanism

The format used for the general encapsulation mechanism for sending data and management information as data frames is defined below. The Ethertype xx-xx is used with these frames.

| Payload Type Field | Payload |
| --- | --- |
| 2 Octets | Variable |

Figure 7-102

EDITORIAL NOTE – Prior to publication of this amendment, an Ethertype will be allocated, and its value inserted above. Ethertypes are assigned by IEEE, see https://standards.ieee.org/regauth/ethertype/ETH-form.shtml.

Type Field Values:

Table 7-59

| Payload Type Field Value | Meaning |
| --- | --- |
| 0 | Reserved |
| 1 | Encapsulated Management Frames, see 7.2.4. |
| 2-65535 | Reserved |

Change the NOTE at end of 11A.1 as follows:

NOTE-- Throughout this clause, the notation Authentication-Request refers to an Authentication frame with the Authentication Transaction Sequence number set to 1; Authentication-Response refers to an Authentication frame with the Authentication Transaction Sequence number set to 2; Authentication-Confirm refers to an Authentication frame with the Authentication Transaction Sequence number set to 3; Authentication-Ack or Authentication-Acknowledgement refers to an Authentication frame with the Authentication Transaction Sequence number set to 4. The first parameter to the above four messages is the Authentication Algorithm, such as "Open System Authentication Algorithm" (or shortened to "Open"), or "Fast BSS Transition Authentication Algorithm" (or shortened to "FT"). The notation FT Request refers to an Authentication frame with the Authentication Transaction Sequence number set to 1, sent using the encapsulated management frame mechanism given in 7.2.4, and sent as a Data frame over the WM from the STA to the currently associated AP. The notation FT Response refers to an Authentication frame with the Authentication Transaction Sequence number set to 2, sent using the encapsulated management frame mechanism given in 7.2.4, and sent as a Data frame over the WM from the currently associated AP to the STA. The notation FT Confirm refers to an Authentication frame with the Authentication Transaction Sequence number set to 3, sent using the encapsulated management frame mechanism given in 7.2.4, and sent as a Data frame over the WM from the STA to the currently associated AP. The notation FT Acknowledgement or FT ACK refers to an Authentication frame with the Authentication Transaction Sequence number set to 4, sent using the encapsulated management frame mechanism given in 7.2.4, and sent as a Data frame over the WM from the currently associated AP to the STA.

Change Figures 11A-5, 11A-7, 11A-9, 11A-11 replaceing “STA, Target AP”in  the parameter list to FT Request/Response/Confirm/Ack with “FT”.

Change third paragraph of 11A.5.3 as follows:

To perform an over-the-DS Fast BSS Transition to a target AP, the STA and the target AP (through the current AP using the encapsulated management frame mechanism given in 7.2.4) shall perform the following exchange:

STA->Target AP: FT Request(FT, 0, RSNIE[PMKR0Name], MDIE, FTIE[SNonce, R0KH-ID])

Target AP->STA: FT Response(FT, Status, MDIE, FTIE[R0KH-ID, R1KH-ID, ANonce, SNonce], RSNIE[PMKR0Name])

Change second paragraph of 11A.5.5 as follows:

To perform an over-the-DS Fast BSS Transition to a target AP in a non-RSN, the STA and the target AP (through the current AP, using the encapsulated management frame mechanism given in 7.2.4) shall perform the following exchange:

STA->Target AP: FT Request(FT, 0, MDIE)

Target AP->STA: FT Response(FT, Status, MDIE)

Change third paragraph of 11A.6.3 as follows:

To perform an Over-the-DS Fast BSS Transition with resource request to a target AP, after completing the FT Request/Response exchange given in 11A.5.3 or 11A.5.5 the STA and target AP shall perform the following exchange via the current AP, using the encapsulated management frame mechanism described in 7.2.4:

STA->Target AP: FT Confirm (FT, 0, MDIE, FTIE[MIC, R0KH-ID, R1KH-ID, ANonce, SNonce], RSNIE[PMKR1Name], RIC-Request)

Target AP->STA: FT Acknowledgement (FT, Status, MDIE, FTIE[MIC, R0KH-ID, R1KH-ID, ANonce, SNonce], RSNIE[PMKR1Name], RIC-Response)

Changed the third and fourth paragraphs of 11A.8.1 as follows:

When a STA invokes the FT protocol, then the first two messages of the sequence are both carried in Authentication frames or both carried in encapsulated management frames, and are described in 11A.8.2 and 11A.8.3

When the STA invokes the FT resource request protocol, then the first four messages of the sequence are all carried in Authentication frames or all carried in encapsulated management frames, and are described in 11A.8.2 through 11A.8.5.

Delete 11A.10

Delete second sentence in second paragraph after Figure 11A-27, starting “For those that require processing by the MAC layer, it generates a MLME-RESERVATION_LOCAL.request…”

Delete 10.3.34.5

Delete 10.3.34.6

Change 10.3.35 as follows, and update the resolutions to LB87 comments #1995 (Counter. MLME-RESERVATION_LOCAL deleted), #1997 (Accepted. MLME-RESERVATION_LOCAL deleted), #2001 (Counter. MLME-RESERVATION_LOCAL deleted), #2003 (Accepted. MLME-RESERVATION_LOCAL deleted):

10.3.35 MLME SAP interface for remote management requests

10.3.35.1 MLME-REMOTE_MANAGEMENT.request

10.3.35.1.1 Function

This primitive is used by the SME to indicate to the MAC that it received an encapsulated management message from a remote STA, and that the MAC should process this management message as if it has been received over the wireless medium.

10.3.35.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-REMOTE_MANAGEMENT.request 	(

PeerMACAddress,

ManagementFrameType

Contents of Management Frame

)

Name	Type	Valid range	Description

PeerMACAddress	MAC Address	Any valid individual MAC address	Specifies the MAC address of the STA that issued the Action Frame.

ManagementFrameType	Octet string	Two octets	Specifies the Frame Control field of the Management Frame issued by the STA.

Content of Management Frame	Sequence of octets	As defined in 7.2.3	The Management Frame received from the STA.

10.3.35.1.3 When generated

This primitive is generated by the SME when it receives a general encapsulation packet over the DS, with Payload Type value 1, indicating an encapsulated Management frame.

10.3.35.1.4 Effect of receipt

Upon receipt of this primitive, the MAC interprets the management frame as if it had received the management message over the wireless medium, and generates MLME-REMOTE_MANAGEMENT.confirm with the response.

10.3.35.2 MLME-REMOTE_MANAGEMENT.confirm

10.3.35.2.1 Function

This primitive is used by the MAC to respond to a MLME-REMOTE_MANAGEMENT.request.

10.3.35.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-REMOTE_MANAGEMENT.confirm 	(

PeerMACAddress,

ManagementFrameType,

Content of Management Frame

)

Name	Type	Valid range	Description

PeerMACAddress	MAC Address	Any valid individual MAC address	Specifies the MAC address of the STA that is the destination of the Action Frame

ManagementFrameType	Octet string	Two octets	Specifies the Frame Control field of the Management Frame response issued by the MAC.

Content of Management Frame	Sequence of octets	As defined in 7.2.3	The Management Frame to send to the STA in response to its request.

10.3.35.2.3 When generated

This primitive is generated by the MAC when it has completed processing of a management frame that was received from a remote STA through the encapsulated Management frame mechanism.

10.3.35.2.4 Effect of receipt

Upon receipt of this primitive, the SME forwards the management frame response to the STA over the DS, using the encapsulated management frame mechanism.

Insert new clause 11.14 as follows:

11.14 Encapsulated management frames

11.14.1 Overview

An encapsulated management frame is a data frame containing a management frame, as described in 7.2.4. Examples are Probe Request, Disassociate, Deauthenticate, and Authentication frames which are generated by the non-AP STA and directed to an AP other than the one to which the STA is currently associated.  This provides a mechanism that allows a STA to communicate and examine capabilities of APs either outside of the radio range, or without leaving the current radio channel. In the following discussion, the currently associated AP is referred to as the current AP, and the destination of the management frame is referred to as the target AP.

An example of this exchange is shown in Figure 204a for an encapsulated Probe Request frame.

Figure 204a: Encapsulated Management frame: Probe Request/Response

11.14.2 Procedures at the non-AP STA

The encapsulated management frame shall be one of the types: Disassociation, Probe Request, Probe Response, Authentication, Deauthentication.

The STA should verify that the target AP is reachable over the DS. Such assurance is provided by the Mobility Domain Identifier being equal to that of the currently associated AP. Other mechanisms that provide this assurance are beyond the scope of this standard.

The SME of the STA generates the encapsulated management frame, and sends it to the MAC using the MA-UNITDATA.request primitive.  In a QoS STA, encapsulated management frames shall be sent using the access category AC_VO.

11.14.3 Procedures at the AP

When the AP receives a data frame over the WM with Ethertype matching the general encapsulation mechanism value, it does no special processing.  It is forwarded over the DS to the destination address given in the data frame, as would any other data frame.

When the AP receives a packet over the DS with (1) Ethertype matching the general encapsulation mechanism value, (2) destination address matching one of its associated STAs, and (3) Content type having the value 1, then the procedures of this paragraph apply. This is the case of the AP that is currently associated with the STA initiating the request having received a reply from the target AP. The AP shall forward the packet to the STA as a data frame.  In a QoS AP, encapsulated management frames shall be sent using the access category AC_VO.

When the AP receives a packet over the DS with (1) Ethertype matching the general encapsulation mechanism value, (2) destination address matching its BSSID, and (3) Content Type value 1, then the procedures of the remainder of this clause apply. This is the case of the target AP receiving the encapsulated management frame. The SME of the AP shall issue a MLME-REMOTE_MANAGEMENT.request primitive to the MAC to request processing of the management frame by the MAC.

When the MAC of the target AP has completed processing of the management frame, and it is a management frame that requires a response, it generates a MLME-REMOTE_MANAGEMENT.confirm primitive, which informs the SME of the results and the information elements to include in the response. The SME encapsulates the response and sends this response as a data packet over the DS to the STA that originated the request.

Processing of the management frame by the MAC may involve MLME primitives to the SME. Such would be the case for Authentication management frames; the SME responds to a MLME-AUTHENTICATE.indication with MLME-AUTHENTICATION.response primitive.

An example of the MLME interaction for an Authentication management frame is shown in Figure 204b.

Figure 204b: Encapsulated Management frame: Authentication frame

Annex A

Change row PC35.5

PC35.5 Encspsulated Management Frames 7.2.4, 7.6, 11.14  PC35:M  Y-N-N/A