IEEE P802.11
Wireless LANs

| MDIE Policy bit Definitions Extensions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Kapil Sood | Intel | 2111 NE 25th Ave JF3-206 Hilsboro OR 97124 | +1-503-264-3759 | Kapil.Sood@intel.com |
| Frank Ciotti | Motorola |  |  | frank.ciotti@motorola.com |

Abstract

This submission is an update to the definition of the policy bits in the Mobility Domain information element, and all the corresponding places in D3.0 that need to stay in sync.  This is an extention/modification to 11-06-1640-00.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Change Figure 112T a follows:

“Reservation over air” to “Fast Transition over-the-air”

“Reservation over DS” to “Fast Transition over-the-DS”

Define bit 3 as “Reservation prior to reassociation supported”

Change second paragraph under Figure 112T as follows:

The policy bits in the MDIE control the behavior of STAs performing a resource request as part of Fast BSS Transitions.

An AP sets Fast BSS Transition capability bit to one to indicate that Fast BSS Transition service is supported by that AP.  If the Fast BSS Transition capability bit is set to 0, then bits 1-3 will also be set to 0.

The AP sets FastTransition-over-air and/or FastTransition-over-DS according to the policy for Fast BSS Transition for the Mobility Domain. At least one of FastTransition-over-air or FastTransition -over-DS is advertised by the AP, if Fast BSS Transition Capability bit is set to one.

If Reservation-prior-to-reassociation-supported is set to one then the STA may perform the resource request protocol of 8A.4, prior to performing reassociation.

If FastTransition-over-air is set to zero, then the STA will not initiate an over-the-air Fast BSS Transition. If FastTransition-over-DS is set to zero, then the STA will not initiate an over-the-DS Fast BSS Transition.

[Note to TGr editor – move this final paragraph to the end of the paragraph describing over-the-air and over-the-DS. Moving it in this submission would obscure the changes being shown by MSWord.]

Change second paragraph of 8A.1.1 as follows:

The Mobility Domain Identifier shall be the value of dot11FTMobilityDomainID. The resource policy bits in the MDIE, Fast Transition over air, Fast Transition over DS, and Reservation prior to reassociation supported, shall be set according to the values of the MIB variables dot11FTOverAirEnabled, dot11FTOverDSEnabled, and dot11FTReservationPriorToReassocaitionSupported, respectively. If Fast BSS-Transition is supported, at least one of Fast Transition over air or Fast Transition over DS shall be set to one. The NAS-ID of the authenticator associated with the AP advertising Fast BSS Transition capability shall be the value of dot11FTNASID.

Insert a new paragraph at start of 8A.3.1:

A STA shall not initiate a Fast BSS Transition over-the-air to a target AP whose MDIE contained the Fast Transition over air bit set to zero.

Insert a new paragraph at start of 8A.3.2:

A STA shall not initiate a Fast BSS Transition over-the-DS to a target AP whose MDIE contained the Fast Transition over DS bit set to zero.

Change third paragraphof 8A.3.3 as follows:

The Information Elements in Fast BSS Transition Confirm and Fast BSS Transition Acknowledgement, and their required contents, shall be as given in 8A.5.3 and 8A.5.4. If the MDIE advertised by the target AP contained the Reservation-Prior-to-Reassociation-Required bit set to one, then the STA may perform the procedures as in 8A.4 prior to sending reassociation request.  If a STA chooses to do so, then STA shall not include a RIC in its Reassociation Request. .

Change third paragraph of 8A.4 as follows:

Availability of the reservation mechanism is advertised by the target AP in the MDIE. An AP that receives a reservation request from a STA, and does not support the reservation mechanism, shall respond with a status code 38 ("The request has not been successful as one or more parameters have invalid values").

Change fifth paragraph of 8A.5.1 as follows:

The MDIE shall contain the Mobility Domain Identifier, and the Fast BSS Transition Capability and policy settings obtained from the target AP. The target AP advertises the MDIE in beacons and probe responses. The Mobility Domain Identifier and all FT policy bits shall be identical to that obtained during the FT Initial Mobility Domain Association exchange.

Change MIB entries:

dot11FTReservationOverAirEnabled to dot11FTOverAirEnabled

dot11FTReservationOverDSEnabled to dot11FTOverDSEnabled

dot11FTReservationRequired to dot11FTReservationPriortoReassociationSupported