IEEE P802.11
Wireless LANs

| The Application of spatial multiplexing Technique |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-10-30 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Yuli Yang | Hisilicon Technologies | Beijing, China | 86-10-8288-2930 | yangyuli@huawei.com |

Abstract

This document comments to use the spatial multiplexing techniques in the multi-antenna systems.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Contents

1 Introduction	3

2 Motivation	3

3 Main Features	4

4 System Model	4

4.1 Antenna Structure	4

4.2 Encoding	4

4.3 Decoding	5

4.3.1 Basic Decoding Algorithm	5

4.3.2 Efficient Decoding Algorithm	7

4.3.3 Improved Decoding Algorithm	8

5 Conclusion	10

Reference	10

# 1 Introduction

It is well known that the multiple input multiple output (MIMO) structure allows much more system capacity gain than the single input single output (SISO) system. And, two opportunities by the use of multiple input multiple output (MIMO) channels can be realized, that is, the spatial diversity and the spatial multiplexing.

Now, in the IEEE 802.11n Standard, the Space Time Block Coding is optional, which is used to enhance the link reliability by capturing spatial diversity. While the spatial multiplexing schemes like BLAST pump multiple information data into MIMO channels for increased throughput, which can be considered for the application in future wireless local area network (WLAN) to provide extraordinarily high bit rates.

In this proposal, we intend to present the signification for the application of spatial multiplexing techniques, especially the Bell Labs Layered Space-Time (BLAST) schemes with successive interference cancellation decoding algorithm. Since the interference from the previous vectors with minimum estimation mean square error (MSE) is cancelled and that from the later vectors is suppressed, the BLAST techniques can be realized with simple encoding structure and decoding algorithm with low computational complexity.

# 2 Motivation

From the information-theoretic point of view, the ultimate limit of multiple-antenna systems has been explored.

If a multiple-antenna system has M transmit and N receive antennas and the narrowband slow fading channel can be modeled as an N×M matrix with i.i.d. complex Gaussian random entries, the average channel capacity of such a system is approximately min(N,M) times higher than that of a single input single output system for the same overall transmitted power.

In the application of the multiple independent channels, high throughput is a main object. And, from the simple transmission structure to the simplification of the decoding, the BLAST scheme becomes more and more available for practice.

BLAST has the potential to increase the capacity of the wireless link by a factor of min(N,M) and is applicable for fixed wireless access (as in a wireless LAN).

As is known, a practical consideration for BLAST is the decoding complexity. And, the transmitter need employ a channel code to guarantee good performance with the majority of fading channel realizations since the transmitter can not have the instantaneous information about the fading channel. Since multi-antenna communication allows for information transmission at very high rates, a major issue of concern is to keep the computational complexity of the decoding algorithm within reasonable bounds.

Obviously, maximum likelihood decoding, for example, is clearly beyond question since it typically requires a search over the prohibitively large set of all possible transmitted signals. Fortunately, some efficient decoding algorithms for BLAST have been invented.

# 3 Main Features

There are three main features that are detailed as follows.

    - Simple encoding architecture. Compared to the spatial diversity techniques, the spatial multiplexing techniques, such as BLAST, have very simple encoding architecture.

    - Flexibility. The BLAST can use the existing system structure to transmit much more throughput than the single input single output system, which makes high throughput realized by the use of the existing system equipments.

    - High data rate. The multiplexing technique can provide much higher data rate than the diversity techniques. This technique has the potential to improve the throughput with increase of the minimum of the number of transmit and receive antennas.

# 4 System Model

## 4.1 Antenna Structure

We refer to a multiple-antenna system in which the BS has M transmit antennas and the CPE has N receive antennas as an (M, N) system. The channels between the transmit antennas and receive antennas are independent and identical complex Gaussian distribution.

## 4.2 Encoding

The spatial multiplexing technique has very simple encoding structure, which allows for information transmission at very high rates. At the BS, the information data are divided into several streams and transmitted from multiple transmit antennas simultaneously.

The encoding process is described as follows. At first, the input information sequence is divided into N sub sequences, and each subsequence is subsequently encoded by a channel code encoder. The output of the coder is transmitted at the transmit antennas according to the models illustrated in Fig.1. In the figures, ct denotes the tth (t=0,1,⋯) output symbol from the channel coder.

- HLST

- DLST

Fig.1 Encoding process of BLAST

In Fig.1, (a) is the encoding process of the Horizontally layered space-time (HLST) structure and (b) is the encoding process of the Diagonally layered space-time (DLST) structure.

It is obvious that the interference between the signals from the transmit antennas of DLST structure is less than that of HLST structure. And however, the data rate of HLST structure is higher than that of DLST structure.

## 4.3 Decoding

### 4.3.1 Basic Decoding Algorithm

One major issue of concern for BLAST is to keep the computational complexity of the decoding algorithm within reasonable bounds.

The maximum likelihood decoding is the algorithm used widely, but it is clearly beyond question for it typically requires a search over the prohibitively large set of all possible transmitted signals. Therefore, the efficient decoding algorithms with successive interference cancellation are invented.

The receive signals can be expressed by an N×1 vector as

x=Hc+v,

where x=x1⋮xN is the received signal vector, and c=c1⋮cM is an M×1 vector for the transmitted signals from M transmit antennas. The equation is fit for both HLST and DLST. The only difference is that there are some zero-positions in some transmitted signal vectors for DLST.

H=H1⋮HN=h1⋯hM is the complex channel matrix with statistically independent entries, where Hi and hj are the ith row and jth column of H, respectively. And, v is the complex Gaussian noise vector with zero mean and covarianceσv2IN, where IN represents the identity matrix with size N.

The linear minimum mean squared error (MMSE) detection of c is

c=G⋅r=HσvIM+r0M,

where the MMSE filtering matrix G=P⋅HH with P=HHH+σv2IM−1, 0M for the M×1 zero vector, and superscripts “+” and “-1” for the pseudo-inverse and inverse operations respectively. It is easy to show that P is the covariance matrix for the detection error c−c. Denoting the first N columns of the pseudo-inverse HσvIM+ by Hα+ and the ith row of Hα+ by Hα,i+, then we have c=Hα+⋅r where the ith element of c is ci=Hα,i+⋅r and Hα,i+is often referred to as the ith MMSE nulling vector.

The conventional V-BLAST detection detects M elements of c iteratively with the optimal ordering. According to the MMSE detection of c, the best detected element of c would be the one with the smallest error variance, i.e., the one for which Pii is the smallest. It can be used to improve the detection of remaining M−1 signals. Suppose that the order of the entries of c is arranged such that this best detected element is the Mth entry. Treating cM as a known quantity, we obtain the following reduced order problem:

r−hMcM=H(M−1)c(M−1)+v

where H(M−1)=h1⋯hM−1 is the deflated channel matrix and c(M−1)=c1c2⋯cM−1T is the reduced signal vector. The solution is now given by

c(M−1)=Hα(M−1)+r−hMcM

and the corresponding error covariance is P(M−1)=H(M−1)HH(M−1)+σv2I(M−1)−1. The detection will proceed until all elements are detected.

The basic decoding algorithm for BLAST can be summarized as the following:

- Find P=HHH+σv2IM−1 and Hα+;

- Find the smallest diagonal entry of P and reorder the entries of c so that the smallest diagonal entry is the last Mth one.

- Form the least-mean-square estimate cM=Hα,M+r.

- Obtain cM (via slicing) from cM=Hα,M+r.

- Cancel the effect of cM and consider the reduced-order problem.

- Continue to find P(M−1) and Hα(M−1)+, and so on.

### 4.3.2 Efficient Decoding Algorithm

In the basic decoding algorithm, the main problem is that the dominant portion, nearly 90%, of the computation complexity involves determining the nulling vectors and optimal ordering in the steps 1 and 6 in the algorithm. Motivated by this, an efficient square-root decoding algorithm is proposed to compute P(m) and Hα(m)+ (m=M,⋯,1, P(M)=P, and H(M)=H) iteratively utilizing unitary transformations and avoiding time-consuming inversions. Since it is a unitary transformation based algorithm without any inversion or squaring operation, it is numerically stable and robust.

Consider the QR decomposition of the augmented channel matrix:

HσvIM=QR=QαQ2R,

where Q is an (N+M)×M matrix with orthonormal columns and partitioned into N×M Qα and M×M Q2, and R is M×M and non-singular. And, P=R−1⋅R−1H and hence R−1 can be identified as a “square-root” of P, i.e., R−1=P12 and P12⋅P12H=P. Therefore, Hα+=P12QαH.

This means the initial nulling vectors may be obtained via P12 and Qα. In fact, for all the reduced order problems with m=M,M−1,⋯,1, the nulling vectors may be obtained by P(m−1)2 and Qα(m−1), where P(m−1)2 is a square-root of P(m−1) and Qα(m−1) is the first (m−1) columns of Qα(m) after a unitary transformation. Meanwhile, the optimal ordering may also be found using P(m−1)2, since the minimum length row of P(m−1)2 corresponds to the minimum diagonal entry of P(m−1). That is, both nulling vectors and the optimal ordering can be found through P(m)2 and Qα(m) (m=M,⋯,1, P(M)2=P12, Qα(M)=Qα). Actually, the square-root algorithm provides an efferent way to calculate P(m)2 and Qα(m) iteratively utilizing unitary transformations.

This algorithm may be summarized as follows.

1) Compute square root of P, i.e., P12 and Qα:

Form the so-called (M+N+1)×(M+1) pre-array

Ωi−1=1HiP|i−1120MP|i−112−eiBi−1

and propagate the square-root algorithm N times:

Ωi−1⋅Θi=0MTP|i12Bi, P|012=1σvIM, B0=0N×M

where ei is an N×1 vector of all zeros except for the ith entry which is unity, P|i12 is the square root of an M×M matrix P|i and P|N12=P12, P|i=j=1iHjH⋅Hj+σv2⋅IM−1, Bi is an N×M sub matrix of Ωi and BN=Qα, “”denotes not relevant entries at this time, and Θi is any unitary transformation that block lower triangularizes the pre-array Ωi.

2) Find the minimum length row of P12 and permute it to be the last Mth row. Permute a accordingly.

3) Find a unitary transformation Σ such that P12Σ is block upper triangular

P12Σ=P(M−1)2PM(M−1)20M−1TpM12

where PM(M−1)2 and pM12 denote the last (M−1)×1 sub-column and the (M, M)th scalar entry, respectively.

4) Update Qα to QαΣ.

5) The nulling vector for the Mth signal is given by pM12qα,MH, where qα,M is the Mth column of Qα.

6) Go back to Step 2, but now with P(M−1)2 and QαM−1 instead of P12 and Qα.

### 4.3.3 Improved Decoding Algorithm

The square-root decoding algorithm introduced in the last sub-section is efficient and numerically stable in the sense that it uses unitary transformations to avoid computing any inverse (or pseudo-inverse, QR decomposition) or squaring. However, it can be seen from the steps of the algorithm that there is still room to increase its efficiency. For example, the algorithm computes the whole nulling matrices Qαm for each deflated subchannel matrix, while only one column of each is used (i.e., the optimum mulling vector); the intermediate results Pm(m−1)2 computed in the algorithm are discarded without any usage. Then, considering these issues, an improved square-root algorithm is proposed, in which the optimum nulling vectors are found with the help of Pm(m−1)2, avoiding the Qαm computation. Most important, this algorithm is maintained without any inverse or squaring operation.

Since P=P12⋅P12H, the MMSE filter G can be written as

G=P⋅HH=P12⋅P12H⋅HH.

Noticing the transform Σ (Step 3 in the square-root algorithm) is unitary and Σ⋅ΣH=IM, so

G=P12Σ⋅P12ΣH⋅HH

which, when

P12Σ=P(M−1)2PM(M−1)20M−1TpM12

is incorporated, yields

G=P(M−1)2PM(M−1)20M−1TpM12⋅P(M−1)2PM(M−1)20M−1TpM12H⋅HH.

Therefore the optimal nulling vector for the Mth signal, which is the Mth row of G is given by

pM12⋅PM(M−1)2HpM12H⋅HH.

This implies that the optimal nulling vector may be obtained utilizing PM(M−1)2 instead of Qαm.

The steps of this algorithm may be summarized as follows.

1) Compute P12.

Form the so-called (M+1)×(M+1) pre-array

Ωi−1=1HiP|i−1120MP|i−112

and propagate the following square-root algorithm

Ωi−1⋅Θi=0MTP|i12, P|012=1σvIM.

2) Find the minimum length row of P12 and permute it to be the last Mth row. Permute a and H accordingly.

3) Find a unitary transformation Σ such that P12Σ is block-upper triangular.

4) The nulling vector for the Mth signal is given by pM12⋅PM(M−1)2HpM12H⋅HH.

5) Go back to step 2, but now with P(M−1)2 and H(M−1), instead of P12 and H.

Compared to the traditional square-root algorithm, the computational efficiency of this algorithm is improved 36% in terms of the number of multiplications and additions required. At the same time, the advantages of the traditional square-root algorithm, such as robustness, hardware friendliness, and numerical stability have been maintained.

# 5 Conclusion

In this comment, we suggest to apply the spatial multiplexing technique in the multi-antenna systems.

Traditionally, the spatial diversity is used in most conditions. Considering the high processing ability of DSP, we comment to use the spatial multiplexing technique in the multi-antenna systems because the spatial multiplexing technique has the excellent characters of simple encoding and transmission structure and the the computation complexity of the decoding algorithm becomes acceptable now.

In conclusion, the spatial multiplexing technique, like BLAST, is available for multi-antenna systems.

# Reference

- G.J.Foschini, “Layered space-time architecture for wireless communication in a fading environment when using multi-element antennas”, Bell Labs Tech. J., pp.41-59, Autumn, 1996.

- D.Shiu and J.M.Khand, “Layered Space-Time Codes for Wireless Communications Using Multiple Transmit Antennas”, ICC 99. Vancouver:1999. pp.436-440.

- B.Hassibi, “An Efficient Square-Root Algorithm for BLAST”, in Proc. IEEE Int. Conf. Acoustics, Speech, Signal Processing, (ICASSP’00), June 2000, pp.737-740.

- H.Zhu, Z.Lei and F.Chin, “An improved square-root algorithm for BLAST”, IEEE Signal Processing Letters, Sep.2004, pp.772-775.