IEEE P802.11
Wireless LANs

| TGn LB84 Submission for Issues in the TXVECTOR and RXVECTOR |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Eldad Perahia | Intel Corporation |  |  | eldad.perahia@intel.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the issues in the TXVECTOR and RXVECTOR

The changes marked in this document are based on TGn Draft version P802 11n D1.05.pdf.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# Proposed Resolution

There is currently no SERVICE field in the TXVECTOR for HT Format.

TGn Editor: In D1.05, pg 186, modify Table n63  as follows

| SERVICE | FORMAT is NON_HT and NONHT MODULATION is either OFDM or DSSS-OFDM | Scrambler initialization, 7 null bits + 9 reserved null bit | Y | N |
| --- | --- | --- | --- | --- |
|  | FORMAT is HT_MM or HT_GF | Scrambler initialization, 7 null bits + 9 reserved null bit | Y | N |
|  | Otherwise | Not present | N | N |

We should define MCS values 77-127 as reserved, similar to the change made to STBC field.

TGn Editor: In D1.05, pg 186, modify Table n63  as follows

| MCS | FORMAT is HT_MM or HT_GF | The MCS field selects the modulation and coding scheme to be used in the transmission of the packet. The value to be used in each modulation and coding scheme is the index defined in 21.6 (Rate dependent parameters for HT Modulation and Coding Schemes (MCS)). Integer: range 0-76. Values of 77-127 are reserved. The interpretation of the MCS index is defined in 21.6 (Rate dependent parameters for HT Modulation and Coding Schemes (MCS)) | Y | Y |
| --- | --- | --- | --- | --- |

Fix CW per editor note. In addition, shouldn’t CW be in TXVECTOR for NON_HT, otherwise what is the point of defining it?  Clarity CW value for 40MHz Upper and Lower Mode.

TGn Editor: In D1.05, pg 186, modify Table n63  as follows

| CW | FORMAT is HT_MM or HT_GF | The CW parameter indicates whether the packet is transmitted using 40 MHz or 20 MHz channel width. Transmission over 40 MHz in non-HT duplicate format may also be indicated. (Ed: Including non-HT duplicate here causes a conflict with the definition of the FORMAT parameter, which says that non-HT duplicate is part of the NON_HT format.)(Ed: HT Duplicate removed according to CIDs 12, 7910) Enumerated type: HT_CW20, for 20 MHz HT_CW40, for 40 MHz For 40 MHz Upper and Lower modes the value for CW is HT_CW20 (Ed: removed HT_CW_20DN for Non-HT Duplicate due to conflict with definition of FORMAT parameter, which includes non-HT duplicate as part of NON_HT format) (Ed: HT_CW_20DH removed according to CIDs 12, 7910) | Y | Y |
| --- | --- | --- | --- | --- |
|  | FORMAT is NON_HT | Enumerated type: NON_HT_CW40 for Non-HT Duplicate NON_HT_CW20 for all other Non-HT (Ed: added NON_HT case to cover non-HT duplicate in a consistent manner) | NY | N |

CH_OFFSET should be contingent on CW. Can we determine CH_OFFSET on RX?

TGn Editor: In D1.05, pg 186, modify Table n63  as follows

| CH_OFFSET | FORMAT is HT_MM or HT_GF | The CH_OFFSET parameter indicates which portion of the 40 MHz channel is used for transmission. Enumerated type: CH_OFF_40 indicates the entire 40 MHz channel CH_OFF_20U indicates the upper 20 MHz of the 40 MHz channel CH_OFF_20L indicates the lower 20 MHz of the 40 MHz channel CH_OFF_20 indicates the use of a 20 MHz that is not part of a 40 MHz channel | Y | YN |
| --- | --- | --- | --- | --- |

Resolve logical problem per editor note.  ANTENNA_SET_ON will be added to TXVECTOR  if resolution in 06/1580r0 is approved. ANTENNA_SET_ON indicates whether ANTENNA_SET is valid.

TGn Editor: In D1.05, pg 186, modify Table n63  as follows

| ANTENNA_SE T | FORMAT is HT_MM or HT_GF | The ANTENNA_SET parameter indicates which antennas of the available antennas are used in the transmission. The length of the field is 8 bits. A 1 in the nth bit from LSB indicates that the nth antenna is used. At most 4 bits out of 8 may be set to 1. (Ed: CID 145, 150, 7380) This field is only present if antenna selection is applied.(Ed: CID 7049, 4665, 3438) (Ed: note that there seems to be a logical problem here in that there is no way defined of indicating whether or not AS is to be applied) | O | N |
| --- | --- | --- | --- | --- |