IEEE P802.11
Wireless LANs

| Waveform generator code for 802.11n |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Micha Anholt | Metalink |  |  | micha_a@metalinkbb.com |

Abstract

This document contains the interface and matlab code for the command line version for the 802.11n waveform generator.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Waveform Generator Description

## Introduction

The proposed phy of IEEE 802.11n standard has an enourmous number of different modes.

There are 20 Mhz modes, 40Mhz modes, duplicate modes SSB modes. Additionaly a packet may be generated using BCC or LDPC coding, STBC coding. Additional features are beamforming and short GI.

In order to reduce the amount of interoperability issues and assist reader of the spec it has been proposed the add to the standard an implementation of a waveform generator that will be able to generate all the different modes described in the spec.

The proposal and the discussions have been posted  on the server as documents 11-06-1096-00-000n-waveform-generator-ppt.ppt

, 11-06-1400-01-000n-waveform-generator.ppt

, 11-06-1714-00-000n-waveform-generator.ppt

.

The proper way to submit the files is been worked out by TGn chair Bruce Kramer. At the moment, as there are no procedures on how to submit these kind of files, only the command line version will be submitted as text in this document. The GUI interfaces will be submitted after the have instruction on how to submit them.

## Interface description

## Input the wave generation function

The input to the generator is a structure which defines the packet parameters. Here is a list of the possible structure elements that may be used:

### Data bit source definition

| parameter name | possible values | meaning |
| --- | --- | --- |
| BitSource | empty string | get bits randomly using matlab random function |
|  | string containing file name | read bits from file (hex byte per line) |

### TXVECTOR parameters

The following parameters names have been chosen to match the TXVECTOR definition. However, for clarity, the enumerations have been replaced by strings.

| parameter name | possible values | meaning |
| --- | --- | --- |
| FORMAT | 'NON_HT' | non ht packet |
|  | ‘HT_MM’ | mixed mode packet |
|  | ‘HT_GF’ | Greenfield packet |
| L_LENGTH | 0 | calculate according to spec |
|  | 1-4095 | override lsig value with this value |
| LENGTH | 0 | ZLF packet |
|  | 1-65535 | packet size in bytes. If read from file LENGTH will be overrided by length of file data |
| MCS | 0-76 | For ht packet this is mcs value, for non ht 0-7 are mapped to non ht rates. |
| SHORT_GI | 0,1 |  |
| STBC | 0,1,2 |  |
| NUM_EXTEN_SS | 0,1,2,3 |  |
| SMOOTHING | 0,1 |  |
| SOUNDING | 0,1 |  |
| LDPC_CODING | 0,1 |  |
| AGGREGATION | 0,1 |  |
| CW | 'HT_CW20' |  |
|  | 'HT_CW40' |  |
|  | 'HT_CW20DN' |  |
| CW_OFFSET | 'CH_OFF_40' |  |
|  | 'CH_OFF_20U' |  |
|  | 'CH_OFF_20L' |  |
| SERVICE.ScramblerStart | 1-127 | oldest bit is msb |
| SERVICE.bits | a vector of 16 bits |  |

Number of transmit antennas

| parameter name | possible values | meaning |
| --- | --- | --- |
| nTx | 1-4 | number of transmit chains |

Reserved bit from HT_SIG

| parameter name | possible values | meaning |
| --- | --- | --- |
| HT_SIG_RESERVED | 0,1 | value of reserved bit. Should be set to 1. 0 value can be used to verify that receiver ignores this bit. |

Output rate in MHz

| parameter name | possible values | meaning |
| --- | --- | --- |
| output_rate | 20,40,80 | sample rate at output in Mhz. |

Window function vector

| parameter name | possible values | meaning |
| --- | --- | --- |
| win | [] – empty vector | no windowing |
|  | win array | samples that represent window shape in transition rise area. Will be mirrored and duplicated for fall area. |

- win – window function vector. Use [] to disable windowing.

Spatial expansion configuration

| parameter name | possible values | meaning |
| --- | --- | --- |
| calib | 0,1 | 1 means generate spatial expansion according to calibration packet |
| BF_Q_source | 'default' | Uses default spatial expansion from draft examples combined with cdd vector values |
|  | 'const+cdd' | use fixed matrix provided by FixedQ var with cdd |
|  | 'general' | use carrier dependent spatial expansion matrices matrices supplied externally by the Q variable. |
| FixedQ | nTx by nSTS matrix | external fixed matrix to be used by 'const+cdd' option |
| post_q_cdd | cdd vector | cdd vector to be used as part of spatial expansion , after multiplication by the matrix. Should be set to 0 vector when using the 'general' option with matrices supplied externally |
| Q | a set of 64 or 128 matrices of size nTx by nSTS | a set of 64 or 128 matrices of size nTx by nSTS to be used for spatial expansion. Matrices should be ordered from carrier 0 to carrier 63 or 127 |

Explanation of channel modes:

| Mode | CW | CW_OFFSET | comments | possible output rates |
| --- | --- | --- | --- | --- |
| 20Mhz | CW20 | N/A | HT and Legacy | 20,40,80 |
| 40Mhz | CW40 | CH_OFF_40 | including mcs32 | 40,80 |
| 40Mhz Duplicate | CW20DN | CH_OFF_40 | only legacy | 40,80 |
| 40Mhz Upper | CW40 | CH_OFF_20U |  | 40,80 |
| 40Mhz Lower | CW40 | CH_OFF_20L |  | 40,80 |

## Running the generator

The function tx_n_highlevel generates the waveform. The first output is the waveform as an array with the first dimension as the antennas and the second dimension the number of samples. The second optional output is the debug structure, arranged as hierarchy of structures containing the signals at all intermediate points between blocks.

Exaples:

total_td_sig=tx_n_highlevel(sim80211nconfig);

[total_td_sig,dbinfo]=tx_n_highlevel(sim80211nconfig);

## Code

The matlab code attached here can be used in several ways. The natural way to use it is to separate it into the different files. There is a text line separator within the block of code that defines the name and limits of each file. Then it may be used either within the command line version or with the GUI interface.

Alternatively, the lazy option is to paste the whole block of code to tx_n_highlevel.m . In this case all the functions of the other files will internal function to this specific file, but the generator can be used.

%%%%%%%%%%%%%%%    Start of File tx_n_highlevel.m

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%5

% function tx_n_highlevel(sim80211nconfig)

%     Input : sim80211nconfig input packet info structure (see documentation)

%     Output:  total_td_sig - time domain signal (nTx x Nsamples)

%              debug_info (optional) - a structure of debug vectors

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%5

function [total_td_sig,debug_info] = tx_n_highlevel(sim80211nconfig)

global tx_n_debug

tx_n_globals

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%5

% Read or Prepare bit stream

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

if (length(sim80211nconfig.BitSource)==0)

source_bits = create_random_bits(sim80211nconfig.LENGTH*8);

else

source_bits = read_bits(sim80211nconfig.BitSource);

sim80211nconfig.LENGTH = length(source_bits)/8;

end

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

%

%   Check consistency and calculate derived parameters

%

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

sim80211nconfig = mcs_info(sim80211nconfig);

sim80211nconfig = init_bf (sim80211nconfig);

tx_n_debug.full_packet_info = sim80211nconfig;

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

%

%   Save tx bits in file

%

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

write_bits(source_bits);

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

%

%   Generate the different segments of the transmission

%

%

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

tx_data_sig    =      generate_data_tx(sim80211nconfig,source_bits);

td_lstf        =      generate_lstf(sim80211nconfig);

td_lltf        =      generate_lltf(sim80211nconfig);

if (~sim80211nconfig.isGF)

td_lsig    =      generate_lsig(sim80211nconfig);

end

if ~(sim80211nconfig.islegacy)

td_htstf   =      generate_ht_stf(sim80211nconfig);

end

% Prepare HT-SIG

if ~(sim80211nconfig.islegacy)

td_htsig   =      generate_ht_sig(sim80211nconfig);

end

if ~(sim80211nconfig.islegacy)

[htlfs_sig_cp,htlf1_sig_cp]    =   generate_ht_ltfs(sim80211nconfig);

end

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

%

%   Merge data and preamble segments

%

%

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

if (sim80211nconfig.islegacy)

total_td_sig = merge_segments(sim80211nconfig.win,td_lstf,td_lltf,td_lsig,tx_data_sig);

else

if (sim80211nconfig.isMM)

total_td_sig = merge_segments(sim80211nconfig.win,td_lstf,td_lltf,td_lsig,td_htsig,td_htstf,htlfs_sig_cp,tx_data_sig);

else % GF

total_td_sig = merge_segments(sim80211nconfig.win,td_htstf,htlf1_sig_cp,td_htsig,htlfs_sig_cp,tx_data_sig);

end

end

if (nargout>1)

debug_info = tx_n_debug;

end

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File PM.m

% function pmat=PM(z,n)

% generate cyclic permutation matrix of size n x n with shift z

function pmat=PM(z,n)

a=zeros(1,n*n);

if (z>=0)

a((0:n-1)*n+(0:n-1)+1)=1;

b=reshape(a,n,n);

pmat=b(:,1+mod((0:n-1)-z,n));

else

pmat=reshape(a,n,n);

end

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File add_cdd.m

% add_cdd

% Input: pilot_and_data_symbols fft_size x n_sym x nsts

%        cdd_vec

% Output:

%       symbols_cdd fft_size x n_sym x nsts

%

function symbols_cdd = add_cdd (pilot_and_data_symbols,cdd_vec)

nsts =     size(pilot_and_data_symbols,3); % or ntx

nsym =     size(pilot_and_data_symbols,2);

fft_size = size(pilot_and_data_symbols,1);

%symbols_cdd = zeros(size(pilot_and_data_symbols));

symbols_cdd = zeros([fft_size nsym nsts]);

for ists = 1:nsts

for isym=1:nsym

t_exp = -cdd_vec(ists)/50; % number of samples for 64 fft

symbols_cdd(:,isym,ists) = pilot_and_data_symbols(:,isym,ists).*(exp(j*2*pi*t_exp/64*(0:fft_size-1)'));%*ones(1,nsym));

end

end

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File add_cp.m

%function td_sig = add_cp(fft_out,shortgi)

% input samples  nfft x nsym x nTx

%       shortgi  0 = normal

%                1 = short

%                2 = double cp and double symbol

function td_sig = add_cp(fft_out,shortgi)

nfft = size(fft_out,1);

nTx =  size(fft_out,3);

nsym = size(fft_out,2);

if (shortgi==2) % This is the preamble. single symbol double cp

cplen = nfft/2;

sym_size = 2*nfft+cplen;

td_sig = zeros(1,nTx,sym_size);

for iTx = 1:nTx

td_sig(1,iTx,1:cplen) = conj(fft_out(nfft-cplen+1:nfft,iTx)');

td_sig(1,iTx,(1+cplen):(cplen+nfft)) = conj(fft_out(:,iTx)');

td_sig(1,iTx,(1+cplen+nfft):(cplen+2*nfft)) = conj(fft_out(:,iTx)');

end

return

end

if (shortgi==1)

cplen = nfft/8;

else

cplen = nfft/4;

end

sym_size = nfft+cplen;

td_sig = zeros(nsym,nTx,sym_size);

for isym = 1:nsym

for iTx = 1:nTx

td_sig(isym,iTx,1:cplen) = conj(fft_out(nfft-cplen+1:nfft,isym,iTx)');

td_sig(isym,iTx,1+cplen:sym_size) = conj(fft_out(:,isym,iTx)');

end

end

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File add_legacy_cdd.m

%function symbols_cdd = add_legacy_cdd (pilot_and_data_symbols)

% adds the cdd for the legacy packet or legacy part of packet

function symbols_cdd = add_legacy_cdd (pilot_and_data_symbols)

global T_ITX_CS; %table n65

nTx =     size(pilot_and_data_symbols,3);

symbols_cdd = add_cdd(pilot_and_data_symbols,T_ITX_CS(nTx,1:nTx));

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File add_sts_cdd.m

% function symbols_cdd = add_sts_cdd (pilot_and_data_symbols)

% adds cdd which is defined per space time stream

%

function symbols_cdd = add_sts_cdd (pilot_and_data_symbols)

global T_ISTS_TS;

nsts        =   size(pilot_and_data_symbols,3);

symbols_cdd =   add_cdd(pilot_and_data_symbols,T_ISTS_TS(nsts,1:nsts));

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File apply_win.m

% symbol_windowed = apply_win(win,symbol)

% apply window on symbol

% Input : win - window function

%         symbol - nTx x symbol_size samples

% Ouput: windowed symbol  nTx x (symbol_size+2*length(win))

function    symbol_windowed = apply_win(win,symbol)

sym_size = size(symbol,2);

nTx      = size(symbol,1);

period = 64*floor(sym_size/64);

winlen = length(win);

padpre  = floor((winlen-1)/2);

padpost = ceil ((winlen+1)/2);

pad_left_sig = symbol(:,period-padpre+1:period);

pad_right_sig = symbol(:,sym_size+1-period:sym_size+padpost-period);

win_ntx = repmat(win,nTx,1);

symbol_expanded = [pad_left_sig symbol pad_right_sig ];

symbol_windowed = symbol_expanded;

symbol_windowed(:,1:winlen)=symbol_windowed(:,1:winlen).*win_ntx;

symbol_windowed(:,sym_size+1:sym_size+winlen)=symbol_windowed(:,sym_size+1:sym_size+winlen).*fliplr(win_ntx);

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File calculate_generator_matrix.m

% function generator=calculate_generator_matrix(blk_size,code_rate_enum)

%  calculates generator matrix

function generator=calculate_generator_matrix(blk_size,code_rate_enum)

% Get code table from the spec

[tab,z]=getcodetable(blk_size,code_rate_enum);

n=blk_size;

n_min_k=size(tab,1)*z;

k=n-n_min_k;

% translate table to parity check matrix

parity_check_table=zeros(size(tab)*z);

for ii=1:size(tab,1)

for jj=1:size(tab,2)

parity_check_table((ii-1)*z+1:ii*z,(jj-1)*z+1:jj*z)=PM(tab(ii,jj),z);

end

end

% translate to standard form

% inverse over reals must contain integer entries so we can use matlab

% inverse functions

generator = mod (inv(parity_check_table(:,(k+1:n)))*parity_check_table(:,(1:k)),2);

generator = round(generator); % make sure enries are integer and not almost integers

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File conv_enc.m

% function conv_enc_bits=conv_enc(inp_bits)

%  input : bits for convolutional encoder

%  outpu : bits after convolutional encoder as column

function conv_enc_bits=conv_enc(inp_bits)

if (size(inp_bits,1)==1)

inp_bits = inp_bits';

end

outputA = mod(conv([1 0 1 1 0 1 1],inp_bits),2);

outputB = mod(conv([1 1 1 1 0 0 1],inp_bits),2);

conv_enc_bits = [outputA(1:end-6) outputB(1:end-6)]';

conv_enc_bits = conv_enc_bits(:);

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File create_random_bits.m

% function bits = create_random_bits(n)

% input : number of bits (mul by 8 if you have number of bytes)

% output: a column of random bits

function bits = create_random_bits(n)

bits = randn(n,1)>0;

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File fd_to_td.m

%function sig_after_ifft = fd_to_td(sig,out_fft_size)

% performs upsampling by zero padding and fft + fft scale factor

% compensation

function sig_after_ifft = fd_to_td(sig,out_fft_size)

sizein=size(sig,1);

inp_to_fft = zeros(out_fft_size,size(sig,2),size(sig,3));

inp_to_fft(1:sizein/2,:,:)=sig(1:sizein/2,:,:);

inp_to_fft(out_fft_size-sizein/2+1:out_fft_size,:,:)=sig(sizein/2+1:size(sig,1),:,:);

sig_after_ifft = out_fft_size*ifft(inp_to_fft);

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File frequency_interleaver.m

%Frequency interleaver function

% frequency_interleaver(org_bits,Ncbpss,Nbpsc,Nrow,Ncol,Nrot,iss)

%

% Input : org_bits bits to interleave (linear array)

%         Ncbpss,Nbpsc,Nrow,Ncol,Nrot mapping parameters

%         iss - input sream (0-(nSS-1))

%

% Output: interleaved bits (linear array)

%

function bits = frequency_interleaver(org_bits,Ncbpss,Nbpsc,Nrow,Ncol,Nrot,iss)

kk = 0:Ncbpss-1;

ii = Nrow*mod(kk,Ncol)+floor(kk/Ncol); % first permutation

s = max(Nbpsc/2,1);

jj = s*floor(ii/s)+mod(ii+Ncbpss-floor(Ncol*ii/Ncbpss),s);% second permutation

if (Nrot>0) % third permutation

rr = mod((jj-(mod(2*iss,3)+3*floor(iss/3))*Nrot*Nbpsc),Ncbpss);

jj=rr;

end

bits = reshape(org_bits,Ncbpss,length(org_bits)/Ncbpss)' ; %reshape to symbols

bits(:,1+jj) = bits; % perfrom the permutation

bits = bits';

bits = bits(:); %reshape to vector

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File gen_pilots.m

% function pilots =gen_pilots(num_syms,nsts,npilots,ists,z_start)

% returns pilots for a single spacetime stream.

%  num_syms - number of symbols to generate

%  nsts     - number of space time streams

%  npilots  - number of pilots 4/6

%  ists     - index of current space time stream

%  z_start  - 1 legacy, 2 mm, 3 gf

function pilots =gen_pilots(num_syms,nsts,npilots,ists,z_start)

% z = 2 mm

% z = 3 gf

PilotPolarity1 = [1  1 1 -1];

PilotPolarity2 = [1  1 -1 -1;...

1 -1 -1 1];

PilotPolarity3 = [ 1   1 -1 -1;...

1  -1  1 -1;...

-1   1  1 -1];

PilotPolarity4 = [ 1   1   1 -1;...

1   1  -1  1;...

1  -1   1  1;...

-1   1   1  1];

PilotPolarity=zeros(4,4,4); % Nsts,ists,npilot

PilotPolarity(1,1,:) = PilotPolarity1;

PilotPolarity(2,1:2,:) = PilotPolarity2;

PilotPolarity(3,1:3,:) = PilotPolarity3;

PilotPolarity(4,1:4,:) = PilotPolarity4;

PilotPolar1_40 = [1  1 1 -1  -1  1];

PilotPolar2_40 = [1  1 -1 -1 -1 -1;...

1  1  1 -1  1  1];

PilotPolar3_40 = [ 1   1 -1 -1 -1 -1 ;...

1   1  1 -1  1  1;...

1  -1  1 -1 -1  1 ];

PilotPolar4_40 = [ 1   1 -1 -1 -1 -1 ;...

1   1  1 -1  1  1;...

1  -1  1 -1 -1  1;...

-1   1  1  1 -1  1];

PilotPolarity_40=zeros(4,4,6); % Nsts,ists,npilot

PilotPolarity_40(1,1,:) = PilotPolar1_40;

PilotPolarity_40(2,1:2,:) = PilotPolar2_40;

PilotPolarity_40(3,1:3,:) = PilotPolar3_40;

PilotPolarity_40(4,1:4,:) = PilotPolar4_40;

if (npilots==4)

PilotTable = squeeze(PilotPolarity(nsts,ists,:));

else

PilotTable = squeeze(PilotPolarity_40(nsts,ists,:));

end

pilots = PilotTable(1+mod((0:num_syms-1)'*ones(1,npilots)+ones(num_syms,1)*(0:npilots-1),npilots));

if (size(pilots,2)==1)

pilots=pilots'; % single sym case

end

% need to multiply by global mask

%Pilot sequence 1-127 cyclic

pilot_seq =[1 1 1 1 -1 -1 -1 1 -1 -1 -1 -1 1 1 -1 1 -1 -1 1 1 -1 1 1 -1 1 1 1 1 1 1 -1 1 ...

1 1 -1 1 1 -1 -1 1 1 1 -1 1 -1 -1 -1 1 -1 1 -1 -1 1 -1 -1 1 1 1 1 1 -1 -1 1 1 ...

-1 -1 1 -1 1 -1 1 1 -1 -1 -1 1 1 -1 -1 -1 -1 1 -1 -1 1 -1 1 1 1 1 -1 1 -1 1 -1 1 ...

-1 -1 -1 -1 -1 1 -1 1 1 -1 1 -1 1 1 1 -1 -1 1 -1 -1 -1 1 1 1 -1 -1 -1 -1 -1 -1 -1];

pilot_seq_bits = pilot_seq(1+mod(z_start+(0:num_syms-1),127));

pilots = pilots.*(pilot_seq_bits'*ones(1,npilots));

pilots=pilots';

pilots = pilots(:);

if (size(pilots,2)==1)

pilots=pilots';

end

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File gen_pilots_leg.m

% function pilots =gen_pilots_leg(num_syms,z_start)

% generates legacy pilots

% Input :num_syms - number of symbols

%        z_start  - the symbol to start from

function pilots =gen_pilots_leg(num_syms,z_start)

% z = 0 signals

% z = 1 data legacy

PilotPolarity = [1  1 1 -1];

pilots = ones(num_syms,1)*PilotPolarity;

% need to multiply by global mask

%Pilot sequence 1-127 cyclic

pilot_seq =[1 1 1 1 -1 -1 -1 1 -1 -1 -1 -1 1 1 -1 1 -1 -1 1 1 -1 1 1 -1 1 1 1 1 1 1 -1 1 ...

1 1 -1 1 1 -1 -1 1 1 1 -1 1 -1 -1 -1 1 -1 1 -1 -1 1 -1 -1 1 1 1 1 1 -1 -1 1 1 ...

-1 -1 1 -1 1 -1 1 1 -1 -1 -1 1 1 -1 -1 -1 -1 1 -1 -1 1 -1 1 1 1 1 -1 1 -1 1 -1 1 ...

-1 -1 -1 -1 -1 1 -1 1 1 -1 1 -1 1 1 1 -1 -1 1 -1 -1 -1 1 1 1 -1 -1 -1 -1 -1 -1 -1];

pilot_seq_bits = pilot_seq(1+mod(z_start+(0:num_syms-1),127));

pilots = pilots.*(pilot_seq_bits'*ones(1,4));

pilots=pilots';

pilots = pilots(:);

if (size(pilots,2)==1)

pilots=pilots';

end

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File generate_data_tx.m

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

%%%%%   function td_data_sig = generate_data_tx(s,source_bits)         %%%%

%%%%%   generate data samples of packet                                %%%%

%%%%%   Input:   s - input structure defining packet                   %%%%

%%%%%            source_bits - data bits                               %%%%

%%%%%   Output:  time domain signal                                    %%%%

%%%%%                                                                  %%%%

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

function td_data_sig = generate_data_tx(s,source_bits)

global tx_n_debug;

% add service and pad bits

pad_and_tail = s.NDataBits-length(source_bits)-16;

source_bits = [s.SERVICE.bits;source_bits;zeros(pad_and_tail,1)];

tx_n_debug.data.source_bits=source_bits;

% scrambling

scr_bits = bitxor(source_bits,scrambler_seq(s.SERVICE.ScramblerStart,length(source_bits))');

tx_n_debug.data.scrambled_bits=scr_bits;

if (s.LDPC_CODING)

after_enc = ldpc_encode(scr_bits,s.ldpc_params);

else    %% BCC start

%separate to encoders

bits_per_enc_before_tail = (s.LENGTH*8+16)/s.NE;

for iES = 1:s.NE

% Get Stream i

stream = scr_bits(iES:s.NE:end);

% insert tail bits and separate to streams

stream(bits_per_enc_before_tail+1:bits_per_enc_before_tail+6)=0;

tx_n_debug.data.stream_bits{iES}=stream;

% conv encode

conv_enc_bits=conv_enc(stream);

tx_n_debug.data.conv_enc_bits{iES}=stream;

puct_bits = puncture_bits(conv_enc_bits,s.PtPattern);

tx_n_debug.data.punct_bits{iES}=stream;

if (iES==1)

after_enc = puct_bits;

else

after_enc = merge_encoders(after_enc,puct_bits,s.Constellations);

end

end

%% BCC end

end

tx_n_debug.data.encoded_bits=after_enc;

% here for loop on streams

for iSS = 1: s.Nss

%streamparse

bits_per_ss_i = stream_parse(after_enc,s.Constellations,iSS);

tx_n_debug.data.stream_parse{iSS}=bits_per_ss_i;

NCBPSS = s.NCBPS*s.Constellations(iSS)/sum(s.Constellations);

Nbpsc  = s.Constellations(iSS);

if (s.LDPC_CODING) % skip freq interleaver for LDPC

input_to_mapper = bits_per_ss_i;

else

%freq interleave

bits_intleaved = frequency_interleaver(bits_per_ss_i,NCBPSS,Nbpsc,s.NROW(iSS),s.NCOL,s.NROT,iSS-1);

tx_n_debug.data.interleaved_bits{iSS}=bits_intleaved;

input_to_mapper = bits_intleaved;

end

%mapper

symbols_i = qam_map(input_to_mapper,Nbpsc);

if (iSS ==1)

symbols = symbols_i;

else

symbols = [symbols ; symbols_i];

end

%stbc

if (iSS<=s.STBC)

symbols = [symbols ; stbc_expand(symbols_i,s.Nsym)];

end

end

tx_n_debug.data.data_symbols=symbols;

% here symbosl is NSTS X (NSYM*Ndatacarriers)

% generate pilots

for ists = 1: s.NSTS

if (s.islegacy)

pilot = gen_pilots_leg(s.Nsym,1);

else

if (s.isMM)

if (s.MCS==32)

pilot = gen_pilots_leg(s.Nsym,3);

else

pilot = gen_pilots(s.Nsym, s.NSTS,s.Npilots,ists,3);

end

else  % GF mode

if (s.MCS==32)

pilot = gen_pilots_leg(s.Nsym,2);

else

pilot = gen_pilots(s.Nsym, s.NSTS,s.Npilots,ists,2);

end

end

end

if (ists==1)

allpilots = pilot;

else

allpilots =  [allpilots;pilot];

end

end

tx_n_debug.data.pilot_symbols=allpilots;

% merge pilot and data to fft_sym

pilot_and_data_symbols = merge_pilot_and_data(symbols,allpilots,s.BW ...

,s.Data_Subcarriers,s.Pilot_Subcarriers);

% here output is carrier x symbol x ists

% make duplicate or modulate or mul by j

pilot_and_data_symbols = modulate_or_duplicate (pilot_and_data_symbols,s.BW,s.BW_20_TO_40);

tx_n_debug.data.full_band_symbols=pilot_and_data_symbols;

if (~s.islegacy)

pilot_and_data_symbols = add_sts_cdd (pilot_and_data_symbols);

tx_n_debug.data.after_sts_cdd_symbols=pilot_and_data_symbols;

end

% multiply by Qk

pilot_and_data_symbols = mul_by_q (pilot_and_data_symbols,s.Q,s.post_q_cdd);

tx_n_debug.data.after_spatial_expansion_symbols = pilot_and_data_symbols;

% Apply Tx chain cdd for legacy transmission

if (s.islegacy)

pilot_and_data_symbols = add_legacy_cdd (pilot_and_data_symbols);

tx_n_debug.data.after_legacy_cdd_symbols = pilot_and_data_symbols;

end

% ifft

scale = tone_field_scale('HT-Data',s.islegacy,(s.MCS ==32),s.use_40_chan)/sqrt(s.NSTS);

td_sig = scale*fd_to_td (pilot_and_data_symbols,s.out_fft_size);

tx_n_debug.data.after_fft_samples = td_sig;

% add cp

td_sig_cp = add_cp(td_sig,s.SHORT_GI);

tx_n_debug.data.after_cp_samples = td_sig_cp;

% window

td_data_sig = td_sig_cp;

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File generate_ht_ltfs.m

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

%      function [htlfs_sig_cp,htlf1_sig_cp ]= generate_ht_ltfs(s)

%      generates the HTLTfs for the preamble

%      Input:  s - packet structure

%      Output:   htlfs_sig_cp - HTLTFs after HT-sig

%                htlf1_sig_cp - HTLTF before HT-sig for gf mode

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

function [htlfs_sig_cp,htlf1_sig_cp ]= generate_ht_ltfs(s)

global PHTLTF;

global tx_n_debug;

% generate HTLTFs

LLTF = [1 1 -1 -1 1 1 -1 1 -1 1 1 1 1 1 1 -1 -1 1 1 -1 1 -1 1 1 1 1   ...

0 1 -1 -1 1 1 -1 1 -1 1 -1 -1 -1 -1 -1 1 1 -1 -1 1 -1 1 -1 1 1 1 1];

HTLTF20 = zeros(64,1);

HTLTF20(1+bitand(64+(-28:28),63)) = [1 1  LLTF  -1 -1 ];

% [-58:58]

HTLTF40 = zeros(128,1);

HTLTF40(1+bitand(128+(-58:58),127)) = [1 1 -1 -1 1 1 -1 1 -1 1 1 1 1 1 1 -1 -1 1 1 -1 1 -1 1 1 1 1   1   ...

1 -1 -1 1 1 -1 1 -1 1 -1 -1 -1 -1 -1 1 1 -1 -1 1 -1 1 -1 1 1 1 1  -1 -1 -1 1 0  ...

0 0 -1 1 1 -1  1 1 -1 -1 1 1 -1 1 -1 1 1 1 1 1 1 -1 -1 1 1 -1 1 -1 1 1 1 1 1  ...

1 -1 -1 1 1 -1 1 -1 1 -1 -1 -1 -1 -1 1 1 -1 -1 1 -1 1 -1 1 1 1 1    ];

htlf1_sig_cp = zeros(0,s.nTx);

BW = s.BW;

BW_20_TO_40 = s.BW_20_TO_40;

if (s.MCS==32)

BW=40;

BW_20_TO_40='none';

end

scale = tone_field_scale('HT-LTF ',0,(s.MCS ==32),s.use_40_chan)/sqrt(s.NSTS);

htltf_data = zeros([64*BW/20 s.NDataLTF  s.NSTS]);

for iLTF = 1:s.NDataLTF

if (BW==40)

htltf = HTLTF40*PHTLTF(1:s.NSTS,iLTF)';

else

htltf = HTLTF20*PHTLTF(1:s.NSTS,iLTF)';

end

htltf_data(:,iLTF,:) = htltf;

end

tx_n_debug.htltf.symbols = htltf_data;

htltf_data = modulate_or_duplicate (htltf_data,BW,BW_20_TO_40);

tx_n_debug.htltf.full_band_symbols = htltf_data;

htltf_data = add_sts_cdd (htltf_data);

tx_n_debug.htltf.after_sts_cdd_symbols = htltf_data;

htltf_data = mul_by_q (htltf_data,s.Q,s.post_q_cdd);

tx_n_debug.htltf.after_q_symbols = htltf_data;

htlfs_sig = scale*fd_to_td (htltf_data,s.out_fft_size);

tx_n_debug.htltf.fft_samples = htlfs_sig;

if (s.isMM)

htlfs_sig_cp = add_cp(htlfs_sig,0);

else

htlf1_sig_cp = add_cp(htlfs_sig(:,1,:),2);

if (s.NSTS>1)

htlfs_sig_cp = add_cp(htlfs_sig(:,2:end,:),0);

else

htlfs_sig_cp = zeros(s.nTx,0);

end

end

tx_n_debug.htltf.after_cp_samples = htlfs_sig_cp;

% generate ExtraLTFs

if (s.NExteneionLTF)

scale = tone_field_scale('HT-LTF ',0,(s.MCS ==32),s.use_40_chan)/sqrt(s.NUM_EXTEN_SS);

for iLTF = 1:s.NExteneionLTF

if (BW==40)

htltf = PHTLTF(iLTF,1:s.NSTS)'*HTLTF40;

else

htltf = PHTLTF(iLTF,1:s.NSTS)'*HTLTF20;

end

if (iLTF==1)

htltf_data = htltf;

else

htltf_data = [htltf_data htltf];

end

end

tx_n_debug.extraltf.symbols=htltf_data;

htltf_data = modulate_or_duplicate (htltf_data,s.BW,s.BW_20_TO_40);

tx_n_debug.extraltf.full_band_symbols=htltf_data;

htltf_data = add_sts_cdd (htltf_data);

tx_n_debug.extraltf.sts_cdd_symbols=htltf_data;

htltf_data = mul_by_q (htltf_data,s.Q(1:s.nTx,s.NSTS+1:s.nTx,:),s.post_q_cdd);

tx_n_debug.extraltf.after_q_symbols=htltf_data;

htlfs_sig = scale*fd_to_td(htltf_data,s.out_fft_size);

tx_n_debug.extraltf.fft_samples=htlfs_sig;

htlfsextra_sig_cp = add_cp(htlfs_sig,0);

tx_n_debug.extraltf.after_cp=htlfsextra_sig_cp;

htlfs_sig_cp = [htlfs_sig_cp htlfsextra_sig_cp];

end

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File generate_ht_sig.m

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

% function td_htsig = generate_ht_sig(s)

%  generates ht-sig samples

%  Input: s - packet structure

%  Output td_htsig - time domain samples for ht-sig

%

%

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

function td_htsig = generate_ht_sig(s)

global PHTLTF;

global tx_n_debug;

if (s.BW==40)

lstf40mode = 'duplicate';

else

lstf40mode = s.BW_20_TO_40;

end

if (s.isMM)

scale = tone_field_scale('HT-SIG ',0,(s.MCS ==32),s.use_40_chan);

else %gf

scale = tone_field_scale('HTSIGGF',0,(s.MCS ==32),s.use_40_chan);

end

htsigbits =[ bitand(floor(s.MCS*(2.^-(0:6))),1)     ...

((s.BW==40)||(s.MCS==32))            ...   % mcs is treated as 20 Mhz with duplicate

bitand(floor(s.LENGTH*(2.^-(0:15))),1)  ...

s.SMOOTHING     ...

s.NOT_SOUNDING&~s.calib   ...           % force to 0 if calibration

s.HT_SIG_RESERVED      ...

s.AGGREGATION   ...

bitand(floor(s.STBC*(2.^-(0:1))),1)  ...

s.LDPC_CODING                 ...

s.SHORT_GI                            ...

bitand(floor(s.NUM_EXTEN_SS*(2.^-(0:1))),1)  ...

];

crc = ones(1,8);

for ii=1:34

c0 = bitand(crc(8)+htsigbits(ii),1);

c1 = bitand(crc(1)+crc(8)+htsigbits(ii),1);

c2 = bitand(crc(2)+crc(8)+htsigbits(ii),1);

crc = [c0 c1 c2 crc(3:7)];

end

crc=bitxor(crc,1);

htsigbits = [htsigbits crc(8:-1:1) 0 0 0 0 0 0];

tx_n_debug.htsig.bits=htsigbits;

htsig_enc_bits  =conv_enc(htsigbits);

tx_n_debug.htsig.coded_bits=htsig_enc_bits;

htsig_intleaved = frequency_interleaver(htsig_enc_bits,48,1,3,16,0,1);

tx_n_debug.htsig.interleaved_bits=htsig_intleaved;

htsigsym        = j*qam_map(htsig_intleaved,1);

tx_n_debug.htsig.data_symbols=htsigsym;

Leg_carriers = ([(-26:-22) (-20:-8) (-6:-1) (1:6) (8:20) (22:26) ]);

htsig_pilots = gen_pilots_leg(2,1);%Nsym,skip

tx_n_debug.htsig.pilot_symbols=htsig_pilots;

htsigsym = merge_pilot_and_data(htsigsym,htsig_pilots,20,Leg_carriers,[-21 -7 7 21]);

tx_n_debug.htsig.all_symbols=htsigsym;

% expand to spacetimestreams

htsig_full_fft = modulate_or_duplicate (htsigsym,20,lstf40mode);

tx_n_debug.htsig.full_band_symbols=htsig_full_fft;

if (s.isMM)

htsig_full_ffttmp = zeros([size(htsig_full_fft,1) 2 s.nTx ]);

for iTx = 1:s.nTx

htsig_full_ffttmp(:,:,iTx)=htsig_full_fft(:,:)/sqrt(s.nTx);

end

htsig_full_fft = htsig_full_ffttmp;

htsig_full_fft = add_legacy_cdd (htsig_full_fft);

tx_n_debug.htsig.legacy_cdd_symbols=htsig_full_fft;

else %gf

htsig_full_ffttmp = zeros([size(htsig_full_fft,1) 2  s.NSTS ]);

for ists = 1: s.NSTS

htsig_full_ffttmp(:,:,ists)=htsig_full_fft(:,:)*PHTLTF(ists,1)/sqrt(s.NSTS);

end

htsig_full_fft = htsig_full_ffttmp;

htsig_full_fft = add_sts_cdd (htsig_full_fft);

tx_n_debug.htsig.sts_cdd_symbols=htsig_full_fft;

htsig_full_fft = mul_by_q (htsig_full_fft,s.Q,s.post_q_cdd);

tx_n_debug.htsig.after_q_symbols=htsig_full_fft;

end

td_htsig = scale*fd_to_td (htsig_full_fft,s.out_fft_size);

tx_n_debug.htsig.fft_samples = td_htsig;

td_htsig = add_cp(td_htsig,0);%sig,shortgi

tx_n_debug.htsig.after_cp = td_htsig;

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File generate_ht_stf.m

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

%   function td_htstf = generate_ht_stf(s)

%   generates HT-stf time domain signal

%   Input : s - structure  defining packet

%   Output: td_htstf - time domain samples

%

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

function td_htstf = generate_ht_stf(s)

global PHTLTF;

global tx_n_debug;

LSTF = zeros(64,1);

LSTF(1+bitand(64+(-26:26),63)) = sqrt(1/2)*[0 0 1+j 0 0 0 -1-j 0 0 0 1+j 0 0 0 -1-j 0 0 0 -1-j 0 0 0 1+j 0 0 0 ...

0 0 0 0 -1-j 0 0 0 -1-j 0 0 0 1+j 0 0 0 1+j 0 0 0 1+j 0 0 0 1+j 0 0];

if (s.BW==40)

lstf40mode = 'duplicate';

else

lstf40mode = s.BW_20_TO_40;

end

% repeat HT-STF with expand to sts,cdd pre,Qmat,new normalization

scale = tone_field_scale('HT-STF ',0,(s.MCS ==32),s.use_40_chan)/sqrt(s.NSTS);

%HT-STF expand to spatial streams

if (s.isMM)

short_full_fft = LSTF*ones(1,s.NSTS);

else %GF

short_full_fft = LSTF*PHTLTF(1:s.NSTS,1)';

end

tx_n_debug.htstf.symbols = short_full_fft;

short_full_fft = modulate_or_duplicate (short_full_fft,20,lstf40mode);

tx_n_debug.htstf.full_band_symbols = short_full_fft;

short_full_fft =  permute(short_full_fft,[1 3 2]);

short_full_fft = add_sts_cdd (short_full_fft);

tx_n_debug.htstf.sts_cdd_symbols = short_full_fft;

short_full_fft = mul_by_q (short_full_fft,s.Q,s.post_q_cdd);

tx_n_debug.htstf.after_q_symbols = short_full_fft;

td_htstf = scale*fd_to_td (short_full_fft,s.out_fft_size);

tx_n_debug.htstf.fft_samples = td_htstf;

if (s.isMM)

td_htstf = add_cp(td_htstf,0);

else

td_htstf = add_cp(td_htstf,2);

end

tx_n_debug.htstf.after_cp = td_htstf;

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File generate_lltf.m

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

%  function td_lltf = generate_lltf(s)

%  Generate L_LTF

%  Input: s - structure defining packet

%  Output: td_lltf - time domain signal for L-LTF

%

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

function td_lltf = generate_lltf(s)

global tx_n_debug;

% generate L-LTF or HT-LTF1

if (s.BW==40)

lstf40mode = 'duplicate';

else

lstf40mode = s.BW_20_TO_40;

end

scale = tone_field_scale('L-LTF  ',s.islegacy,(s.MCS ==32),s.use_40_chan);

LLTF = zeros(64,1);

LLTF(1+bitand(64+(-26:26),63)) = [1 1 -1 -1 1 1 -1 1 -1 1 1 1 1 1 1 -1 -1 1 1 -1 1 -1 1 1 1 1   ...

0 1 -1 -1 1 1 -1 1 -1 1 -1 -1 -1 -1 -1 1 1 -1 -1 1 -1 1 -1 1 1 1 1];

tx_n_debug.lltf.symbols = LLTF;

long_full_fft = modulate_or_duplicate (LLTF,20,lstf40mode);

tx_n_debug.lltf.full_band_symbols = long_full_fft;

long_full_fft=long_full_fft*ones(1,s.nTx)/sqrt(s.nTx);

long_full_fft =  permute(long_full_fft,[1 3 2]);

long_full_fft = add_legacy_cdd (long_full_fft);

tx_n_debug.lltf.after_legacy_cdd_symbols = long_full_fft;

td_lltf = scale*fd_to_td (long_full_fft,s.out_fft_size);

tx_n_debug.lltf.fft_samples = td_lltf;

td_lltf = add_cp(td_lltf,2);

tx_n_debug.lltf.after_cp_samples = td_lltf;

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File generate_lsig.m

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

% function td_lsig=generate_lsig(s)

% Input:s - structure defining packet

% Output: td_lsig - time domain signal for L-SIG

%

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

function td_lsig=generate_lsig(s)

global tx_n_debug;

if (s.BW==40)

lstf40mode = 'duplicate';

else

lstf40mode = s.BW_20_TO_40;

end

scale = tone_field_scale('L-SIG  ',s.islegacy,(s.MCS ==32),s.use_40_chan);

lsigbits =[ bitand(floor(s.LSigRateField*(2.^-(0:3))),1)  0 ...

bitand(floor(s.L_LENGTH*(2.^-(0:11))),1)  ];

parity = bitand(sum(lsigbits),1);

lsigbits = [lsigbits parity 0 0 0 0 0 0];

tx_n_debug.lsig.bits = lsigbits;

lsig_enc_bits  =conv_enc(lsigbits);

tx_n_debug.lsig.coded_bits = lsig_enc_bits;

lsig_intleaved = frequency_interleaver(lsig_enc_bits,48,1,3,16,0,1);

tx_n_debug.lsig.frequency_interleaved_bits = lsig_intleaved;

lsigsym        = qam_map(lsig_intleaved,1);

tx_n_debug.lsig.data_symbols = lsigsym;

Leg_carriers = ([(-26:-22) (-20:-8) (-6:-1) (1:6) (8:20) (22:26) ]);

lsig_pilots = gen_pilots_leg(1,0);%Nsym,skip

tx_n_debug.lsig.pilot_symbols = lsig_pilots;

lsigsym = merge_pilot_and_data(lsigsym,lsig_pilots,20,Leg_carriers,[-21 -7 7 21]);

tx_n_debug.lsig.all_symbols = lsigsym;

lsig_full_fft = modulate_or_duplicate (lsigsym,20,lstf40mode);

tx_n_debug.lsig.full_band_symbols = lsig_full_fft;

lsig_full_fft=lsig_full_fft*ones(1,s.nTx)/sqrt(s.nTx);

lsig_full_fft =  permute(lsig_full_fft,[1 3 2]);

lsig_full_fft = add_legacy_cdd (lsig_full_fft);

tx_n_debug.lsig.legacy_cdd_symbols = lsig_full_fft;

td_lsig = scale*fd_to_td (lsig_full_fft,s.out_fft_size);

tx_n_debug.lsig.fft_sampels = td_lsig;

td_lsig = add_cp(td_lsig,0);

tx_n_debug.lsig.after_cp_samples = td_lsig;

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File generate_lstf.m

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

% function td_lstf = generate_lstf(s)

% generate L-STF

% Input: s - structure defining packet

% Output: td_lstf - time domain samples for L-STF

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

function td_lstf = generate_lstf(s)

global tx_n_debug;

if (s.BW==40)

lstf40mode = 'duplicate';

else

lstf40mode = s.BW_20_TO_40;

end

scale = tone_field_scale('L-STF  ',s.islegacy,(s.MCS ==32),s.use_40_chan);

LSTF = zeros(64,1);

LSTF(1+bitand(64+(-26:26),63)) = sqrt(1/2)*[0 0 1+j 0 0 0 -1-j 0 0 0 1+j 0 0 0 -1-j 0 0 0 -1-j 0 0 0 1+j 0 0 0 ...

0 0 0 0 -1-j 0 0 0 -1-j 0 0 0 1+j 0 0 0 1+j 0 0 0 1+j 0 0 0 1+j 0 0];

%L-STF

tx_n_debug.lstf.symbols = LSTF;

long_full_fft = modulate_or_duplicate (LSTF,20,lstf40mode);

tx_n_debug.lstf.full_band_symbols = long_full_fft;

%copy for all antennas

long_full_fft=long_full_fft*ones(1,s.nTx)/sqrt(s.nTx);

long_full_fft =  permute(long_full_fft,[1 3 2]);

long_full_fft = add_legacy_cdd (long_full_fft);

tx_n_debug.lstf.after_legacy_cdd_symbols = long_full_fft;

td_lstf = scale*fd_to_td (long_full_fft,s.out_fft_size);

tx_n_debug.lstf.fft_samples = td_lstf;

td_lstf = add_cp(td_lstf,2);

tx_n_debug.lstf.after_cp = td_lstf;

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File getcodetable.m

% function [tab,z]=getcodetable(block,code_rate_enum)

% returns the code to be used

function [tab,z]=getcodetable(block,code_rate_enum)

% Table n103

% Matrix prototypes of parity check matrices for codeword block length n= 648 bits.

% Subblock size is Z= 27 bits.

% (a)

% Code rate R= 1/2.

if (block==648)

z= 27;

if (code_rate_enum==0)

tab=[...

0 -1 -1 -1 0 0 -1 -1 0 -1 -1 0 1 0 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1;

22 0 -1 -1 17 -1 0 0 12 -1 -1 -1 -1 0 0 -1 -1 -1 -1 -1 -1 -1 -1 -1;

6 -1 0 -1 10 -1 -1 -1 24 -1 0 -1 -1 -1 0 0 -1 -1 -1 -1 -1 -1 -1 -1;

2 -1 -1 0 20 -1 -1 -1 25 0 -1 -1 -1 -1 -1 0 0 -1 -1 -1 -1 -1 -1 -1;

23 -1 -1 -1 3 -1 -1 -1 0 -1 9 11 -1 -1 -1 -1 0 0 -1 -1 -1 -1 -1 -1;

24 -1 23 1 17 -1 3 -1 10 -1 -1 -1 -1 -1 -1 -1 -1 0 0 -1 -1 -1 -1 -1;

25 -1 -1 -1 8 -1 -1 -1 7 18 -1 -1 0 -1 -1 -1 -1 -1 0 0 -1 -1 -1 -1;

13 24 -1 -1 0 -1 8 -1 6 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 0 0 -1 -1 -1;

7 20 -1 16 22 10 -1 -1 23 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 0 0 -1 -1;

11 -1 -1 -1 19 -1 -1 -1 13 -1 3 17 -1 -1 -1 -1 -1 -1 -1 -1 -1 0 0 -1;

25 -1 8 -1 23 18 -1 14 9 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 0 0;

3 -1 -1 -1 16 -1 -1 2 25 5 -1 -1 1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 0;

];

elseif (code_rate_enum==1)

% (b)

% Code rate R= 2/3.

tab=[...

25 26 14 -1 20 -1 2 -1 4 -1 -1 8 -1 16 -1 18 1 0 -1 -1 -1 -1 -1 -1;

10 9 15 11 -1 0 -1 1 -1 -1 18 -1 8 -1 10 -1 -1 0 0 -1 -1 -1 -1 -1;

16 2 20 26 21 -1 6 -1 1 26 -1 7 -1 -1 -1 -1 -1 -1 0 0 -1 -1 -1 -1;

10 13 5 0 -1 3 -1 7 -1 -1 26 -1 -1 13 -1 16 -1 -1 -1 0 0 -1 -1 -1;

23 14 24 -1 12 -1 19 -1 17 -1 -1 -1 20 -1 21 -1 0 -1 -1 -1 0 0 -1 -1;

6 22 9 20 -1 25 -1 17 -1 8 -1 14 -1 18 -1 -1 -1 -1 -1 -1 -1 0 0 -1;

14 23 21 11 20 -1 24 -1 18 -1 19 -1 -1 -1 -1 22 -1 -1 -1 -1 -1 -1 0 0;

17 11 11 20 -1 21 -1 26 -1 3 -1 -1 18 -1 26 -1 1 -1 -1 -1 -1 -1 -1 0;

];

elseif (code_rate_enum==2)

% (c)

% Code rate R= 3/4.

tab=[...

16 17 22 24 9 3 14 -1 4 2 7 -1 26 -1 2 -1 21 -1 1 0 -1 -1 -1 -1;

25 12 12 3 3 26 6 21 -1 15 22 -1 15 -1 4 -1 -1 16 -1 0 0 -1 -1 -1;

25 18 26 16 22 23 9 -1 0 -1 4 -1 4 -1 8 23 11 -1 -1 -1 0 0 -1 -1;

9 7 0 1 17 -1 -1 7 3 -1 3 23 -1 16 -1 -1 21 -1 0 -1 -1 0 0 -1;

24 5 26 7 1 -1 -1 15 24 15 -1 8 -1 13 -1 13 -1 11 -1 -1 -1 -1 0 0;

2 2 19 14 24 1 15 19 -1 21 -1 2 -1 24 -1 3 -1 2 1 -1 -1 -1 -1 0;

];

elseif (code_rate_enum==3)

% (d)

% Code rate R= 5/6.

tab=[...

17 13 8 21 9 3 18 12 10 0 4 15 19 2 5 10 26 19 13 13 1 0 -1 -1;

3 12 11 14 11 25 5 18 0 9 2 26 26 10 24 7 14 20 4 2 -1 0 0 -1;

22 16 4 3 10 21 12 5 21 14 19 5 -1 8 5 18 11 5 5 15 0 -1 0 0;

7 7 14 14 4 16 16 24 24 10 1 7 15 6 10 26 8 18 21 14 1 -1 -1 0;

];

end

elseif(block==1296)

%Table 1 n104

%Matrix prototypes of parity check matrices for codeword block length n= 1296 bits.

% Subblock size is Z= 54 bits.

% (a)

% Code rate R= 1/2.

z= 54;

if (code_rate_enum==0)

tab=[...

40 -1 -1 -1 22 -1 49 23 43 -1 -1 -1 1 0 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1;

50 1 -1 -1 48 35 -1 -1 13 -1 30 -1 -1 0 0 -1 -1 -1 -1 -1 -1 -1 -1 -1;

39 50 -1 -1 4 -1 2 -1 -1 -1 -1 49 -1 -1 0 0 -1 -1 -1 -1 -1 -1 -1 -1;

33 -1 -1 38 37 -1 -1 4 1 -1 -1 -1 -1 -1 -1 0 0 -1 -1 -1 -1 -1 -1 -1;

45 -1 -1 -1 0 22 -1 -1 20 42 -1 -1 -1 -1 -1 -1 0 0 -1 -1 -1 -1 -1 -1;

51 -1 -1 48 35 -1 -1 -1 44 -1 18 -1 -1 -1 -1 -1 -1 0 0 -1 -1 -1 -1 -1;

47 11 -1 -1 -1 17 -1 -1 51 -1 -1 -1 0 -1 -1 -1 -1 -1 0 0 -1 -1 -1 -1;

5 -1 25 -1 6 -1 45 -1 13 40 -1 -1 -1 -1 -1 -1 -1 -1 -1 0 0 -1 -1 -1;

33 -1 -1 34 24 -1 -1 -1 23 -1 -1 46 -1 -1 -1 -1 -1 -1 -1 -1 0 0 -1 -1;

1 -1 27 -1 1 -1 -1 -1 38 -1 44 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 0 0 -1;

-1 18 -1 -1 23 -1 -1 8 0 35 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 0 0;

49 -1 17 -1 30 -1 -1 -1 34 -1 -1 19 1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 0;

];

% (b)

% Code rate R= 2/3.

elseif (code_rate_enum==1)

tab=[...

39 31 22 43 -1 40 4 -1 11 -1 -1 50 -1 -1 -1 6 1 0 -1 -1 -1 -1 -1 -1;

25 52 41 2 6 -1 14 -1 34 -1 -1 -1 24 -1 37 -1 -1 0 0 -1 -1 -1 -1 -1;

43 31 29 0 21 -1 28 -1 -1 2 -1 -1 7 -1 17 -1 -1 -1 0 0 -1 -1 -1 -1;

20 33 48 -1 4 13 -1 26 -1 -1 22 -1 -1 46 42 -1 -1 -1 -1 0 0 -1 -1 -1;

45 7 18 51 12 25 -1 -1 -1 50 -1 -1 5 -1 -1 -1 0 -1 -1 -1 0 0 -1 -1;

35 40 32 16 5 -1 -1 18 -1 -1 43 51 -1 32 -1 -1 -1 -1 -1 -1 -1 0 0 -1;

9 24 13 22 28 -1 -1 37 -1 -1 25 -1 -1 52 -1 13 -1 -1 -1 -1 -1 -1 0 0;

32 22 4 21 16 -1 -1 -1 27 28 -1 38 -1 -1 -1 8 1 -1 -1 -1 -1 -1 -1 0;

];

% (c)

% Code rate R= 3/4.

elseif (code_rate_enum==2)

tab=[...

39 40 51 41 3 29 8 36 -1 14 -1 6 -1 33 -1 11 -1 4 1 0 -1 -1 -1 -1;

48 21 47 9 48 35 51 -1 38 -1 28 -1 34 -1 50 -1 50 -1 -1 0 0 -1 -1 -1;

30 39 28 42 50 39 5 17 -1 6 -1 18 -1 20 -1 15 -1 40 -1 -1 0 0 -1 -1;

29 0 1 43 36 30 47 -1 49 -1 47 -1 3 -1 35 -1 34 -1 0 -1 -1 0 0 -1;

1 32 11 23 10 44 12 7 -1 48 -1 4 -1 9 -1 17 -1 16 -1 -1 -1 -1 0 0;

13 7 15 47 23 16 47 -1 43 -1 29 -1 52 -1 2 -1 53 -1 1 -1 -1 -1 -1 0;

];

elseif (code_rate_enum==3)

%(d)

% Code rate R= 5/6.

tab=[...

48 29 37 52 2 16 6 14 53 31 34 5 18 42 53 31 45 -1 46 52 1 0 -1 -1;

17 4 30 7 43 11 24 6 14 21 6 39 17 40 47 7 15 41 19 -1 -1 0 0 -1;

7 2 51 31 46 23 16 11 53 40 10 7 46 53 33 35 -1 25 35 38 0 -1 0 0;

19 48 41 1 10 7 36 47 5 29 52 52 31 10 26 6 3 2 -1 51 1 -1 -1 0;

];

end

elseif(block==1944)

%Table 1 n105

% Matrix prototypes of parity check matrices for codeword block length n=1944 bits.

% Subblock size is Z = 81 bits.

%(a)

% Code rate R= 1/2.

z= 81;

if (code_rate_enum==0)

tab=[...

57 -1 -1 -1 50 -1 11 -1 50 -1 79 -1 1 0 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1;

3 -1 28 -1 0 -1 -1 -1 55 7 -1 -1 -1 0 0 -1 -1 -1 -1 -1 -1 -1 -1 -1;

30 -1 -1 -1 24 37 -1 -1 56 14 -1 -1 -1 -1 0 0 -1 -1 -1 -1 -1 -1 -1 -1;

62 53 -1 -1 53 -1 -1 3 35 -1 -1 -1 -1 -1 -1 0 0 -1 -1 -1 -1 -1 -1 -1;

40 -1 -1 20 66 -1 -1 22 28 -1 -1 -1 -1 -1 -1 -1 0 0 -1 -1 -1 -1 -1 -1;

0 -1 -1 -1 8 -1 42 -1 50 -1 -1 8 -1 -1 -1 -1 -1 0 0 -1 -1 -1 -1 -1;

69 79 79 -1 -1 -1 56 -1 52 -1 -1 -1 0 -1 -1 -1 -1 -1 0 0 -1 -1 -1 -1;

65 -1 -1 -1 38 57 -1 -1 72 -1 27 -1 -1 -1 -1 -1 -1 -1 -1 0 0 -1 -1 -1;

64 -1 -1 -1 14 52 -1 -1 30 -1 -1 32 -1 -1 -1 -1 -1 -1 -1 -1 0 0 -1 -1;

-1 45 -1 70 0 -1 -1 -1 77 9 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 0 0 -1;

2 56 -1 57 35 -1 -1 -1 -1 -1 12 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 0 0;

24 -1 61 -1 60 -1 -1 27 51 -1 -1 16 1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 0;

];

% (b)

% Code rate R= 2/3.

elseif (code_rate_enum==1)

tab=[...

61 75 4 63 56 -1 -1 -1 -1 -1 -1 8 -1 2 17 25 1 0 -1 -1 -1 -1 -1 -1;

56 74 77 20 -1 -1 -1 64 24 4 67 -1 7 -1 -1 -1 -1 0 0 -1 -1 -1 -1 -1;

28 21 68 10 7 14 65 -1 -1 -1 23 -1 -1 -1 75 -1 -1 -1 0 0 -1 -1 -1 -1;

48 38 43 78 76 -1 -1 -1 -1 5 36 -1 15 72 -1 -1 -1 -1 -1 0 0 -1 -1 -1;

40 2 53 25 -1 52 62 -1 20 -1 -1 44 -1 -1 -1 -1 0 -1 -1 -1 0 0 -1 -1;

69 23 64 10 22 -1 21 -1 -1 -1 -1 -1 68 23 29 -1 -1 -1 -1 -1 -1 0 0 -1;

12 0 68 20 55 61 -1 40 -1 -1 -1 52 -1 -1 -1 44 -1 -1 -1 -1 -1 -1 0 0;

58 8 34 64 78 -1 -1 11 78 24 -1 -1 -1 -1 -1 58 1 -1 -1 -1 -1 -1 -1 0;

];

% (c)

% Code rate R= 3/4.

elseif (code_rate_enum==2)

tab=[...

48 29 28 39 9 61 -1 -1 -1 63 45 80 -1 -1 -1 37 32 22 1 0 -1 -1 -1 -1;

4 49 42 48 11 30 -1 -1 -1 49 17 41 37 15 -1 54 -1 -1 -1 0 0 -1 -1 -1;

35 76 78 51 37 35 21 -1 17 64 -1 -1 -1 59 7 -1 -1 32 -1 -1 0 0 -1 -1;

9 65 44 9 54 56 73 34 42 -1 -1 -1 35 -1 -1 -1 46 39 0 -1 -1 0 0 -1;

3 62 7 80 68 26 -1 80 55 -1 36 -1 26 -1 9 -1 72 -1 -1 -1 -1 -1 0 0;

26 75 33 21 69 59 3 38 -1 -1 -1 35 -1 62 36 26 -1 -1 1 -1 -1 -1 -1 0;

];

%(d)

% Code rate R= 5/6.

elseif (code_rate_enum==3)

tab=[...

13 48 80 66 4 74 7 30 76 52 37 60 -1 49 73 31 74 73 23 -1 1 0 -1 -1;

69 63 74 56 64 77 57 65 6 16 51 -1 64 -1 68 9 48 62 54 27 -1 0 0 -1;

51 15 0 80 24 25 42 54 44 71 71 9 67 35 -1 58 -1 29 -1 53 0 -1 0 0;

16 29 36 41 44 56 59 37 50 24 -1 65 4 65 52 -1 4 -1 73 52 1 -1 -1 0;

];

end

else

errordlg('Invalid block rate')

end

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File init_bf.m

% function s_out = init_bf(s_in)

%  Verifies BF configuration and expands Q matrix

%  Input  - packet info structure

%  Output - modified structure

function s_out = init_bf(s_in)

s_out = s_in;

global T_ITX_CS; %table n65

if (s_out.islegacy)

s_out.post_q_cdd=zeros(1,s_out.nTx);

% in this case Q will be used to duplicate the streams to antennas

% cdd that will take effect will be legacy cdd in a separate module.

end

if (s_out.calib)

% initialize calibration, override external Q or cdd

if (s_out.NUM_EXTEN_SS ~= s_out.nTx-s_out.NSTS)

errordlg ('number of tx antennas should match total number of probed streams')

end

Q=PHTLTF(1:s_out.nTx,1:s_out.NSTS+s_out.NUM_EXTEN_SS);

s_out.post_q_cdd=T_ITX_CS(s_out.nTx,:);

elseif (strcmp(s_out.BF_Q_source,'default'))

if (s_out.nTx<s_out.NSTS)

errordlg('Not enough Tx antennas to support rate');

end

if (s_out.NUM_EXTEN_SS>0)

errordlg('cannot use extendion LTF with direct conversion');

end

if (length(s_out.post_q_cdd)<s_out.nTx)

errordlg('not enough cdd values');

end

if (s_out.nTx==s_out.NSTS)

Q = eye(s_out.NSTS);

else

if (s_out.NSTS==1)

Q = 1/sqrt(s_out.nTx)*ones(s_out.nTx,1);

else

if (s_out.NSTS==2 && s_out.nTx==3)

Q = sqrt(2/3)*([ 1 0 ; 0 1 ; 1 0]);

end

if (s_out.NSTS==2 && s_out.nTx==4)

Q = sqrt(2/4)*([1 0 ; 0 1; 1 0 ; 0 1]);

end

if (s_out.NSTS==3 && s_out.nTx==4)

Q =sqrt(3/4)*([1 0 0 ; 0 1 0 ; 0 0 1; 1 0 0 ;]);

end

end

end

s_out.Q = reshape(repmat(Q,1,s_in.in_fft_size),size(Q,1),size(Q,2),s_in.in_fft_size);

elseif (strcmp(s_out.BF_Q_source,'const+cdd'))

% verify Q exists and sizes are correct

if (size(s_out.FixedQ,1)~=s_out.nTx)

errordlg('BF matrix should have nTx number of rows')

end

if (size(s_out.FixedQ,2)~=(s_out.nSTS+s_out.NUM_EXTEN_SS))

errordlg('BF matrix should have nSTS+NextLTF number of columns')

end

if (length(s_out.post_q_cdd)<s_out.nTx)

errordlg('not enough cdd values');

end

s_out.Q = reshape(repmat(FixedQ,1,in_fft_size),size(FixedQ,1),size(FixedQ,2),s_in.in_fft_size);

elseif (strcmp(s_out.BF_Q_source,'general'))

if (size(s_out.Q,3)~=s_out.in_fft_size)

errordlg('Should supply 64 or 128 BF matrices accroding to mode')

end

if (size(s_out.Q,1)~=s_out.nTx)

errordlg('BF matrix should have nTx number of rows')

end

if (size(s_out.Q,2)~=(s_out.nSTS+s_out.NUM_EXTEN_SS))

errordlg('BF matrix should have nSTS+NextLTF number of columns')

end

else

errordlg('invalid BF_Q_source value')

end

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File ldpc_encode.m

% function bits = ldpc_encode(inp,ldpc_params)

% performs ldpc encoding.

% Input: inp - input bits

%        ldpc_params - encoding params structure for packet

% Ouput: encoded bits

function bits = ldpc_encode(inp,ldpc_params)

bits=zeros(ldpc_params.N_avbits,1);

generator_matrix = calculate_generator_matrix(ldpc_params.N_Blk,ldpc_params.code_rate_ind);

in_blk_start_ptr=1; % where the next block starts

out_blk_start_ptr=1; % where the next block starts

for ii=1:ldpc_params.N_cw

ldpc_bits=zeros(ldpc_params.K_Blk,1);% initialize to 0

% calculate size of data,parity and repeat

blk_size = ldpc_params.K_Blk-ldpc_params.shortvec(ii);

parity_size = ldpc_params.N_Blk-ldpc_params.K_Blk-ldpc_params.puntvec(ii);

rep_size = ldpc_params.repvec(ii);

% copy systematic part to ldpc encoder input buffer

ldpc_bits(1:blk_size)=inp(in_blk_start_ptr:in_blk_start_ptr+blk_size-1);

in_blk_start_ptr=in_blk_start_ptr+blk_size;

% generate parity

parity = bitand((generator_matrix* ldpc_bits) , 1);

% combine the two after shortening and pucturing

ldpc_data_and_parity = [ldpc_bits(1:blk_size);parity(1:parity_size)];

% generate repeat bits by copying (possibly cyclically) data and parity

rep_bits=ldpc_data_and_parity(1+mod((0:rep_size-1),length(ldpc_data_and_parity)));

% combine entire block

ldpc_block= [ldpc_data_and_parity;rep_bits];

blk_out_size = length(ldpc_block);

% copy  to output

bits(out_blk_start_ptr:out_blk_start_ptr+blk_out_size-1)=ldpc_block;

out_blk_start_ptr = out_blk_start_ptr+blk_out_size;

end

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File ldpc_encoder_params.m

%function ldpc_params=ldpc_encoder_params(length,code_rate_ind,mstbc,Ncpbs)

% This function generates a structure of ldpc related parameters

% Inputs:

%        length - packet length inbytes

%        code_rate_ind - 0,1,2,3 representing 1/2,2/3,3/4,5/6 respectively

%        mstbc         - 1,2 representing non stbc and stbc respectively

%        Ncpbs         - total coded bits for OFDM symbol

function ldpc_params=ldpc_encoder_params(length,code_rate_ind,mstbc,Ncpbs)

global num_vec denum_vec;

num=num_vec(1+code_rate_ind);

denum=denum_vec(1+code_rate_ind);

R=num/denum;

N_pld = length*8+16;

N_avbits=Ncpbs*mstbc*ceil(N_pld/Ncpbs/R/mstbc);

if (N_avbits<=648)

N_cw=1;

if(N_avbits>=N_pld+912/denum) % 1-R = 1/denum

N_Blk = 1296;

else

N_Blk = 648;

end

elseif(N_avbits<=1296)

N_cw=1;

if(N_avbits>=N_pld+1464/denum) % 1-R = 1/denum

N_Blk = 1944;

else

N_Blk = 1296;

end

elseif(N_avbits<=1944)

N_cw=1;

N_Blk = 1944;

elseif(N_avbits<=2592)

N_cw=2;

if(N_avbits>=N_pld+2916/denum) % 1-R = 1/denum

N_Blk = 1944;

else

N_Blk = 1296;

end

else

N_Blk = 1944;

N_cw=ceil(N_pld/(N_Blk*R));

end

N_shrt = N_cw* N_Blk*R-N_pld;

N_punct = max(0,N_cw* N_Blk-N_avbits-N_shrt);

% Floating point operation have been replaced by integer operations using

% 1-R = 1/denum

% R/(1-R) = num

%

if ((10*N_punct>N_cw* N_Blk/denum) && (10*N_shrt<12*N_punct*num)) ||(10*N_punct>3*N_cw* N_Blk/denum)

N_avbits=N_avbits+Ncpbs*mstbc;

N_punct = max(0,N_cw* N_Blk-N_avbits-N_shrt);

end

Nsym = N_avbits/Ncpbs;

Nrep=max(0,N_avbits-N_cw* N_Blk/denum-N_pld);

shortvec=floor(N_shrt/N_cw)*ones(1,N_cw);;

inx=N_shrt-N_cw*floor(N_shrt/N_cw);

shortvec(1:inx)=shortvec(1:inx)+1;

puntvec=floor(N_punct/N_cw)*ones(1,N_cw);

ind=N_punct-N_cw*floor(N_punct/N_cw);

puntvec(1:ind)=puntvec(1:ind)+1;

repvec=floor(Nrep/N_cw)*ones(1,N_cw);

ind=Nrep-N_cw*floor(Nrep/N_cw);

repvec(1:ind)=repvec(1:ind)+1;

ldpc_params.code_rate_ind = code_rate_ind;

ldpc_params.N_cw=N_cw;

ldpc_params.N_Blk=N_Blk;

ldpc_params.K_Blk=N_Blk*num/denum;   % N_Blk*R

ldpc_params.N_pld=N_pld;

ldpc_params.Nsym=Nsym;

ldpc_params.N_avbits=N_avbits;

ldpc_params.shortvec=shortvec;

ldpc_params.puntvec=puntvec;

ldpc_params.repvec=repvec;

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File mcs_info.m

% function s_out = mcs_info(s_in)

% This functions validates the input structure and calculates derived

% internal parameters. The additinal parameters are appended to the

% structure and returned as the output

function s_out = mcs_info(s_in)

global Legacy_Info   MCS_Info20   MCS_Info40;

global PHTLTF;

global Punct num_vec denum_vec;

global Pilot_Subcarriers_20 Pilot_Subcarriers_40;

s_out = s_in;

s_out.islegacy = strcmp(s_in.FORMAT,'NON_HT');

s_out.isMM     = strcmp(s_in.FORMAT,'HT_MM');

s_out.isGF     = strcmp(s_in.FORMAT,'HT_GF');

% Translate the extern CW and CW_OFFSET parameters from TXVECTOR to

% internal parameters which are

% BW=20 if there is one band or if subchannels are not independet

%    40 40 Mhz HT operation

% BW_20_TO_40 =

%         'upper' upper mode

%         'lower' lower mode

%         'duplicate' for 40Mhz duplicate and(!) mcs 32

s_out.BW_20_TO_40='none'; % default

s_out.BW=20;

s_out.in_fft_size = 128;

if (strcmp(s_in.CW,'HT_CW40'))

if (strcmp(s_in.CW_OFFSET,'CH_OFF_20U'))

s_out.BW_20_TO_40='upper';

s_out.use_40_chan = 0;           % for scaling table

elseif(strcmp(s_in.CW_OFFSET,'CH_OFF_20L'))

s_out.BW_20_TO_40='lower';

s_out.use_40_chan = 0;           % for scaling table

elseif(strcmp(s_in.CW_OFFSET,'CH_OFF_40'))

s_out.use_40_chan = 1;           % for scaling table

if (s_in.MCS~=32)

s_out.BW=40;

else

s_out.BW_20_TO_40='duplicate';

end

else

errordlg('Invalid value for CW_OFFSET')

end

elseif (strcmp(s_in.CW,'HT_CW20DN'))

s_out.BW_20_TO_40='duplicate';

s_out.use_40_chan = 1;               % for scaling table

elseif(strcmp(s_in.CW,'HT_CW20'))

if (s_in.MCS==32)

errordlg('MCS 32 is transmitted with CW = HT_CW40 only')

end

s_out.in_fft_size = 64;

s_out.use_40_chan = 0;               % for scaling table

else

errordlg('Invalid value for CW')

end

if (s_in.output_rate~=20 && s_in.output_rate~=40 && s_in.output_rate~=80)

errordlg ('invalid output rate. Pick 20,40,80' )

end

if(~strcmp(s_in.CW,'HT_CW20') && s_in.output_rate==20)

errordlg ('invalid output rate.Pick 40,80 ' )

end

s_out.out_fft_size =64*(s_in.output_rate/20); % should be 20/40/80

% check for sanity

if (s_out.islegacy)

if (s_in.SHORT_GI)

errordlg('Short Gi is not allowed in Legacy Mode');

end

if (s_in.LENGTH>4095)

errordlg('Length is too large for legacy packet');

end

if (s_in.MCS>7)

errordlg('Mcs too large for legacy packet');

end

end

if (s_out.islegacy)

if (s_in.LENGTH<=0 || s_in.LENGTH>4095)

errordlg('Length is incompatible with legacy packet' )

end

s_out.CodeRateInd = Legacy_Info(s_in.MCS+1,2);

s_out.Nss=1;

s_out.NSTS=1;

s_out.NBPSC         =Legacy_Info(s_in.MCS+1,3);

s_out.Constellations=Legacy_Info(s_in.MCS+1,3);

s_out.Ndata=48;

s_out.Npilots=4;

s_out.NCBPS=s_out.Ndata * s_out.NBPSC;

s_out.NDBPS=s_out.Ndata * s_out.NBPSC* num_vec(1+s_out.CodeRateInd)/denum_vec(1+s_out.CodeRateInd);

s_out.NE=1;

s_out.NCOL = 16;

s_out.NROW = s_out.NCBPS/16;

s_out.NROT = 0;

s_out.Nsym = ceil((16+8*s_in.LENGTH+6)/s_out.NDBPS);

s_out.NDataBits = s_out.Nsym*s_out.NDBPS;

s_out.Pilot_Subcarriers=Pilot_Subcarriers_20;

s_out.Data_Subcarriers=([(-26:-22) (-20:-8) (-6:-1) (1:6) (8:20) (22:26) ]);

s_out.L_LENGTH = s_in.LENGTH;

s_out.LSigRateField = Legacy_Info(s_in.MCS+1,4);

s_out.L_DATARATE    = s_out.NDBPS/4; % 4 is symbol duration in usec Rate is in Mbps

else  % not in legacy packet

if (s_in.LENGTH<0 || s_in.LENGTH>=2^16)

errordlg('Length is incompatible with 802.11n packet' )

end

if (s_out.BW == 20 )

s_out.Pilot_Subcarriers=Pilot_Subcarriers_20;

s_out.Npilots = 4;

if (s_in.MCS==32)

s_out.Data_Subcarriers=([(-26:-22) (-20:-8) (-6:-1) (1:6) (8:20) (22:26) ]);

mcs_line=[32 0 1 1 0 0 0];

else

mcs_line = MCS_Info20(find(MCS_Info20(:,1)==s_in.MCS),:);

s_out.Data_Subcarriers=([(-28:-22) (-20:-8) (-6:-1) (1:6) (8:20) (22:28)]);

end

else

mcs_line = MCS_Info40(find(MCS_Info40(:,1)==s_in.MCS),:);

s_out.Pilot_Subcarriers=Pilot_Subcarriers_40;

s_out.Data_Subcarriers=([(-58:-54) (-52:-26) (-24:-12) (-10:-2) (2:10) (12:24) (26:52) (54:58)]);

s_out.Npilots = 6;

end

s_out.Ndata =length(s_out.Data_Subcarriers);

s_out.CodeRateInd = mcs_line(2);

s_out.Nss=mcs_line(3);

s_out.NSTS=s_out.Nss+s_in.STBC; % not all combinations are allowed here

s_out.Constellations=mcs_line(4:4+s_out.Nss -1);

s_out.NBPSC=sum(s_out.Constellations);

s_out.NCBPS=s_out.NBPSC*s_out.Ndata;

s_out.NDBPS=s_out.NCBPS*num_vec(1+s_out.CodeRateInd)/denum_vec(1+s_out.CodeRateInd);

if (s_in.SHORT_GI)

Rate = s_out.NDBPS/3.6;

else

Rate = s_out.NDBPS/4;

end

if (s_out.MCS == 32)

s_out.NCOL = 16;

s_out.NROW = s_out.NCBPS/16;

s_out.NROT = 0;

elseif (s_out.BW == 20)

s_out.NCOL = 13;

s_out.NROW = 4*s_out.Constellations;

s_out.NROT = 11;

else

s_out.NCOL = 18;

s_out.NROW = 6*s_out.Constellations;

s_out.NROT = 29;

end

if (s_out.Nss==1)

s_out.NROT = 0;

end

mstbs = 1+(s_out.STBC>0);

if (s_in.LDPC_CODING)

s_out.ldpc_params = ldpc_encoder_params(s_in.LENGTH,s_out.CodeRateInd,mstbs,s_out.NCBPS);

s_out.Nsym=s_out.ldpc_params.Nsym;

s_out.NDataBits=s_out.ldpc_params.N_pld;

else

% start of BCC specific

s_out.NE=1+(Rate>300);  % here condition may change to 1+(RateLongCP>300)

s_out.Nsym = mstbs*ceil((16+8*s_in.LENGTH+6*s_out.NE)/(s_out.NDBPS*mstbs));

%end of bcc specific

s_out.NDataBits = s_out.Nsym*s_out.NDBPS;

end

s_out.NDataLTF = s_out.NSTS+(s_out.NSTS==3);

s_out.NExteneionLTF = s_out.NUM_EXTEN_SS+(s_out.NUM_EXTEN_SS==3);

s_out.NTotalLTF = s_out.NDataLTF+s_out.NExteneionLTF;

% calculate Lsig length if not specified externally

% if spedified externally will take external value

if ((s_in.L_LENGTH==0) && s_out.isMM)

if (s_in.SHORT_GI)

symbolTime=9/10;

else

symbolTime=1;

end

s_out.L_LENGTH = 3*(ceil(s_out.Nsym*symbolTime)+s_out.NTotalLTF+3)-3;

end

s_out.LSigRateField   = Legacy_Info(0+1,4);

s_out.L_DATARATE      = 6;

end

s_out.PtPattern = Punct{s_out.CodeRateInd +1};

stbc_1_msc_list = ([(0:23) (33:39)  41 43 46 48 50 ]);

stbc_2_msc_list = [(8:15) (33:38)];

if (s_in.STBC)

if (s_in.STBC==1)

if length(find(stbc_1_msc_list==s_in.MCS))==0

errordlg('mcs is not compatible with stbc')

end

elseif (s_in.STBC==2)

if length(find(stbc_2_msc_list==s_in.MCS))==0

errordlg('mcs is not compatible with stbc')

end

end

end

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File merge_encoders.m

%function bits = merge_encoders(bitsA,bitsB,bspc_streams)

% input : two column vectors of bits

% output: merge for before stream parser

function bits = merge_encoders(bitsA,bitsB,bspc_streams)

bspc_streams_axis = max(bspc_streams/2,1);

sum_s = sum(bspc_streams_axis);

bits=[reshape(bitsA,sum_s,length(bitsA)/sum_s)' reshape(bitsB,sum_s,length(bitsB)/sum_s)']';

bits= bits(:);

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File merge_pilot_and_data.m

% function symbols = merge_pilot_and_data(data_sym,pilot_sym,BW,data_c,pilot_c)

% merges pilot and data to fft symbol

% BW = 20/40

% data_c,pilot_c - carrier indices for data and pilot

function symbols = merge_pilot_and_data(data_sym,pilot_sym,BW,data_c,pilot_c)

nSTS = size(data_sym,1);

if (nSTS>4)

errordlg('data size is wrong')

end

Nsym = size(data_sym,2)/length(data_c);

ndata = size(data_c,2);

npilot = size(pilot_c,2);

if BW==20

fft_size = 64;

else

fft_size = 128;

end

symbols =   zeros(fft_size,Nsym,nSTS);

for iSTS = 1:nSTS

sts_sym    =  data_sym(iSTS,:);

p_sym      =  pilot_sym(iSTS,:);

sts_sym    =  reshape(sts_sym,length(data_c),Nsym);

p_sym      =  reshape(p_sym,length(pilot_c),Nsym);

symbols (1+ bitand(data_c+fft_size,fft_size-1),:,iSTS) = sts_sym(1:ndata,:);

symbols (1+ bitand(pilot_c+fft_size,fft_size-1),:,iSTS) = p_sym(1:npilot,:);

end

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File merge_segments.m

% function td_sig = merge_segments(win,varargin)

% merges different signals to a single signal

% If window is empty then they are concatenated.

% If window is not empty the signals are windowed and added with overlap

function td_sig = merge_segments(win,varargin)

len = 0;

% calculate signal as sum of signals length

for ii=1:nargin-1

len=len+size(varargin{ii},1)*size(varargin{ii},3);

end

nTx = size(varargin{2},2);

if (length(win)==0)

td_sig = zeros(nTx,len); % maybe + extras

start = 1;

for ii=1:nargin-1

if (~prod(size(varargin{ii})))

continue;

end

sym_size=size(varargin{ii},3);

nsym=size(varargin{ii},1);

for jj=1:nsym

td_sig(:,start:start+sym_size-1)=shiftdim(varargin{ii}(jj,:,:),1);

start=start+sym_size;

end

end

else

winlen=length(win);

td_sig = zeros(nTx,len+winlen); % maybe + extras

start = 1;

for ii= 1:nargin-1

if (~prod(size(varargin{ii})))

continue;

end

sym_size=size(varargin{ii},3);

% find periodicity (cyclic prefix could be different) 64*floor(/64)

nsym=size(varargin{ii},1);

for jj=1:nsym

symbol=zeros(nTx,sym_size);

symbol(:,:) = varargin{ii}(jj,:,:);

symbol_expanded = apply_win(win,symbol);

td_sig(:,start:start+sym_size+winlen-1)=td_sig(:,start:start+sym_size+winlen-1)+symbol_expanded;

start=start+sym_size;

end

end

end

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File modulate_or_duplicate.m

% function mod_sym = modulate_or_duplicate (sym,org_bw,exp_bw_string)

% This function will create 128 fft point as functino of 64 fft input

% according to :

%

% This function also multiplies the upper part by j

%

%

% Input    sym:Nfft(Beforeshift)x nsym x nsts

%          org_bw 20/40

%          exp_bw_string

%                       'none' - do nothing

%                       'upper'

%                       'lower'

%                       'duplicate'

%

% Ouput

%            mod_sym: Nfft x nsym x nsts

function mod_sym = modulate_or_duplicate (sym,org_bw,exp_bw_string)

if (org_bw==40 )

if (size(sym,1)~=128)

errordlg('input vector has wrong format')

end

mod_sym = sym;

mod_sym(1+(0:63),:,:)=mod_sym(1+(0:63),:,:)*j;

return;

end

if (size(sym,1)~=64)

errordlg('input vector has wrong format')

end

if (strcmp(exp_bw_string,'none') )

mod_sym = sym;

return ;

end

mod_sym = zeros(128,size(sym,2),size(sym,3));

if (strcmp(exp_bw_string,'lower') )

mod_sym(1+(96:96+31),:,:)= sym(1+(0:31),:,:);

mod_sym(1+(64:64+31),:,:)=sym(1+(32:63),:,:);

end

if (strcmp(exp_bw_string,'upper') )

mod_sym(1+(32:32+31),:,:)=sym(1+(0:31),:,:);

mod_sym(1+(0:31),:,:)=sym(1+(32:63),:,:);

end

if (strcmp(exp_bw_string,'duplicate') )

mod_sym(1+(0:31),:,:)=sym(1+(32:63),:,:) *j;

mod_sym(1+(32:32+31),:,:)=sym(1+(0:31),:,:) *j;

mod_sym(1+(64:64+31),:,:)=sym(1+(32:63),:,:);

mod_sym(1+(96:96+31),:,:)=sym(1+(0:31),:,:);

end

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File mul_by_q.m

% function sym_tx_ant =  mul_by_q (sts_symbols,q_mat,post_q_cdd)

% multiply by  Q matrix and adds cdd

% The cdd could be added on top of Q instead of defining it separately but

% for spatial expansion it is more clear to separate const matrix and cdd

% Input

%           sts_symbols :fft_size x nsym x nsts

%           q_mat   :    nTx x nSTS x nFFT

%           post_q_cdd: cdd to apply after the multiplication

% Output

%           sym_tx_ant fft_size x nsym x nTx

%

function sym_tx_ant = mul_by_q (sts_symbols,q_mat,post_q_cdd)

nsts =     size(sts_symbols,3);

nsym =     size(sts_symbols,2);

fft_size = size(sts_symbols,1);

nTx = size(q_mat,1);

q_bin = zeros(nTx,nsts);

sym_tx_ant = zeros([fft_size nsym nTx]);

for nfft = (1:fft_size)

for isym = 1:nsym

q_bin = q_mat(1:nTx,1:nsts,nfft); % extract Q for specific bin

sym_tx_ant(nfft,isym,:) = q_bin*squeeze(sts_symbols(nfft,isym,:));

end

end

sym_tx_ant=add_cdd(sym_tx_ant,post_q_cdd);

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File puncture_bits.m

% puncturing function

% function punct_bits = puncture_bits(bits,pattern)

% input : coded bits

%         puncture patterns vector (1 retain 0 puncture)

%

function punct_bits = puncture_bits(bits,pattern)

len = length(pattern);

punct_bits = bits(find(pattern(1+mod((0:length(bits)-1),len))==1));

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File qam_map.m

% maps a single spatial stream

% function symbols = qam_map(bits,bpsc)

% Input : bits - bits to map

%         bpsc - bits per symbol (1,2,4,6)

% Output: Symbols

%

function symbols = qam_map(bits,bpsc)

MAP1     =            [-1 1];

MAP2     = sqrt(1/2) *[-1 1];

MAP4     = sqrt(1/10)*[-3 -1 3 1];

MAP6     = sqrt(1/42)*[-7 -5 -1 -3 7 5 1 3];

if (bpsc==1)

symbols = MAP1(bits+1);

return

end

sym_ind = [reshape(bits,(bpsc/2),length(bits)/(bpsc/2))' ]*2.^(bpsc/2-1:-1:0)';

if (bpsc ==2 )

sym_re = MAP2(1+sym_ind);

end

if (bpsc ==4 )

sym_re = MAP4(1+sym_ind);

end

if (bpsc ==6 )

sym_re = MAP6(1+sym_ind);

end

symbols = sym_re(1:2:end)+j*(sym_re(2:2:end));

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File read_bits.m

%function bits = read_bits (name)

% input : file name that contains rows with byte in each row

% output: column of bits generated byte by byte with lsb first

%

function bits = read_bits (name)

fid = fopen (name,'rt');

hex_read = fscanf(fid,'%s\n'); % appends all words

fclose(fid);

bytes = reshape(hex_read,2,length(hex_read)/2)'; % resahpe to bytes

bytes_dec = hex2dec(bytes);     % convert to int

bits = bitand(floor(bytes_dec*(2.^-(0:7))),1); % separate to bits

bits=bits';                                    %

bits=bits(:);                                  % create bit stream in correct order

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File scrambler_seq.m

% scrambler function

% function scr_bits = scrambler_seq(start,size)

%      Input: start - scrambler initialization

%             size  - length of sequence to scrambler

%      Output: scrambler sequence to xor with bits

%

function scr_bits = scrambler_seq(start,size)

% periodic scrambler sequence

scr_seq =...

[...

0     0     0     0     1     1     1     0     1     1     1     1 ...

0     0     1     0     1     1     0     0     1     0     0     1 ...

0     0     0     0     0     0     1     0     0     0     1     0 ...

0     1     1     0     0     0     1     0     1     1     1     0 ...

1     0     1     1     0     1     1     0     0     0     0     0 ...

1     1     0     0     1     1     0     1     0     1     0     0 ...

1     1     1     0     0     1     1     1     1     0     1     1 ...

0     1     0     0     0     0     1     0     1     0     1     0 ...

1     1     1     1     1     0     1     0     0     1     0     1 ...

0     0     0     1     1     0     1     1     1     0     0     0 ...

1     1     1     1     1     1     1];

%map scrambler bits to register value

scr_init_seq = filter (2.^(0:6),1,[scr_seq scr_seq(1:6)]);

% find where matches start value

init_loc  = find(scr_init_seq(7:end)==start)+6;

% generate periodic sequence that starts from init value

scr_bits = scr_seq(1+mod(init_loc+(0:size-1),127));

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File set_song_parameters.m

% This is a default configuration to generate the 802.11a Annex G packet.

% Should generate the exact waveform of Annex G if window is modified

clear sim80211nconfig     % clear configuration

sim80211nconfig.BitSource='';        % or filename

sim80211nconfig.BitSource='song.hex';        % or filename

sim80211nconfig.FORMAT='NON_HT'; %Legacy  % MixedMode %GreenField

sim80211nconfig.L_LENGTH = 0;    % autocalculate spoofed length

sim80211nconfig.LENGTH   = 5;       % bytes could be overriden by file

sim80211nconfig.MCS      = 5;       % for legacy use 0->7

sim80211nconfig.SHORT_GI  = 0;

sim80211nconfig.STBC     = 0;

sim80211nconfig.NUM_EXTEN_SS =0;

sim80211nconfig.SMOOTHING =1;

sim80211nconfig.NOT_SOUNDING =1;

sim80211nconfig.LDPC_CODING =0;

sim80211nconfig.AGGREGATION =0;

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

% The mapping of CW and CW_OFFSET from the TXVECTOR definition

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

%                        CW              CW_OFFSET         comments

%                        -----           ---------         ---------

%   20Mhz                CW20              N/A             HT and Legacy

%   40Mhz                CW40              CH_OFF_40       including  mcs32

%   40Mhz Duplicate      CW20DN            CH_OFF_40       only legacy

%   40Mhz Upper          CW40              CH_OFF_20U

%   40Mhz Lower          CW40              CH_OFF_20L

%

sim80211nconfig.CW        = 'HT_CW20';  %'HT_CW20','HT_CW40','HT_CW20DN'

sim80211nconfig.CW_OFFSET = 'CH_OFF_40';  %'CH_OFF_40','CH_OFF_20U','CH_OFF_20L'

sim80211nconfig.output_rate = 20;

sim80211nconfig.nTx      = 1;

sim80211nconfig.SERVICE.ScramblerStart  = hex2dec('5d');

sim80211nconfig.SERVICE.bits            = zeros(16,1)  ; % may change reserved to

% verify ignores

% reseverd bits

sim80211nconfig.HT_SIG_RESERVED =1; % It is illegal to set to 0 according to spec

% but can be used in testing to verify

% rx ignores this bit

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

% Spatial expansion and Beamforming initializations

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

sim80211nconfig.post_q_cdd = zeros(1:sim80211nconfig.nTx); % This is not relevant to Legacy

sim80211nconfig.calib       = 0; % Setting the bit to 1 will for calibration Q

sim80211nconfig.BF_Q_source ='default'; % 'default' 'const+cdd','general'

%sim80211nconfig.FixedQ      =0;         % specify Q for  'const+cdd'

%sim80211nconfig.Q           = zeros(nTx,nSTS,64); % specify Q for 'general'

sim80211nconfig.win          =[]; % window at sample rate.Set to 0.5 to get exact Annex G

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File start.m

function start(in)

addpath gui

addpath data

if nargin==0

txn

else

addpath lab

addpath lab\11b

txn_lab

end

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File stbc_expand.m

% function sym_stbc = stbc_expand(sym,Ndata)

% input: symbol in linear array and Number of data symbols

% output: added new spatial stream in linear array

function sym_stbc = stbc_expand(sym,Ndata)

sym_stbc = reshape(sym,length(sym)/Ndata,Ndata)';  % reshape and conj each line in ofdm sym

sym_stbc = sym_stbc((1+bitxor((1:size(sym_stbc,1))-1,1)),:);% switch line pairs

sym_stbc(1:2:end,:)=-sym_stbc(1:2:end,:);          % sign invert for first lines

sym_stbc = conj(sym_stbc');                        %transpose

sym_stbc = sym_stbc(:);                            %linear again

sym_stbc = conj(sym_stbc');

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File stream_parse.m

% stream parse function

%

% function bits_per_ss_i = stream_parse(bits,bspc_streams,istream)

%

% Input: bits -  input bit vector

%        bspc_streams - vector of bit per carrier per stream

%        istream - chosen stream 1-nSS

function bits_per_ss_i = stream_parse(bits,bspc_streams,istream)

bspc_streams_axis = max(bspc_streams/2,1);

sum_s = sum(bspc_streams_axis);

sum_to_stream = sum(bspc_streams_axis(1:istream-1));

npbs_i = bspc_streams_axis(istream);

% collect bits for spatial stream istream

bits_per_ss_i = bits(find((mod(0:length(bits)-1,sum_s)>=sum_to_stream) & (mod(0:length(bits)-1,sum_s)<sum_to_stream+npbs_i)));

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File tone_field_scale.m

% function scale = tone_field_scale(sig_name,islegacy,ismcs32,uses_double_chan)

% returns scale which is 1/sqrt(NTonesFIeld)

% Inputs: sig_name - string containing signal name: 'L-STF  ','L-LTF  ' etc

%         islegacy,ismcs32,uses_double_chan - the other attributes which are

%         needed for scale determinatino

function scale = tone_field_scale(sig_name,islegacy,ismcs32,uses_double_chan)

names =     ['L-STF  ';'L-LTF  ';'L-SIG  ';'HT-SIG ';'HT-STF ';'HT-LTF ';'HT-Data';'HTSIGGF';];

NTonesField=[12 24;    52 104;   52 104;   52 104;    12 24;    56 114;   56 114;   56 114;];

index = strmatch(sig_name,names );

if ((islegacy||ismcs32) && (index==7))

index = 2;

end

ToneField =  NTonesField(index,1+uses_double_chan);

scale = 1/sqrt(ToneField);

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File tx_n_globals.m

% tx_n_globals file

% Initializes tables, resets debug struct

function  tx_n_globals()

global Legacy_Info   MCS_Info20   MCS_Info40;

global PHTLTF;

global T_ITX_CS T_ISTS_TS; %table n65

global tx_n_debug;

global num_vec denum_vec;

tx_n_debug=[];

%mcs    puncturing-ind  NBPSC   Lsig Rate Field

Legacy_Info = [ ...

0   0   1   11  ;

1   2   1   15  ;

2   0   2   10  ;

3   2   2   14  ;

4   0   4   9   ;

5   2   4   13  ;

6   1   6   8   ;

7   2   6   12  ];

%mcs    puncturing-ind  NSS constel1    constel2    constel3    constel4

MCS_Info20 = [ ...

0     0     1     1     0     0     0;

1     0     1     2     0     0     0;

2     2     1     2     0     0     0;

3     0     1     4     0     0     0;

4     2     1     4     0     0     0;

5     1     1     6     0     0     0;

6     2     1     6     0     0     0;

7     3     1     6     0     0     0;

8     0     2     1     1     0     0;

9     0     2     2     2     0     0;

10     2     2     2     2     0     0;

11     0     2     4     4     0     0;

12     2     2     4     4     0     0;

13     1     2     6     6     0     0;

14     2     2     6     6     0     0;

15     3     2     6     6     0     0;

16     0     3     1     1     1     0;

17     0     3     2     2     2     0;

18     2     3     2     2     2     0;

19     0     3     4     4     4     0;

20     2     3     4     4     4     0;

21     1     3     6     6     6     0;

22     2     3     6     6     6     0;

23     3     3     6     6     6     0;

24     0     4     1     1     1     1;

25     0     4     2     2     2     2;

26     2     4     2     2     2     2;

27     0     4     4     4     4     4;

28     2     4     4     4     4     4;

29     1     4     6     6     6     6;

30     2     4     6     6     6     6;

31     3     4     6     6     6     6;

33     0     2     4     2     0     0;

34     0     2     6     2     0     0;

35     0     2     6     4     0     0;

36     2     2     4     2     0     0;

37     2     2     6     2     0     0;

38     2     2     6     4     0     0;

39     0     3     4     2     2     0;

40     0     3     4     4     2     0;

41     0     3     6     2     2     0;

42     0     3     6     4     2     0;

43     0     3     6     4     4     0;

44     0     3     6     6     2     0;

45     0     3     6     6     4     0;

46     2     3     4     2     2     0;

47     2     3     4     4     2     0;

48     2     3     6     2     2     0;

49     2     3     6     4     2     0;

50     2     3     6     4     4     0;

51     2     3     6     6     2     0;

52     2     3     6     6     4     0;

53     0     4     4     2     2     2;

54     0     4     4     4     2     2;

55     0     4     4     4     4     2;

56     0     4     6     2     2     2;

57     0     4     6     4     2     2;

58     0     4     6     4     4     2;

59     0     4     6     4     4     4;

60     0     4     6     6     2     2;

61     0     4     6     6     4     2;

62     0     4     6     6     4     4;

63     0     4     6     6     6     2;

64     0     4     6     6     6     4;

65     2     4     4     2     2     2;

66     2     4     4     4     2     2;

67     2     4     4     4     4     2;

68     2     4     6     2     2     2;

69     2     4     6     4     2     2;

70     2     4     6     4     4     2;

71     2     4     6     4     4     4;

72     2     4     6     6     2     2;

73     2     4     6     6     4     2;

74     2     4     6     6     4     4;

75     2     4     6     6     6     2;

76     2     4     6     6     6     4;

];

MCS_Info40 = [ ...

0     0     1     1     0     0     0;

1     0     1     2     0     0     0;

2     2     1     2     0     0     0;

3     0     1     4     0     0     0;

4     2     1     4     0     0     0;

5     1     1     6     0     0     0;

6     2     1     6     0     0     0;

7     3     1     6     0     0     0;

8     0     2     1     1     0     0;

9     0     2     2     2     0     0;

10     2     2     2     2     0     0;

11     0     2     4     4     0     0;

12     2     2     4     4     0     0;

13     1     2     6     6     0     0;

14     2     2     6     6     0     0;

15     3     2     6     6     0     0;

16     0     3     1     1     1     0;

17     0     3     2     2     2     0;

18     2     3     2     2     2     0;

19     0     3     4     4     4     0;

20     2     3     4     4     4     0;

21     1     3     6     6     6     0;

22     2     3     6     6     6     0;

23     3     3     6     6     6     0;

24     0     4     1     1     1     1;

25     0     4     2     2     2     2;

26     2     4     2     2     2     2;

27     0     4     4     4     4     4;

28     2     4     4     4     4     4;

29     1     4     6     6     6     6;

30     2     4     6     6     6     6;

31     3     4     6     6     6     6;

32     0     1     1     0     0     0;

33     0     2     4     2     0     0;

34     0     2     6     2     0     0;

35     0     2     6     4     0     0;

36     2     2     4     2     0     0;

37     2     2     6     2     0     0;

38     2     2     6     4     0     0;

39     0     3     4     2     2     0;

40     0     3     4     4     2     0;

41     0     3     6     2     2     0;

42     0     3     6     4     2     0;

43     0     3     6     4     4     0;

44     0     3     6     6     2     0;

45     0     3     6     6     4     0;

46     2     3     4     2     2     0;

47     2     3     4     4     2     0;

48     2     3     6     2     2     0;

49     2     3     6     4     2     0;

50     2     3     6     4     4     0;

51     2     3     6     6     2     0;

52     2     3     6     6     4     0;

53     0     4     4     2     2     2;

54     0     4     4     4     2     2;

55     0     4     4     4     4     2;

56     0     4     6     2     2     2;

57     0     4     6     4     2     2;

58     0     4     6     4     4     2;

59     0     4     6     4     4     4;

60     0     4     6     6     2     2;

61     0     4     6     6     4     2;

62     0     4     6     6     4     4;

63     0     4     6     6     6     2;

64     0     4     6     6     6     4;

65     2     4     4     2     2     2;

66     2     4     4     4     2     2;

67     2     4     4     4     4     2;

68     2     4     6     2     2     2;

69     2     4     6     4     2     2;

70     2     4     6     4     4     2;

71     2     4     6     4     4     4;

72     2     4     6     6     2     2;

73     2     4     6     6     4     2;

74     2     4     6     6     4     4;

75     2     4     6     6     6     2;

76     2     4     6     6     6     4;

];

PHTLTF = [ 1 -1  1   1 ;

1  1 -1   1 ;

1  1  1  -1 ;

-1  1  1   1 ;];

% Table n65

T_ITX_CS     = [  0    0    0    0;

0 -200    0    0;

0 -100 -200    0;

0  -50 -100 -150;];

T_ISTS_TS    = [  0    0    0    0;

0 -400    0    0;

0 -400 -200    0;

0 -400 -200 -600;];

% Code rate numerator and denumerator

num_vec   = [ 1 2 3 5];

denum_vec = [ 2 3 4 6];

global Punct Pilot_Subcarriers_20 Pilot_Subcarriers_40

Punct_1_2 = [1 ] ;

Punct_2_3 = [1 1 1 0 1 1 1 0 1 1 1 0] ;

Punct_3_4 = [1 1 1 0 0 1 1 1 1 0 0 1 1 1 1 0 0 1] ;

Punct_5_6 = [1 1 1 0 0 1 1 0 0 1 ] ;

Punct = cell(4,1);

Punct{1}= Punct_1_2;

Punct{2}= Punct_2_3;

Punct{3}= Punct_3_4;

Punct{4}= Punct_5_6;

Pilot_Subcarriers_20=[-21 -7 7 21];

Pilot_Subcarriers_40=[-53 -25 -11 11 25 53] ;

%%%%%%%%%%%%%%     End of File

%%%%%%%%%%%%%%%    Start of File write_bits.m

% function  write_bits (bits)

% writes bits to file grouped to bytes

function  write_bits (bits)

bitgroup = reshape(bits,8,length(bits)/8)'; % resahpe to bytes

bytes = bitgroup*(2.^(0:7)');

fid = fopen ('bits.hex','wt');

fprintf(fid,'%02x\n',bytes); % appends all words

fclose(fid);

%%%%%%%%%%%%%%     End of File