IEEE P802.11
Wireless LANs

| Normative Text Proposal for QoS aware Load Balancing |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-1-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Yong Bai, Lan Chen, Lei Du | DoCoMo Beijing Labs Inc. | 7/F Raycom Infotech Park Tower A, No.2 Kexueyuan South Road, Haidian District, Beijing, P.R.China | +86-10-82861501 {ex.336, 123, 312} | {bai, chen, du}@docomolabs-beijing.com.cn |
| Sunghyun Choi | Seoul National University | Kwanak P.O.Box 34, Seoul 151-600, Korea | +82-2-880-1753 | schoi@snu.ac.kr |
| Jin Sunggeun | ETRI | 161 Gajeong-Dong, Yuseong-gu, Daejeon, 305-700, Korea | +82-2-880-1755 | sgjin@etri.re.kr |
| Moo Ryong Jeong, Fujio Watanabe | DoCoMo USA Labs Inc. | 3240 Hillview Ave, Palo Alto, CA 94304-1201, USA | +1-650-493-9600 | {jeong, watanabe}@docomolabs-usa.com |
| Akira Yamada | NTT DoCoMo Inc. | 3-5, Hikarinooka, Yokosuka-shi, Kanagawa, 239-8536, Japan | +81-46-840-6527 | yamadaakir@nttdocomo.co.jp |

Abstract

This document contains a normative text proposal in support of an AP load balancing protocol to improve wireless network throughput and QoS effectiveness in accordance with REQ2030.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- 7. Frame formats

- 7.2 Format of individual frame types

- 7.2.3 Management frames

- 7.2.3.1 Beacon frame format

##### Insert new rows into table 8 as follows:

Table 8—Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 30 | AC Station Count | The AC Station Count element shall be present in a QoS AP where dot11RadioMeasurementEnabled is true. |

- 7.2.3.9 Probe Response frame format

##### Insert new rows into table 15 as follows:

Table 15—Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 29 | AC Station Count | The AC Station Count element shall be present in a QoS AP where dot11RadioMeasurementEnabled is true. |

- 7.3 Management frame body components

### 7.3.2 Information Elements

### Change the table 26 as follows:

Table 26－Element IDs

| Information Element | Element ID | Length (in octets) |
| --- | --- | --- |
| BSS Available Admission Capacity (see 7.3.2.43) | 67 | 4 to 36 |
| BSS AC Access Delay (see 7.3.2.44) | 68 | 6 |
| AC Station Count (see 7.3.2.44a) | 69 | 3 to 11 |
| Reserved | 69 70-126 |  |

- 7.3.2.22 Measurement Report element

- 7.3.2.22.8 STA Statistics Report

- Change the order 10 in table 31b as follows:

Table 31b－ Group Identity for a STA Statistics Report (continued)

| Group Identity Requested | Statistics Returned |
| --- | --- |
| 10 | dot11BSSAverageAccessDelay Group (only available at an AP): dot11STAStatisticsAPAverageAccessDelay (INTEGER), dot11STAStatisticsAverageAccessDelayBestEffort (INTEGER), dot11STAStatisticsAverageAccessDelayBackGround (INTEGER), dot11STAStatisticsAverageAccessDelayVideo (INTEGER), dot11STAStatisticsAverageAccessDelayVoice (INTEGER), dot11STAStatisticsACStationCount (INTEGER), |
| 11－255 | None |

- Change Figure 85k as follows:

|  | dot11STA StatisticsAP AverageAccess Delay | dot11STA Statistics AverageAccess DelayBestEffort | dot11STA Statistics AverageAccess DelayBackGround | dot11STA Statistics AverageAccess DelayVideo | dot11STA Statistics AverageAccess DelayVoice | dot11STA Statistics ACStation Count |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | 2 3-11 |

Figure 85k---Measurement Report field format for dot11BSSAverageAccessDelay Group

- 7.3.2.27 Neighbor Report element

- Change the table 43B in 802.11v Draft 0.05 as follows:

- Table 43B—Optional Extensions

| Sub-Element ID | Name |
| --- | --- |
| 0 | Reserved |
| 1 | TSF Information |
| 2 | Roaming Candidate Preference |
| 3 | QBSS Available Admission Capacity |
| 4 | AC Station Count |
| 4 5-10 | Reserved |
| 11 | QBSS Load |
| 12-255 | Reserved |

- Insert the following clause after 7.2.3.44:

- 7.3.2.44a AC Station Count

The AC Station Count element contains a list of Station Count fields at different Access Categories as shown in Figure X1.

|  | Element ID | Length | Station Count Bitmask | Station Count List |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 2 x (total number of non-zero bits in Station Count Bitmask) |

Figure X1--- AC Station Count element format

The Length field in octets shall be set to 2+2Nnz , where Nnz equals the total number of non-zero bits in Station Count Bitmask).

The Station Count Bitmask field indicates the AC values that have Station Count specified in the following Station Count List. The format of the Station Count Bitmask is defined in Table 44a.

Table 44a－ Station Count Bitmask Definition

| Bit (s) | Station Count Reported |
| --- | --- |
| 0 | AC 0 (VO) |
| 1 | AC 1 (VI) |
| 2 | AC 2 (BE) |
| 3 | AC 3 (BK) |
| 4-7 | Reserved |

The bit is set to 1 to indicate that the Station Count for the corresponding access category is present in the Station Count List field. The bit is set to 0 to indicate that the Station Count for the corresponding access category is not present in the Station Count List field.

The Station Count List comprises a sequence of Station Count fields corresponding respectively to the non-zero bits in the Station Count Bitmask field.

The Station Count field is 2 cotets long and contains an unsigned integer that specifies the number of STAs that are currently associated with this QBSS for the corresponding AC traffic. The field is helpful for roaming non-AP QSTAs to select a QAP that is likely to result in higher total throughput and QoS effectiveness.

- 10. Layer management

- 10.3 MLME SAP Interface

### 10.3.2 Scan

- 10.3.2.2 MLME-SCAN.confirm

- 10.3.2.2.2 Semantics of the service primitive

- Insert the following rows at the end of the BSSDescription table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| AC Station Count | As defined in frame format | As defined in frame format | The values from the AC Station Count information element if such an element was present in the probe response or beacon, else null. Present on when the MIB atttibute dot11RadioMeasurementEnabled is true |