IEEE P802.11
Wireless LANs

| Key Lifetime for PSK |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bill Marshall | TGr Editor | 180 Park Ave, Florham Park, NJ 07932 | 973-360-8718 | wtm@research.att.com |

Abstract

Current TGr limits the Key Lifetime to a maximum of 45 days (65,535 minutes).

This submission proposes that a Key Lifetime value of zero (0) indicate a PSK that has no expiration.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Overview

Comment #879 of LB87 pointed out a situation where the STA and R1KH can get out of sync in their understanding of remaining key lifetime, when the negotiated AKM is PSK. This came as a result of the removal of the Nonce in the key name calculation, resulting in the PMKR0Name value being constant for PSK.

This submission includes the normative text based on the results of the straw poll conducted at the TGr ad-hoc session 11/13/2006 (result of straw poll for this resolution was yes-10, no-1), which proposed that this problem be rectified by making the key lifetime of the PMK-R0 generated by PSK infinite.

Changes to D3.0

Change third paragraph of 8.5a.1 as follows:

The lifetime of the PMK-R0, PMK-R1, and PTK keys are bound to the lifetime of the PSK or MSK. For example, the 802.1X AS may communicate the MSK key lifetime with the MSK. If such an attribute is provided, the lifetime of the PMK-R0 shall be not more than the lifetime of the MSK. If the MSK lifetime attribute is not provided, or for PSK, the key lifetime shall be the value of the MIB variable dot11FTR0KeyLifetime. For PSK, the key lifetime shall be infinite.

Change final paragraph of 7.3.2.47 as follows:

For the Reassociation Deadline interval, a value of zero indicates no deadline exists. For the Key Lifetime interval, a value of zero indicates the key lifetime is infinite, and will not expire is reserved.

Change description of dot11FTR0KeyLifetime as follows:

"This attribute shall specify the default lifetime of the PMK-R0, in minutes, when derived from a PSK, or when a Session-Timeout attribute is not provided during the EAP authentication. This attribute does not apply to PSK, where the key lifetime is infinite."

Resolve LB87 comments as follows:

| 856 | Counter. Initial association for PSK changed so that the key lifetime of the generated PMK-R1 matches the infinite lifetime of the PSK itself. Text changes in 11-06-1745. |
| --- | --- |
| 859 | Counter. Initial association for PSK changed so that the key lifetime of the generated PMK-R1 matches the infinite lifetime of the PSK itself. Text changes in 11-06-1745. |
| 897 | Counter. Initial association for PSK changed so that the key lifetime of the generated PMK-R1 matches the infinite lifetime of the PSK itself. Text changes in 11-06-1745. |
| 899 | Counter. Initial association for PSK changed so that the key lifetime of the generated PMK-R1 matches the infinite lifetime of the PSK itself. Text changes in 11-06-1745. |
| 911 | Accepted. With PMK-R0NameSalt, PMKR0Name isn't needed in formula for PMK-R1. |
| 1143 | Accepted |