IEEE P802.11
Wireless LANs

| Extended Channel Switch Announcement |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-12-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bjorn A. Bjerke | Qualcomm, Inc. | 9 Damonmill Square, Suite 2A, Concord, MA 01742, USA | +1 781-276-0912 | bbjerke@qualcomm.com |
| Eldad Perahia | Intel Corporation |  |  | eldad.perahia@intel.com |
| Adrian Stephens | Intel Corporation | 15 JJ Thompson Avenue, Cambridge CB3 0FD, UK | +44 1223 763457 | adrian.p.stephens@intel.com |
| Peter Ecclesine | Cisco Systems | MS SJ-10-5 170 West Tasman Dr, San Jose, CA 95134-1706, USA | +1 408-527-0815 | pecclesi@cisco.com |

Abstract

This submission contains normative text related to adding an Extended Channel Switch Announcement to the 802.11n draft.  The intention is to extend the DFS capabilities introduced in 802.11h for Channel Switch Announcement to all regulatory classes (including 40 MHz operation, and operation in 2.4 GHz and 5 GHz). This document borrows significantly from the Extended Channel Switch Announcement proposals adopted by TGv and TGy (e.g., 06/1432r2), and authored by Jari Jokela (Nokia), Emily Qi (Intel) and Allan Thomson (Cisco).

The submission also addresses 8 comments received on P802.11n Draft 1.0. The CIDs are: 1065, 2820, 6880, 1066, 4533, 6881, 7511, and 7512.

The text is aligned with P802.11-REVma/D9.0 and P802.11n-D1.07.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Submission note: The following 8 CIDs are concerned with su clause 11.9.8.3 (11.9.5.3 in Draft 1.0). The proposed text contained in this document address these comments.

CIDs and their proposed resolution:

| 1065 | 150 | 20 | 11.9.5.3 | it is not clear that an AP "has" a channel when it powers up, but I think that I can see the intent here - but there is no period of time specified, so the statement is sort of pointless as written | change the sentence beginning with "An HT AP" to "An HT AP shall receive the beacons of other HT BSSs on its desired channel of operation for ProbeDelay time before starting a BSS." | Counter: New text in 1775r1 avoids the problem |
| --- | --- | --- | --- | --- | --- | --- |
| 2820 | 150 | 20 | 11.9.5.3 | Can't mandate that a STA receive anything. Too many unknowns, like signal strength and interference | Change to "An HT AP shall listen for Beacons of other…" | Counter: New text in 1775r1 avoids the problem |
| 6880 | 150 | 22 | 11.9.5.3 | How can the AP do "background scanning"? This places a requirement for the AP to have multiple radios. | Delete this sentence | Counter: new text in 1775r1 avoids problem |
| 1066 | 150 | 25 | 11.9.5.3 | seems like it is not necessary for a STA to dissociate - why is it not possible for the STA to simply also switch to the 20MHz mode? | change "The STA" to "A STA" change "de-associate" to "disassociate" and change the "shall" to "may" | Counter: new text in 1775r1 avoids problem |
| 4533 | 150 | 27 | 11.9.5.3 | This needs to be much more specific | Specify rules that the AP must follow to ensure that power-save STAs are informed about the channel transition | Counter: new text in 1775r1 provides guidelines addressing STAs in power save mode |
| 6881 | 150 | 27 | 11.9.5.3 | Too vague. | Specify rules that the AP must follow to ensure that power-save STAs are informed about the channel transition | Counter: new text in 1775r1 provides guidelines addressing STAs in power save mode |
| 7511 | 150 |  | 11.9.5.3 | "An HT AP shall receive the beacons of other HT BSSs on its channel when it first powers up. " This doesn't say how long, or what "first powers up" means. | This is actually a requirement on an SME. It should be described in terms of the MLME primitives. | Counter: new text in 1775r1 avoids problem |
| 7512 | 150 |  | 11.9.5.3 | "In any transition of channel width, the AP should take in consideration any STAs that asleep. " Considerate APs are nice. But what this means is opaque. Unlike the channel switch announcement, the channel width change takes place immediately. It is not clear how the AP can be considerate. | Remove the sentence. | Counter: new text in 1775r1 uses the extended channel switch announcement |

TGn Editor: The following text should be inserted into the 802.11n draft as instructions to the 802.11 base standard editor.

- 7.2.3.1 Beacon frame format

Insert new row into Table 8 as shown below:

- Table 8—Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
|  | Extended Channel Switch Announcement | Extended Channel Switch Announcement element may be present if dot11ExtendedChannelSwitchImplemented is true. |

- 7.2.3.4 Association Request frame format

Insert new row into Table 10 as shown below:

- Table 10—Association Request frame body

| Order | Information | Notes |
| --- | --- | --- |
|  | Supported Regulatory Classes | Supported Regulatory Classes information element is present if dot11ExtendedChannelSwitchImplemented is true. |

- 7.2.3.5 Association Response frame format

Insert new row into Table 11 as shown below:

- Table 11—Association Response frame body

| Order | Information | Notes |
| --- | --- | --- |
|  | Supported Regulatory Classes | Supported Regulatory Classes information element is present if dot11ExtendedChannelSwitchImplemented is true. |

- 7.2.3.6 Reassociation Request frame format

Insert a new row into Table 12 as shown below:

- Table 12—Reassociation Request frame body

| Order | Information | Notes |
| --- | --- | --- |
|  | Supported Regulatory Classes | Supported Regulatory Classes information element is present if dot11ExtendedChannelSwitchImplemented is true. |

- 7.2.3.7 Reassociation Response frame format

Insert new row into Table 13 as shown below:

- Table 13—Reassociation Response frame body

| Order | Information | Notes |
| --- | --- | --- |
|  | Supported Regulatory Classes | Supported Regulatory Classes information element is present if dot11ExtendedChannelSwitchImplemented is true. |

- 7.2.3.9 Probe Response frame format

Insert new row into Table 15 as shown below:

- Table 15—Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
|  | Extended Channel Switch Announcement | Extended Channel Switch Announcement element is present if dot11ExtendedChannelSwitchImplemented is true. |

- 7.3.2 Information Elements

Insert Element ID x into Table 26 and change the Reserved row accordingly:

Table 26—Element IDs

| Information Element | Element ID | Length (in octets) |
| --- | --- | --- |
| Extended Channel Switch Announcement | x | 6 |

TGn Editor: In subclause 7.2.3.1, Table 8 (Beacon Frame format), delete the “Secondary Channel Offset” row. This affects CID 1742. New resolution (counter): The row in question has been removed, as it is redundant. The secondary channel offset information is present in the HT Information element.

TGn Editor: In subclause 7.2.3.9, Table 15 (Probe Response Frame format), delete the “Secondary Channel Offset” row. This affects CID 1742 (same new resolution as outlined above).

TGn Editor: Delete the entire subclause 7.3.2.20a (Secondary Channel Offset element). The CIDs affected are 1742 (same new resolution as above) and 632. New resolution for 632 (counter): The subclause in question has been removed, so the clarifications suggested are not needed.

Insert the following new subclause 7.3.2.36:

- 7.3.2.36 Extended Channel Switch Announcement element

The Extended Channel Switch Announcement element is used by an AP in a BSS or a STA in an IBSS to advertise

when it is changing to a new channel and a new regulatory class. The announcement includes both the regulatory class and the channel number of the new channel, or, in the case of 40 MHz operation, the primary channel of the new channel pair. The format of the Extended Channel Switch Announcement element is shown in Figure 112b.

|  | Element ID | Length | Channel Switch Mode | New Regulatory Class | New Channel Number | Channel Switch Count |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | 1 |

Figure 112b-Extended Channel Switch Announcement element

The Length field shall be set to 4.

The Channel Switch Mode field indicates any restrictions on transmission until a channel switch. An AP in a

BSS or a STA in an IBSS sets the Channel Switch Mode field to either 0 or 1 on transmission. A Channel

Switch Mode set to 1 means that the STA in a BSS to which the frame containing the element is addressed transmits no further frames within the BSS until the scheduled channel switch. A STA in an IBSS may treat a Channel Switch Mode field set to 1 as advisory. A Channel Switch Mode set to 0 does not impose any requirement on the receiving STA.

The New Regulatory Class field shall be set to the number of the regulatory class to which the STA is moving, as defined in Annex J.

The New Channel Number field shall be set to the number of the channel to which the STA is moving. The channel number shall be a channel from the STA’s new Regulatory Class as defined in Annex J. In the case where the STA is moving to a 40 MHz channel (i.e., a pair of 20 MHz channels), the channel number refers to the primary channel. The position of the secondary channel (above or below the primary channel) is uniquely specified by the New Regulatory Class field.

The Channel Switch Count field either shall be set to the number of TBTTs until the STA sending the Channel

Switch Announcement element switches to the new channel or shall be set to 0. A value of 1 indicates that the switch shall occur immediately before the next TBTT. A value of 0 indicates that the switch shall occur at any time after the frame containing the element is transmitted.

TGn Editor: Update the references to section numbers below

The Extended Channel Switch Announcement element may be included in Channel Switch Announcement frames, as described in 7.4.1.5, and Extended Channel Switch Announcement frames, as described in 7.4.1.6, and may be included in Beacon frames, as described in 7.2.3.1, and Probe Response frames, as described in 7.2.3.9. The use of Extended Channel Switch Announcement elements and frames is described in 11.9.7 and 11.9.8.3.

Insert the following new subclause 7.3.2.37:

- 7.3.2.37 Supported Regulatory Classes element

The Supported Regulatory Classes element is used by a STA to advertise the Regulatory Classes that it supports. The format of the Supported Regulatory Classes element is shown in Figure 112c.

|  | Element ID | Length | Current Regulatory Class | Alternate Regulatory Classes |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | Length-1 |

Figure 112c-Supported Regulatory Classes element

The length of the Supported Regulatory Classes element is between 1 and 32 octets. The Current Regulatory Class octet indicates the Regulatory Class in use. The Alternate Regulatory Classes field contains (Length-1) regulatory class values, each in a single octet subfield.  These values represent alternative regulatory classes that the STA supports, excluding the Current Regulatory Class value. The values in the Alternate Regulatory Classes field are in increasing order.

The Supported Regulatory Classes element may be included in Association Request frames, as described in 7.2.3.4, Association Response frames, as described in 7.2.3.5, in Reassociation Request frames, as described in 7.2.3.6, and Reassociation Response frames, as described in 7.2.3.7.

TGn Editor: Change clause 7.4.1 as shown below:

- 7.4.1 Spectrum management action details

Five Several Action frame formats are defined for spectrum management. An Action Value field, in the octet field

immediately after the Category field, differentiates the five formats. The Action Value field values associated

with each frame format are defined in Table 44.

- Table 44—spectrum management action value field values

| Action Value field value | Description |
| --- | --- |
| 0 | Measurement Request |
| 1 | Measurement Response |
| 2 | TPC Request |
| 3 | TPC Report |
| 4 | Channel Switch Announcement |
| 5 | Extended Channel Switch Announcement |
| 56-255 | Reserved |

TGn Editor: Change clause 7.4.1.5 as shown below:

##### 7.4.1.5 Channel Switch Announcement frame format

Replace Figure 117 with the following figure:

EDITORIAL NOTE—The changes comprise insertion of the Secondary Channel Offset element field at the end of the existing structure.

|  | Category | Action value | Channel Switch Announcement element | Extended Channel Switch Announcement element |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 5 | 6 |

Figure 117--Channel Switch Announcement frame body format

Delete the paragraph at the end of subclause 7.4.1.5 and insert the following new paragraph:

The Secondary Channel Offset element is defined in 7.3.2.20a.  This element is present when switching to a 40 MHz channel. It may be present when switching to a 20 MHz channel (in which case the secondary channel offset is set to 0).

The Extended Channel Switch Announcement element shall be set as described 7.3.2.36. This element may be present if dot11ExtendedChannelSwitchImplemented is true.

Insert the following new subclause 7.4.1.6:

##### 7.4.1.6 Extended Channel Switch Announcement frame format

The Extended Channel Switch Announcement frame uses the Action frame body format and is transmitted by an AP in a BSS or a STA in an IBSS to advertise a channel switch. The format of the Extended Channel Switch Announcement frame body is shown in Figure 117a

|  | Category | Action | Extended Channel Switch Announcement Element |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 |

Figure 117a—Extended Channel Switch Announcement frame format

The Category field shall be set to 0 (representing spectrum management).

The Action Value field shall be set to 5 (representing an Extended Channel Switch Announcement frame).

The Extended Channel Switch Announcement element shall be set as described 7.3.2.36.

- 10.3 MLME SAP Interface

Change 10.3.6 as follows:

- 10.3.6 Associate

- 10.3.6.1 MLME-ASSOCIATE.request

        - Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-ASSOCIATE.request	(
PeerSTAAddress,
AssociateFailureTimeout,
CapabilityInformation,
ListenInterval,
Supported Channels,
RSN,
QoSCapability,

HT Capabilities,
SupportedRegulatoryClasses, 
VendorSpecificInfo
)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SupportedRegulatoryClasses | As defined in the Supported Regulatory Classes element | As defined in the Supported Regulatory Classes element | Specifies the Supported Regulatory Classes capabilities of the non-AP STA. This parameter shall only be present if the MIB attribute dot11ExtendedChannelSwitchImplemented is true. |

- 10.3.6.2 MLME-ASSOCIATE.confirm

- 10.3.6.3 MLME-ASSOCIATE.indication

- 10.3.6.4 MLME-ASSOCIATE.response

- 10.3.6.4.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-ASSOCIATE.response	(
PeerSTAAddress,
ResultCode,
CapabilityInformation,
AssociationID,
EDCAParameterSet, 
SupportedRegulatoryClasses, 
VendorSpecificInfo
)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SupportedRegulatoryClasses | As defined in the Supported Regulatory Classes element | As defined in the Supported Regulatory Classes element | Indicates the Supported Regulatory Classes capabilities of the AP. This parameter shall only be present if the MIB attribute dot11ExtendedChannelSwitchImplemented is true. |

Change 10.3.7 as follows:

- 10.3.7 Reassociate

- 10.3.7.1 MLME-REASSOCIATE.request

- 10.3.7.1.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-REASSOCIATE.request	(
NewAPAddress,
ReassociateFailureTimeout,
CapabilityInformation,
ListenInterval,
Supported Channels,
RSN,
QoSCapability,

HT Capabilities
SupportedRegulatoryClasses, 
VendorSpecificInfo
)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SupportedRegulatoryClasses | As defined in the Supported Regulatory Classes element | As defined in the Supported Regulatory Classes element | Specifies the Supported Regulatory Classes of the non-AP STA. This parameter shall only be present if the MIB attribute dot11ExtendedChannelSwitchImplemented is true. |

- 10.3.7.2 MLME-REASSOCIATE.confirm

- 10.3.7.3 MLME-REASSOCIATE.indication

- 10.3.7.4 MLME-REASSOCIATE.response

- 10.3.7.4.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-REASSOCIATE.response	(
PeerSTAAddress,
ResultCode,
CapabilityInformation,
AssociationID,
EDCAParameterSet, 
SupportedRegulatoryClasses, 
VendorSpecificInfo
)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SupportedRegulatoryClasses | As defined in the Supported Regulatory Classes element | As defined in the Supported Regulatory Classes element | Specifies the Supported Regulatory Classes of the non-AP STA. This parameter shall only be present if the MIB attribute dot11ExtendedChannelSwitchImplemented is true. |

TGn Editor: Delete the entire subclause 10.3.15 Channel Switch from the TGn Draft

Submission note – The changes to the base standard specified in subclause 10.3.15 of 802.11n-D1.07 are not needed, as Secondary Channel Offset is no longer needed after TGn adopted regulatory classes.

Insert the following new subclause:

10.3.30 Extended channel switch

The following MLME primitives support the signaling of Extended Channel Switch Announcement.

10.3.30.1 MLME-EXTCHANNELSWITCH.request

10.3.30.1.1 Function

This primitive requests that an Extended Channel Switch Announcement frame be sent by an AP.

10.3.30.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-EXTCHANNELSWITCH.request	(

Mode,

Regulatory Class,

Channel Number,

Channel Switch Count,

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Mode | Integer | 0,1 | Channel switch mode, as defined for the Extended Channel Switch Announcement element. |
| Regulatory Class | Integer | As defined in Annex J | Specifies the new regulatory class. |
| Channel Number | Integer | As defined in Annex J | Specifies the new channel number. |
| Channel Switch Count | Integer | 0 – 255 | Specifies the number of TBTTs until the channel switch event, as described for the Extended Channel Switch Announcement element. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

10.3.30.1.3 When generated

This primitive is generated by the SME to request that an Extended Channel Switch Announcement frame be sent to a non-AP STA that is associated to the AP.

10.3.30.1.4 Effect of receipt

On receipt of this primitive, the MLME constructs an Extended Channel Switch Announcement frame. The AP then attempts to transmit this to the STA that is associated.

10.3.30.2 MLME-EXTCHANNELSWITCH.confirm

10.3.30.2.1 Function

This primitive reports the result of a request to switch channel.

10.3.30.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-EXTCHANNELSWITCH.confirm	(

ResultCode,

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID PARAMETERS or UNSPECIFIED FAILURE | Reports the result of an extended channel switch request. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

10.3.30.2.3 When generated

This primitive is generated by the MLME when an extended channel switch request completes. Possible unspecified failure causes include an inability to schedule an extended channel announcement.

10.3.30.2.4 Effect of receipt

The SME is notified of the results of the extended channel switch procedure.

10.3.30.3 MLME-EXTCHANNELSWITCH.indication

10.3.30.3.1 Function

This primitive indicates that an Extended Channel Switch Announcement frame was received from an AP.

10.3.30.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-EXTCHANNELSWITCH.indication	(

Peer MAC Address,

Mode,

Regulatory Class,

Channel Number,

Channel Switch Count,

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerMAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity from which the Measurement Report frame was received. |
| Mode | Integer | 0,1 | Channel switch mode, as defined for the Extended Channel Switch Announcement element. |
| Regulatory Class | Integer | As defined in Annex J | Specifies the new regulatory class. |
| Channel Number | Integer | As defined in Annex J | Specifies the new channel number. |
| Channel Switch Count | Integer | 0 – 255 | Specifies the number of TBTTs until the channel switch event, as described for the Extended Channel Switch Announcement element. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

10.3.30.3.3 When generated

This primitive is generated by the MLME when a valid Extended Channel Switch Announcement frame is received.

10.3.30.3.4 Effect of receipt

On receipt of this primitive, the SME decides whether to accept the switch request.

10.3.30.4 MLME-EXTCHANNELSWITCH.response

10.3.30.4.1 Function

This primitive is used to schedule an accepted extended channel switch.

10.3.30.4.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-EXTCHANNELSWITCH.response	(

Mode,

Regulatory Class,

Channel Number,

Channel Switch Count,

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Mode | Integer | 0,1 | Channel switch mode, as defined for the Extended Channel Switch Announcement element. |
| Regulatory Class | Integer | As defined in Annex J | Specifies the new regulatory class. |
| Channel Number | Integer | As defined in Annex J | Specifies the new channel number. |
| Channel Switch Count | Integer | 0 – 255 | Specifies the number of TBTTs until the channel switch event, as described for the Extended Channel Switch Announcement element. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

10.3.30.4.3 When generated

This primitive is generated by the SME to schedule an accepted extended channel switch request.

10.3.30.4.4 Effect of receipt

On receipt of this primitive, the MLME schedules the extended channel switch.  The actual channel switch is made at the appropriate time through the MLME-PLME interface using the PLME-SET primitive of the dot11CurrentFrequency MIB attribute. If Mode = 0, STA shall refrain from transmitting until the time of the extended channel switch.

11.9 DFS procedures

…

Change clause 11.9.7.1 as follows:

11.9.7.1 Selecting and advertising a new channel in an infrastructure BSS

The decision to switch to a new operating channel in an infrastructure BSS shall be made only by the AP. An

AP may make use of the information in Supported Channel and Supported Regulatory Classes elements and the results of measurements undertaken

by the AP and other STAs in the BSS to assist the selection of the new channel. The algorithm to

choose a new channel is beyond the scope of this standard, but shall satisfy applicable regulatory requirements,

including uniform spreading rules and channel testing rules. The AP shall attempt to select a new

channel that is supported by all associated STAs, although it should be noted that this might not always be

possible.

In the following text wherever Channel Switch Announcement is referred to, the following rules apply:

- If an AP is switching to a new channel in a different regulatory class then the AP shall use the Extended Channel Switch Announcement elements and frames instead of the Channel Switch Announcement elements and frames.

- If an AP is switching to a channel within the same regulatory class, and dot11ExtendedChannelSwitchImplemented is true, then the AP shall send the Extended Channel Switch Announcement element and frame, else the AP shall send the Channel Switch Announcement element and frame. Optionally the Extended Channel Switch Announcement element may be sent with the Channel Switch Announcement frame.

An AP shall inform associated STAs that the AP is moving to a new channel and maintain the association by

advertising the switch using Channel Switch Announcement elements in Beacon frames, Probe Response

frames, and Channel Switch Announcement frames until the intended channel switch time. The AP may

force STAs in the BSS to stop transmissions until the channel switch takes place using the Channel Switch

Mode field in the Channel Switch Announcement element. If possible, the channel switch should be scheduled

so that all STAs in the BSS, including STAs in power save mode, have the opportunity to receive at

least one Channel Switch Announcement element before the switch. The AP may send the Channel Switch

Announcement frame in a BSS without performing a backoff, after determining the WM is idle for one PIFS

period.

A STA that receives a Channel Switch Announcement element may choose not to perform the specified switch, but to take alternative action. For example, it may choose to move to a different BSS.

A STA in a BSS that is not the AP shall not transmit the Channel Switch Announcement element.

Insert new first paragraph in clause 11.9.7.2 as follows:

11.9.7.2 Selecting and advertising a new channel in an IBSS

In the following text wherever Channel Switch Announcement is referred to, the following rules apply:

- If a DFS owner is switching to a new channel in a different regulatory class then the DFS owner shall use the Extended Channel Switch Announcement elements and frames instead of the Channel Switch Announcement elements and frames.

- If DFS owner is switching to a channel within the same regulatory class, and dot11ExtendedChannelSwitchImplemented is true, then the DFS owner shall send the Extended Channel Switch Announcement element and frame, else the DFS owner shall send the Channel Switch Announcement element and frame. Optionally the Extended Channel Switch Announcement element may be sent with the Channel Switch Announcement frame.

TGn Editor: Make changes to the TGn Draft as shown below:

Insert the following new subclause:

11.9.8 Channel selection methods for 20/40 MHz operation

TGn Editor: Insert the following sentence at the beginning of the clause:

For an HT STA, the following MIB attributes are true: dot11RegulatoryClassesImplemented, dot11RegulatoryClassesRequired, and dot11ExtendedChannelSwitchImplemented.

Insert the following new subclause:

11.9.8.3 Channel management at the AP

An HT AP shall receive the beacons of other HT BSSs on its channel when it first powers up.

It may receive these beacons later as part of background scanning.

HT BSSs shall only overlap HT BSSs with the same primary channel and secondary channel location. 1

An AP operating a 20/40 MHz BSS may decide to move the BSS and/or adjust its channel width if, for example, another BSS starts to operate in either or both of the primary or secondary channels, or radar is detected in either or both of the primary or secondary channels. Specifically, the AP may move its BSS to a different pair of channels, or change from a 20/40 MHz BSS to a 20 MHz BSS using either the primary channel of the previous channel pair or any other available 20 MHz channel, The AP may also change from a 20 MHz BSS to a 20/40 MHz BSS.

An AP may switch the BSS capability between 20/40 MHz and 20 MHz by changing the value of the Supported

Channel Width Set field of the HT capabilities element in the Beacon. The STAs shall may disassociate/associate

or re-associate to accommodate the new capability. In any transition of channel width, the AP should take

into consideration any STAs that are in power-save mode.

An AP that wishes to maintain existing associations while making a channel width change or while performing a channel pair relocation may inform associated HT STAs that it is making the change by including an Extended Channel Switch Announcement element in Beacon, Probe Response, and Channel Switch Announcement frame transmissions until the intended channel switch time.  The New Channel Number of the Extended Channel Switch Announcement element represents the new channel in the case that the BSS after relocation/width change will be a 20MHz BSS, or the primary channel of the new pair of channels in the case that the BSS after relocation/width change will be a 20/40 MHz BSS.  When changing to a new pair of channels, the New Regulatory Class field specifies the position of the secondary channel relative to the new primary channel, i.e., either above or below.   An AP operating in a 20/40 MHz BSS may also send a Channel Switch Announcement element to maintain existing

associations while making a primary channel change, and the new primary channel is in the same regulatory class as the existing primary channel.

When changing from one channel to a new channel, or from one pair of channels to a new pair of channels, the Supported Channel Width Set field of the HT capabilities element in the Beacon remains unchanged.

When the Extended Channel Switch Announcement is used to switch from 20/40 MHz BSS mode of operation to 20 MHz BSS mode of operation on the primary channel, the Supported Channel Width Set field of the HT capabilities element in the Beacon remains unchanged.

When an AP uses the Extended Channel Switch Announcement element to announce a channel switch or channel width adjustment, the values for New Channel Number and New Regulatory Class shall comply with the values given in Table nx. The values for Supported Channel Width Set are also given.

Table nx - Channel switching and channel width adjustment combinations

| Original channel | Destination channel |  |  |  |
| --- | --- | --- | --- | --- |
|  | Same 20 MHz channel | Different 20 MHz channel | Pair of 20 MHz channels with same primary channel as original | Pair of 20 MHz channels with different primary channel than original |
| 20 MHz channel | N/A | New Channel Number: any number different from the current number, satisfying channel allocation rules specified in 21.3.14.3 and Annexes I and J. Supported Channel Width Set: 0 (unchanged). | New Channel Number: same as the current number, satisfying channel allocation rules specified in definitions in 21.3.14.3 and Annexes I and J. Supported Channel Width Set: 1. | New Channel Number: any number different from the current number, satisfying channel allocation rules specified in 21.3.14.3 and Annexes I and J. Supported Channel Width Set: 1. |
| Pair of 20 MHz channels | New Channel Number: same as the current primary channel, satisfying channel allocation rules specified in 21.3.14.3 and Annexes I and J. Supported Channel Width Set: 1 (unchanged). | New Channel Number: any number different from the current primary channel, satisfying channel allocation rules specified in 21.3.14.3 and Annexes I and J. Supported Channel Width Set: 0. | N/A | New Channel Number: any number different from the current number, satisfying channel allocation rules specified in 21.3.14.3 and Annexes I and J. Supported Channel Width Set: 1 (unchanged). |

Movements of 20/40 MHz BSSs from one channel pair to a different channel pair and changes between 20 MHz BSS and 20/40 MHz BSS modes of operation should be scheduled so that all STAs in the BSS, including STAs in power save mode, have the opportunity to receive at least one Extended Channel Switch Announcement element before the switch. The AP may send the Extended Channel Switch Announcement element in a Beacon without performing a backoff, after determining the WM is idle for one PIFS period.

A STA shall support the Extended Channel Switch Announcement if dot11ExtendedChannelSwitchImplemented is true.

When the Extended Channel Switch Announcement element is transmitted in bands where dot11SpectrumManagementRequired is true, the Channel Switch Announcement element may also be transmitted.  If both the Extended Channel Switch Announcement element and the Channel Switch Announcement element are transmitted, the New Channel Number of both elements shall be identical.  HT STAs associated to an HT AP shall ignore the Channel Switch Announcement element.

For 20 MHz operation when the New Regulatory Class signifies 40 MHz channel spacing, the 20 MHz channel is the primary channel of the 40 MHz channel.

### A. 4.3 IUT configuration

TGn Editor: Make additional changes and additions to the PICS relative to P802.11n D1.08 as specified below:

Change PICS  items CF10 and CF11and insert CF15 at the end of the table:

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| *CF10 | Is spectrum management operation supported? | 7.3.1.4, 11.6 | (CF6 OR CF15):O | Yes □ No □ |
| *CF11 | Is regulatory classes capability implemented? | 7.3.2.12, 17.3.8.3.2, 17.3.8.6, 17.4.2, Annex I, Annex J | (CF6 OR CF15)& CF8&CF10:O | Yes □ No □ N/A □ |
| *CF15 | Enhancements for Higher Throughput | 7.3.2.49 (HT Capabilities element) (Ed: CID 685) | O.2 | Yes □ No □ (Ed: CID 686) |

TGn Editor: Modify Table I.3 in Annex I as follows (highlights are only used to emphasize the change to D1.08, and should not be part of the draft)

Table I.3—Behavior limits sets

| Behavior limits set | USA | Europe | Japan |
| --- | --- | --- | --- |
| 11 40MHz Primary channel with Secondary = 1; 20MHz operation on primary channelb | Reserved | Reserved | Reserved |
| 12 40MHz Primary channel with Secondary = -1; 20MHz operation on primary channelb | Reserved | Reserved | Reserved |
| 13 11-255 | Reserved | Reserved | Reserved |

bFor 20 MHz operation where the regulatory class signifies 40 MHz channel spacing, the 20 MHz channel corresponds to the channel number indicated.

TGn Editor: Add the following footnote to Table J.1 in Annex J.

The Channel Spacing for Regulatory Class 13-24 is for the supported bandwidth rather than the operating bandwidth. In these regulatory classes, the AP operates in 20/40 MHz mode, and the operating bandwidth for a non-AP STA is either 20 MHz or 40 MHz.