IEEE P802.11
Wireless LANs

| Legacy Protection Mechanism – Proposed text changes |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Brian Hart | Cisco Systems, Inc. | 170 West Tasman Drive, San Jose, CA 95134 | 408-525-3346 | brianh@cisco.com |
| Douglas Chan | Cisco Systems, Inc. | 170 West Tasman Drive, San Jose, CA 95134 | 408-527-9344 | dougchan@cisco.com |
| Luke Qian | Cisco Systems, Inc. | 170 West Tasman Drive, San Jose, CA 95134 | 330-523-2051 | lchia@cisco.com |
| Andrew Myles | Cisco Systems, Inc. | 170 West Tasman Drive, San Jose, CA 95134 | 61-2-8446-1010 | amyles@cisco.com |

Abstract

The current legacy protection mechanism given in the text does not adequately address the issue of collisions between Greenfield packets with 11abg packets. To ensure proper interoperability between devices, we propose changes to the current text to allow HT devices to report the presence of an OBSS HT/non-HT situation, and thereby provide overlapping HT devices with information that optional counter-measures may be required. The proposed method duplicates the 11g/11b mechanism to avoid the OBSS HT/non-HT report rippling outwards in domino fashion

This document contains a text proposed change to the P802.11n-D1.06 Draft to address CIDs 10292, 12030, 12048, 12049.  It augments 06/1572r1.

Abstract

The current legacy protection mechanism given in the text does not adequately address the issue of collisions between Greenfield packets with 11abg packets. To ensure proper interoperability between devices, we propose changes to the current text to allow HT devices to report the presence of an OBSS HT/non-HT situation, and thereby provide nearby HT devices with information that optional counter-measures may be required. The proposed method duplicates the 11g/11b mechanism to avoid the OBSS HT/non-HT report rippling outwards in domino fashion

This document contains a text proposed change to the P802.11n-D1.08 Draft to address CIDs 12030, 4522, 10292, 12048, 12049.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

| 12030 | Douglas, Brett | T | Y | 159 | 24 | 20.1 | Green Field Mode introduced into the 2.4 and 5 GHz bands will introduce the same type of interoperability problems as 11b/11g in the 2.4 GHz band. We should either: 1.Not allow it to be used in the 2.4 and 5 GHz bands, or 2. Introduce very strong mechanisms to make sure it will never be used in the presence of 11a/b/g devices. |
| --- | --- | --- | --- | --- | --- | --- | --- |

Introduction:

## Motion to Adopt

A motion to adopt the changes defined in this submission means that the editing instructions and any changed or added material are actioned in the current TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or set in context the response to the omission.

- Text Proposal:

TGn Editor:  Change Figure n36 in section 7.3.2.50  as indicated by the following changes:

| B0 B1 | B2 | B3 | B4 | B45 B15 |
| --- | --- | --- | --- | --- |
| Operating Mode | Non-Greenfield STAs Present | Transmit Burst Limit | OBSS Non-HT STAs Present | Reserved |
| 2 |  |  |  |  |

TGn Editor:  Insert the following row below the “Transmit Burst Limit” row of Table n28 in section 7.3.2.50 as indicated by the following additions:

| OBSS Non-HT STAs Present | Indicates if the use of protection for non-HT STAs by overlapping BSSs is determined to be desirable. Present in Beacon and Probe response frames transmitted by an AP.Otherwise reserved. See 9.13.3.4 (Use of OBSS Non-HT STAs Present) | Set to 1 if the use of protection for non-HT STAs by overlapping BSSs is determined to be desirable. Examples of when this bit may be set to 1 include, but are not limited to, when - one or more non-HT STAs are associated - a non-HT BSS is overlapping (a non-HT BSS may be detected by the reception of a Beacon where the supported rates only contain clause 15, 17, 18 or 19 rates) - a management frame (excluding a Probe Request) is received where the supported rate set includes only Clause 15, 17, 18 and 19 rates. Set to 0 otherwise. |
| --- | --- | --- |

TGn Editor:  Insert the following new subclause:

9.13.3.4 Use of OBSS Non-HT STAs Present

The OBSS Non-HT STAs Present field allows HT devices to report the presence of other non-HT STAs that cannot interpret the HT transmissions correctly. See 9.13.3 (Protection mechanisms for different HT PHY options).

A second HT AP that detects a first HT AP’s beacon with the OBSS Non-HT STAs Present field set to 1 may, in conjunction with radio resource measurements and/or heuristics, cause greenfield and RIFS sequence transmissions of the second AP’s BSS to be protected by setting the Operating Mode field to 3. If the NonERP_Present bit is set to 1 in the first AP’s beacon, then the Use_Protection bit to 1 may also be set to 1 by the second AP.  See Annex n061818

HT STAs may also scan for the presence of non-HT devices either autonomously or after the STA’s AP transmits a HT Information element with the Operating Mode field set to 1. Non-HT devices may be detected as follows :

- one or more non-HT STAs are associated

- a non-HT BSS is overlapping (a non-HT BSS may be detected by the reception of a Beacon where the supported rates only contain clause 15, 17, 18 or 19 rates)

- management frame (excluding a Probe Request) is received where the supported rate set includes only Clause 15, 17, 18 and 19 rates.

- or when a HT Information element with OBSS Non-HT STAs Present field equal to 1 is received.

When non-HT devices are detected, the STA may enable protection of its Greenfield and RIFS sequence transmissions.

TGn Editor:  Insert the following new Annex:

# Annex n061818

(Informative)

Summary of typical use of the OBSS Non-HT STAs Present field, NonERP_Present bit and Use_Protection bit.

Separate fields and bits are used for status reporting (OBSS Non-HT STAs Present and NonERP_Present) and for protection (Operating Mode and Use_Protection) so that protection extends to the BSS directly detecting the non-HT STAs and to the nearby BSSs, but not beyond. Table n061818 describes typical uses of these fields and bits. A first AP has non-HT STAs associated to its BSS or directly observes a non-HT BSS, and reports this status to its neighbors (Layer 1c). A second AP’s BSS overlaps the first AP’s BSS, and may respond to the status report by requiring protection (Layer 2b or  2c).  The second AP does not relay the status report further. A third AP’s BSS overlaps the second AP’s BSS but not the first, and so the third AP sees no report of non-HT STAs and so requires no protection (Layer 3).

Table n061818: Summary of typical use of the OBSS Non-HT STAs Present field, NonERP_Present bit and Use_Protection bit

| Layer | Indication from overlapping BSS |  | Indication to Associated STAs |  | Indication to overlapping BSSs |  | Typical Use |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | OBSS Non-HT STAs Present from overlapping BSS | ERP IE is present and NonERP_Present equals 1 from overlapping BSS | Operating Mode | ERP IE is present and Use_Protection equals 1 | OBSS Non-HT STAs Present | ERP IE is present and NonERP_present equals 1 |  |
| 1a | 0 | 0 | 1 | 0 | 0 | 0 | Non-HT STAs may be present in both the primary and the secondary channel, but protection by overlapping BSSs is determined to not be necessary. |
| 1b | 0 | 0 | 3 | 0, 1 | 0 | 0 | Non-HT STAs are associated to and protected by the BSS, but protection by overlapping BSSs is determined to not be necessary. |
| 1c | 0 | 0 | 3 | 0 | 1 | 0 | Non-HT STAs are associated to and protected by the BSS, and protection by overlapping BSSs is determined to be necessary. |
|  |  |  |  | 1 |  | 1 |  |
| 1d | 1 | 0 | 3 | 0 | 1 | 0 | Both the BSS and an overlapping BSS have determined that protection by the BSS is necessary, and protection by overlapping BSSs is determined to be necessary. |
|  |  | 0 |  | 1 |  | 1 |  |
|  |  | 1 |  | 0 |  | 1 |  |
|  |  | 1 |  | 1 |  | 1 |  |
| 2a | 1 | 0, 1 | 1 | 0 | 0 | 0 | An overlapping BSS has determined that protection by the BSS is necessary, but the BSS is advising that there may be non-HT STAs, rather than requiring protection. |
| 2b | 1 | 0 | 3 | 0 | 0 | 0 | An overlapping BSS has determined that protection by the BSS is necessary, the BSS has no associated non-ERP STAs, and the BSS is requiring the determined protection. |
|  |  | 1 |  | 1 |  |  |  |
| 2c | 1 | 0, 1 | 3 | 1 | 0 | 0 | An overlapping BSS has determined that protection by the BSS is necessary, the BSS has associated non-ERP STAs, and the BSS is requiring protection. |
| 3 | 0 | 0 | 0, 2 | 0 | 0 | 0 | No non-HT STAs are present. (Layer 3) |
| Various | Other combinations of values |  |  |  |  |  | None or unusual. |