IEEE P802.11
Wireless LANs

| MIMO Testing in a Conducted Environment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Charles R. Wright | Azimuth Systems, Inc. | 31 Nagog Park Acton MA 01720 | 978-263-6610 | charles_wright @azimuthsystems.com |

Abstract

This document contains proposed changes to the P802.11.2 draft (D0.10) that would define test methodologies for 802.11n MIMO devices.  The proposal addresses the following CIDs: 49, 270

This document is currently a work-in-progress provided for information and comment.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# 5.3.2.2 Test Setup

Insert the following figure and caption after Figure 3:

Figure 4---Connection of components in a MIMO system with a MIMO channel.  The box around the MIMO channel and attenuators is a logical box and does not dictate implementation.

Insert the following new subclause at the appropriate place in the draft:

## 5.3.2.2.1 MIMO Channel Setup

Figure 4 shows the minimal setup for a conducted setup for testing MIMO devices.  A MIMO channel is inserted in the path between the WLCP and the DUT, and a bank of programmable attenuators are used, one for each possible antenna connection.  The diagram shows the maximal case of 4x4 MIMO systems.  If the DUT has fewer antenna connections, then fewer attenuators are required.  Likewise, if the WLCP has fewer than four antennas, then fewer connections to the MIMO channel are required.

Depending on the needs of a specific metric, the transmit power of the dominant transmitter and the path loss of the MIMO channel and attenuators may need to be characterized.

The power meter is used to measure the power transmitted by the dominant data transmitting device, as determined by to which device the traffic generator is connected.  This is done because it is usually easier and more accurate to measure the power of a high-level source, rather than a low-level one.  However, if a means of measuring power due to the WLCP directly on the DUT side of the MIMO channel and attenuators is sufficiently accurate, this is also acceptable.   If the traffic generator is connected to the DUT, then transmit power is measured at the DUT.  If the traffic is bidirectional, the power is measured at both devices.  For MIMO devices, the power should be measured at each antenna connection.

The MIMO channel can be any of a number of possible channel matrices.  The following describe a few baseline channels

### Butler Matrix Channel

A Butler matrix is one such possibility and is recommended as the baseline method for connecting two MIMO devices.  Figure XX below shows the schematic for a 4x4 Butler matrix on the left, while its matrix representation is shown on the right.  In the matrix representation, each element in the matrix is a unit amplitude complex number whose phase is specified after the angle symbol.

H=1∠90∠135∠−135∠90−1∠45∠135∠135∠45−1∠90∠−135∠135∠901

Figure XX.  Passive RF circuit for a Butler matrix on the left; matrix representation of same on the right.

### Identity Channel

The simplest of all possible channel matrices is the identity channel, which is depicted in Figure YY below.  This channel is simply the direct connection of antennas on the WLCP to antennas on the DUT through the programmable attenuators.  This channel matrix is not applicable for general MIMO systems due to (a) the possibility of a differing number of antenna connections on the WLCP and DUT, (b) the fact that this channel does not exercise receiver diversity, and (c) performance that does not correlate with a real, over-the-air system.  For these reasons, the identity matrix is not recommended for general use.  However, it can be useful for connecting two devices and exercising protocol-level tests.

H=1000010000100001,

Figure YY.  The identity channel

### MIMO Multipath Channels

The performance of 802.11 MIMO devices in multipath is important due to the strong dependence of this PHY layer on the characteristics of the MIMO multipath channel.  For measuring the performance of 802.11 MIMO devices, this document recommends using a multipath emulator capable implementing the models defined in the TGn channel models document.  The emulator should be capable of providing multipath emulation for signals passing through it in both directions (i.e., “bi-directional”).  The emulation provided should create the conditions of a reciprocal channel.

The parameters of the multipath models are listed in Table QQ and should be reported with the measurement of any metric using these models.

| Model | A, B, C, D, E, F |
| --- | --- |
| Transmit antenna spacing (d/) | ½, 1, 4 |
| Receive antenna spacing (d/) | ½, 1, 4 |
| Line-of-sight component | Yes/no |
| Doppler rate (Hz) | 2.7 / 5.8 |

Table QQ.  Parameters of the multipath models.

# 5.3.4 Modifiers and variations

Change item (c) in the list to state:

c)	A multipath channel emulator can be inserted in the RF path to evaluate the effects of multipath on the system.

References:

[1] IEEE 802.11-03/940r4, “TGn Channel Models”, V. Erceg, et al

[2] IEEE 802.11-06/1839r1, “MIMO testing in a conducted environment”, C. Wright

[3] IEEE 802.11-06/1501r0, “Multipath testing in a conducted environment”, C. Wright