IEEE P802.11
Wireless LANs

| TGn LB84 Submission CCA sensing in 40MHz |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2026-07-30 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Assaf Kasher | Intel Corporation | Matam Industrial Park Haifa 31015, Israel | +972-4-8651547 | Assaf.kasher@intel.com |
| Matthew Fischer | Broadcom | 190 Mathilda Place, Sunnyvale, CA 94086 | +1 408 543 3370 | mfischer@broadcom.com |
| Vinko Erceg | Broadcom |  |  | verceg@broadcom.com |
| Ali Raissinia | Airgo |  | 650-475-1997 | ali@airgonetworks.com |
| Gal Basson | Intel | POB 1659, Haifa 31015, Israel |  | Gal.basson@intel.com |
| Srini Kandala | Airgo |  |  | srini@airgonetworks.com |
| Richard Van Nee | Airgo |  |  | vannee@airgonetworks.com |
| Allert VanZelst | Airgo |  |  | allert@airgonetworks.com |
| VK Jones | Airgo |  |  | vkjones@airgonetworks.com |
| William McFarland | Atheros |  |  | billm@atheros.com |
| Jason Trachewsky | Broadcom |  |  | jat@broadcom.com |
| Solomon Trainin | Intel Corporation | POB 1659, Haifa 31015, Israel | 97248655738 | solomon.trainin@intel.com |
| Adrian Stephens | Intel Corporation | 15 JJ Thompson Avenue, Cambridge, CB3 0FD, United Kingdom | 44 1223 763457 | adrian.p.stephens@intel.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB84 comments:

- Too numerous to list here – please see main body of document

The changes marked in this document are based on TGn Draft version 1.06.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# CID :

CCA on secondary:

CCA on secondary:

14, 49, 69, 106, 174, 175, 291, 295, 449, 704, 712, 1069, 1450, 1495, 1503, 1517, 1524, 1559, 1625, 1635, 1657, 1657, 1661, 1718, 1750, 2730, 2733, 2737, 2738, 3117, 3118, 3264, 3452, 3453, 3513, 3514, 3515, 3882, 4007, 4048, 4084, 4288, 4532, 4574, 4640, 4791, 4792, 4804, 6769, 6813, 6938, 7012, 7178, 7282, 7319, 7320, 7372, 7424, 7448, 7473, 7767, 7839, 7840, 7873, 7878, 7879, 7893, 7894, 7913, 7922, 7923, 7926, 8021, 8045, 8127, 8200, 8263, 8281, 8284, 10016, 10017, 10018, 10020, 10293, 10379, 11730, 11975, 12111, 12200, 12201, 12202, 12245

TGn editor: Insert the following text at the appropriate alphabetically-sorted location within subclause 3 of TGn draft D1.06:

3.n34 20MHz PPDU: either a clause 17 PPDU, or clause 19 OFDM PPDU, or a clause 21 20MHz HT-PPDU.

3.n35 40MHz PPDU: 40MHz HT PPDU as defined in clause 21.

TGn Editor: Replace the entire text of subclause “9.20.2 STA CCA sensing 20/40/ MHz BSS” with following text of TGn draft D1.06 on about page 144:

Submission Note: CCA Sensing sections refer to both the 2.4GHz and the 5GHz band

9.20.2 STA CCA sensing 20/40 MHz BSS

A station may transmit a 20 MHz PPDU in the primary channel following the rules in 9.9.1.

A STA transmitting a 40 MHz PPDU that begins a TXOP as described in 9.9.1.3, shall sense CCA on both the 20 MHz primary channel and the 20 MHz secondary channel before the 40 MHz transmission shall commence. A STA transmitting a 20 MHz PPDU in the primary channel shall sense CCA of the 20 MHz primary channel before transmitting and is not required to sense CCA of the secondary channel before transmitting. Secondary channel CCA shall be deemed BUSY during transmission or reception of a 20 MHz frame in the primary channel. Secondary channel CCA shall be deemed BUSY during transmission of a 20 MHz frame in the secondary channel.

At the specific slot boundaries, determined by the STA based on the 20 MHz primary channel CCA, when the transmission begins a TXOP (as described in 9.9.1.3), the STA shall be permitted to transmit using a 40 MHz rate only if the secondary channel has been idle for a duration of at least a the minimum of AIFS and DIFS immediately preceding the impending 40 MHz transmission and may either transmit no PPDU or transmit a 20 MHz PPDU if the secondary channel has not been idle for a duration of at least a the minimum of AIFS and DIFS immediately preceding the impending transmission. The next earliest 40 MHz transmission for the STA, if it chooses not to transmit either a 40 MHz PPDU or a 20 MHz PPDU shall be when an additional backoff (determined using the same contention window value as the previous backoff) on the primary channel has expired and the secondary channel has been idle for at least a the minimum of AIFS and DIFS immediately preceding the impending 40 MHz transmission.

When the TXOP is obtained for a 40 MHz transmission, the station may transmit 40 MHz frames and/or 20 MHz frames during the TXOP. When the TXOP is obtained by the exchange of 20 MHz transmissions only in the primary channel, the station shall not transmit 40 MHz frames during the TXOP.

TGn Editor:  Insert the following text as the new first paragraph of “21.3.21.5 Clear channel assessment (CCA) sensitivity” on page 266 of TGn draft D1.06 as shown:

The receiver of a 20/40 MHz STA shall provide CCA on both the primary and secondary channels.

TGn Editor:  Change the text of “21.3.21.5 Clear channel assessment (CCA) sensitivity” on page 266 of TGn draft D1.06 as would appear after accounting for the changes from the adopted document 11-06-1571r5 and the accepted changes to the sensitivity levels as agreed to by passing motion during the PM2 TGn session of November 15, 2006, beginning with the words “The start of a valid HT transmission” as is shown:

When the secondary channel is idle, the start of a valid 20 MHz HT transmission in the primary channel that is not also a valid Clause 17 or Clause 19 transmission at a receive level equal to or greather than the minimum modulation and coding rate sensitivity of -82 dBm shall cause the PHY to set PHY-CCA.indicate(BUSY) with a probability > 90% within 4 usec. The start of a valid 40 MHz HT transmission in both the primary channel and the secondary channel at a receive level equal to or greather than the minimum modulation and coding rate sensitivity  of -79 dBm shall cause the PHY to set PHY-CCA.indicate(BUSY) for both the primary channel and the secondary channel with a probability per channel > 90% within 4 usec. In a 40 MHz channel CCA shall be indicated for a signal at that level at the primary channel.

TGn Editor:  Change, in subclause “21.3.21.5 Clear channel assessment (CCA) sensitivity” on page 266 of TGn draft D1.06, in the middle of the first paragraph, the text that begins with the words “any signal 20 dB above the minimum modulation” as is shown:

any signal 20 dB above the minimum modulation and coding rate sensitivity (–62 dBm for either 20 MHz channel provided that the other 20 MHz channel is IDLE, -59 dBm for a 40 MHz channel)

TGn Editor:  Add, in subclause “21.3.21.5 Clear channel assessment (CCA) sensitivity” on page 266 of TGn draft D1.06, after above changed sentence the following:

The receiver shall hold the 20 MHz primary channel CCA signal busy for any signal 20 dB above the minimum modulation and coding rate sensitivity (–62 dBm) in the 20 MHz primary channel. When the primary channel is idle, the receiver shall hold the 20 MHz secondary channel CCA signal busy for any signal 20 dB above the minimum modulation and coding rate sensitivity (–62 dBm) in the 20 MHz secondary channel. For a 40 MHz transmission, both primary channel CCA and secondary channel CCA shall indicate busy for any signal level above -59 dBm.  The receiver shall hold both the 20MHz primary channel CCA and the 20MHz secondary channel CCA busy for any 40MHz signal 20dB above the minimum modulartion and coding rate sensitivity (-59dBm)

TGn Editor:  Insert the following text as the new last paragraph of “21.3.21.5 Clear channel assessment (CCA) sensitivity” on page 266 of TGn draft D1.06 as shown:

The primary channel is the channel indicated in the MIB attribute dot11CurrentPrimaryChannel. The secondary channel is the channel indicated in the MIB attribute dot11CurrentSecondaryChannel.