IEEE P802.11
Wireless LANs

| Lb84 miscelanous PHY comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-12-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Assaf Kasher | Intel Corporation | MATAM industrial Park, Haifa, ISRAEL | +972-4-8651547 | assaf.kasher@intel.com |

Abstract

This document suggest resolution to lb84 comments with CIDs:  11975, 7121, 691, 3456, 10057, 11935, 12172, 3407.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

This document attempts to resolve several PHY comments:

| 7121 | In Table n101 and n102, there are MCS's that require 1 or 2 encoders depending on whether a 800ns or 400ns GI is used. This will cause interoperability problems. For example, consider a device that only has one decoder. It may still be capable of receiving MCS 50 with 800ns GI, and capable of receiving 400ns GI for other MCS's. There is currently no way to indicate this condition. | Add HT capability field for number of encoded streams a receiver is capability of decoding. |
| --- | --- | --- |
| 691 | The number of FEC encoders is determined as a throughput threshold which raises an issue since depending on the use of short GI or not, the same MCS uses a different number of encoders. This issue applies to 5 40MHz modes (MCS 50,51,64,67,69) | Mandate that same MCS for 400ns and 800ns uses the same number of FEC encoders, i.e. use not a rate based threshold. |
| 3456 | Number of encoders is a function of GI length. | Include GI length as part of the calculation. |

Suggestion: Counter

- Change the text in 20.3.10.3 (coding) – replace “When the BCC FEC encoder is used, two encoders will be used when the PHY rate is greater than 300Mbps.” with “When the BCC FEC encoder is used, two encoders will be used when the PHY rate of the used MCS is greater than 300Mbps with normal GI in use”.

- Change table n91 so that NES is 1 for MCS 50 and 51 in 40MHz.

- Change table n92 so that NES is 1 for MCS 64 and 67 and 69 in 40MHz.

- Remove the notes under these tables.

| 10057 | There is a description that "Qk shall not be an identity matrix when extension HT-LTFs are present." I understand that someone may worry about transient response, but it would depending on implementors effort. | Replace "shall" with "should" or "may". And if we think transient response using direct mapping may cuase proble, add specification on this. |
| --- | --- | --- |
| 11935 | Feels that the text "'Q_k shall not be an identity matrix when extension HT-LTFs are present (N_ESS >0)." is not required. IMHO, if Q_k is an identity matrix, it would mean that the extension streams get switched off when transmitting data portion and data streams gets switched off when EHTLTFs are transmitted. | Kindly remove the text from the subclause. |
| 12172 | No explanation is provided why an identity matrix should not be used for Q with extension HT-LTFs | Provide an explanation |

Suggestion: Reject

If the Identity matrix is used, there is powering OFF and ON of transmit chains that does not happen any where else in packet transmission.  Allowing this will require defiinng the transient response requirement for these and adding tests to test them.  This will add unwated complication to the standard.

| 3407 | Y | According to this section RCPI parameter needs to be send to MAC entity. There is no RCPI parameter defined for RXVECTOR. | Define a RCPI parameter for RXVECTOR. |
| --- | --- | --- | --- |

Suggestion: Accept.

This was taken care of in Draft 1.07