IEEE P802.11
Wireless LANs

| LB84-submission-simplifying-formulas |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-12-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Assaf Kasher | Intel Corporation | Matam Industrial Park Haifa 31015, Israel | +972-4-8651547 | Assaf.kasher@intel.com |

Abstract

This document attempts to resolve CID 12164 by simplifying formulas in the document (D1.07), continuing the effort of 06-0935r3

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

This document attemts to resolve CID 12164 by simplifying some formulas in the document:

| The waveform equations such as (20-10), inside table n66, etc etc, are unnecessarily ugly. They contain much duplication and therefore are much more difficult to verify than they should be. | (1) Define a function uppercaseUpsilon_k(BW) as a generalization of the existing uppercaseUpsilon. For BW=20MHz, this is always 1, For 40MHz, this is 1 then j. In this way, the two summations over disjoint k may be simplified to a single summation over all of k. (2) Replace the 47 by a named constant (3) Use a named constant for the 90deg rotation in the HT-SIG, to avoid confusion with other j factors floating around. (4) The summation are sometimes from some negative N to some positive N, and sometimes 0 to some positive M. Pick a convention and stick with it. For instance (20-53) is particularly ugly (5) The separate summations over data tones and pilot tones are ugly and would be better if merged into a single summation. Define a new function, basically the X of p210 line 3, which is the multiplexing of data and pilot subcarriers - basically the spatial mapping input. (6) Order the sqrt(1/N) term and w(t) term consistently |
| --- | --- |

Suggested Resolution: Counter as per 06-1924r0

Discussion:

- was defined in 06-935r3, it is not usefull in equations in which data is duplicated over the two adjacent channels.

- This submission creates a new variable Dk,n which exists for k=-NSR:NSR and takes the values from dk,n which exists for k=0:NSD-1. This enables using a single summation for the data and the pilots and therefore simplifies equations.

- It is now possible simplify the duplication in frequency so as little as possible is duplicated.  The 90° rotation in table n59 is now clearer and simpler.

- The constant NSR is used when its value in table n52 is the value used in the equation.  This does not happen in all of the “legacy” part of a mixed format packet or in HT and non-HT duplicate packet.

- The sqrt(1/N) and windowing term were ordered consitently were possible

- An error was fixed in equation 21-58 (summation over symbol index was missing)

In general, I think that together with 06-935r3, the equations are much simpler and clearer.

TGn Editor: Remove table n56 on page 229 (draft D1.07)

| Modulation method | Conversion to time domain signal on Tx chain |
| --- | --- |
| 20 MHz transmission on several transmit chains |  |
| 40 MHz transmission on several transmit chains |  |
| are defined in 17.3.5.9. p0 is the first pilot value in the sequence defined in 17.3.5.9. |  |

TGn Editor Replace the following part of lines 29:31 in page 228 of section 21.3.9.3

denoted by .  The conversion of these complex numbers into a time domain signal is decribed in the Tabel n56 (Generation of the Non-hT SIGNAL Field)

With

denoted by .  The time domain waveform of the Non-HT SIGNAL field in 20MHz transmission is:

In a 40MHz transmission the time domain waveform is:

where:

are defined in 17.3.5.9.

p0 is the first pilot value in the sequence defined in 17.3.5.9.

TGn Editor: remove table n59 on page 232 (draft D1.07)

| Modulation method | Conversion to time domain signal on Tx chain |
| --- | --- |
| 20 MHz transmission |  |
| 40 MHz transmission |  |

TGn Editor: Replace the following part of lines 49-52 of page 231 of section 21.3.9.4.5

two groups of 48 complex numbers:  .  The conversion of these into the time domain signal is according to Table n59 (Modulcation of the HT SIGNAL field in an HT mixed format packet)

With

two groups of 48 complex numbers:  .  The time domain waveform for the HT SIGNAL field in an HT mixed format packet in a 20MHz transmission is:

For a 40MHz transmission the time domain waveform is:

where:

is defined in 21.3.9.3

TGn Editor: replace equation 21-24 on page 241 (D1.07)

With

TGn Editor: replace line 46 from page 241 of (D1.07):

M(k), Pk, and pn are defined in 17.3.5.9,

With

Pk and pn are defined in 17.3.5.9,

Dk,n is defined in 21.3.9.4.3

TGn Editor: replace equation 21-25 on page 242 (D1.07)

With

TGn Editor: replace line 24 from page 242 of (D1.07):

M(k), Pk, and pn are defined in 17.3.5.9,

With

Pk and pn are defined in 17.3.5.9,

Dk,n is defined in 21.3.9.4.3

TGn Editor: replace equation 21-53 on the top of page 260 (D1.07)

With

TGn Editor: replace the following lines (19-32) from page 260 of (D1.07):

M(k) in the 20 MHz case is

, and	(20-1)

With

, and

TGn Editor: replace equation 21-55 on page 261 (D1.07)

With

TGn Editor: replace the following lines (44-58) from page 261 of (D1.07):

M(k) in the 40 MHz case is

, and	(20-2)

With

In the 40MHz case:

, and

TGn Editor: replace equation 21-57 on page 262 (D1.07)

With

TGn Editor: replace line 36 from page 262 of (D1.07):

M(k), Pk, and pn are defined in 17.3.5.9,

With

Pk and pn are defined in 17.3.5.9,

Dk,n is defined in 21.3.9.4.3

TGn Editor: replace equation 21-58 on page 263 (D1.07)

With

TGn Editor: replace line 36 from page 263 of (D1.07):

,, and pn and are defined in 17.3.5.9,

With

and pn and are defined in 17.3.5.9,

Dk,n is defined in 21.3.9.4.3