IEEE P802.11
Wireless LANs

| Tx MCS Set Definition |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-01-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Vinko Erceg | Broadcom | San Diego | 858 521-5885 | verceg@broadcom.com |
| Jim Petranovich | Conexant |  |  | jim.petranovich@conexant.com |
| Matthew Fischer | Broadcom |  |  | mfischer@broadcom.com |

Abstract

This submission covers tramsnit MCS set definition and is based on Draft 1.05 and TGn adhoc accepted submission 06-1904r1.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@npx.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

TGn Editor: Changes in red are based on the changes to the Draft 1.05 proposed in the submission 06-1904r1 (accepted by TGn adhoc).

Change the first paragraph of 7.3.2.46.4 “Supported MCS Set field”, on page 57,   of TGn D1.05, as follows:

The Supported MCS Set field indicates which MCSs a STA supports. The structure of the field is defined in Figure n40 (Supported MCS Set field). Bits b80 to b89 represent the Highest Supported Data Rate that the STA is able to receive.  Resolution of the Highest Supported Data Rate is 1 Mbps. Bits b0 to b76 of represent the receive MCS Bitmask Field indicate support of correspond to MCS index values 0 to 76. When a bit is set to 1, the corresponding MCS is supported. When a bit in the receive MCS Bitmask Field is set to 0, the corresponding MCS is not supported. When a bit in the receive MCS Bitmask Field is set to 1 the corresponding MCS is supported by the STA on receive, if the int [Data_rate] of the corresponding MCS is less than or equal to the HighestSupportedDataRate in Mbps or the value of HighestSupportedDataRate is set to 0. (Ed: CID 8086, 8075) See 21.6 (Rate dependent parameters for HT Modulation and Coding Schemes (MCS)) for the definition of MCS index values (Ed: CID 10311) and corresponding Data rate. Bits b96 to b112 define the Supported Tx MCS Set. When the Tx Rx MCS Set Not Equal bit b96 is set to 0, the Tx Supported MCS Set is equal to the Rx Supported MCS Set and bits b97 to b112 are set to 0. In the case the bit b96 is set to 1, the supported Tx MCS set is not equal to the Rx Supported MCS Set and bits b97 to b112 are set accordingly. Bits b97 to b98 define the Tx Maximum Number of Spatial Streams Supported (00 – 1 spatial stream; 01 – 2 spatial streams; 10 – 3 spatial streams; 11 – 4 spatial streams).  Bits b99 to b104 indicate the Tx Maximum Modulation Supported (00 – BPSK; 01 – QPSK; 10 – 16-QAM; 11 – 64-QAM) for each of the supported spatial streams 2 through 4. Bits b105 to b110 indicate the Tx Maximum Coding Rate Supported (00 – rate 1/2; 01 – rate 2/3; 10 – rate 3/4; 11 – rate 5/6) for each of the supported spatial streams 2 through 4. Bit b111 indicates Tx Unequal Modulation Supported (0 – unequal modulation not supported; 1 – unequal modulation supported). Bit b112 indicates Tx MCS32 Supported (0 – MCS32 not supported; 1 – MCS32 supported).

TGn Editor: Change the Figure n40 of 7.3.2.46.4 “Supported MCS Set field”, on page 58, of TGn D1.05, as follows:

Figure n40—Supported MCS Set field

TGn Editor: Change AnnexD on page 340 at line 51 as follows:

dot11BeamFormingOptionEnabled TruthValue, }

dot11HighestSupportedDataRate Integer,

dot11TxRxMCSSetNotEqual Integer,

dot11TxMaximumNumberSpatialStreamsSupported Integer,

dot11TxMaximumModulationSupported2 Integer,

dot11TxMaximumModulationSupported3 Integer,

dot11TxMaximumModulationSupported4 Integer,

dot11TxMaximumCodingRateSupported2 Integer,

dot11TxMaximumCodingRateSupported3 Integer,

dot11TxMaximumCodingRateSupported4 Integer,

dot11TxUnequalModulationSupported Integer,

dot11TxMCS32Supported Integer}

TGn Editor: Change AnnexD on page 345 at line 38. After definitions of dot11BeamFormingOptionEnabled insert as follows:

dot11HighestSupportedDataRate OBJECT-TYPE

SYNTAX INTEGER (0..600)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute shall specify the Highest Data Rate in Mbps at which the station may receive data. The default value of this attribute shall be 0."

::= { dot11PhyHTEntry 21 }

dot11TxRxMCSSetNotEqual OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute indicates if the supported Tx and Rx MCS sets are not equal. The default value of this attribute is FALSE."

::= { dot11PhyHTEntry 22 }

dot11TxMaximumNumberSpatialStreamsSupported OBJECT-TYPE

SYNTAX INTEGER (0..3)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute indicates the Tx maximum number of spatial streams supported. The default value of this attribute is 0."

::= { dot11PhyHTEntry 23 }

dot11TxMaximumModulationSupported2 OBJECT-TYPE

SYNTAX INTEGER (0..3)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute indicates the Tx maximum modulation support for the spatial stream 2 in the case the spatial stream 2 is supported. The default value of this attribute is 0."

::= { dot11PhyHTEntry 24 }

dot11TxMaximumModulationSupported3 OBJECT-TYPE

SYNTAX INTEGER (0..3)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute indicates the Tx maximum modulation support for the spatial stream 3 in the case the spatial stream 3 is supported. The default value of this attribute is 0."

::= { dot11PhyHTEntry 25 }

dot11TxMaximumModulationSupported4 OBJECT-TYPE

SYNTAX INTEGER (0..3)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute indicates the Tx maximum modulation support for the spatial stream 4 in the case the spatial stream 4 is supported. The default value of this attribute is 0."

::= { dot11PhyHTEntry 26 }

dot11TxMaximumCodingRateSupported2 OBJECT-TYPE

SYNTAX INTEGER (0..3)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute indicates the Tx maximum coding rate support for the spatial stream 2 in the case the spatial stream 2 is supported. The default value of this attribute is 0."

::= { dot11PhyHTEntry 27 }

dot11TxMaximumCodingRateSupported3 OBJECT-TYPE

SYNTAX INTEGER (0..3)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute indicates the Tx maximum coding rate support for the spatial stream 3 in the case the spatial stream 3 is supported. The default value of this attribute is 0."

::= { dot11PhyHTEntry 28 }

dot11TxMaximumCodingRateSupported4 OBJECT-TYPE

SYNTAX INTEGER (0..3)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute indicates the Tx maximum coding rate support for the spatial stream 4 in the case the spatial stream 4 is supported. The default value of this attribute is 0."

::= { dot11PhyHTEntry 29 }

dot11TxUnequalModulationSupported OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute indicates the Tx unequal modulation support. The default value of this attribute is FALSE."

::= { dot11PhyHTEntry 30 }

dot11TxMCS32Supported OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute indicates the Tx MCS32 mode support. The default value of this attribute is FALSE."

::= { dot11PhyHTEntry 31 }

References:

IEEE P802.11n/D1.05 October 2006