IEEE P802.11
Wireless LANs

| Tx MCS Set Definition |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-01-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Vinko Erceg | Broadcom | San Diego | 858 521-5885 | verceg@broadcom.com |
| Jim Petranovich | Conexant |  |  | jim.petranovich@conexant.com |
| Matthew Fischer | Broadcom |  |  | mfischer@broadcom.com |

Abstract

This submission covers tramsnit MCS set definition and is based on the Draft 1.05 and 1.08 and TGn adhoc accepted submission 06-1904r1.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@npx.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

TGn Editor: Changes in red are based on the changes to the Draft 1.05 proposed in the submission 06-1904r1 (accepted by TGn adhoc).

Change the first paragraph of 7.3.2.46.4 “Supported MCS Set field”, on page 57, of TGn D1.05, as follows:

The Supported MCS Set field indicates which MCSs a STA supports. The structure of the field is defined in Figure n40 (Supported MCS Set field). Bits b80 to b89 represent the Highest Supported Data Rate that the STA is able to receive.  Resolution of the Highest Supported Data Rate is 1 Mbps. Bits b0 to b76 of represent the receive MCS Bitmask Field indicate support of correspond to MCS index values 0 to 76. When a bit is set to 1, the corresponding MCS is supported. When a bit in the receive MCS Bitmask Field is set to 0, the corresponding MCS is not supported. When a bit in the receive MCS Bitmask Field is set to 1 the corresponding MCS is supported by the STA on receive, if the int [Data_rate] of the corresponding MCS is less than or equal to the HighestSupportedDataRate in Mbps or the value of HighestSupportedDataRate is set to 0. (Ed: CID 8086, 8075) See 21.6 (Rate dependent parameters for HT Modulation and Coding Schemes (MCS)) for the definition of MCS index values (Ed: CID 10311) and corresponding Data rate. Bits b96 to b100 define the transmit Supported MCS Set. The Tx MCS Set Defined bit b96 equal to 0 indicates that the Tx MCS Set is not defined in which case bits b97 to b100 are set to 0. The Tx MCS Set Defined bit b96 equal to 1 indicates that the Tx MCS Set is defined in which case bits b97 to b100 are set accordingly. When the Tx Rx MCS Set Not Equal bit b97 is set to 0, the Tx Supported MCS Set is equal to the Rx Supported MCS Set and bits b98 to b100 are set to 0. In the case the bit b97 is set to 1, the supported Tx MCS set is not equal to the Rx Supported MCS Set and bits b98 to b100 are set accordingly. Bits b98 to b99 define the Tx Maximum Number of Spatial Streams Supported (00 – 1 spatial stream; 01 – 2 spatial streams; 10 – 3 spatial streams; 11 – 4 spatial streams). Bit b100 indicates Tx Unequal Modulation Supported (0 – unequal modulation not supported; 1 – unequal modulation supported).

TGn Editor: Change the Figure n40 of 7.3.2.46.4 “Supported MCS Set field”, on page 58, of TGn D1.05, as follows:

Figure n40—Supported MCS Set field

TGn Editor: Change AnnexD on page 340 at line 51 as follows:

dot11BeamFormingOptionEnabled TruthValue, }

dot11HighestSupportedDataRate Integer,

dot11TxMCSSetDefined TruthValue,

dot11TxRxMCSSetNotEqual TruthValue,

dot11TxMaximumNumberSpatialStreamsSupported Integer,

dot11TxUnequalModulationSupported TruthValue }

TGn Editor: Change AnnexD on page 345 at line 38. After definitions of dot11BeamFormingOptionEnabled insert as follows:

dot11HighestSupportedDataRate OBJECT-TYPE

SYNTAX INTEGER (0..600)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute shall specify the Highest Data Rate in Mbps at which the station may receive data. The default value of this attribute shall be 0."

::= { dot11PhyHTEntry 21 }

dot11TxMCSSetDefined OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute indicates if the Tx MCS set is defined. The default value of this attribute is FALSE."

::= { dot11PhyHTEntry 22 }

dot11TxRxMCSSetNotEqual OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute indicates if the supported Tx and Rx MCS sets are not equal. The default value of this attribute is FALSE."

::= { dot11PhyHTEntry 23 }

dot11TxMaximumNumberSpatialStreamsSupported OBJECT-TYPE

SYNTAX INTEGER (0..3)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute indicates the Tx maximum number of spatial streams supported. The default value of this attribute is 0."

::= { dot11PhyHTEntry 24 }

dot11TxUnequalModulationSupported OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute indicates the Tx unequal modulation support. The default value of this attribute is FALSE."

::= { dot11PhyHTEntry 25 }

TGn Editor: Add the following MIB attributes to the Table n72, Draft 1.05:

| Dot11TxMCSSetDefined | False/Boolean | Static |
| --- | --- | --- |
| dot11TxRxMCSSetNotEqual | False/Boolean | Static |
| dot11TxMaximumNumberSpatialStreamsSupported | False/Boolean | Static |
| dot11TxUnequalModulationSupported | False/Boolean | Static |

TGn Editor: Change the sentence in Sec.  9.6.6, pg. 96, ln. 34, Draft 1.08:

“then the CandidateMCSSet is the MCS Set consisting of the logical AND of the receive Supported MCS Set

of the STA that sent the frame that is eliciting the response and the set of MCSs that the responding

STA is capable of transmitting.”

References:

IEEE P802.11n/D1.05 October 2006

IEEE P802.11n/D1.08 December 2006