IEEE P802.11
Wireless LANs

| Updates to Clause 10 and Clause 11 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-01-31 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Nancy Cam-Winget | Cisco | 190 W Tasman Dr, San Jose, CA 95134 | 408 853 0532 | ncamwing@cisco.com |
| Kapil Sood | Intel Corporation | 2111 NE 25th Ave JF3-206 Hillsboro OR 97124 | +1-503-264-3759 | kapil.sood@intel.com |
| Jesse Walker | Intel | 2111 NE 25th Ave JF3-206 Hillsboro OR 97124 | +1-503-712-1849 | jesse.walker@intel.com |
| Abhijit Choudhury | Cisco Systems | 3625 Cisco Way, San Jose CA 95134 | +1-408-525-8682 | Abhijitc@cisco.com |

Abstract

This contribution addresses LB88 comments 404 and 561.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

### Overview

Comment 404 points out the lack of MLME interfaces for broadcast/multicast in Clauses 10 and 11.  Comment 561 requests a behavioural description for the delivery of MMPDUs  when the APs support both TGw and non-TGw STAs.

The following editorial instructions are based on the TGw Draft 1.01 and 802.11ma D9.0:

### 10.3.17.1  MLME-SETKEYS.request

### 10.3.17.1.2 Semantics of the service primitive

Change the 'Key Type' entry in the SetKeyDescriptor of Clause 10.3.17.1.2  as follows:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Key Type | Integer | Group, Pairwise, Peerkey, IGTK | Defines whether this key is a group key, pairwise key, or PeerKey, or the Integrity Group key. |

### 10.3.18.1  MLME-DELETEKEYS.request

### 10.3.18.1.2 Semantics of the service primitive

Change the 'Protect Type' and 'Key Type' entries in the SetKeyDescriptor of Clause 10.3.18.1.2  as follows:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Key Type | Integer | Group, Pairwise, Peerkey, IGTK | Defines whether this key is a group key, pairwise key, or PeerKey, or the Integrity Group key. |

### 10.3.22.1 MLME-SETPROTECTION.request

### 10.3.22.1.2 Semantics of the service primitive

Change the 'Key Type' entry  in the ProtectList  of Clause 10.3.22..21 as follows:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Protect Type | Enumeration | None, Rx, Tx, Rx_Tx, Rx_MMPDU, Tx_MMPDU, Rx_Tx_MMPDU | The protection value for this MAC. |
| Key Type | Integer | Group, Pairwise, Peerkey, IGTK | Defines whether this key is a group key, pairwise key, or PeerKey or the Integrity Group key. |

### Change Clause 10.3.22.1.4 as follows:

### 10.3.22.1.4 Effect of receipt

Receipt of this primitive causes the MAC to set the protection and to protect data frames and Robust Management frames as indicated in the ProtectType element of the Protectlist parameter:

- None: Specifies that data frames neither from the MAC address nor to the MAC address is protected.

- Rx: Specifies that data frames from MAC address is protected

- Tx: Specifies that data frames to MAC address is protected

- Rx_Tx: Specifies that data frames to and from MAC address is protected.

- Rx_MMPDU: Specifies that MMPDU frames from MAC address is protected

- Tx_MMPDU: Specifies that MMPDU frames to MAC address is protected

- Rx_Tx_MMPDU: Specifies that MMPDU frames to and from MAC address is protected.

Once it is specified that data or MMPDU frame is protected to or from a MAC address, this is reset by the MLME-SETPROTECTION.request primitive.  The MLME-SETPROTECTION.request primitive deletes the state by specifying None.

Change Clause 11.18 of 802.11w Draft 1.01 as follows:

11.18 Broadcast and multicast Robust Management Frame procedures

When Robust Management frame protection is enabled, the MLME shall provide an encapsulation service for robust broadcast/multicast management frames. All Robust management frames shall be submitted to this service for encapsulation and transmission.

The broadcast/multicast frame protection service shall take the following actions:

- Robust Management frame protection for multicast/broadcast shall be set using the MLME-SETPROTECTION.request with the Protectlist including a Key Type value of IGTK.  A non-AP STA shall also set the Protect Type value to Rx_MMPDU. An AP shall set the Protect Type value to Tx_MMPDU.

- The IGTK shall be installed using the MLME-SETKEYS.request with the value IGTK for the Key Type field in the Key Descriptor element.

- All Robust broadcast/multicast action mRobust Management frames, the broadcast/multicast management frame shall be encapsulated and protected using BIP (see 8.3.4).

The broadcast/multicast frame protection service shall be used by other services internal to the MLME layer and shall take as an input the body of the frame as described in 7.2.3.12. The destination address is assumed to be the broadcast/multicast address. The service shall return a status result indicating acceptance of the frame for processing or rejection of the frame.