IEEE P802.11
Wireless LANs

| Protected Management Frame Pseudo-code |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-07-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Abhijit Choudhury | Cisco Systems | 3625 Cisco Way, San Jose CA 95134 | +1-408-525-8682 | abhijitc@cisco.com |
| Nancy Cam-Winget | Cisco Systems | 190 W Tasman, San Jose CA 95134 | +1-408-853-0532 | ncamwing@cisco.com |
| Jesse Walker | Intel | 2111 NE 25th Ave JF3-206 Hillsboro OR 97124 | +1-503-712-1849 | jesse.walker@intel.com |
| Dorothy Stanley | Aruba | 1322 Crossman Ave, Sunnyvale, CA 94089 | +1-630-363-1389 | dstanley@arubanetworks.com |
| Kapil Sood | Intel Corporation | 2111 NE 25th Ave JF3-206 Hillsboro OR 97124 | +1-503-264-3759 | kapil.sood@intel.com |

Abstract

This document contains proposed changes to the Per-frame Pseudo-code in Clause 8.7 in TGw draft D2.0 to address LB 102 comments (#368, 369, 163, 370, 116, 164, 223, 165, 371, 372, 373, 374, 663, 375, 166, 739, 167, 740, 664, 376, 168, 741, 169, 377, 378, 379, 70, 71, 170, 171).

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

8.7.2.1 Per-MSDU Tx pseudo-code

8.7.2.1a Per-MMPDU Tx pseudo-code

if (dot11RSNAEnabled = TRUE) then

if ((dot11RSNAProtectedManagementFramesEnabled = TRUE) and (FrameControl.SubType

is one of Disassociation, Deauthentication or Action)) then

// Management Frame Protection is enabled and frame is eligible for protection

if (MMPDU has an individual RA) then

if (MMPDU Protection for RA is off for Tx)) then

// Check for legacy operation

Transmit the MMPDU without protection, after fragmentation

else if (Pairwise key exists for the MMPDU's RA) then

// Note that it is assumed that no entry in the key

// mapping table will be of an unsupported cipher type

Transmit the MMPDU, to be protected after fragmentation

// see 8.7.2.2a

else

// pairwise key was not found

Discard the entire MMPDU and generate an MLME.confirm primitive,

if it exists, to notify the SME that the MMPDU was undeliverable

endif

else // MMPDU has a multicast/broadcast RA

if (IGTK exists) then

// if we find a suitable IGTK

Set the Key ID subfield of the MMIE to corresponding IGTK KeyID

Transmit the MMPDU with BIP

else

Discard the entire MMPDU and generate an MLME.confirm primitive, if it exists, to notify the SME that the MMPDU was undeliverable

endif

endif

else

// either Management Frame Protection is not enabled OR

// frame is not eligible for protection

Transmit the MMPDU without protection

endif

endif

8.7.2.2a Per-MPDU Tx pseudo-code for MMPDU

if (dot11RSNAEnabled = TRUE) then

if (MPDU is member of an MMPDU that is to be transmitted without   protection) then

Transmit the MPDU without protection

else if (MPDU has an individual RA) then

Protect the MPDU using entry's TK and selected cipher from RSN IE

Transmit the MPDU

else

// MPDU has a multicast/broadcast RA

Protect the MPDU using IGTK and BIP

Transmit the MPDU

endif

endif

8.7.2.3A Per-MPDU Rx pseudo-code for MMPDU

if (dot11RSNAEnabled = TRUE) then

if (dot11RSNAProtectedManagementFramesEnabled = TRUE) then

if (Protection for TA is off for Rx) then

// (dot11RSNALegacyManagementFrames = TRUE for Rx) and

// TA does not support Management Frame Protection

if (Protected Frame subfield of the Frame Control field is set to 1) then

Discard the frame

else

Receive the MPDU

endif

else

// Management Frame Protection is expected

if (MPDU has individual RA) then

if (Protected Frame subfield of the Frame Control field is

set to 0) then

// unprotected frame

if ((Pairwise key exists) or

((Pairwise key does not exist) and

(FrameControl.SubType is Action))) then

Discard the frame

if (security association has an AES-CCM key) then

Increment dot11RSNAStatsCCMPDecryptErrors

endif

endif

else if (security association has an AES-CCM key) then

if (PN is not valid) then

Discard the MPDU as a replay

Increment dot11RSNAStatsRobustMgmtCCMPReplays

else

Decrypt frame using AES-CCM key

if (the integrity check fails) then

Discard the frame

Increment dot11RSNAStatsCCMPDecryptErrors

else

Make the MPDU available for further processing

endif

endif

else

// if (any other cipher exists) then

//     Process the frame using other cipher

// else

//     Discard the frame

// endif

endif

else if (MPDU has multicast/broadcast RA) then

if (MMIE is not present) then

// unprotected frame

if ((IGTK exists) or

((IGTK does not exist) and

(FrameControl.SubType is Action))) then

Discard the frame

if (security association has an AES-128-CMAC IGTK) then

Increment dot11RSNAStatsCMACICVErrors

endif

endif

else if (security association has an AES-128-CMAC IGTK) then

if (PN is not valid) then

Discard the MPDU as a replay

Increment dot11RSNAStatsCMACReplays

else

Check integrity of the frame using AES-128-CMAC key

if (the ICV fails) then

Discard the frame

Increment dot11RSNAStatsCMACICVErrors

else

Make the MPDU available for further processing

endif

endif

else

// if (any other cipher exists) then

//     Process the frame using other cipher

// else

//     Discard the frame

// endif

endif

endif // if (MPDU has individual RA) then

endif // if (Protection for TA is true for Rx) then

else // if (dot11RSNAProtectedManagementFramesEnabled = TRUE)  then

Receive the MPDU

endif

endif

8.7.2.4A Per-MMPDU Rx pseudo-code

if (dot11RSNAEnabled = TRUE) then

if(dot11RSNAProtectedManagementFramesEnabled = TRUE)) then

if (the MMPDU was not protected) then

Receive the MMPDU unprotected

Make the MMPDU available to higher layers

else //Have a protected MMPDU

if ((MMPDU has individual RA) and (security association has an AES-CCM key))

if (the MMPDU has only one MPDU or multiple MPDUS with

sequential PNs) then

Receive the MMPDU

Make the MMPDU available to higher layers

else

Discard the MMPDU as a replay

Increment dot11RSNAStatsRobustMgmtCCMPReplays

endif

else if ((MMPDU has multicast/broadcast RA) and

(security association has an AES-128-CMAC IGTK)) then

Receive the MMPDU

Make the MMPDU available to higher layers

else

// if (any other cipher exists) then

//     Process the frame using other cipher

// else

//     Discard the frame

// endif

endif

endif

endif

endif

References:

IEEE 802.11 WG LB102: IEEE P802.11w/2.0, March 2007