IEEE P802.11
Wireless LANs

| LB97 PHY Comment Resolutions: Preamble |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-29 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bjorn A. Bjerke | Qualcomm, Inc. | 9 Damonmill Sq. Suite 2A Concord, MA 01742, USA | +1 781-276-0912 | bbjerke@qualcomm.com |

Abstract

This submission suggests resolutions of LB97 PHY comments related to the sub-topic Preamble. The following CIDs are addressed: 281, 338, 414, 656, 657, 658, 2648, 2699, 3129, 3130, 3131, 3144.

The changes marked in this document are based on TGn Draft 2.0

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

| 281 | 257.54 | 20.3.9.4.6 | Error in Figure n71: "N_STS+N_ELTF" | Change to "N_STS+N_ESS" |
| --- | --- | --- | --- | --- |

Suggested resolution: Accept

in the middle block of Figure n71 needs to be replaced by  so that the columns numbered  through (and not ) of matrix Q are used for the spatial mapping.

TGn Editor: in Figure n71, replace  in the middle block by

| 338 | 255.30 | 20.3.9.4.6 | If the receiver advertised ability to receive ELTF, then it shouldn't be optional for the receiver to decode the PPDU. | Remove "When an HT packet includes one or more extension HT-LTFs, it is optional for a receiver to decode the data poriton of the PPDU." |
| --- | --- | --- | --- | --- |

Suggested resolution: Counter

Make it optional to decode the PPDU for a receiver that has not advertised its capability to receive Extension HT-LTFs

TGn Editor: on page 255, line 30, change the sentence beginning with “When an HT packet…” as follows:

When an HT packet includes one or more Eextension HT-LTFs, it is optional for a receiver that has not advertised its capability to receive Extension HT-LTFs to decode the data portion of the PPDU.

| 414 | 255.30 | 20.3.9.4.6 | If the receiver advertised ability to receive ELTF, then it shouldn't be optional for the receiver to decode the PPDU. | Remove "When an HT packet includes one or more extension HT-LTFs, it is optional for a receiver to decode the data poriton of the PPDU." |
| --- | --- | --- | --- | --- |

Duplicate; same suggested resolution as CID 338

| 656 | 251.00 | 20.3.9.4.3 | Please mention that convolutional coding of rate 1/2 and QBPSK modulation of subcarriers is used, or refer to Clause 17.3.4. It will be good also to mention brief reason on why the HT-SIG field of mixed mode format is transmitted like other non-HT transmissions, in different to the HT-SIG of greenfield format. |  |
| --- | --- | --- | --- | --- |

Suggested resolution: Accept

TGn Editor: on page 251, line 46, change the sentence beginning with “The HT-SIG parts…” as follows:

The HT-SIG parts are encoded at R = 1/2, interleaved, mapped to a BPSK constellation, and have pilots inserted following the steps described in 17.3.5.5, 17.3.5.6, 17.3.5.7, and 17.3.5.8.  The BPSK constellation is rotated by 90° relative to the non-HT SIGNAL field in order to accommodate detection of the start of the HT SIGNAL field.

TGn Editor: on page 252, line 38, add the following text at the end of the NOTE as follows:

NOTE—this definition results in a BPSK modulation in which the constellation of the data tones is rotated by 90º relative to the non-HT SIGNAL field in HT mixed format frames, and relative to the first HT-LTF in greenfield format frames (see Figure n68 (Data tone constellations in the non-HT SIGNAL field (L-SIG) and the HT SIGNAL field (HT-SIG) in an HT mixed format PPDU)).  In HT mixed format frames, the HT SIGNAL field is transmitted with the same number of subcarriers and the same cyclic shifts as the preceding non-HT portion of the preamble.  This is done to accommodate the estimation of channel parameters needed to robustly demodulate and decode the information contained in the HT SIGNAL field.

| 657 | 257.00 | 20.3.9.4.6 | In Figures n70 and n71, it seems that the [P_HTLTF] 1, i_STS to [P_HTLTF] N_STS, i_STS seems not correct. The column indices shall be 'n' for the n'th DLTF or ELTF as per the equations 20-25 and 20-26. The CSD block before the IDFT in these Figures is also confusing, as equations 20-25 and 20-26 suggest the shifts done after IDFT in time domain signals. | correction or clarification required. |
| --- | --- | --- | --- | --- |

Suggested resolution: Counter

- In Figure n70, the rightmost subscript of both and   must be changed from to , and in Figure n71, the rightmost subscript of both and   must be changed from to .

- The CSD may be implemented in the time-domain or the frequency domain.  In a frequency domain implementation, the CSDs are scalar multiplications applied on a per spatial stream basis, where each stream corresponds to a row in the matrix .  In this case, the ordering of the CSD block and the IDFT is arbitrary, so the figures should be left as they are.

TGn Editor: in Figure n70, replace both occurrences of  by

TGn Editor: in Figure n71, replace both occurrences of  by

| 658 | 260.20 | 20.3.9.5.3 | It would be helpful to add some description on why the HT Signal field needs to be processed using the Spatial Mapping like all other HT fields except HT-Sig in mixed mode format. It is not described in 20.3.9.4.3. |  |
| --- | --- | --- | --- | --- |

Suggested resolution: Accept

TGn Editor: on page 260, at the end of the paragraph that ends on line 20, add the following text:

In HT greenfield format frames, the HT SIGNAL field is transmitted with the same number of subcarriers, the same cyclic shifts, and the same spatial mapping as the preceding portions of the preamble.  This is done to accommodate the estimation of channel parameters needed to robustly demodulate and decode the information contained in the HT SIGNAL field.

| 2648 | 220.45 | 20.3.2 | "This parameter shall be set to 6 Mb/s when FORMAT is HT_MF." If the usage is defined in 9.13.4, this statement is a contradiction. It is also unnecessary. The PHY doesn't care what goes in this rate field, but the MAC does. So the PHY shouldn't attempt to prescribe its contents. If this is not a true statement, this parameter should be not present, and the PHY should insert a value of 6 Mbps whenever HT_MF is used. | Remove the quoted sentence. |
| --- | --- | --- | --- | --- |

Suggested resolution: Accept

The sentence is redundant, as the proper setting of the parameter is defined in 9.13.4.

TGn Editor: on page 220, line 45, delete the sentence that starts with “This parameter shall be set to…”.

| 2699 | 255.11 | 20.3.9.4.6 | "it may discard a frame including Extension HT-LTFs as an unknown frame type." This is not so. An unknown frame type still requires the MAC to respect the duration/ID field. | The action of the STA needs to be described in terms of what it signals to the MAC in this case. I suggest that the PHY signals a PHY-RXEND.indication (UnsupportedRate), or we could add a new status code: UnsupportedFormat. Perhaps there should be a statement somewhere in the SAP description that includes this and all other PHY options (e.g. Short GI, LDPC...) that says that a PPDU received that indicates the use of any feature not supported generates a PHY-RXEND.indication of some specified error condition. |
| --- | --- | --- | --- | --- |

Suggested resolution: Counter

The primitive PHY-RXEND.indicate(FormatViolation) is used to indicate unsupported preamble in clause 20.3.23 PLCP receive procedure.  The text in question should be modified to be aligned with the PLCP receive procedure.

TGn Editor: on page 255, change the sentence on line 11 as follows:

If a receiver has not advertised its ability to receive Extension HT-LTFs, it shall either may discard a frame including Extension HT-LTFs as an unknown frame type issue a PHY-RXEND.indicate(FormatViolation) primitive upon reception of a frame that includes Extension HT-LTFs, or decode that frame.

TGn Editor: in clause 20.3.23 (PLCP receive procedure), page 305, on and around line 47, add the text  as shown below:

- If the HT-SIG indicates Extension spatial streams, i.e., Extension HT-LTFs are present, and the receiver has not advertised its ability to receive Extension HT-LTFs, the HT PHY shall maintain PHY-CCA.indication(BUSY) until the received level drops below the CCA sensitivity level (minimum modulation and coding rate sensitivity + 20 dB) specified in 20.3.21.1.

TGn Editor: in clause 20.3.23 (PLCP receive procedure), page 305, on line 53, add the text as shown below:

Subsequent to an indication of a valid HT-SIG CRC, a PHY-RXSTART.indication(RXVECTOR) shall be issued.  The RXVECTOR associated with this primitive includes the parameters specified in Table n63 (TXVECTOR and RXVECTOR parameters).  Upon reception of a GF preamble by an HT STA that does not support GF, the FORMAT field of RXVECTOR is set to HT_GF and the remaining fields may be empty, and the PHY shall issue the error condition PHY-RXEND.indication(FormatViolation).  Upon reception of Extension HT-LTFs by an HT STA which does not support Extension HT-LTFs, the PHY shall issue the error condition  PHY-RXEND.indication(FormatViolation).  If the HT-SIG is not completely recognizable and supported the PHY shall issue the error condition PHY-RXEND.indication(UnsupportedRate).

TGn Editor: in clause 20.3.23 (PLCP receive procedure), page 308, modify Figure n86 (PLCP receive state machine) as follows (and shown in red in the figure below):

- Add a box “Check for Extension HT-LTFs”between the “Evaluate HT-SIG” and “Setup PSDU RX” boxes; text inside should read: “Determine no. of extension spatial streams from HT-SIG”

- Label arrow from new box to “Setup PSDU RX” box: “ELTFs supported”

- Label arrow from new box to “End of wait” box on the left: “ELTFs not supported”

- Add description below last arrow: “Set PHY_RXSTART.ind(RXVECTOR) then set PHY_RXEND.indicate(FormatViolation)”

- In “End of Wait” box modify text as follows: “For Reserved HT-SIG indication and ELTFs not supported: set PHY_CCA.ind(IDLE)…”

| 3129 | 254.00 | 20.3.9.4.5. | Wrong notation | Change HTS(k) to HTS_k |
| --- | --- | --- | --- | --- |

Suggested resolution: Accept

The proposed change is consistent with the convention for training sequences in the base standard.

TGn Editor: in Equation (20-21) on page 254, change to

| 3130 | 258.00 | 20.3.9.4.6 | Wrong notation | Change HTLTF(k) to HTLTF_k |
| --- | --- | --- | --- | --- |

Suggested resolution: Accept

The proposed change is consistent with the convention for training sequences in the base standard.

TGn Editor: in Equation (20-25) on page 258, change to

| 3131 | 258.00 | 20.3.9.4.6 | Wrong notation | Change HTLTF(k) to HTLTF_k |
| --- | --- | --- | --- | --- |

Suggested resolution: Counter

The proposed change is consistent with the convention for training sequences in the base standard.  For complete consistency, the proposed change must also be implemented in Equation (20-31) on page 261.

TGn Editor: in Equation (20-26) on page 258, change to

TGn Editor: in Equation (20-31) on page 261, change to

| 3144 | 315.00 | 20.4.3 | No reference for N_LTF | Refer equation or section where N_LTF is defined |
| --- | --- | --- | --- | --- |

Suggested resolution: Accept

The parameter is introduced and defined in Section 20.3.9.4.6 “The HT-LTF long training field” and in Eq. (20-22).

TGn Editor: on page 315, after line 11 add the sentence:

is the number of HT-LTFs and is defined in Equation (20-22)