IEEE P802.11
Wireless LANs

| LB97 Submission to resolve PHY HT Legacy spec comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Assaf Kasher | Intel Corporation | Matam Industrial Park Haifa Israel 31015 | +972-4-8651547 | Asssaf.kasher@intel.comment |

Abstract

This document suggests resolution for PHY HT Legacy spec comments with the following CIDs: 1237, 2897, 2898, 2900, 2903, 332, 932, 1268, 1483, 2634, 2640, 2925, 3169.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

This documents suggests resolution to comments related to the HT legacy spec in the PHY (clause 20)

| 1237 | How can a STA that is compliant with PHY specifications in 17/18/19 do any data rate other than those listed in 17/18/19? | reword this statement to qualify the "shall be compliant with PHY specifications" |
| --- | --- | --- |

Suggestion: counter

TGn Editor: Change Line 14 in page 211 of Draft 2.0 as follows:

orthogonal frequency division multiplexing (OFDM) system. When operating in a 20MHz channel width the HT STA shall also be compliant with PHY

| 2897 | The current definition of non-HT PPDU includes "A Clause 20 PHY PPDU with the TXVECTOR FORMAT parameter set to NON_HT and the CH_BANDWIDTH parameter set to NON_HT_CBW20" that actually means that only way to use the already defined PHYs is to use the clause 20. | Change the definition to cover in non-HT PPDUs all the existing PHY w/o any need to use the clause 20 specification. |
| --- | --- | --- |

Suggestion: Reject

Reason:  The PLME specification in clause 20 defines the interface for a clause 20 STA.  Such a STA is also compliant with clauses 17, 18, 19 and therefore the interface to be used when such modulations are transmitted or received by this device must be defined.

| 2898 | Current definition of non-HT PPDU and HT-PPDU shares the same legacy rates and modulations. The HT-PPDU has the legacy rates and modulations for duplicated legacy frames and for frames sent by the legacy rates and modulation with CH_BANDWIDTH = NON_HT_CBW40 and CH_OFFSET = CH_OFF_20U and CH_OFF_20L. Current definition complicates understanding of the entire specification. | Extend definition of the non-HT PPDU: include in the non-HT PPDU frames of the clause 20 that use the legacy rates and modulation. |
| --- | --- | --- |
| 1483 | It seems unnatural that non-HT duplicate frame (PPDU) is considered to be part of HT PPDU. This creates unnecessary verbiage in subclause 9.6 and creates confusion to the reader. | Define non-HT duplicate PPDU and add in the definition of HT PPDU that non-HT duplicate PPDU is not a part of HT PPDU. Search for the usage of HT PPDU and revise the text to fit the new "natural" definition. |

Suggestion: Counter (Accept in principle)

See below

| 2900 | "NON_HT indicates Clause 15, 17, 18 or 19 PPDU formats, or non-HT duplicated PPDU format …" but on the page 217 line 8 two more cases are defined in addition to the non-HT duplicated - CH_OFF_20U and CH_OFF_20L w/o any relation to NON_HT format | make it consistent |
| --- | --- | --- |

Suggestion: Counter

Legacy upper and lower modulation types where added (see below).   These parameters apply to both non-HT and HT transmissions so the text is now consistent.

| 2903 | The clause 20 does not specify relation of the HT PHY to other PHYs. One example is the new definition of the CCA and CCA sensitivity that differs from other PHYs and impacts operation that uses the legacy rates and modulations. These changes should be presented. The subclause 19.1.2 Operational modes may be used as an example. | Add subclause Operating modes to explain changes of HT PHY in relation to legacy PHY features like CCA. |
| --- | --- | --- |

Suggestion: Reject

Reason: CCA rules are specified well for all types of transmission in 20.3.21.5 and changes specified in 07-457.  There is no need to add a new subclause.

| 332 | A maximum L_LENGTH of 4095 octets can led to the medium being consumed for too long a time and affect performances of applications, especially those that are time-critical, like voice-related services. | Change the maximum of L_LENGTH to 2400 octets or a reasonable value in this range. |
| --- | --- | --- |

Suggestion: Transfer to MAC subgroup.

Reason:  The MAC sets the length, the limit defined here is only a physical one due to the number of bits used.

| 932 | Since HT devices are being required to be compliant with a/b/g, clauses 15, 17, 18, and 19 should be deleted by this amendment. | Delete clauses 15, 17, 18, and 19. Copy the parts cross referenced in clause 20 into clause 20. Call clause 20 "The PHY". Change PICS entries for 15/17/18/19 to reference the corresponding parts of clause 20. Delete clause 16 as useless. Shorten this huge spec, rather than lengthen it. |
| --- | --- | --- |

Suggestion: Reject

Clause 15, 17, 18, 19 are needed because devices complying with these clauses only can still be build.

| 1268 | identify the MCS index that corresponds to 11a, 11b, and 11g PHYs | Insert a table showing the corresponding values for legacy 11a, 11b, and 11g devices. |
| --- | --- | --- |

Suggestion: Reject

Legacy modulation and coding rate are selected according to rates and not MCS indices.  They use the L_DATARATE for this purpose as part of an HT STA.

| 2634 | "The HT STA shall be compliant with PHY specifications as defined in Clause 17 for operation in the 5 GHz bands, and Clauses 18 and 19 for operation in the 2.4 GHz bands ." Which specifications? Clearly not all of them all the time, for example during 40MHz transmission the 20MHz mask specified in Clauses 17, 19 is not applicable, and the STA clearly does not comply with them. | Indicate which PHY specifications it is compliant with, or remove the quoted sentence. |
| --- | --- | --- |

Suggestion: Counter

The sentenece was modified above to include the word “also” and the limitation to 20MHz operating mode.   With this word it is now clear that the PHY is compliant with these clauses when transmitting and receiving frames compliant with these clauses.

| 2640 | "HT mixed format: packets of this format contain a preamble compatible with the non-HT receivers." Not so for non-OFDM receivers | Reword: "HT mixed format: packets of this format contain a preamble compatible with the Clause 17 and Clause 19 receivers." |
| --- | --- | --- |

Suggestion: accept

TGn Editor: Change the following text in line 51 of page 212 of Draft D2.0:

— HT mixed format: packets of this format contain a preamble compatible with the clause 17 and clause 19 non-HT receivers.

| 2925 | No mention of ERP-OFDM in L_DATARATE | Add ERP-OFDM in the value column alongwith the data rates. |
| --- | --- | --- |
| 3169 | No mention of ERP-OFDM in L_DATARATE | Add ERP-OFDM in the value column alongwith the data rates. |

Suggestion: Accept

See below

TGn Editor: Change the NON_HT_MODULATION line of table n56 TXVECTOR and RXVECTOR in lines 26-46 of page 218 of Draft D2.0:

| Parameter | Condition | values | Parameter is Present in which? |  |
| --- | --- | --- | --- | --- |
|  |  |  | TXVECTOR Y/N/O | RXVECTOR Y/N/O |
| NON_HT_MODULATION |  | When FORMAT is NON_HT: Enumerated Type: ERP-DSS ERP-CCK ERP-OFDM ERP-PBCC DSS-OFDM OFDM NON-HT-DUP-OFDM UPPER-20-OFDM LOWER-20-OFDM | Y | Y |

TGn Editor: Change the L_DATARATE line of table n56 TXVECTOR and RXVECTOR in lines 10-37 of page 220 of Draft D2.0:

| Parameter | Condition | values | Parameter is Present in which? |  |
| --- | --- | --- | --- | --- |
|  |  |  | TXVECTOR Y/N/O | RXVECTOR Y/N/O |
| L_DATARATE | FORMAT is NOT_HT | Indicates the rate used to transmit the PSDU in Mb/s. Allowed values depend on the value of the NON-HT MODULATION parameter thus: DSSS: 1 and 2 CCK: 5.5 and 11 PBCC: 5.5, 11, 22 and 33 DSSS-OFDM: 6, 9, 12, 18, 24, 36, 48 and 54 ERP-OFDM: NON-HT-DUP-OFDM, UPPER-20-OFDM, LOWER-20-OFDM: 6, 9, 12, 18, 24, 36, 48 and 54 OFDM (depends on channel spacing): 20 MHz channel spacing: 6, 9, 12, 18, 24, 36, 48, and 54 10 MHz channel spacing: 3, 4.5, 6, 9, 12, 18, 24, and 27 (See Note 1) 5 MHz channel spacing: 1.5, 2.25, 3, 4.5, 6, 9, 12, and 13.5 (See Note 1) | Y | Y |

TGn Editor: Change the CH_OFFSET line of table n56 TXVECTOR and RXVECTOR in lines 35-57 of page 223 of Draft D2.0:

| Parameter | Condition | values | Parameter is Present in which? |  |
| --- | --- | --- | --- | --- |
|  |  |  | TXVECTOR Y/N/O | RXVECTOR Y/N/O |
| CH_OFFSET | FORMAT is HT_MF or HT_GF, or NON_HT with modulation NON-HT-DUP-OFDM, or UPPER-20-OFDM, or LOWER-20-OFDM | The CH_OFFSET parameter indicates which portion of the channel is used for transmission. Enumerated type: CH_OFF_20 indicates the use of a 20 MHz channel (that is not part of a 40 MHz channel) CH_OFF_40 indicates the entire 40 MHz channel CH_OFF_20U indicates the upper 20 MHz of the 40 MHz channel CH_OFF_20L indicates the lower 20 MHz of the 40 MHz channel | Y | N |

TGn Editor: Change the following lines in table n57 (lines 28-31 of page 231 draft D2.0) (and the title of this table):

| HT PHY Parameter | 2.4 GHz Operation defined by Clause 19 | 5 GHz Operation defined by clasue 17 |
| --- | --- | --- |
| CH_BANDWIDTH | CH_BANDWIDTH | CH_BANDWIDTH |
| CH_OFFSET | CH_OFFSET | CH_OFFSET |

TGn Editor: Add the following after the paragraph in lines 46-49 of page 243 (clause 20.3.8 Transmission in the upper/lower 20MHz of a 40MHz channel):

This applies to 20MHz HT transmissions, to clause 17 transmissions, and to clause 19 OFDM transmissions.