IEEE P802.11
Wireless LANs

| Updated Texts for Clause 11A.7 RA-OLSR |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-05-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Youiti Kado | National Institute of Information and Communications Technology (NICT) | 3-5 Hikaridai, Seika-cho, Soraku-gun, Kyoto 619-0289, JAPAN | +81-774-98-6900 | kado@nict.go.jp |
| Kyeongsoo Kim | STMicroelectronics, Inc. | 1060 East Brokaw Road, MS 212, San Jose, CA 95131, USA | +1-408-451-8137 | kyeongsoo.kim@st.com |
| Azman-Osman Lim | National Institute of Information and Communications Technology (NICT) | 3-5 Hikaridai, Seika-cho, Soraku-gun, Kyoto 619-0289, JAPAN | +81-774-98-6868 | aolim@nict.go.jp |
| Yong Liu | Samsung Information System America | 75 West Plumeria Drive, San Jose, CA 95134, USA | +1-408-544-5649 | yo.liu@samsung.com |
| Kenichi Mase | Niigata University | 8050 Igarashi 2, Niigata 950-2181, JAPAN | +81-25-262-6755 | mase@ie.niigata-u.ac.jp |
| Masanori Nozaki | Oki Electric Industry Co., Ltd. | 2-5-7 Honmachi, Chuo-ku, Osaka 541-0053, JAPAN | +81-6-6260-0700 | nozaki765@oki.com |
| Hiraku Okada | Niigata University | 8050 Igarashi 2, Niigata 950-2181, JAPAN | +81-25-262-6133 | hiraku@ie.niigata-u.ac.jp |
| Mineo Takai | University of California, Los Angeles (UCLA) | 3532 Boelter Hall, Los Angeles, CA 90095-1596, USA | +1-310-825-2303 | mineo@ieee.org |
| Xudong Wang | Kiyon, Inc. | 9381 Judicial Drive, Suite 160, San Diego, CA 92121, USA | +1-858-453-3491 | xudong@kiyon.com |
| Bing Zhang | National Institute of Information and Communications Technology (NICT) | 3-5 Hikaridai, Seika-cho, Soraku-gun, Kyoto 619-0289, JAPAN | +81-774-98-6820 | zhang@nict.go.jp |

Abstract

This submission provides new updated texts for RA-OLSR protocol, which completely replace those in Clause 11A.7 of the current draft IEEE P802.11sTM/D1.03.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

### Background

This document provides the updated texts for RA-OLSR protocol and specifications in the Clause 11A.7, which resolve part of the CIDs given in [2].

Affected CIDs:

| Category | CID |
| --- | --- |
| Clarification/Correction: | 1037, 2160, 2366, 2783, 3392, 3395, 4559, 4565, 5046, 5049, 5054, 5056, 5058, 5059, 5062, 5063, 5068, 5070, 5072, 5074, 5079, 5087, 5089, 5093, 5415, 5423, 5437, 5471, 5472, 5474, 5475, 5491 |
| Parameters: | 1562, 3650, 4608, 4609, 4610, 5060, 5708, 5710, 5711, 5712, 5713 |
| Element Format: | 35, 2362, 2364 |
| Operations (Basic/Proxy): | 562, 2161, 2787, 3649, 3651, 4605, 4606, 4607, 5382, 5697, 5699, 5700, 5703, 5705, 5706 |

Expected resolutions:

- Correction, clarification, informative/normative of the current text for clause 11A.7 RA-OLSR.

- Clarify the parameters and values.

- Submissions that improve basic and proxy operations.

- Reorganize and modify the explanation of element generation, forwarding, and processing.

Summary of changes:

- Redrawn the PICS that used in the clause 11A.7 RA-OLSR.

- Replaced “message” with “element,” “MD5” with “CRC32,” etc.

- Reorganized and modified the ABBR element generation, forwarding and processing.

- Reorganized and modified the associated station discovery.

- The following is normative text proposed as an amendment to P802.11s/D1.03.

Replace the whole texts in clause 7.3.2 with those given in the followinig pages:

- Information elements

Insert the following entries in Table 26 and change the Reserved row as shown.

| - Element IDs |  |  |
| --- | --- | --- |
| Information element | Element ID | Length (in octets) |
| Mesh Capability | <ABSB> (see note below) | 18 |
| Active Mesh Profile Announcement | <ABSB> (see note below) | 9 |
| Mesh ID | <ABSB> (see note below) | 0 to 255 |
| Local Link state announcement | <ABSB> (see note below) | 4 |
| Target Transmission Rate | <ABSB> (see note below) | 18 |
| Offered Traffic Load | <ABSB> (see note below) | 16 |
| Neighborhood Congestion | <ABSB> (see note below) | 3 |
| Peer Link Management | <ABSB> (see note below) | 3 to 7 |
| Mesh Portal Reachability | <ABSB> (see note below) | 1 to 255 |
| Mesh Portal/Root Announcement | <ABSB> (see note below) | 28 to 255 |
| Mesh Channel Switch Announcement | <ABSB> (see note below) | 13 |
| Mesh Neighbor List | <ABSB> (see note below) | 1 to 255 |
| Mesh DTIM | <ABSB> (see note below) | 2 |
| Beacon Timing | <ABSB> (see note below) | 5 to 255 |
| MDAOP Setup Request | <ABSB> (see note below) | 8 to 255 |
| MDAOP Setup Reply | <ABSB> (see note below) | 3 |
| MDAOP Advertisements Request | <ABSB> (see note below) | 0 |
| MDAOP Advertisements | <ABSB> (see note below) | 1 to 255 |
| MDAOP Set Teardown | <ABSB> (see note below) | 7 |
| Connectivity Report | <ABSB> (see note below) | 14 to 255 |
| Portal Announcement (PANN) | <ABSB> (see note below) | 17 |
| Root Announcement (RANN) | <ABSB> (see note below) | 21 |
| Path Request (PREQ) | <ABSB> (see note below) | 36 to 255 |
| Path Reply (PREP) | <ABSB> (see note below) | 32 to 255 |
| Path Error (PERR) | <ABSB> (see note below) | 13 |
| Proxy Update (PU) | <ABSB> (see note below) | 12 to 255 |
| Proxy Update Confirmation (PUC) | <ABSB> (see note below) | 10 |
| HELLO | <ABSB> (see note below) | 27 to 255 |
| Topology Control (TC) | <ABSB> (see note below) | 25 to 255 |
| Multiple Interface Declaration (MID) | <ABSB> (see note below) | 19 to 255 |
| Local Association Base Advertisement (LABA) | <ABSB> (see note below) | 22 to 255 |
| Local Association Base Checksum Advertisement (LABCA) | <ABSB> (see note below) | 18 to 255 |
| Association Base Block Request (ABBR) | <ABSB> (see note below) | 14 to 255 |
| Mesh Security Capability | <ABSB> (see note below) | 7 |
| MSA Handshake | <ABSB> (see note below) | 88 to 255 |
| Mesh Key Holder Security | <ABSB> (see note below) | 98 |
| Mesh Encrypted Key | <ABSB> (see note below) | 82 to 255 |
| EAP Authentication | <ABSB> (see note below) | 42 |
| EAP Message | <ABSB> (see note below) | 2 to 255 |
| Reserved | <ABSB> (see note below) |  |

Replace the whole texts in clause 7.3.2.78 with those given in the followinig pages:

- RA-OLSR common information elements

RA-OLSR elements are carried in RA-OLSR frames (defined in Error: Reference source not found) and have the common information element format shown in RA-OLSR .

| Octets: 1 | 1 | 1 | 6 | 1 | 1 | 2 | Variable |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Element ID | Length | Vtime | Originator Address | TTL | Hop Count | Sequence Number | IE-Specific |
| - RA-OLSR common information element format |  |  |  |  |  |  |  |

The Element ID is set to the value given inElement IDs for the information element (IE).

The Vtime field indicates the validity time (in seconds) during which an MP considers the information contained in the information element as valid after its reception, unless a more recent update to the information is received. The validity time is represented by its mantissa (four highest bits of Vtime field) and by its exponent (four lowest bits of Vtime field). In other words:

Vtime=C×1+a16×2

where ‘a’ is the integer represented by the four highest bits of Vtime field.

The Originator Address is the MAC address of the MP that has originally generated this information element. This may be different from the sender-address in case that multiple IEs are encapsulated in one frame.

The Time to Live (TTL) field indicates the maximum number of hops allowed for this information element.

The Hop Count field indicates the number of hops an information element has attained.

The Sequence Number field indicates a sequence number assigned by the originator MP, which ensures that each element can be uniquely identified in the network.

The IE-Specific fields are fields specific to each element and are described in the following subclauses.

- HELLO element

The fields specific to the HELLO element are shown in Fields specific to RA-OLSR LABA .

| Octets: 1 | 1 | 1 | 1 | 6 | 4 | … | 6 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Htime | Willingness | Block 1 Link Code | Block 1 Number of Neighbor Interface Address | Block 1 Neighbor Interface Address 1 | Block 1 Link Metric 1 | … | Block 1 Neighbor Interface Address X | Block 1 Link Metric X |

| … | 1 | 1 | 6 | 4 | … | 6 | 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| … | Block N Link Code | Block N Number of Neighbor Interface Address | Block N Neighbor Interface Address 1 | Block N Link Metric 1 | … | Block N Neighbor Interface Address Y | Block N Link Metric Y |  |  |
| - Fields specific to HELLO element |  |  |  |  |  |  |  |  |  |

The Htime field indicates the HELLO emission interval (in seconds) used by the MP on this particular interface. The HELLO emission interval is represented by its mantissa (four highest bits of Htime field) and by its exponent (four lowest bits of Htime field). In other words:

Htime=C×1+a16×2b

where ‘a’ is the integer represented by the four highest bits of Htime field and ‘b’ the integer represented by the four lowest bits of Htime field.

The Willingness field indicates the willingness of an MP to carry and forward traffic for other MPs (i.e., to be selected as MPR). The value of willingness is 0-7: WILL_NEVER(0), WILL_DEFAULT(3), and WILL_ALWAYS(7). The values 8-255 are reserved.

The Link Code field indicates the link state. The value of link code is defined as: NOT_NEIGH(0), SYM_NEIGH(1) and MPR_NEIGH(2). The values 3-255 are reserved.

The Number of Neighbor Interface Addresses field indicates the number of neighbor interface addresses of each Link Code.

The Neighbor Interface Address field indicates the MAC address of an interface of a neighbor MP.

The Link Metric field indicates the metric of the link.

- Topology Control (TC) element

The fields specific to the Topology Control (TC) element are shown in Fields specific to RA-OLSR LABA .

| Octets: 2 | 6 | 4 | … | 6 | 4 |
| --- | --- | --- | --- | --- | --- |
| Advertised Neighbor Sequence Number (ANSN) | Advertised Neighbor Main Address 1 | Link Metric 1 | … | Advertised Neighbor Main Address N | Link Metric N |
| - Fields specific to TC element |  |  |  |  |  |

The ANSN field indicates a sequence number associated with the advertised neighbor set. Every time an MP detects a change in its advertised neighbor set, it increments this sequence number (“Wrap-around” is handled as described in 11A.7.15). This number is sent in this ANSN field of the TC element to keep track of the most recent information. When an MP receives a TC element, it can decide on the basis of this advertised ANSN, whether or not the received information about the advertised neighbors of the originator MP is more recent than what it already has.

The Advertised Neighbor Main Address field indicates the main address of a neighbor MP.

The Link Metric field indicates the metric of the link.

- Multiple Interface Declaration (MID) element

The fields specific to the Multiple Interface Declaration (MID) element are shown in Fields specific to RA-OLSR LABA .

| Octets: 6 | … | 6 |  |
| --- | --- | --- | --- |
| RA-OLSR Interface Address 1 | … | RA-OLSR Interface Address N |  |
| - Fields specific to MID element |  |  |  |

The RA-OLSR Interface Address field indicates the address of an RA-OLSR interface of the MP, excluding the MPs main address (which is already indicated in the Originator Address field).

- Local Association Base Advertisement (LABA) element

The fields specific to the Local Association Base Advertisement (LABA) element are shown in Fields specific to RA-OLSR LABA .

| Octets: 1 | 1 | 6 | 1 | … | 6 | 1 | … |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Block 1 Index | Block 1 Number of STA | Block 1 STA Address 1 | Block 1 STA Sequence Number 1 | … | Block 1 STA Address X | Block 1 STA Sequence Number X | … |

| 1 | 1 | 6 | 1 | … | 6 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Block N Index | Block N Number of STA | Block N STA Address 1 | Block N STA Sequence Number 1 | … | Block N STA Address Y | Block N STA Sequence Number Y |  |
| - Fields specific to RA-OLSR LABA element |  |  |  |  |  |  |  |

The Block Index field indicates the index of a block that stores a list of STA addresses. The value of Block Index is 0-253.

The Block Number of STA field indicates the size of a block counted in octets and measured from the beginning of the preceding “Block Index” field until the next “Block Index” field (or to the end of the information element if there are no more blocks).

The STA Address field indicates the MAC address of the associated station. The station address does not include the “Group MAC address bit” in the 48-bit MAC address as described for Local Association Base (LAB) and Global Association Base (GAB) in 11A.7.4.

The STA Sequence Number field indicates the sequence number in the association management frame sent by the STA.

- Local Association Base Checksum Advertisement (LABCA) element

The fields specific to the Local Association Base Checksum Advertisement (LABCA) element are shown in Fields specific to LABCA.

| Octets: 1 | 4 | … | 1 | 4 |
| --- | --- | --- | --- | --- |
| Block Index 1 | Block Checksum 1 | … | Block Index N | Block Checksum N |
| - Fields specific to LABCA element |  |  |  |  |

The Block Index field indicates the index of a block that stores a list of STA addresses. The value of Block Index is 0-253.

The Block Checksum field indicates the checksum of a block. This field length is calculated using CRC32 method.

- Association Base Block Request (ABBR) element

The fields specific to the Association Base Block Request (ABBA) element are shown in Fields specific to ABBR.

| Octets: 1 | … | 1 |
| --- | --- | --- |
| Block Index 1 | … | Block Index N |
| - Fields specific to ABBR element |  |  |

The Block Index field indicates the index of a block that has been detected to be inconsistent and is requested for readvertisement by the MP generating this information element, except the Block Index is 254, whereby an originator MP generates an element when the entire local association information of a MAP is needed, or the Block Index is 255, whereby an originator MP generates an element when the global association information is needed.

Replace the whole texts in clause 11A.7 with those given in the followinig pages:

- Radio Aware OLSR path selection protocol (Optional)

- Introduction

Radio Aware Optimized Link State Routing (RA-OLSR) protocol is a proactive, link-state wireless mesh path selection protocol based on Optimized Link State Routing (OLSR) protocol [IETF RFC 3626] with extensions from Fisheye State Routing (FSR) protocol and uses radio-aware metrics in forwarding path and multipoint relay (MPR) set calculation. RA-OLSR enables the discovery and maintenance of optimal paths based on a predefined metric, given that each MP has a mechanism to determine the metric cost of a link to each of its neighbors. In order to propagate the metric link cost information between MPs, a metric field is used in RA-OLSR information elements. In disseminating topology information over the network, RA-OLSR adopts the following approaches in order to reduce the related control overhead:

- It uses only a subset of MPs in the network, called MPRs, in flooding process;

- It optionally controls (and thereby reduces) the element exchange frequencies based on the Fisheye scopes.

The RA-OLSR protocols also include an association discovery and maintenance protocol to support non-mesh STAs both internal (associated with MAPs) and external (with MPPs): When a non-mesh STA is a source or a destination of an IEEE 802 data link, RA-OLSR protocol sets up a path selection path to the MAP or the MPP associated with that STA by complementing path selection information among MPs with STA association information.

- Overview

The optimization in RA-OLSR mainly focuses on the minimization of flooding overhead: First, in RA-OLSR only a selected subset of 1-hop neighbor MPs (i.e., MPRs) is used by an MP in forwarding information elements. The MPRs are selected such that a broadcast element, forwarded by these MPRs, can reach all 2-hop neighbor MPs of the selecting MP (i.e., MPR selector). The information required to perform MPR selection is acquired through the periodic exchange of HELLO elements. In addition, RA-OLSR can also optionally control the element exchange frequencies based on the fisheye scopes to further reduce the amount of information elements exchanges. These techniques significantly reduce the number of forwarding transmissions required to flood a element to all MPs in the network. Second, RA-OLSR requires only partial link state to be flooded in order to provide best path paths. The minimal set of link state information required is that all the MPs selected as MPRs shall declare the links to their MPR selectors. Additional topological information, if present, may be utilized, e.g., for redundancy purposes.

In wireless mesh networks, unlike traditional mobile ad hoc networks where all mobile MPs are directly participating in path selection procedures, associated STAs that do not support mesh services are indirectly participating in path selection procedures through their associated MAPs. To support these STAs, RA-OLSR provides information repositories for associated STAs at MAPs — Local Association Base (LAB) and Global Association Base (GAB) — with efficient advertisement and management schemes, which complements a path selection table at each MP during path selection process. This is detailed in Associated station discovery.

RA-OLSR may optimize the reactivity to topological changes by reducing the maximum time interval for periodic information element transmissions. Furthermore, as RA-OLSR continuously maintains paths to all destinations in the network, the protocol is beneficial for traffic patterns where a large subset of MPs are communicating with another large subset of MPs, and where the [source, destination] pairs are changing over time. The protocol is particularly suited for large and dense networks, as the optimization done using MPRs works well in this context.

RA-OLSR is designed to work in a completely distributed manner and does not depend on any central entity. The protocol does not require reliable transmission of information elements: Each MP sends information elements periodically, and can therefore sustain a reasonable loss of some of such elements. Such losses occur frequently in radio networks due to collisions or other transmission problems.

Also, RA-OLSR does not require sequenced delivery of elements. Each information element contains a sequence number that is incremented for each element. The recipient of an information element can, if required, easily identify which information is more recent — even if elements have been re-ordered while in transmission.

Given a network with only single-interface MPs, an MP may deduct the neighbor set directly from the information exchanged as part of link sensing: the “Main Address” of a single interface MP is, by definition, the address of the only interface on that MP. In a network with multiple-interface MPs, additional information is required in order to map interface addresses to main addresses (and, thereby, to MPs). This additional information is acquired through multiple interface declaration (MID) elements, described in Multiple .

- Terminology

| - Terminology for RA-OLSR |  |
| --- | --- |
| Terminology | Description |
| Main Address | Primary MAC address of the MP, if it has more than one PHY |
| Originator Address | The main address of an MP, which sent a given element |
| Sender Interface Address | The address of the interface, over which the element was last transmitted |
| Receiving Interface | The interface, over which the element was received |
| Associated Station | A station that is associated with an MAP |
| Local Association Base (LAB) | The table of the associated stations to a given MAP |
| Global Association Base (GAB) | The information base that maintains the list of the associated stations to all the MAPs in the Mesh (in other terms, all the Local Association Base of all the MAPs of the Mesh) |
| Association Tuple | The information about associated stations is maintained in entries called “association tuples”, either “Local Association Tuple”, in the LAB, or “Global Association Tuple”, in the GAB |
| Checksum | The value obtained by applying CRC32 function to the information in the LAB / GAB (or subsets of these Association Bases) |

Terminology for RA-OLSR is listed in Terminology for RA-OLSR.

- element processing and forwarding

Here we describe a general rule for processing and forwarding elements (see 7.3.2.78 for details and 7.4.9.12 for its format).

- element processing and flooding

Upon receiving an RA-OLSR management frame, an MP examines each of the included elements. Based on the “ID” value, the MP can determine the further processing of the element.

An MP may receive the same element several times in a wireless mesh network. Therefore, each MP maintains a “Duplicate Set” where the MP records information about the most recently received elements, by which any duplicate processing of those elements can be avoided. For such the element, an MP records a “Duplicate Tuple” as follows:

(D_addr, D_seq_num, D_forwarded, D_iface_list, D_time)

| Field | Description |
| --- | --- |
| D_addr | The originator address of the element |
| D_seq_num | The sequence number of the element |
| D_forwarded | A boolean indicating if the element has already been forwarded |
| D_iface_list | A list of the addresses of the interfaces on which the element has been received |
| D_time | Expiration time of the tuple |

In an MP, the set of Duplicate Tuples are denoted the “Duplicate set”.

Upon receiving an RA-OLSR management frame, an MP shall perform the following tasks for each encapsulated element:

- If the RA-OLSR frame contains no elements, the frame shall be silently discarded.

- If the TTL of the element is less than or equal to zero, or if the element was sent by the receiving MP (i.e., the Originator Address of the element is the main address of the receiving MP), then the element shall be silently dropped.

- Processing condition:

- If there exists a tuple in the duplicate set, where:

D_addr = Originator Address, AND

D_seq_num = Sequence Number.

Then the element has already been processed and shall not be processed again.

- Otherwise, if the MP implements the element type (i.e., element ID), the element shall be processed according to the specifications for the element ID.

- Forwarding condition:

- If there exists a tuple in the duplicate set, where:

D_addr = Originator Address, AND

D_seq_num = Sequence Number, AND

the receiving interface (address) is in D_iface_list.

Then the element has already been considered for forwarding and should not be forwarded again.

- Otherwise:

- If the MP implements the element type (i.e., element ID), the element shall be processed according to the specifications for the element ID.

- Otherwise, the MP implements the element type (i.e., element ID), the element shall be processed according to the default forwarding algorithm described in RA-OLSR default forwarding algorithm.

- RA-OLSR default forwarding algorithm

The default forwarding algorithm is the following:

- If the sender interface address of the element is not detected to be in the symmetric 1-hop neighborhood of the MP, the forwarding algorithm shall silently stop here (and the element shall not be forwarded).

- If there exists a tuple in the duplicate set where:

D_addr = Originator Address, AND

D_seq_num = Sequence Number.

Then the element is further considered for forwarding if and only if:

D_forwarded is false, AND

the (address of the) interface that received the element is not included among the addresses in D_iface_list.

- Otherwise, if such an entry doesn’t exist, the element is further considered for forwarding.

If the element is not considered for forwarding after steps a) through c), the processing of this clause stops here (i.e., steps 4 to 8 are ignored). Otherwise, the following algorithm is used:

- If the sender interface address is an interface address of an MPR selector of this MP AND the TTL of the element is greater than ‘1’, the element shall be forwarded (as described later in steps f) to h)).

- If there exists an entry in the duplicate set with the same Originator Address and Sequence Number, the entry is updated as follows:

D_time = current time + DUP_HOLD_TIME

The receiving interface MAC address is added to D_iface_list.

D_forwarded is set to true if and only if the element is forwarded according to step d).

Otherwise a new entry is added to the duplicate set with:

D_addr = Originator MAC Address.

D_seq_num = Sequence Number.

D_time = current time + DUP_HOLD_TIME.

D_iface_list contains the receiving interface address.

D_forwarded is set to true if and only if the element is forwarded according to step d).

If, and only if, according to step d), the element shall be forwarded then:

- The TTL of the element is decremented by one.

- The hop-count of the element is increased by one.

- The element is broadcast on all interfaces.

- Considerations on processing and forwarding

It should be noted that the processing and the forwarding of elements are two different actions, conditioned by different rules. Processing is related to using the content of the elements, while forwarding is related to forwarding the same element for other MPs in the network.

Notice that this standard includes a description for both the forwarding and the processing of each known element type (i.e., Element ID). elements with known element types shall not be forwarded “blindly” by the algorithm described in RA-OLSR default forwarding algorithm. Forwarding (and setting the correcteelement field in the forwarded, knowneelement) is the responsibility of the algorithm specifying how theeelement is to be handled and, if necessary, forwarded. This enables aeelement type to be specified such that theeelement can be modified while in transit (e.g., to reflect the path theeelement has taken). In this regard, it would be possible to bypass the MPR flooding mechanism if for some reasons, classical flooding is required for aeelement type; in such a case, the forwarding algorithm for thateelement type specifies simple forwarding of receivedeelements, regardless of MPRs.

NOTE--By defining a set of element types, which shall be recognized by all implementations of RA-OLSR, it is possible to extend the protocol through introduction of additional element types, while still being able to maintain compatibility with older implementations.

- Element emission and jitter

As a basic implementation requirement, the synchronization of information elements should be avoided.  As a consequence, RA-OLSR information elements should be emitted such that they avoid synchronization.

Emission of management traffic from neighbor MPs may — for various reasons (mainly timer interactions with frame processing) — become synchronized such that several neighbor MPs attempt to transmit management traffic simultaneously. This may lead to collisions and hence element loss, possibly the loss of several subsequent elements of the same type.

To avoid such synchronizations, the following simple strategy for emitting information elements is recommended.  An MP should add an amount of jitter to the interval at which elements are generated. The jitter shall be a pseudo-random value for each element generated. For an MP utilizing jitter:

Actual element interval = ELEMENT_INTERVAL – jitter

where jitter is a pseudo-random value in [0, MAXJITTER] and ELEMENT_INTERVAL is the value of the element interval specified for the element being emitted (e.g., HELLO_INTERVAL for HELLO elements, TC_INTERVAL for TC-elements, etc.).

Jitter may be introduced when forwarding elements. The following simple strategy may be adopted: When a element is to be forwarded by an MP, it should be kept in the MP during a short period of time:

Keep element period = jitter

where jitter is a pseudo-random value in [0, MAXJITTER]. Notice that when the MP sends an information element, the opportunity to piggyback other elements (before their keeping period is expired) maybe taken to reduce the number of frame transmissions.

- Information repositories

Through the exchange of RA-OLSR information elements, each MP accumulates information about the network. This information is stored according to the descriptions in this subclause.

- Link set

An MP records a set of “link tuples”:

(L_local_iface_addr, L_neighb_iface_addr, L_time, L_link_metric)

describing links between local and remote interfaces. The tuples in this set are maintained by some other component of this standard and populated using Neighbor Table Entry whose link state is available as described in Error: Reference source not found.

| Field | Description |
| --- | --- |
| L_local_iface_addr | The interface on the local MP |
| L_neighb_iface_addr | The interface on the remote MP, with which there exists a symmetric link |
| L_time | The expiration time of this tuple |
| L_link_metric | The value representing the metric cost of the link. An example is the Airtime cost given in Error: Reference source not found |

- Neighbor set

An MP records a set of “neighbor tuples”:

(N_neighb_main_addr, N_willingness)

describing neighbors.

| Field | Description |
| --- | --- |
| N_neighb_main_addr | The interface on the local MP |
| N_willingness | An integer, between 0 and 7, specifying the MPs willingness to carry traffic on behalf of other MPs |

- Interface association set

For each destination in the network, “Interface Association Tuples” are recorded:

(I_iface_addr, I_main_addr, I_time)

| Field | Description |
| --- | --- |
| I_iface_addr | An interface address of an MP |
| I_main_addr | The main address of an MP |
| I_time | The expiration time of the tuple |

- 2-hop neighbor set

An MP records a set of “2-hop tuples”:

(N_neighb_main_addr, N_2hop_addr, N_time)

describing symmetric links between its neighbors and the symmetric 2-hop neighborhood.

| Field | Description |
| --- | --- |
| N_neighb_main_addr | The main address of a neighbor |
| N_2hop_addr | The main address of a neighbor, which has a symmetric link to N_neighbor_main_addr |
| N_time | The expiration time of the tuple |

- MPR set

An MP maintains a set of neighbors that are selected as MPR.  Their main addresses are listed in the MPR Set.

- MPR selector set

An MP records a set of “MPR-selector tuples”:

(MS_main_addr, MS_time)

describing the neighbors that have selected this MP as an MPR.

| Field | Description |
| --- | --- |
| MS_main_addr | The main address of an MPR-selector |
| MS_time | The expiration time of the tuple |

- Topology set

Each MP in the network maintains topology information about the network. This information is acquired from TC elements and is used for path selection table calculations. For each destination in the network, at least one “Topology Tuple”:

(T_dest_addr, T_last_addr, T_seq, T_time, T_link_metric)

is recorded.

| Field | Description |
| --- | --- |
| T_dest_addr | The main address of an MP, which may be reached in one hop from the MP with main address T_last_addr |
| T_last_addr | An MP that can reach T_dest_addr in one hop |
| T_seq | A sequence number |
| T_time | The expiration time of the tuple |
| T_link_metric | The value representing the metric cost of the link. If more than one link exists, the minimum cost (i.e., the cost of the link with the best quality) should be used. An example of link metric is the Airtime cost given in Error: Reference source not found. |

11A.7.4.8 Local Association Base (LAB)

Each MAP keeps a set of associated stations, denoted Local Association Base” (LAB) in this standard, which holds “Local Association Tuple”, one for each associated station. Additionally, to provide support for a large number of stations, each MAP divides its LAB into blocks of local association tuples: each block is a subset of the LAB, and the LAB is the union of those subsets. The blocks of the LAB are numbered; hence each block is identified by an integer, the “block index”.

Each element of the LAB, a “Local Association Tuple” of an associated station, has the following fields:

(block_index, sta_address, sta_sequence_number)

| Field | Description |
| --- | --- |
| block_index | Index of the block the “Local Association Tuple” belongs to |
| sta_address | The Mac address of the associated station. A station address does not include the “Group MAC address bit” in the 48 bit MAC address |
| sta_sequence_number | The sequence number (i.e., the whole 2-octet “Sequence Control” field) of the management frame that a STA sent to the MAP when the STA is associated with or disassociated with the MAP |

11A.7.4.9 Global Association Base (GAB)

Each MAP maintains information concerning which station is associated to which MAP in the entire Mesh, in a “Global Association Base” (GAB) that is union of all LAB of each MAP in the Mesh. It is a set of “global association tuples” containing

(block_index, MAP_address, sta_address, sta_sequence_number, expiration_time)

| Field | Description |
| --- | --- |
| block_index | Index of block associated to the station address by the MAP in its LAB |
| MAP_address | The MAC address of the MAP |
| sta_address | The MAC address of the associated STA. A STA address does not include the “Group MAC address bit” in the 48 bit MAC address |
| sta_sequence_number | The sequence number (i.e., the whole 2-octet “Sequence Control” field) of the management frame that a STA sent to the MAP when the station associated with or disassociated with the MAP |
| expiration_time | The time at which the entry is no longer valid |

- Multiple Interface Declaration (MID) element

The relationship between RA-OLSR interface addresses and main addresses is defined through the exchange of Multiple Interface Declaration (MID) elements.  This subclause describes how MID elements are exchanged and processed.

Each MP with multiple interfaces shall announce, periodically, information describing its interface configuration to other MPs in the network.  This is accomplished through flooding a MID element to all MPs in the network.

- Conditions for generating a MID element

A MID element shall be sent by an MP to declare its multiple interfaces (if any) within a certain refreshing period (MID_INTERVAL). All interface addresses other than the main address of an MP are put in the RA-OLSR interface address field in a MID element. If the resulting element cannot fit into one element (due to maximum allowed element size), more MID elements are generated for interface addresses that have not been transmitted and carried in separate frames until the entire interface addresses have been sent. The information diffused in the network by these MID elements help each MP to calculate its path selection table.  An MP that has only a single interface address participating in the mesh network running RA-OLSR, shall not generate any MID element.

The “Vtime” and “TTL” fields of the MID element shall be set to constant MID_HOLD_TIME and 255, respectively.

- Conditions for forwarding a MID element

MID elements are broadcast and forwarded by the MPRs in order to diffuse the elements in the entire network.  The “default forwarding algorithm” shall be used for forwarding of MID elements.

- Conditions for processing a MID element

The tuples in the multiple interface association set are recorded with the information that is exchanged through MID elements.

Upon receiving a MID element, the “validity time” shall be computed from the Vtime field of the element (as described in element processing and forwarding). Then the Multiple Interface Association Information Base should be updated as follows:

- If the sender interface (NB: not originator) of this element is not in the symmetric 1-hop neighborhood of this MP, the element shall be discarded.

- For each interface address listed in the MID element:

- If there exists a tuple in the interface association set where:

I_iface_addr = interface address, AND

I_main_addr = originator address.

Then the holding time of that tuple is set to:

I_time = current time + validity time.

- Otherwise, a new tuple is recorded in the interface association set where:

I_iface_addr = interface address,

I_main_addr = originator address,

I_time = current time + validity time.

- Mapping interface addresses and MP addresses

In networks with multiple interface MPs operating within a common RA-OLSR area, it is required to be able to map any interface address to the corresponding main address.

Given an interface address:

- if there exists some tuple in the interface association set where:

I_iface_addr = interface address.

Then the result of the main address search is the originator address I_main_addr of the tuple.

- Otherwise, the result of the main address search is the interface address itself.

- HELLO element

HELLO elements are exchanged between neighbor MPs, and serve the purpose of populating the 2-hop neighbor set as well as carry MPR signaling.

- Conditions for generating a HELLO element

This involves transmitting the Neighbor Set and the MPR Set. HELLO element shall be tranmitted periodically, and its period is specified by HELLO_INTERVAL. HELLO element is scoped for the MP’s neighborhood only (TTL=1). The list of addresses declared in a HELLO element is a list of neighbor interface addresses computed as follows: For each non-expired tuple in the Link Set, where L_local_iface_addr is the interface where the HELLO is to be transmitted, L_neighb_iface_addr is advertised. If the MP, to which L_neighb_iface_addr is associated is selected as MPR (i.e., a corresponding tuple exists in the MPR-set), a neighbor-tlv with type = MPR is associated with this address. The originator address for a HELLO element is set to the main address of the MP, generating the HELLO element.

The willingness of an MP may be set to any integer value from 0 to 7, and specifies how willing an MP is to be forwarding traffic on behalf of other MPs.  MPs, by default, have a willingness of WILL_DEFAULT. WILL_NEVER indicates an MP that does not wish to carry traffic for other MPs, for example due to resource constraints (like being low on battery).  WILL_ALWAYS indicates that an MP always should be selected to carry traffic on behalf of other MPs, for example due to resource abundance (like permanent power supply, high capacity interfaces to other MPs). An MP may dynamically change its willingness as its conditions change. One possible application is, for example, for an MP, connected to a permanent power supply and with fully charged batteries, to advertise a willingness of WILL_ALWAYS.  Upon being disconnected from the permanent power supply, a willingness of WILL_DEFAULT is advertised.  As battery capacity is drained, the willingness would be further reduced.

- Conditions for forwarding a HELLO element

Each HELLO element generated is broadcast by the MP on one interface to its neighbors (i.e., the interface for which the HELLO was generated).  HELLO elements SHALL not be forwarded.

- Conditions for processing a HELLO element

An MP processes incoming HELLO elements for the purpose of conducting link sensing (detailed in Topology discovery), neighbor detection and MPR selector set population (detailed in Populating the MPR selector set). Onlgeelements from peer MPs (as described in Error: Reference source not found) are accepted

- Populating the neighbor set

An MP maintains a set of neighbor tuples, based on the link tuples. This information is updated according to changes in the Link Set.

The Link Set keeps the information about the links, while the Neighbor Set keeps the information about the neighbors. There is a clear association between those two sets, since an MP is a neighbor of another MP if and only if there is at least one link between the two MPs. Only elements from peer MPs (as described inError: Reference source not found) are accepted

In any case, the formal correspondence between links and neighbors is defined as follows:

The “associated neighbor tuple” of a link tuple, is, if it exists, the neighbor tuple where:

N_neighb_main_addr = main address of L_neighb_iface_addr

The “associated link tuples” of a neighbor tuple, are all the link tuples, where:

N_neighb_main_addr = main address of L_neighb_iface_addr

The Neighbor Set shall be populated by maintaining the proper correspondence between link tuples and associated neighbor tuples, as follows:

Creation

Each time a link appears, that is, each time a link tuple is created, the associated neighbor tuple shall be created, if it doesn't already exist, with the following values:

N_neighb_main_addr = main address of L_neighb_iface_addr (from the link tuple)

Removal

Each time a link is deleted, that is, each time a link tuple is removed, the associated neighbor tuple shall be removed if it has no longer any associated link tuples.

These rules ensure that there is exactly one associated neighbor tuple for a link tuple, and that every neighbor tuple has at least one associated link tuple.

- HELLO element processing

The “Originator Address” of a HELLO element is the main address of the MP, which has emitted the element. Likewise, the “willingness” shall be computed from the Willingness field of the HELLO element.

Upon receiving a HELLO element, an MP should update its Neighbor Set as follows:

If the Originator Address is the N_neighbor_main_addr from a neighbor tuple included in the Neighbor Set:

Then, the neighbor tuple should be updated as follows:

N_willingness = willingness from the HELLO element

- Populating the 2-hop neighbor set

The 2-hop neighbor set describes the set of MPs that have a symmetric link to a symmetric neighbor.  This information set is maintained through periodic exchange of HELLO elements as described in this subclause.

- HELLO element processing

The “Originator Address” of a HELLO element is the main address of the MP, which has emitted the element.

Upon receiving a HELLO element, an MP should update its 2-hop Neighbor Set. Notice that a HELLO element shall neither be forwarded nor be recorded in the duplicate set.

Upon receiving a HELLO element, the validity time shall be computed from the Vtime field of the element header (see element processing and forwarding).

The 2-hop Neighbor Set should be updated as follows:

- For each address (henceforth: 2-hop neighbor address), listed in the HELLO:

- If the main address of the 2-hop neighbor address = main address of the receiving MP:

silently discard the 2-hop neighbor address.

(in other words: an MP is not its own 2-hop neighbor).

- Otherwise, a 2-hop tuple is created with:

N_neighb_main_addr = Originator address;

N_2hop_addr = main address of the 2-hop neighbor;

N_time = current time + validity time.

This tuple may replace an older similar tuple with same N_neighb_main_addr and N_2hop_addr values.

- Populating the MPR set

MPRs are used to flood information elements from an MP into the network while reducing the number of forwarding transmissions that occur in a region. The concept of MPR is an optimization of a classical flooding mechanism.

Each MP in the network selects, independently, its own set of MPRs among its 1-hop neighborhood. The neighbor interfaces, which are selected as MPR, are advertised with an associated neighbor-tlv with type=MPR in HELLO elements.

The MPR set shall be calculated by an MP in such a way that it, through the neighbors in the MPR-set, can reach all strict 2-hop neighbors with optimal radio-aware metric. (Notice that an MP, a, which is a direct neighbor of another MP, b, is not also a strict 2-hop neighbor of MP b). This means that the union of the symmetric 1-hop neighborhoods of the MPR MPs contains the strict 2-hop neighborhood.  MPR set recalculation should occur when changes are detected in the symmetric neighborhood or in the symmetric strict 2-hop neighborhood. MPR set recalculation will also occur when a radio-aware metric is changed.

MPRs are computed per interface, the union of the MPR sets of each interface makes up the MPR set for the MP.

While it is not essential that the MPR set is minimal, it is essential that all strict 2-hop neighbors can be reached through the selected MPR MPs. An MP should select an MPR set such that any strict 2-hop neighbor is covered by at least one MPR MP.  Keeping the MPR set small ensures that the overhead of the protocol is kept at a minimum.

The MPR set can coincide with the entire neighbor set. This could be the case at network initialization (and correspond to classic link-state path selection).

The heuristic for the selection of MPRs in the original OLSR does not take into account the radio-aware metric. It computes an MPR set with minimal cardinality and therefore links with lower radio-aware metric can be omitted. Consequently, the path calculated between two MPs using the known partial topology is not optimal (in terms of radio-aware metric) in the whole network.

The decision of how each MP selects its MPRs is essential to determinate the optimal radio-aware metric path in the network. In the MPR selection, links with low radio-aware metric should not be omitted.

- MPR Computation

The following specifies a recommended heuristic for selection of MPRs. It constructs an MPR-set that enables an MP to reach any MP in the strict 2-hop neighborhood through relaying by one MPR with willingness different from WILL_NEVER.  The heuristic shall be applied per interface: The MPR set for an MP is the union of the MPR sets found for each interface.  The following terminology is used in describing the heuristics:

| Terminology | Description |
| --- | --- |
| Neighbor of an interface | An MP is a “neighbor of an interface” if the interface (on the local MP) has a link to any one interface of the neighbor MP. |
| 2-hop neighbors reachable from an interface | The list of 2-hop neighbors of the MP that can be reached from neighbors of this interface. |
| MPR set of an interface | A (sub)set of the neighbors of an interface with a willingness different from WILL_NEVER, selected such that through these selected MPs, all strict 2-hop neighbors reachable from that interface are reachable. |
| N | N is the subset of neighbors of the MP, which are neighbor of the interface I. |
| N2 | The set of 2-hop neighbors reachable from the interface I, excluding: (i) the MPs only reachable by members of N with willingness WILL_NEVER (ii) the MP performing the computation (iii) all the neighbors: the MPs for which there exists a link to this MP on some interface |
| D(y) | The degree of a 1-hop neighbor MP y (where y is a member of N), is defined as the number of symmetric neighbors of MP y, EXCLUDING all the members of N and EXCLUDING the MP performing the computation. |

The recommended heuristic is as follows:

- Start with an MPR set made of all members of N with N_willingness equal to WILL_ALWAYS.

- Calculate D(y), where y is a member of N, for all MPs in N.

- Add to the MPR set those MPs in N, which are the only MPs to provide reachability to an MP in N2.  For example, if MP ‘b’ in N2 can be reached only through a symmetric link to MP ‘a’ in N, then add MP ‘a’ to the MPR set. Remove the MPs from N2 that are now covered by an MP in the MPR set.

- While there exist MPs in N2 that are not covered by at least one MP in the MPR set:

- For each MP in N, calculate the reachability, i.e., the number of MPs in N2 that are not yet covered by at least one MP in the MPR set, and which are reachable through this 1-hop neighbor;

- Select as an MPR the MP with the highest N_willingness among the MPs in N with non-zero reachability. In case of multiple choices, we use tie-breakers in the following order:

Maximum reachability (i.e., reachability to the maximum number of MPs in N2);

Maximum degree (D(.))

Optimal radio-aware metric (i.e., the best link quality according to the radio-aware metric);

Remove the MPs from N2 that are now covered by an MP in the MPR set.

- An MP’s MPR set is generated from the union of the MPR sets for each interface. As an optimization, process each MP ‘y’, in the MPR set in increasing order of N_willingness. If all MPs in N2 are still covered by at least one MP in the MPR set excluding MP ‘y’, and if N_willingness of MP ‘y’ is smaller than WILL_ALWAYS, then MP ‘y’ may be removed from the MPR set.

Other algorithms, as well as improvements over this algorithm, are possible.  For example, assume that in a multiple-interface scenario there exists more than one link between MPs ‘a’ and ‘b’. If MP ‘a’ has selected MP ‘b’ as MPR for one of its interfaces, then MP ‘b’ can be selected as MPR without additional performance loss by any other interfaces on MP ‘a’.

- Populating the MPR selector set

The MPR selector set of an MP, n, is populated by the main addresses of the MPs that have selected n as MPR.  MPR selection is signaled through HELLO elements.

- HELLO element processing

Upon receiving a HELLO element, if an MP finds one of its own interface addresses in the list with a Link Code field  equal to MPR_NEIGH, information from the HELLO element shall be recorded in the MPR Selector Set.

The “validity time” shall be computed from the Vtime field of the element (see element processing and forwarding). The MPR Selector Set should then be updated as follows:

- If there exists no MPR selector tuple with:

MS_main_addr = Originator address

then a new tuple is created with:

MS_main_addr = Originator address

- The tuple (new or otherwise) with

MS_main_addr = Originator address

is then modified as follows:

MS_time = current time + validity time.

Deletion of MPR selector tuples occurs in case of expiration of the timer or in case of link breakage as described in the “Neighborhood and 2-hop Neighborhood Changes”.

- Neighborhood and 2-hop neighborhood changes

A change in the neighborhood is detected when:

- The L_SYM_time field of a link tuple expires. This is considered as a neighbor loss if the link described by the expired tuple was the last link with a neighbor MP (on the contrary, a link with an interface may break while a link with another interface of the neighbor MP remains without being observed as a neighborhood change).

- A new link tuple is inserted in the Link Set with a non expired L_SYM_time or a tuple with expired L_SYM_time is modified so that L_SYM_time becomes non-expired. This is considered as a neighbor appearance if there was previously no link tuple describing a link with the corresponding neighbor MP.

A change in the 2-hop neighborhood is detected when a 2-hop neighbor tuple expires or is deleted according to Populating the 2-hop neighbor set.

The following processing occurs when changes in the neighborhood or the 2-hop neighborhood are detected:

- In case of neighbor loss, all 2-hop tuples with N_neighb_main_addr = Main address of the neighbor shall be deleted.

- In case of neighbor loss, all MPR selector tuples with MS_main_addr = Main address of the neighbor shall be deleted.

- The MPR set shall be re-calculated when a neighbor appearance or loss is detected, or when a change in the 2-hop neighborhood is detected.

- An additional HELLO element may be sent when the MPR set changes.

- Topology discovery

The link sensing and neighbor detection part of the protocol offers, to each MP, a list of neighbors with which it can communicate directly and, in combination with the frame format and forwarding part, an optimized flooding mechanism through MPRs.  Based on this, topology information is disseminated through the network. The present subclause describes which part of the information given by the link sensing and neighbor detection is disseminated to the entire network and how it is used to construct paths.

Paths are constructed through advertised links and links with neighbors. An MP shall at least disseminate links between itself and the MPs in its MPR-selector set, in order to provide sufficient information to enable path selection.

- Advertised neighbor set

A TC element is sent by an MP in the network to declare a set of links, called advertised link set, which shall include at least the links to all MPs of its MPR Selector set, i.e., the neighbors that have selected the sender MP as an MPR.

The advertised neighbor sequence number (ANSN) associated with the advertised neighbor set is also sent with the list. The ANSN number shall be incremented when links are removed from the advertised neighbor set; the ANSN number should be incremented when links are added to the advertised neighbor set.

- Conditions for generating a TC element

In order to build the topology information base, each MP, which has been selected as MPR, broadcasts Topology Control (TC) elements.  TC elements are flooded to all MPs in the network and take advantage of MPRs.  MPRs enable better scalability in the distribution of topology information.

The list of addresses can be partial in each TC element, but parsing of all TC elements describing the advertised link set of an MP shall be complete within a certain refreshing period (TC_INTERVAL). The information diffused in the network by these TC elements help each MP calculate its path selection table.

When the advertised link set of an MP becomes empty, this MP should still send (empty) TC elements during the duration equal to the “validity time” (typically, this is equal to TOP_HOLD_TIME) of its previously emitted TC elements, in order to invalidate the previous TC elements. It should then stop sending TC elements until some MP is inserted in its advertised link set.

An MP may transmit additional TC elements to increase its reactiveness to link failures. When a change to the MPR selector set is detected and this change can be attributed to a link failure, a TC element should be transmitted after an interval shorter than TC_INTERVAL.

In order to realize the frequency control, when an MP initiates a TC element, RA-OLSR sets different maximum hop count value for the frame according to the different frequencies. With the different maximum hop count values, the frames can only reach different designed scopes. The “TTL” field of the TC elements is used to control the scope. In the default behavior, the TTL is alternatively set to 2, 4, and the maximum TTL value in every TC_INTERVAL.

- Conditions for forwarding a TC element

TC elements are broadcast and forwarded by the MPRs in order to diffuse the elements in the entire network.  TC elements shall be forwarded according to the default forwarding algorithm (described in element processing and forwarding).

- Conditions for processing a TC element

Upon receiving a TC element, the “validity time” shall be computed from the Vtime field of the element (see element processing and forwarding).  The topology set should then be updated as follows:

- If the sender interface (NB: not originator) of this element is not in the symmetric 1-hop neighborhood of this MP, the element shall be discarded.

- If there exist some tuple in the topology set where:

T_last_addr = originator address, AND

T_seq > ANSN,

then further processing of this TC element shall not be performed and the element shall be silently discarded (case: element received out of order).

- All tuples in the topology set where:

T_last_addr = originator address, AND

T_seq < ANSN

shall be removed from the topology set.

- For each of the advertised neighbor main addresses received in  the TC element:

- If there exist some tuple in the topology set where:

T_dest_addr = advertised neighbor main address, AND

T_last_addr = Originator address,

then the holding time of that tuple shall be set to:

T_time = current time + validity time, AND

T_link_metric = Link Metric of the advertised neighbor main address.

- Otherwise, a new tuple shall be recorded in the topology set where:

T_dest_addr = Advertised neighbor main address,

T_last_addr = Originator address,

T_seq = ANSN,

T_time = current time + validity time, AND

T_link_metric = Link Metric of the advertised neighbor main address.

- Path selection table calculation

- General

Each MP maintains a path selection table that allows it to path data destined for the other MPs in the network.  The path selection table is based on the information contained in the local link information base and the topology set.  Therefore, if any of these sets are changed, the path selection table is recalculated to update the path information about each destination in the network.  The path entries are recorded in the path selection table in the following format:

1. R_dest_addr#1    R_next_addr#1    R_dist#1    R_metric#1   R_iface_addr#1

2. R_dest_addr#2    R_next_addr#2    R_dist#2    R_metric#2  R_iface_addr#2

3.         ,,                       ,,                  ,,              ,,                    ,,

Each entry in the table consists of R_dest_addr, R_next_addr, R_dist, R_metric and R_iface_addr. Such entry specifies that the MP identified by R_dest_addr is estimated to be R_dist hops away from the local MP with the cumulative radio-aware metric equal to R_metric, that the symmetric neighbor MP with interface address R_next_addr is the next hop MP in the path to R_dest_addr, and that this symmetric neighbor MP is reachable through the local interface with the address R_iface_addr.  Entries are recorded in the path selection table for each destination in the network for which a path is known.  All the destinations, for which a path is broken or only partially known, are not recorded in the table.

More precisely, the path selection table is updated when a change is detected in either:

- the link set,

- the neighbor set,

- the 2-hop neighbor set,

- the topology set,

- the Multiple Interface Association Information Base,

More precisely, the path selection table is recalculated in case of neighbor appearance or loss, when a 2-hop tuple is created or removed, when a topology tuple is created or removed or when multiple interface association information changes.  The update of this path selection information does not generate or trigger any elements to be transmitted, neither in the network, nor in the 1-hop neighborhood.

- Path selection algorithm

To construct the path selection table of MP X, a shortest path algorithm is run on the directed graph containing the arcs X -> Y where Y is any symmetric neighbor of X (with Link Code equal to SYM_NEIGH), the arcs Y -> Z where Y is a neighbor MP with willingness different of WILL_NEVER and there exists an entry in the 2-hop Neighbor set with Y as N_neighb_main_addr and Z as N_2hop_addr, and the arcs U -> V, where there exists an entry in the topology set with V as T_dest_addr and U as T_last_addr.

The optimal path is selected through the following procedure using the neighbor set, the link set, the 2-hop neighbor set and the topology set:

- All the entries from the path selection table are removed. Clear the list of candidate MPs. Initialize the shortest-path tree to only the root MP.

- Call the MP just added to the tree MP V. For each MP W, which is the one-hop neighbor ,and the link between V and W is the SYM link, calculate the link cost (the sum of radio-aware metric) D of the resulting path from the root MP to W.  D is equal to the sum of the link cost of the (already calculated) shortest path to vertex V and the advertised cost of the link between vertices V and W.  If D is:

- Greater than or equal to the value that already appears for vertex W on the candidate list, then examine the next MP.

- Less than the value that appears for W on the candidate list, or if W does not yet appear on the candidate list, then set the entry for W on the candidate list to indicate D from the root MP. The next hop that results from the candidate path for W accordingly is set to the same as the next hop of V.

- If at this step the candidate list is empty, the shortest-path tree (of transit vertices) has been completely built and the algorithm terminates. Otherwise, choose the MP belonging to the candidate list that is closest to the root MP, and add it to the shortest-path tree (removing it from the candidate list in the process).

- The new path entries for the destination MP W is recorded in the path selection table.

- Iterate the algorithm by returning to Step 2.

- Associated station discovery

- Overview

There are three elements for associated station discovery — (1) “Local Association Base Advertisement (LABA)” element, (2) “Local Association Base Checksum Advertisement (LABCA)” element, and (3) “Association Base Block Request (ABBR)” element.

A MAP shall advertise the contents of its LAB in a LABA element or a LABCA element periodically, and its period is specified by LABA_INTERVAL. An ABBR element is sent when it is needed. The policy for choosing to generate LABCA elements instead of LABA elements is left as a choice to the MAP, but the following is suggested:

- If there have been no changes in the Local Association Table for a duration greater than LABA_INTERVAL, the MAP generates LABCA elements.

- If there is a change (i.e., station association or disassociation), it switches back to sending LABA elements.

When a checksum mismatch is detected by an MP receiving the LABCA element, an MP has no contents in its GAB, or an MP needs the required LABA elements, the MP issues an ABBR element, with the list of indexes of required blocks.

- Local Association Base Advertisement (LABA) element

This subclause describes the function, generation, sending, forwarding, and processing of the LABA element.

11A.7.13.2.1 Function

The purpose of the LABA element is

- to advertise the contents of its LAB in a broadcast manner

- to send the contents of its LAB to an MPP

- to reply to the ABBR element

11A.7.13.2.2 Conditions for generating and sending a LABA element

An MP shall generate a LABA element in the following cases:

Case A: Original Transmission

An MAP generates a LABA element if all of the following applies:-

- one of the following applies:

- the LAB of the MAP is changed from the previous transmission of the LABA

- the MAP is not configured to send a LABCA

- the LABA_INTERVAL has expired

As an optimization to increase responsiveness of the station discovery protocol to changes in the association tables, a MAP may generate an earlier LABA element in case of change.

When the conditions for the generating a LABA element are satisfied, the MAP shall send out the LABA element in the following cases:

Case A1: Unicast to MPP

The MAP may send the LABA element to an MPP if all of the following applies:

— the MAP is configured to send a LABA to an MPP in a unicast manner if possible.

— the MAP knows the MAC address of the MPP

— the path to the MPP is established

The content of a LABA element in Case A1 shall be as shown in Table s??.

Table s??—Content of a LABA element in Case A1

| Field | Value |
| --- | --- |
| Element ID | Value given in Table 26 for the LABA element |
| Length | Length of the element |
| Vtime | Validity time during which an MAP considers the information contained in the information element as valid after its reception, e.g. GA_HOLD_TIME |
| Originator address | The main address of the MAP |
| TTL | Maximum number of hops allowed for this information element |
| Hop count | 0 |
| Sequence number | Sequence number of the LABA |
| Block index | Index of the block in the LAB |
| Number of STA | The number of STAs in the block |
| STA address | MAC address of an associated STA |
| STA sequence number | The sequence number of the management frame that an STA sent to the MAP when the STA is associated with or disassociated with the MAP |

Case A2: Broadcast

Otherwise, the MAP shall broadcast the LABA. The “TTL” field of the LABA IEs is used to control the scope. In the default behavior, the TTL is alternatively set to 1 and the maximum TTL value in every LABA_INTERVAL.

The content of a LABA element in Case A2 shall be as shown in Table s??.

Table s??—Content of a LABA element in Case A2

| Field | Value |
| --- | --- |
| Element ID | Value given in Table 26 for the LABA element |
| Length | Length of the element |
| Vtime | Validity time during which an MAP considers the information contained in the information element as valid after its reception, e.g. GA_HOLD_TIME |
| Originator address | The main address of the MAP |
| TTL | Maximum number of hops allowed for this information element, or 1 |
| Hop count | 0 |
| Sequence number | Sequence number of the LABA |
| Block index | Index of the block in the LAB |
| Number of STA | The number of STAs in the block |
| STA address | MAC address of an associated STA |
| STA sequence number | The sequence number of the management frame that an STA sent to the MAP when the STA is associated with or disassociated with the MAP |

Case B: Reply to ABBR element

An MP generates a LABA element if all of the following applies:

— The Destination Address of the ABBR is the same as MAC address of the receiving MP

— The Sequence Number of the ABBR is greater than the Sequence Number of the last ABBR received from the same originator address.

When the above conditions for the generation of a LABA are satisfied, the MP shall send out the LABA element in the following cases:

Case B1: Block Index is not 254 nor 255

The MP shall send a LABA element to the originator address of the receiving ABBR in a unicast manner, or broadcast a LABA element. The content of a LABA element in Case B1 shall be as shown in Table s??.

Table s??—Content of a LABA element in Case B1

| Field | Value |
| --- | --- |
| Element ID | Value given in Table 26 for the LABA element |
| Length | Length of the element |
| Vtime | Validity time during which an MP considers the information contained in the information element as valid after its reception, e.g. GA_HOLD_TIME |
| Originator address | The main address of the MP |
| TTL | Maximum number of hops allowed for this information element |
| Hop count | 0 |
| Sequence number | Sequence number of the LABA |
| Block index | Index of the block in the LAB, which is contained in the received ABBR, or all blocks in the LAB |
| Number of STA | The number of STAs in the block |
| STA address | MAC address of an associated STA in the block |
| STA sequence number | The sequence number of the management frame that an STA sent to the MAP when the STA is associated with or disassociated with the MAP |

Case B2: Block Index = 254

The MP shall generate a LABA element to contain all contents of its LAB. The MP shall send the LABA element to the originator address of the receiving ABBR in a unicast manner. The content of each LABA element in Case B2 shall be as shown in Table s??.

Table s??—Content of a LABA element in Case B2

| Field | Value |
| --- | --- |
| Element ID | Value given in Table 26 for the LABA element |
| Length | Length of the element |
| Vtime | Validity time during which an MP considers the information contained in the information element as valid after its reception, e.g. GA_HOLD_TIME |
| Originator address | The main address of the MAP |
| TTL | Maximum number of hops allowed for this information element |
| Hop count | 0 |
| Sequence number | Sequence number of the LABA |
| Block index | Index of the block in the LAB |
| Number of STA | The number of STAs in the block |
| STA address | MAC address of an associated STA in the block |
| STA sequence number | The sequence number of the management frame that an STA sent to the MAP when the STA is associated with or disassociated with the MAP |

Case B3: Block Index = 255

The MP shall generate several LABA elements to contain all contents of its GAB. The contents of the GAB are divided into LABA elements which have the same MAC address of the MAP, MAP_address. The MP shall send these LABA elements to the originator address of the receiving ABBR in a unicast manner. The content of each LABA element in Case B2 shall be as shown in Table s??.

Table s??—Content of a LABA element in Case B3

| Field | Value |
| --- | --- |
| Element ID | Value given in Table 26 for the LABA element |
| Length | Length of the element |
| Vtime | Validity time during which an MP considers the information contained in the information element as valid after its reception, e.g. GA_HOLD_TIME |
| Originator address | The MAC address of the MAP in the GAB |
| TTL | Maximum number of hops allowed for this information element |
| Hop count | 0 |
| Sequence number | Sequence number of the LABA |
| Block index | Index of the block in the LAB |
| Number of STA | The number of STAs in the block |
| STA address | MAC address of an associated STA in the block |
| STA sequence number | The sequence number of the management frame that an STA sent to the MAP when the STA is associated with or disassociated with the MAP |

11A.7.13.2.3 Conditions for forwarding a LABA element

An MP forwards a received LABA element in the following cases:

Case A: Broadcast

The MP shall forward a LABA according to the “default forwarding algorithm” (described inRA-OLSR default forwarding algorithm) if all of the following applies:

— The received LABA element was broadcasted.

— the MP has received and accepted a LABA – See 11A.7.13.2.4.1

Case B: Unicast

A LABA shall be propagated if all of the following applies:

— The received LABA element was unicasted.

— The Sequence Number of the LABA is greater than the Sequence Number of the last LABA received from the same originator address.

— The MP is not the destination of the LABA

— The TTL of the LABA is more than 1

The content of a LABA element in Case B shall be as shown in Table s??)

Table s??—Content of a LABA element in Case B

| Field | Value |
| --- | --- |
| Element ID | Value given in Table 26 for the LABA element |
| Length | As received |
| Vtime | As received |
| Originator address | As received |
| TTL | As received -1 |
| Hop count | As received +1 |
| Sequence number | As received |
| Block index | As received |
| Number of STA | As received |
| STA address | As received |
| STA sequence number | As received |

11A.7.13.2.4 Conditions for processing a LABA element

Received LABA elements are subject to certain acceptance criteria. Processing and actions taken depend on the contents of the LABA and the information available to the receiving MP.

11A.7.13.2.4.1 Acceptance criteria

The LABA is discarded if any of the following is true:

— The Sequence Number <= previous Sequence Number

— (The Destination Address = MAC address of the receiving MP) AND (the received LABA was unicasted)

Otherwise, the LABA element is accepted.

11A.7.13.2.4.1 Effect of receipt

The following applies only to a LABA that has not been discarded:

- The receiving MP shall update its proxy information with the block index, STA address and STA sequence number, and set the LABA originator address as the corresponding proxy.

- For each STA address in the LABA element, if there exists the STA address in the local association information and its STA sequence number in the LABA > an STA sequence number in the local association information, the receiving MP may delete the STA address and the STA sequence number in its local association information.

- 11A.7.13.3 Local Association Base Checksum Advertisement (LABCA)

11A.7.13.3.1 Conditions for generating, forwarding, and processing a LABCA element

- The generation of LABCA elements can replace the generation of LABA elements, and is straightforward from the LABCA element format description, and from the checksum calculation described in a following subclause.

The forwarding of LABCA elements is done according to the “default forwarding algorithm”.

The processing of LABCA elements is also simple: as indicated, a verification of all the Checksum Status Values is performed for all blocks.

In case of one or more mismatches an ABBR element is generated. However, for each block that matches, the GA_expiration_time of tuples in the matching blocks is updated, and also each entry in a block that has disappeared is deleted:

- For each tuple in the Global Association Base (GAB) where:

- GA_MAP_address = Originator Address, AND

- GA_block_index = in the range from 0 to the number of blocks-1 in the element

The field GA_expiration_time is updated as follows:

- GA_expiration_time = current time + validity time

- For each tuple in the Global Association Base (GAB) where:

- GA_MAP_address = Originator address, AND

- GA_block_index is greater than last block index in the LABCA element,

The tuple is deleted.

11A.7.13.3.2 Checksum calculation

The other subclauses use the fact that a checksum is calculated and verified. In this subclause, a procedure for performing checksum calculation is provided: For a block with a given index,

- All the corresponding association tuples, appropriately either in LAB or in the GAB, are retrieved.

- They are converted in sequence of octets (8): (MAC address, sequence number)

- They are sorted by increasing order.

- They are concatenated as a sequence of octets.

- CRC32 function is applied to the sequence.

- The result of the CRC32 function is used to represent the checksum

11A.7.13.4 Association Base Block Request (ABBR) element

11A.7.13.4.1 Conditions for generating, forwarding, and processing an ABBR element

An MP shall generate an ABBR element in the following cases:

Case A: An MP checks its own GAB corresponding to the received LABCA element of the originator MAP AND detects a mismatch. An ABBR element that includes a list of the mismatched Block Indexs is generated and is sent to the originator MAP of the mismatching LABCA element.

Case B: An MP needs the required LABA elements (e.g., the MP wants to maintain the GAB proactively even through some LABA elements are expired). An ABBR element that includes the list of the required Block Indexs is generated and is sent to a MP/MAP or an MPP.

Case C: An MP needs the entire LAB of a MAP. An ABBR element that includes only the Block Index of 254 is generated and is sent to a MP/MAP or an MPP.

Case D: An MP needs the GAB. An ABBR element that includes only the Block Index of 255 is generated and is sent to a MP/MAP or an MPP.

The ABBR element is forwarded in a similar way the data frame is forwarded, i.e., hop by hop and using the path selection table to reach the destination MP/MAP/MPP.

Received ABBR element is subject to certain acceptance criteria. A MP shall process the received ABBR element and actions taken depend on the contents of the ABBR element as listed in the following cases:

Case A: ABBR element contains a list of the mismatched Block Indexs, an MP

- (default) unicasts the entire LABA element (with all Local Association Bases), OR

- unicasts a LABA element with the requested Block Index, OR

- broadcasts the entire LABA element (with all Local Association Bases), OR

- broadcasts a LABA element with the requested Block Index for which there was (at least) one request, AND broadcasts one LABCA element in addition.

Case B: ABBR element contains a list of the required Block Indexs, an MP

- unicasts the required LABA elements.

Case C: ABBR element contains a Block Index of 254, an MP

- unicasts the entire LABA element of a MAP.

Case D: ABBR element contains a Block Index of 255, an MP

- unicasts all LABA elements.

- Associated station address search and population of the path selection table

The intent of the maintenance of the different Association Tables is that when an MP needs a mesh path for an associated station address, it proceeds as follows: it searches in its Local Association Base, and in its Global Association Base, for association tuples with the required address. If more than one were found to be present, the MP chooses the association tuple with the highest GA_STA_sequence_number.

Because the paths are proactively computed, instead of performing this search, the MP complements the path selection table calculation described in another subclause in an equivalent way with the following step:

For each entry in the Global Association Base where:

- There is no local association tuple in the LAB, nor global association tuple in the GAB with the same STA_address, and a higher STA_sequence_number, AND

- There exists a recorded path selection entry such that:

R_dest_addr = GA_MAP_address

Then a path entry is created with:

R_dest_addr = GA_STA_address

R_next_addr = R_next address (of the recorded path selection entry)

R_dist = R_dist (of the recorded path selection entry)

R_iface_addr = R_iface_addr (of the recorded path selection entry)

11A.7.13.6 Considerations for support of STAs without mesh functionality

The verification of disjunct MAC addresses between a non-MAP STA without mesh functionality and MPs during authentication/association of the non-MAP STA (11.3.3) may be done by a lookup of the MAC address of the non-MAP STA in the forwarding information of the MAP with mesh functionality.

The MAC address of the non-MAP STA does already exist in the mesh network if the MAP with mesh functionality already contains the MAC address in its forwarding information.

- Recommended values for constants

This subclause lists the values for the constants used in the description of the protocol.

- Setting emission intervals and holding times

The recommended value for constant C is the following:

C = 0.0625 seconds

C is a scaling factor for the “validity time” calculation (“Vtime” and “Htime” fields of elements, see element processing and forwarding and HELLO element). The “validity time” advertisement is designed such that MPs in a network may have different and individually tunable emission intervals, while still interoperates fully. For protocol functioning and interoperability to work:

- The advertised holding time shall always be greater than the refresh interval of the advertised information. Moreover, it is recommended that the relation between the interval (from Multiple ), and the hold time is kept as specified in Multiple , to allow for reasonable frame loss.

- The constant C should be set to the suggested value.  In order to achieve interoperability, C shall be the same on all MPs.

- The emission intervals, along with the advertised holding times (subject to the above constraints may be selected on a per MP basis.

The timer resolution of a given implementation might not be sufficient to wake up the system on precise refresh times or on precise expire times: the implementation should round up the ‘validity time’ (“Vtime” and “Htime” fields of elements) to compensate for coarser timer resolution, at least in the case where “validity time” could be shorter than the sum of emission interval and maximum expected timer error.

- Emission intervals

HELLO_INTERVAL = 2 seconds

REFRESH_INTERVAL = 2 seconds

TC_INTERVAL = 5 seconds

MID_INTERVAL = TC_INTERVAL

LABA_INTERVAL = TC_INTERVAL

LABCA_INTERVAL = TC_INTERVAL

MAXJITTER = HELLO_INTERVAL / 4

- Holding time

NEIGHB_HOLD_TIME = 3 * REFRESH_INTERVAL

TOP_HOLD_TIME = 3 * TC_INTERVAL

DUP_HOLD_TIME = 30 seconds

MID_HOLD_TIME = 3 * MID_INTERVAL

GA_HOLD_TIME = 3 * LABA_INTERVAL

The Vtime of the elements (seeelement processing and forwarding), and the Htime of theeHELLO element (seeHELLO element) are the fields that hold information about the above values in mantissa and exponent format (rounded up). In other words:

Value = C*(1+a/16)*2b [in seconds]

where a is the integer represented by the four highest bits of the field and b the integer represented by the four lowest bits of the field.

Notice, that for the previous recommended value of C, (1/16 seconds), the values, in seconds, expressed by the formula above can be stored, without loss of precision, in binary fixed point or floating point numbers with at least 8 bits of fractional part.  This corresponds with NTP time-stamps and single precision IEEE Standard 754 floating point numbers.

Given one of the above holding times, a way of computing the mantissa/exponent representation of a number T (of seconds) is the following:

- Find the largest integer ‘b’ such that: T/C >= 2b

- Compute the expression 16*(T/(C*(2b))-1), which may not be a integer, and round it up.

This results in the value for ‘a’

- if ‘a’ is equal to 16: increment ‘b’ by one, and set ‘a’ to 0 now,

- ‘a’ and ‘b’ should be integers between 0 and 15, and the field is a byte holding the value a*16+b

For instance, for values of 2 seconds, 6 seconds, 15 seconds, and 30 seconds respectively, ‘a’ and ‘b’ would be: (a=0, b=5), (a=8,b=6), (a=14,b=7) and (a=14,b=8) respectively.

- Sequence number

Sequence numbers are used in RA-OLSR with the purpose of discarding “old” information, i.e., elements received out of order. However with a limited number of bits for representing sequence numbers, wrap-around (that the sequence number is incremented from the maximum possible value to zero) occurs. To prevent this from interfering with the operation of the protocol, the following shall be observed:

The sequence number S1 is said to be “greater than” the sequence number S2 if:

S1 > S2 AND S1 - S2 <= MAXVALUE/2 OR

S2 > S1 AND S2 - S1 > MAXVALUE/2

where the term MAXVALUE designates the largest possible value for a sequence number.

When comparing two elements, it is possible — even in the presence of wrap-around — to determine which element contains the most recent information.

References:

- Draft in progress, IEEE P802.11sTM/D1.03 Draft STANDARD for Information Technology - Telecommunications and information exchange between systems - Local and metropolitan area networks - Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications: Amendment: ESS Mesh Networking, April 2007.

- Comments Resolution of IEEE P802.11s, 11-07/0023r26.

In the specification of LABCA information element in 7.3.2.78.5.